3GPP TSG-RAN WG3 #111-e	[draft]R3-210960

Online, 25 Jan - 4 Feb 2021

Agenda Item:	9.3.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on the Overlapping Band Handling over F1

Document for:	Approval

# Introduction

CB: # 7_OverlappingBandsF1AP

RAN2: updating band during RRC connection setup or RRC connection resume phase is not supported; SelectedBandCombinationIndex field itself cannot unambiguously indicate a specific band of serving cell

ZTE

Extend the scope of the Selected BandCombinationIndex IE, CRs in R3-20XXXX/XX should be agreed according to the LS reply from RAN2.

whether additional text description is needed for UE Context Setup procedure needs to be further confirmed by the group.

Nok

Extend the scope of the Selected BandCombinationIndex IE to signal a request to change the band selection to the gNB-CU.

The update of Measurement Object values due to a change in the selected band is carried out via a gNB-CU initiated UE Context Modification Update procedure.

E///,Vz

actual band chosen by the gNB-DU is signalled over F1 by the gNB-DU to the gNB-CU.

servingCellMO (serving cell measurement object) chosen by the gNB-CU for a given cell remains the same, but it is associated to the band for that cell that the gNB-DU selected

HW

If network decides to change the frequency band initially selected by UE, it should be the CU to make the decision

- note LS

- positions are not far; attempt to converge

- merge/revise CRs as needed; check details

- reply LS needed?

(ZTE - moderator)

Summary of offline disc R3-210960

Note:

The email discussion plan to be end at end of Friday of the first week.(Friday 17:00 UTC 2021-1-29)

# For the Chairman’s Notes

Propose the following:

# Discussion

PhaseII:

- Considering that solution Option1 with majority support, and it does not preclude the possibility for any other enhancements in the future, therefore, as the moderator, we propose to approve Option1 based on the LS from RAN2.

Pls companies check the detail of stage3 CRs in the phase2 draft box, and provide updates, if any.

- So far, majority companies would like to follow the previous agreement in RAN3 that CU’s responsibility to generate MeasConfig except MeasGapConfig and trigger an update of the Measurement Objects via a UE Context Modification procedure. Therefore, there is no changes to the existing specifications.

Question: Any view on the above proposals?

| Company | Comment |
| --- | --- |

PhaseI:

3.1 Solution decision

The summary of offline discussion was noted in R3-205585 in RAN3#109e meeting. An LS to RAN2 on band selection and indication on single connectivity was agreed in R3-205690. Meanwhile, R3-205673/74 are considered as BLs for this issue; if no technical concerns, agreement on both CRs is to be expected.

And some further discussion was continued in RAN3#110e meeting, it was confirmed that RAN3 should decide the solution based on the the feedback from RAN2.

Two scenarios were identified in previous meetings:

Scenario1: Single connectivity, PCell supports multiple overlapping bands.

Scenario2:  In CA and DC cases,  the UE may support different band combinations with overlapping bands, and the network shall select the band combination with highest throughput to provide the best performance for the UE.

And two solutions are on the table according to the contributions for this meeting:

Option1) Extend the scope of the Selected BandCombinationIndex IE, CRs in R3-206086/87.

Option2) Introduce a new IE to indicate the selected band per cell from DU to CU, CRs in R3-206217/18.

According to the LS reply from RAN2 in R3-210033:

Regarding the band updation scenario, it is possible for the network to change UE’s served band only after RRC connection is established in Uu interface and UE’s radio capability is obtained as well as AS security is activated. So updating band during RRC connection setup or RRC connection resume phase is not supported.

The above text in LS from RAN2 perspective clarifies that updating band during Uu RRC connection setup or RRC connection resume phase is not supported, take UE initial access procedure as an example, such change is not allowed between step1 and step5. While RAN3 is talking about the band updating possibility during F1 UE context setup procedure, precisely, in step11.

Figure 8.1-1: UE Initial Access procedure (See in TS38.401)

In addition, RAN2 discussed and concluded that: Selected BandCombinationIndex field itself cannot unambiguously indicate a specific band of serving cell, the band combination indicated by Selected BandCombinationIndex includes the current serving band(s) and may include other bands, but companies in RAN2 understand CU can determine the new selected band based on Selected BandCombinationIndex and other information (i.e. UE’s serving cell, supported band of this serving cell) that is likely present in the CU.

It’s clear that the Selected BandCombinationIndex IE can be reused by CU to determine the new selected band together with other available local information in CU. That is, it is not necessary to introduce new IEs and Option 1 is preferable, see R3-210066, R3-210142.

In R3-210324, it provides the opinion that the CU may still determine which overlapping band(s) the DU selected for the chosen SpCell and SCells by means of specific implementation mechanisms, bringing higher complexity. Furthermore, while the above procedure may be a possible implementation, it would not be justifiable to leave such important aspect of the split RAN design up to implementation. If for example an implementation does not follow the steps described herein, e.g. due to high implementation complexity, there would be inter vendor interoperability issues. the standard would not be detailed enough to specify how such an important feature should work.

ZTE: How the gNB-CU determines the new selected band based on Selected BandCombinationIndex and other information belongs to RRM implementation, nothing special here compared with other RAN node related RRM handling. And it processed within the gNB-CU, therefore, no inter vendor interoperability issues raised here.

Finally, if in the future RAN4 defined a band combination index including multiple overlapping bands, it would be impossible to identify the selected band at the gNB-CU. This is because the Selected BandCombinationIndex IE would include multiple overlapping bands, while the serving cell could also support multiple overlapping bands.

ZTE: It has already been discussed in RAN2 about all the possible band combinations, there is no need to talk about the uncertainty possibility.

The signalling procedure proposed in R3-210324 is shown as below:

- Figure 3.1-1: Description of possible solution

Therefore, the LS can be noted and no reply LS is needed. Option1 should be agreed in RAN3#111e meeting and this topic can be closed.

Question 1: Which option can be agreed according to the LS reply from RAN2?

| Company | Comment |
| --- | --- |
| ZTE | Option1 can be agreed. A lot discussion has been made in RAN2, it seems not necessary to repeat the same discussion in RAN3 again. Further comments from our side can be seen as above (highlight with blue text). Furthermore, according to the minutes in RAN3#108e, R3-205673/74 are considered as BLs for this issue; if no technical concerns, agreement on both CRs is to be expected. |
| Ericsson | We prefer Option 2. Option 1 uses the Selected BandCombinatinoIndex, which is an IE used in DC. This IE has not been used for functions like CA or single connectivity, namely, this IE belongs to a function (DC) that needs to calculate which band combinations have been selected by the DU in order to communicate this information from MN to SN. In cases of CA the DU does not have to communicate the selected band combination index to the CU because there is no band combination signalling between MN and SN. Therefore using the Selected BandCombinationIndex for CA means to force a node to support DC functionalities, when that node may only be required to support CA. For single connectivity the situation is even worst because the UE is not configured with multiple bands, but only with one band! There is no reason why the DU should derive a BandCombinationIndex in the case of single connectivity because there is no combination of bands at all. For that, Option 1 mixes the DC function with the single connectivity band selection. This is a technical issue for which we cannot agree to Option 1. ZTE:The scenario mentioned above has been throughly discussed in RAN2, and they confirmed the feasibility of Option1 in this case. We don't think we should waste our time to discuss this in RAN3 again. We should respect the respond from RAN2. Both CU and DU know the UE is operating on single band, and they are aware of the absolute frequency of serving cell. So it is easy to derive the band information from indicated BC, there is no confusion at all. In addition, please note that, UE is not allowed to report all fallback BC in UE radio capability, so even if the UE is configured with CA, when DU sends the BandCombinationIndex to CU, the associated BC may include multiple band entries that is more than currently configured. if your concern is valid, then the issue also exists in normal CA case! The statement in the LS from RAN2 is that there is a “believe” that CU implementation may work out the selected band for the UE. This “believe” leaves a fundamental function like band selection up to implementation, which is likely to lead to interoperability issues. Note that there are plenty of interoperability issues regarding band selection today, which is reflected by the discussions in RAN3 and RAN2, hence deriving the selected band is not “business as usual” but is something that if not clearly specified will lead to failures. For that Option 2 provides a simple addition to the F1 (the addition of 1 IE) that cannot be misinterpreted and that provides unequivocal information, removing any interoperability failure and removing the misuse of a DC IE for CA and single connectivity. ZTE: As I explained above, there is no confusion and IOT issue for Option1, BandCombinationIndex IE is a standardized IE over F1 interface since it was introduced. While for option2, it brings more standard effort over F1 without additional benefits comapred with option1. |
| Verizon | We prefer Option 2 as it is a complete solution. Option 1 is a retrofit solution option that does not cover single connectivity scenario. Explicit band indication from DU minimizes implementation ambiguity/complexity as well as avoid potential interoperability issues in a multi-vendor deployment. ZTE: Pls see above. |
| Nokia | Option 1. The CU can determine the selected band from the information contained in Selected BandCombinationIndex IE, and hence there is no reason to deviate from prior RAN3 agreement to utilize this approach as the basis upon RAN2 reply, which was already received. |
| CATT | Option 1. Based on the feedback from RAN2,the combined with Selected BandCombinationIndex IE and other information,Cu could determined the selected band,so we think no extra information is needed. |
| Samsung | Option 1. The CRs in this meeting is R3-210067/68 RAN2’s feedback has clearly address our concern. The SelectedBandCombinationIndex IE can be used, and the selected band can be determined by CU. |
| China Telecom | Option 1. RAN2 has clear conclusion on which IE need to be extended. |
| China Unicom | Prefer Option1. |
| Huawei | Before we go for option 1 or option 2, we would like to clarify that it should be the CU who makes the final decision whether the band could be changed or not, as discussed in R3-210810, we think CU has more knowledge than DU about the load situation. As to the change indication from DU to CU, we see both pros and cons for either options, option 1 is simpler with less spec impact, option 2 is more straight forward. But firstly we would like to see CU as the entity to make the final decision. BTW, I suppose the Figure 3.1-1 is not correct? Since according to RAN2 LS, frequency band could not be changed till after the RRC connection is established. |
| CMCC | Based on the feeback from RAN2, Option 1 is preferred. The SelectedBandCombinationIndex IE can be used, and wether the band can be changed is determined by CU. |

Summary:

Option1: 8 companies (one companies would like to confirm the CU makes the final decision firstly)

Option2: 2 companies

3.2 Stage3 detail on how the gNB-DU updates the MeasObject and measurement gap

Another open issue raised in last meeting, whether a solution is needed to be specified in order to address the mis-match between MeasConfig prepared by the Gnb-CU (before the UE band was changed) and MeasConfig after the gNB-DU assigns a new band to the UE.

Two alternatives are on the table from last meeting:

Option A) the gNB-CU triggers another round of UE Context Modification procedure

Option B) the gNB-DU updates the MeasObject and measurement gap when it switches the band before it sends the reply to the gNB-CU.

In R3-210142, it believes that Option A is aligned with existing RAN3 framework. That is, the gNB-DU can request the gNB-CU to update the band selected for the UE based on signaling from the gNB-DU. However, the change itself should be based upon a gNB-CU initiated UE Context Modification Procedure. Similarly, this incurs no additional specification changes. In contrast, we consider Option B as conflicting with RAN3 existing framework. In a similar way that a gNB-DU cannot force a change in Pcell/Scell autonomously, it should not be able to force a change in the selected band without involvement of the gNB-CU, and have the gNB-CU triggering a UE context modification procedure correspondingly to carry out the operation. The process of changing the selected band itself is still an intra-cell handover process. Hence, it should follow the same logic and trigger an update of the Measurement Objects via a UE Context Modification procedure.

In R3-210801, it clarifies that if network decides to change the frequency band initially selected by UE, it should be the CU to make the decision (there are other considerations as a whole package of RRM policy, such as radio coverage, service distribution among frequency bands, etc., and such info are first learned by CU), and both nodes (CU and DU) should be on the same page of the new selected band info, no matter which node makes the decision of band change.

Ericsson Comments on R3-210142 and R3-210066: The current status quo is that the DU selects the bands on which the UE is served. This is proved by the fact that in DC The DU at the MN selects the band combination and signals it to the CU by means of the Selected BandCombination Index as well as within the CellGroupConfig. In the case of DC (to giv an example) it is not the CU to chose bands for the UE. Hence, the interpretation that it is the CU to chose the bands for the UE is incorrect. The selected band is part of the CGC, which is encoded autonomously by the DU. The latter should serve as another hint to the fact that bands are selected by the DU.

In R3-210066, there should be a way to allow gNB-CU modify the improper Measurement Objects proposal from DU (the MeasObjectId included in the CellGroupConfig IE is the update configuration proposed by gNB-DU), e.g., the gNB-CU triggers another round of UE Context Modification procedure towards gNB-DU.

If the change proposed by gNB-DU can be accepted by the gNB-CU, then according to current procedure text in 8.3.1.2 of TS38.473:

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

It seems the highlight text has already covered the successful case.

For the case that gNB-CU decides to update the proposed change from the gNB-DU, then gNB-CU should be allowed to trigger another around of UE Context Modification procedure towards gNB-DU, which is also supported with current spec.

Question 2: Option A or Option B? Is any additional procedure text proposed above is needed ?

| Company | Comment |
| --- | --- |
| ZTE | Option A seems preferable. No additional procedure text is needed. |
| Ericsson | We prefer Option B. There are two issues with Option A. The first (most important) issue is that measurement configuration at the UE is needed as soon as possible and waiting for a new round of UE context Modification Procedures implies extra delays for measurement configuration. This would delay any further procedure such as handovers, DC setup, CA setup, etc. and it may lead to more failures (e.g. RLF due to too late HO). Note that other UE configurations depend on gap configurations. E.g. DRX, CSI-RS configurations. If the DU needs to wait to configure gaps properly, DU will also need to wait to configure these other features properly. The CellGroupConfig 7ignaled by DU to CU at UE Context Setup Response will need to contain a temporary configuration for features like DRX and CSI-RS, while the DU waits for CU to confirm the new MeasConfig. At the UE Context Modification, a new reconfiguration of the UE for these depending features would be needed. This is sub-optimal. If such reconfiguration is delayed for whatever reason, the UE ends up remaining on a sub-optimal configuration for a long time. ZTE: As I said, the delay will happen only when the change proposed by DU can not be accepted by CU and such delay is necessary, in this case, CU have to trigger another round of UE Context Modification procedure. The second issue is that at reception of the MeasConfig in UE Context Setup Request, the DU is tasked to collect measurements for the frequencies and bands indicated by the CU in the MeasObjectNR. What should the DU do? If the DU configures measurements for those frequencies/bands, the measurement configuration may be wrong (measurement configurations, e.g. gaps, are frequency/band specific). Or should the DU not configure any measurement? This would break the current specifications that state the DU shall configure gaps. Hence even with Option A, changes to the standard are needed. ZTE: For measurement gap, the logic is the same, if the value proposed by DU is acceptable, CU will pass it to UE, if not, CU has the right to modify it. For example, for the case UE connected to multiple gNB-Dus, the previous common understanding is that CU should send the existing measurement gap to the newly added DU, and the newly added DU should respect the received gap, but, if the newly added DU cannot accept the received gap configuration because of its own capability, the measurement cannot be carried out in this DU. If not, then this DU shall generate the new gap configuration and transfer it to CU. On the other hand, the CU also has the capability to re-generate the gap configuration for multiple Dus. After that, the CU shall combine the new gap configuration with the existing gap configuration and decide whether sends the combined gap configuration to the already existed DU and newly added DU. And the combined gap configuration is appropriate for both Dus. Otherwise, without the appropriate gap configuration for both Dus, the UE cannot perform the measurement, which will affect the mobility performance in the network, such as handover and cell re-selection. This is an example to explain why CU should be able to modify the improper band/gap configuration from DU, because CU has more local infor among Dus to make a proper decision. |
| Verizon | While both solutions seem to work, we prefer Option B. Option A has unnecessary extra latency added due to an additional round of context modification procedures and risks UE ending up in wrong/sub-optimal configuration for long time. ZTE: Pls see above. |
| Nokia | Option A. The process of changing the selected band itself is still an intra-cell handover process. Hence, it should follow the same logic and trigger an update of the Measurement Objects via a UE Context Modification procedure and not introduce unnecessary complexity conflicting with the existing framework. This also incurs no changes to the existing specifications. In contrast, in case of Option B, this would also require revisiting prior agreements on RAN3 regarding Pcell/Scell selection and we do not see these as trivial changes. It is not appropriate to tweak existing framework only for the overlapping band intra-cell handover case. Therefore, if a solution such as the one indicated in Option B is to be studied, then also discussion on Pcell/Scell selection mechanism needs to be reopened together with it in order to have a common approach to covering Pcell/Scell selection (including the overlapping band case) as a whole. |
| CATT | Option A . We have agreed before that all information inMeasObject except Measgapconfig should be decided by CU.So,we think the right procedure is DU send the selectedbandcombination to CU and CU further send updated Measobject to CU.CU generates cellgroupconfig which is transparent to CU accordingly. |
| Samsung | Option A. It is CU’s responsibility to generate MeasConfig except MeasGapConfig. We need follow this. |
| China Telecom | Option A. we agree with the highlight analysis above. Changing the selected band via intra-cell handover is aligned with the existing RAN3 framework. |
| China Unicom | Option A. It is more proper behavior that CU triggers UE Context Modification procedure. |
| Huawei | See comments to Q1. If the frequency band is finally changed, DU should be informed about the final decision and generate gap according to legacy procedure. |
| CMCC | Option A |

Summary:

OptionA: 8 companies (one companies would like to confirm the CU makes the final decision firstly)

OptionB: 2 companies

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-210033 | Reply LS on band selection and indication on single connectivity (RAN WG2) | LS in Move to 9.3.2 |
| --- | --- | --- |
| R3-210066 | Correction on Overlapping Band Handling over F1 (ZTE) | discussion |
| R3-210067 | Correction on Overlapping Band Handling over F1 (ZTE, CATT, Samsung, Nokia, Nokia Shanghai Bell) | CR0631r3, TS 38.473 v15.12.0, Rel-15, Cat. F |
| R3-210068 | Correction on Overlapping Band Handling over F1 (ZTE, CATT, Samsung, Nokia, Nokia Shanghai Bell) | CR0632r4, TS 38.473 v16.4.0, Rel-16, Cat. A |
| R3-210142 | Discussion on Overlapping Bands (Nokia, Nokia Shanghai Bell) | discussion |
| R3-210324 | Overlapping bands handling (Ericsson, Verizon Wireless) | discussion |
| R3-210325 | Overlapping band handling CR 38.473 (Ericsson, Verizon Wireless) | CR0584r3, TS 38.473 v15.12.0, Rel-15, Cat. F |
| R3-210326 | Overlapping band handling CR 38.473 (Ericsson, Verizon Wireless) | CR0585r3, TS 38.473 v16.4.0, Rel-16, Cat. A |
| R3-210810 | Further discussions on frequency band overlapping (Huawei) | discussion |