3GPP TSG-RAN WG3 Meeting #111e	R3-21xxxx

- 25 Jan - 5 Feb 2020

Agenda item:    15.2

Source:         Moderator(ZTE)

Title:            TP for Per slice QoE measurement

Document for:   Discussion and approval

# Introduction

This contribution provides our TP for Per slice QoE measurement of TR38.890.

-------------------------------------------------- Start of change ------------------------------------------------------------

## 6.9 Per slice QoE measurement

### 6.9.1 	Scenario and requirements

Figure 6.9.1-1: An example of the same service type served by different slices

Figure 6.9.1-1 is an example of the same service type served by different slices. As shown in the figure, UE1 is served by Slice #1 and UE 2 is served by Slice #2. If the Service Level Agreement (SLA) of Slice #1 and Slice #2 are different, QoE of UE1 and UE2 may be different for the same service type.

Collecting QoE per slice is beneficial for both management system (e.g. OAM) and network functions (e.g. UDM, NWDAF, etc.).

Below are scenarios identified for per slice QoE measurement collection:

| Scenario 1 | Different service types uses different slices | service type 1 –slice 1 service type 2 –slice 2 |
| --- | --- | --- |
| Scenario 2 | Different service types uses the same slice | service type 1 –slice 1 service type 2 –slice 1 |
| Scenario 3 | The same service type using different slice | service type 1 –slice 1 service type 1 –slice 2 |

### 6.9.2 	Solution

Solution 1: Network based solution

- OAM/CN transmit the QoE measurement configuration to NG-RAN node, including Slice Scope.

- NG-RAN node can map the Slice Scope to the ongoing PDU session list and send the QoE measurement configuration with the PDU session list to UE.

- UE receives the QoE measurement configuration and sends it to the corresponding application layer according to the PDU session list.

- UE sends the QoE report with PDU session ID to NG-RAN node.

- NG-RAN node can remap the PDU session ID back to slice ID and attach it in the QoE report.

- NG-RAN node forwards the QoE report with slice ID to the MCE.

Solution 2: UE based solution

- OAM/CN transmits the QoE measurement configuration to NG-RAN node, including Slice Scope.

- NG-RAN node checks the Slice Scope with all of the ongoing PDU sessions, and sends QoE measurement configuration to UE with qualified PDU session, including Slice Scope.

- UE receives the QoE measurement configuration and sends it to the corresponding application layer according to the Slice Scope.

- UE sends the QoE report with slice ID to NG-RAN node.

- NG-RAN node forwards the QoE report with slice ID to the MCE.

- For both solutions, Slice Scope is outside of the QoE configuration container which defined by SA4 and also needs confirmation with SA5. Slice Scope is applied for all configured Area scope.

- For S-based Slice scope (e.g. list of S-NSSAIs) should be transmitted to the target gNB during mobility.

# 7	Potential Impacts on NR specifications

Editor’s NOTE: Description of potential Specification impact, according to the potential solutions of NR QoE.

For Per slice QoE measurement, the key impacts of the solutions can be found in the table below:

Table 7.x: Key impacts of the solutions

| Criteria Solution | RAN impact | Core impact | OAM impact | UE impact | SA4 impact |
| --- | --- | --- | --- | --- | --- |
| Network based | Introduce Slice info in message. Slice area check. | N/A | Slice info outside Configuration | NAS/AS layer | N/A |
| UE based | Same | N/A | Same impact | Application layer | FFS, e.g. slice info inside Configuration |

# 8	Conclusion

The following feature is recommended by RAN3 to be specified in normative phase:

-	Per slice QoE measurement

Solution down-selection and finalization can be performed in normative phase.

--------------------------------------------------- End of change ------------------------------------------------------------