3GPP TSG-RAN WG3 Meeting #112-e 	R3-212752

- E-Meeting, May 17-28, 2021

Source:	Nokia (moderator)

Title:	Summary of Offline Discusion - CB: # 117_MaxCGI_E1CB

Agenda Item:	9.3.8.1

Document for:	Approval

# 1	Introduction

CB: # 117_MaxCGI_E1

- Issue is acknowledged: seems insufficient capacity?

- check why we introduced this?

- How much is the n. of supported cells a “proxy” for CU-UP capacity?

- proposed solution: increase n. >512?

- check details

(Nok - moderator)

Summary of offline disc R3-212752

# 2	For the Chairman Nodes

[To be Updated]

# 3	Discussion

The issue under discussion is in regard to the maximum number of NR-CGI that can be reported over E1, which currently has an upper limit of 512.

This IE was introduced at RAN3#100 based on R3-182929 and adopted at the same meeting. This contribution also did not limit its use to any specific deployment. This is further captured in Stage 2 and 3 specifications.

<<excerpt of R3-182929>>

Supported cell information Basically, there are 2 purposes of the cell information. Cell information should be provided as cell list containing at least NR-CGI. - To ensure that gNB-CU-CP performs gNB-CU-UP relocation (inside the same gNB) when the UE leaves the cells served by a particular gNB-CU-UP. This is based on the UE measurements, but the cells supported by a particular gNB-CU-UP needs to be known by both gNB-CU-CP and gNB-CU-UP. - To ensure that gNB-CU-UP performs PM collection at cell level.

<< end of excerpt of R3-182929>>

The existing limit regarding the maximum number of NR-CGIs that can be signalled by a gNB-CU-UP is artificially restrictive and limits the existing functionality that relies on the information conveyed by this IE, such as gNB-CU-UP relocation scenario and mobility. One example of a service relying on this information and feature (signalling of supported NR-CGIs) is URLLC purpose, which shall satisfy a very tight delay constraint. In regard to the existing limit, multiband scenarios can easily exceed 512 cells even in early deployments and cloud-based deployments. Thus, functions for gNB-CU-UP relocation and selection mechanism are hindered and otherwise lead to force to deploy additional gNB-CU-UPs (or instances of gNB-CU-UPs) in order to properly indicate the NR-CGIs for a gNB-CU-UP and keep the same level of operation and performance during gNB-CU-UP relocation cases that rely on this IE just for this purpose.

Consider the following simple scenario, in which several CU-UPs are deployed at different locations, and which would not be able to properly support the CU-UP relocation correctly with the existing max number of NR-CGI in E1AP.

- CU-CP and multiple CU-UPs (CU-UP4 to CU-UPX) are deployed at a centralized location in a virtualized environment and intended for general traffic.

- Two distributed locations are deployed with CU-UP and DU distributed

- DU1, DU2, DU3 and DU4 provide services with a critical delay component

- CU-UP1 can ensure that the service quality is sustained for such services for DU1 and DU2

- CU-UP2 and CU-UP3 can ensure that the service quality is sustained for such services for DU3 and DU4

- All the network elements depicted belong to the same gNB

Figure 1. Example of CU-UPs deployed at different locations

Questions 1: Do companies agree that in large configurations, the existing limit of maximum number of NR-CGI is unnecessarily restrictive?

| Company | Comment |
| --- | --- |
| Nokia | Agree. |

Questions 2: Do companies have a concern the NR-CGIs signalled over E1 would be a “proxy” for gNB-CU-UP capacity? If so, please elaborate on the concern.

| Company | Comment |
| --- | --- |
| Nokia | No concern. The list of NR-CGIs from gNB-CU-UP is for purpose of aiding the gNB-CU-CP on its selection of an adequate gNB-CU-UP. Similarly, the IE itself is of optional nature. |

Questions 3: Do companies agree to extend the maximum number of NR-CGI that can be signalled over E1 interface as proposed in [1]?

| Company | Comment |
| --- | --- |
| Nokia | Agree. |

# 4	Conclusion, recommendations [if needed]

[To be Updated]

# 5	References

- R3-212383, Maximum number of NRCGI over E1, Nokia, Nokia Shanghai Bell, Verizon Wireless, Samsung