3GPP TSG-RAN WG3 #112-e	R3-212639

Online, 17 – 27 May, 2021

Agenda Item:	15.2.1.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on CB #: NRQoE3-RANConfig

Document for:	Approval

# Introduction

CB: # NRQoE3-RANConfig

- Detail infor of QoE configurations? E.g., Service Type, Container for application layer measurement configuration, QOE reference ID, Area Scope of QMC defined either at a cell/TA/TAI/PLMN level, Slice Scope...

- How to handle multiple simultaneous QoE configuration? Send an LS to RAN2 to define ad-hoc UE capabilities for supporting multiple QoE measurements?

- How to support per slice QoE measurements? Impact on configuration and report? Send LS to other groups with clarification questions, if any? Support roaming users for which the slice scope may relate to HPLMN slices in case of signalling based activation?

- How to support MR-DC case? QoE measurement configuration transfer is NOT supported from MN to SN?

- How to support QoE measurement handling at RAN overload ? Including a QoE paused indication?

- Capture agreements as TP for BL CRs, if agreeable

- List open issues for next meeting in the summary

(HW - moderator)

Summary of offline disc R3-212639

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion [if needed]

## To reuse and update the existing Trace Activation IE (like we did in LTE), or to create a new IE NR QoE activation

This is overlapped with the CB: # NRQoE2-Activation_Deactivation, main reason is that this issue is a basic one and is the starting point for configuration, so companies just need to provide your views.

Q1: To reuse existing IE or to create a new IE?

| Company | To Reuse or to create a new one | Comment |
| --- | --- | --- |
| Samsung | Reuse Trace Activation IE | As we commented in CB QoE2, using Trace activation IE had already be agreed in the TR38.890 and also agreed by RAN2. |

## If to reuse, is a new IE like “QoE Reference ID” needed or not

In LTE, there is only one ID “E-UTRAN Trace ID” which is shared by all the trace related tasks, include interface trace, MDT and QMC, which implies that if all the three tasks are configured at the same, there is only one ID to refer; in other words, we could also use different messages to configure different tasks with different Trace ID respectively.

Q2: is a new IE like “QoE Reference ID” needed or not if reusing existing IE?

| Company | A new IE needed or not? | Comment |
| --- | --- | --- |
| Samsung | New IE needed | To support multiple QMC |

## If a new IE like “QoE Reference ID” is needed in the same message, should it be service type specific if multi-service type is configured, or shared by multi-service types in one message?

Here the main scenario is that when multi-service type or multi-QoE measurement (even two QoE measurement for the same service type towards the same UE) is configured in one message at the same time, do we need to introduce different “QoE Reference ID” for each service, or it is shared by multi-service?

Q3: If “QoE Reference ID” is needed, should it be service type specific, or shared by multi-service?

| Company | “QoE Reference ID” is service type specific, or shared by multi-service, in one message | Comment |
| --- | --- | --- |
| Samsung | Not sure | We noticed that companies have different views, we think it’s not clear for RAN3 that whether one QoE reference is associated with one service type or multiple service types, since this Multiple QMC is the request from SA5 and QoE reference is defined by SA5, shall we ask them the relationship between the two, and then make decisions? |

## If to reuse, is a new IE like “Measurement Collection Entity IP Address” needed or not

In LTE, there is only one ID “Trace Collection Entity IP Address” which is shared by all the trace related tasks, include interface trace, MDT and QMC, which implies that if all the three tasks are configured at the same, there is only one collection IP address to refer; in other words, we could also use different messages to configure different tasks with different collection IP address respectively.

Q4: is a new IE like “Measurement Collection Entity IP Address” needed or not if reusing existing IE?

| Company | A new IE needed or not? | Comment |
| --- | --- | --- |
| Samsung | May be needed |  |

## If a new IE like “Measurement Collection Entity IP Address” is needed, in the same message, should it be service type specific if multi-service type is configured, or shared by multi-service types in one message

Here the main scenario is that when multi-service type or multi-QoE measurement is configured in one message at the same time, do we need to introduce different “Measurement Collection Entity IP Address” for each service, or it is shared by multi-service, or it is configured per QoE measurement?

Q5: If  “Measurement Collection Entity IP Address” is needed, should it be service type specific, or shared by multi-service?

| Company | “Measurement Collection Entity IP Address” is service type specific, or shared by multi-service. | Comment |
| --- | --- | --- |
| Samsung | QoE reference specific | In 28.405, it’s said that QoE reference is used to identify the QoE measurement collection job in the traffic nodes and in the measurement collection centre. So we think this entity IP address should be QoE reference specific. |

## Configuration and report message if QoE measurement with Slice is requested

Here the issues are mainly about what should be configured over NG, Uu for QMC with slice and what should be included in the RRC message for QoE measurement report over Uu. According to the discussion papers, the main questions are:

- The first one is, if application layer is aware of the mapping between service types and slice. Assuming the slice info (a slice ID or a list of slice IDs?) is configured in parallel with the container, should it also be added into the container, for the configuration message over NG and Uu? (An LS maybe needed to send to SA4)

- Then for QoE measurement report, should slice ID be included as an explicit IE in the RRC message, or just leave it to application layer to include slice ID info the QoE measurement report container?

Companies please share your views in the following table and indicate your preferred configuration over NG and Uu.

Q6: if application layer is aware of the mapping between service types and slice or not?

| Company | Comments and views |
| --- | --- |
| Samsung | Q1: Firstly, the application is aware of the mapping between service type and slice, in TS 27.007, the S-NSSAI associated to the PDU session can be obtained by Application layer through AT command (10.1.23 PDP context read dynamic parameters +CGCONTRDP). The same as service type, slice scope should be finally checked in UE application layer to decide whether to start QoE measurement, no matter it’s transmitted in or out of the container. If we learn from “service type” IE in LTE QMC, slice ID can be out of the container, but finally it should be transmitted to the application layer. Q2: slice ID should be included in QoE report, but regarding it’s inside or outside of the report container, we have no strong view. Maybe it’s better to let application to do it. |

Q7: should slice info also be added into the container, i.e. as part of the transparent container?

Moderator: here I assume slice info is anyway configured in parallel with the container (outside of the container), please companies give your views if think otherwise, and also provide your view on a slice ID or a list of slice IDs

| Company | Comments and views |
| --- | --- |
| Samsung | If it’s anyway configured in parallel with container, it can also be transmitted to the application layer, just the same as service type. |

Q8: should slice ID be included as an explicit IE in the QoE measurement report RRC message, or just leave it to application layer to include slice ID info the container?

| Company | Comments and views |
| --- | --- |
| Samsung | Both are OK for us. No strong view |

There is another issue, which is about the support of roaming UE for which the slice scope may relate to HPLMN slices in case of signalling based activation. Please companies share your views.

Q9: Support of Roaming UEs

| Company | Comments and views |
| --- | --- |
| Samsung | If the QoE configuration is from the Home network, does home network really need or is home network allowed to collect QoE in the visited network slice? |

## MR-DC support

Here the main question is: whether QoE measurement configuration should be transferred from MN to SN or not?

Q10: whether QoE measurement configuration should be transferred from MN to SN or not?

| Company | Yes or No | Comment |
| --- | --- | --- |
| Samsung | No |  |

## Overload handling

From the discussion papers, we could see that there are several proposals, the main intention is whether any guidance from OAM/CN is needed, including

- an explicit pause indication?

- the priorities of the QoE measurements of different service types or slices for the RAN to pause or release ongoing QoE measurements in case of RAN overload?

- prioritization mechanism for UE to send to RAN pending QoE reports when RAN overload is solved?

Please companies share your views to the three sub-questions.

Q11: over NG/ the interaction between RAN and OAM, a pause indication and/or priorities for RAN to pause or release, priorities for UE to send to RAN pending QoE reports when RAN overload is solved?

| Company | Comments and views |
| --- | --- |
| Samsung | At least we should have pause indication, priorities mechanism may need further discussion. |

## Other parameters?

As could be seen from discussion papers, I could see consensus on the introduction of the following IE:

- Container

- UE Application layer measurement configuration IE for each service type

- a numerated IE indicating service type (Streaming services, MTSI services, VR, MBMS, XR)

- Area scope, could be a list of cells/TA/TAI/PLMN

- Slice scope, could be a list of S-NSSAI

Q12: Any comments to the four parameters above, e.g. yes or no, additional comments.

| Company | Comment |
| --- | --- |
| Samsung | A little confused, what’s the difference between “container” and “UE Application layer measurement configuration IE for each service type”? We think the container, service type, area scope and slice scope should be inside of the UE Application layer measurement configuration IE. And to support multiple QMC, a list of UE Application layer measurement configuration IE should be included in Trace Activation IE. |

In addition, I could also see some proposals on introducing criteria from OAM/CN to RAN a set of criteria, including one or more time-based criteria, one or more threshold-based criteria and one or more event-based criteria, to be used for triggering and/or stopping QoE measurements. Please companies share your views whether any or all of these criteria are needed or not.

Q13. If any or all of these criteria, one or more time-based, one or more threshold-based, one or more event-based, are needed or not.

| Company | Comment |
| --- | --- |
| Samsung | It’s good to have some conditions for triggering/stopping the QoE measurement, it’s more flexible to collect QoE in different scenarios |

## If to introduce a new IE NR QoE activation, what else needed

Here we should note that all the discussions and conclusions (except 3.1) above are also applied here for this issue, e.g. one QoE reference ID or not, one TCE address or not, slice configuration, etc.

Q14. If to introduce a new IE NR QoE activation, what else needed, in addition to the agreements reached above?

| Company | Comment |
| --- | --- |
| Samsung | I may not understand the question. Why we need a new IE NR QoE activation? What’s the issue? |

## Send LS to other groups

[Moderator] I think pending on the final agreements we will reach, we may need send LS at least to RAN2, SA4 and SA5, we could leave this discussion to the phase II.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-211732	NR QoE configuration and reporting	Qualcomm Incorporated

- R3-211733	TP for 38.413 and 38.423) NR QoE configuration and reporting	Qualcomm Incorporated

- R3-211734	Per slice QoE measurements	Qualcomm Incorporated

- R3-211836	Discussion on NR QoE configuration details	CATT

- R3-211837	Discussion on per-slice QoE measurement	CATT

- R3-211986	QoE Configuration and Reporting	Ericsson

- R3-212324	Principles for configuration of NR QoE measurements	Nokia, Nokia Shanghai Bell

- R3-212381	CR TS 38.413 QoE Configuration and Reporting - Signalling Design	Ericsson

- R3-212435	Discussion on NR QoE Configuration	ZTE Corporation, China Telecom

- R3-212440	TP for TS 38.300) Introduce NR QoE	ZTE Corporation, China Telecom

- R3-212441	Discussion on configuration details in NR standalone mode	LG Electronics

- R3-212442	TP for TS 38.413) NR QOE configuration	ZTE Corporation, China Telecom

- R3-212443	TP for TS 38.423) NR QOE configuration	ZTE Corporation, China Telecom

- R3-212498	Per-slice QoE measurement	CMCC

- R3-212518	Further analysis on spec impacts of the potential solutions to QoE measurement per slice	Huawei

- R3-212521	Further analysis on spec impacts on RAN intervention of QoE measurement	Huawei

- R3-212591	On per-slice QoE measurement	Nokia, Nokia Shanghai Bell

- R3-211978	RAN3 impacts for supporting QoE Measurement Collection in NR	Samsung

- R3-211979	Support of NR QoE Measurement Collection	Samsung

- R3-211980	Support of NR QoE Measurement Collection	Samsung

- R3-211835	Discussion on NR QoE configuration procedures	CATT

- R3-211985	Procedures for Configuration, Activation and Deactivation of QMC	Ericsson

- R3-212322	Baseline and stage 2 aspects for NR QoE	Nokia, Nokia Shanghai Bell

- R3-212323	Introduction of QoE Measurement Collection for NR	Nokia, Nokia Shanghai Bell

- R3-212434	Consideration on NR QoE activation procedure	ZTE Corporation, China Telecom, China Unicom

- R3-212519	Further analysis on spec impacts on configuration and reporting	Huawei

- R3-212520	CR to 38.413 on Introduction of QoE measurement	Huawei