3GPP TSG-RAN WG3#113-e                                             	                     R3-213437

E-Meeting, 16 – 26 August 2021

Agenda item:	9.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title: 	Support of 4g-5g Direct Data Forwarding over E1 with shared gNB

Document for:	Approval

# Introduction

At the last RAN3#112 the case of 4g to 5g was not solved and the following open points were listed in the summary of discussions [2]:

Non-shared case:

Two solutions are left on the table to support direct data forwarding from EPS to 5GS in scenario 4. Comparison and down selection is performed at next meeting.

Solution 1: CU-CP requests one data forwarding address from the CU-UP using the existing signalling. CU-CP feedback the tunnel address to the two E-RABs in Handover Request Ack message to 5GC. With this, the data from the two E-RABs in the source node will be sent to one DRB buffer in the target (ref R3-211957/R3-212545/R3-212356)

Solution 3: Add Data Forwarding from E-UTRAN Request List to the DRB To Setup List in PDU Session Resource To Setup List within Bearer Context Setup Request message and Data Forwarding from E-UTRAN Response List to the DRB Setup List in PDU Session Resource Setup List within Bearer Context Setup Request message (ref R3-211642/R3-211642/R3-211958).

Shared SgNB case:

Bearer Context Setup Request message is used from the CU-CP to the CU-UP.

Regarding how to support internal data forwarding in the SgNB, the following two options will be further evaluated at the next meeting:

Alternative 1: Add gNB-CU-UP E1AP ID in Bearer Context Setup Request message

Alternative 2: Add S1 DL UP Transport Layer Information per DRB in Bearer Context Setup Request message.

This paper provides our view on the shared case.

# Discussion on the Shared case

In the case of shared SgNB/gNB there are two logical nodes. Therefore, the situation cannot be compared with the case of inter-MN handover without SN change in which a context ID is exchanged to retrieve/reuse the context.

With two logical nodes, the idea is just to enable internal direct forwarding, a kind of “short-cut” for the forwarding tunnels.

Therefore, it is just needed to transfer the transport ID i.e. the S1 DL UP Transport Layer Information as proposed by alternative 2.

Alternative 1 is not needed but also not strictly correct since there is no context reuse.

Proposal 1: agree the alternative 2 (S1 DL UP TLI) in [3] for the shared case which is the one technically correct.

# Conclusion and Proposal

This paper has discussed the shared case and drawn the following conclusions:

Proposal 1: agree the alternative 2 (S1 DL UP TLI) in [3] for the shared case which is the one technically correct.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- R3-212624, Summary of offline discussion on Direct Data Forwarding over E1 for inter-system 4g to 5g

- R3-213438, Support of Direct Data Forwarding for Inter-system Handover, CR TS 38.463