3GPP TSG-RAN WG3 Meeting #113-e	R3-213736

E-meeting, 16-26 Aug 2021

Title: 	Finalizing the PDU session level Expected UE Activity Behaviour

Source: 	Huawei, Lenovo, Motorola Mobility, Qualcomm Incorporated, ZTE

Agenda item:	8.3.2

Document Type:	Discussion

1. Introduction

Last RAN3#112-e meeting discussed the PDU session level Expected UE activity behaviour, with the following agreements.

For NGAP, the PDU session level “Expected UE Activity Behavior” is included in the following messages. - PDU SESSION RESOURCE SETUP REQUEST - INITIAL CONTEXT SETUP REQUEST - HANDOVER REQUEST - PATH SWITCH REQUEST ACKNOWLEDGE - PDU SESSION RESOURCE MODIFY REQUEST For XnAP, the PDU session level “Expected UE Activity Behavior” is included in the S-NODE MODIFICATION REQUEST message. Continue to study whether the PDU session level “Expected UE Activity Behavior” is included within or outside of the SMF container for NGAP To be continued...

This document continues to discuss this issue, and provides corresponding CRs.

2. Discussion

After the discussion at last RAN3-112-e meeting, it made consensus that the PDU session level “Expected UE Activity Behaviour” should be provided to NG-RAN node. The remaining question is whether the PDU session level “Expected UE Activity Behavior” is included within or outside of the SMF container for NGAP?

Currently the basic procedure on the PDU session level “Expected UE Activity Behavior” is clearly described section in 5.4.6.2 of TS 23.501 as follows.

-	The SMF derives and sends the “CN assisted RAN parameters tuning” to the AMF.

-	Based on this, the AMF determines the PDU session level “Expected UE activity behaviour”, associated with a PDU Session.

<Except from Section 5.4.6.2 of TS 23.501> The SMF uses the SMF-Associated parameters (e.g. Expected UE Behaviour parameters or Network Configuration parameters of the UE) to derive the SMF derived CN assisted RAN parameters tuning. The SMF sends the SMF derived CN assisted RAN parameters tuning to the AMF during the PDU Session establishment procedure and if the SMF-Associated parameters change the PDU Session modification procedure is applied. The AMF stores the SMF derived CN assisted RAN parameters tuning in the PDU Session level context. The AMF uses the SMF derived CN assisted RAN parameters tuning to determine a PDU Session level "Expected UE activity behaviour" parameters set, which may be associated with a PDU Session ID, as described below in this clause.

Also it is clearly specified that the AMF provides to the RAN the PDU session level “Expected UE activity behaviour”.

- "Expected UE activity behaviour", i.e. the expected pattern of the UE's changes between CM-CONNECTED and CM-IDLE states or the duration of CM-CONNECTED state. This may be derived e.g. from the statistical information, or Expected UE Behaviour or from subscription information. The AMF derives one or more sets of the "Expected UE activity behaviour" parameters for the UE as follows: - AMF may derive and provide to the RAN a UE level of "Expected UE activity behaviour" parameters set considering the Expected UE Behaviour parameters or Network Configuration parameters received from the UDM (see clauses 4.15.6.3 or 4.15.6.3a of TS 23.502 [3]) and the SMF derived CN assisted RAN parameters tuning associated with a PDU Session using Control Plane CIoT 5GS Optimisation. This set of "Expected UE activity behaviour" parameters is valid for the UE; and - AMF may provide to the RAN a PDU Session level "Expected UE activity behaviour" parameters set, e.g. considering the SMF derived CN assisted RAN parameters tuning, per established PDU Session. The PDU Session level "Expected UE activity behaviour" set of parameters is associated with and valid for a PDU Session ID. The RAN may consider the PDU Session level "Expected UE activity behaviour" parameters when the User Plane resources for the PDU Session are activated;

When further looking into the detailed information elements, it can be found that the CN assisted RAN parameters tuning defined in TS 29.518/29.502 and Expected UE activity behaviour defined in TS 38.413 are totally different.

- The “CN assisted RAN parameters tuning” sent by the SMF to the AMF in TS 29.502

CnAssistedRanPara; 6.1.6.2.48; SMF derived CN assisted RAN parameters tuning. ##### 6.1.6.2.48 Type: CnAssistedRanPara Table 6.1.6.2.48-1: Definition of type CnAssistedRanPara Attribute name; Data type; P; Cardinality; Description. stationaryIndication; StationaryIndication; O; 0..1; Identifies whether the UE is stationary or mobile (see 3GPP TS 23.502 [3] clause 4.15.6.3).. communicationDurationTime; DurationSec; O; 0..1; Indicates for how long the UE will normally stay in CM-Connected for data transmission (see 3GPP TS 23.502 [3] clause 4.15.6.3).. periodicTime; DurationSec; O; 0..1; Identifies interval time of periodic communication (see 3GPP TS 23.502 [3] clause 4.15.6.3).. scheduledCommunicationTime; ScheduledCommunicationTime; O; 0..1; Identifies time and day of the week when the UE is available for communication (see 3GPP TS 23.502 [3] clause 4.15.6.3).. scheduledCommunicationType; ScheduledCommunicationType; O; 0..1; Indicates that the Scheduled Communication Type (see 3GPP TS 23.502 [3] clause 4.15.6.3). (NOTE 2). trafficProfile; TrafficProfile; O; 0..1; Identifies the type of data transmission: single packet transmission (UL or DL), dual packet transmission (UL with subsequent DL or DL with subsequent UL), and multiple packets transmission (see 3GPP TS 23.502 [3] clause 4.15.6.3).. batteryIndication; BatteryIndication; O; 0..1; Indicates the power consumption type(s) of the UE (see 3GPP TS 23.502 [3] clause 4.15.6.3).. NOTE 1: At least one of optional parameters above shall be present. NOTE 2: The value of attribute "scheduledCommunicationType" shall be used together with the value of "scheduledCommunicationTime"..

- The PDU session level “Expected UE Activity Behaviour” sent by the AMF to the NG-RAN in TS 38.413

#### 9.3.1.94 Expected UE Activity Behaviour This IE indicates information about the expected "UE activity behaviour" as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Expected Activity Period; O; INTEGER (1..30¦40¦50¦60¦80¦ 100¦120¦150¦180¦ 181, ...); If set to "181" the expected activity time is longer than 180 seconds. The remaining values indicate the expected activity time in [seconds].. Expected Idle Period; O; INTEGER (1..30¦40¦50¦60¦80¦ 100¦120¦150¦180¦ 181, ...); If set to "181" the expected idle time is longer than 180 seconds. The remaining values indicate the expected idle time in [seconds].. Source of UE Activity Behaviour Information; O; ENUMERATED (subscription information, statistics, ...); If "subscription information" is indicated, the information contained in the Expected Activity Period IE and the Expected Idle Period IE, if present, is derived from subscription information. If "statistics" is indicated, the information contained in the Expected Activity Period IE and the Expected Idle Period IE, if present, is derived from statistical information..

We understand the views that SMF can generate and send PDU session level “Expected UE activity behaviour” to the RAN in the SMF container. But this may require the following specification impacts.

- In TS 23.501, it should update the related descriptions on the SMF deriving the CN assisted RAN parameters tuning and sending it to the AMF.

- In TS 29.502/TS 29.518, the “Expected UE activity behaviour” should be included from the SMF to the AMF. Note that the existing CN assisted RAN parameters tuning is still needed for the a PDU Session using Control Plane CIoT 5GS Optimisation

Further, the “within SMF container” approach may require the CN products upgrade to support the PDU session level “Expected UE activity behaviour”. It is better for RAN3 to simply follow the CN descriptions at this late R16 stage for the SA groups.

- The PDU session level “Expected UE Activity Behaviour” is provided by the AMF to the NG-RAN node, outside of SMF container.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- The PDU session level “Expected UE Activity Behaviour” is provided by the AMF to the NG-RAN node, outside of SMF container.

The corresponding CRs for NGAP and XnAP are provided in [3-4].

5. Reference

- TS 23.501

- TS 29.502

- R3-213748	Correction on Expected UE activity behaviour	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated, ZTE

- R3-213737	Expected UE Activity Behaviour	Huawei, Lenovo, Motorola Mobility, Qualcomm Incorporated, ZTE