3GPP TSG-RAN WG3 Meeting #113-e 	R3-214269

- E-Meeting, August 16-27, 2021

Source:	Nokia (moderator)

Title:	Summary of Offline Discusion - CB: # 113_ GNBIDfailure

Agenda Item:	9.3.4.1

Document for:	Approval

# 1	Introduction

CB: # 113_GNBIDfailure

- Whether the issue is valid?

- Whether any solution is needed?

(Nok - moderator)

Summary of offline disc in R3-214269

# 2	For the Chairman Nodes

[To be Updated]

# 3	Discussion

The issue under discussion is in regard to F1 Setup Failure scenarios due to conflict of the gNB-DU-ID.

The gNB-DU-ID is a unique identifier that is configured at gNB-DU. Therefore, in multi-vendor scenarios, it is possible that gNB-DUs from different vendors have been configured with the same gNB-DU-ID. The consequence from this duplicated gNB-DU-ID configuration, is that gNB-CU will fail F1 Setup Requests if it detects that the gNB-DU-ID is already in use by a different gNB-DU. However, there is no suitable cause value in the current F1AP specification to hint the reason for the failure. Hence, it is likely that the gNB-DU reattempts the F1 Setup procedure, only to fail again.

Introducing a new cause value is a straightforward approach that would help deal with the issue above, since the gNB-DU would be able to deduce the reason for the failure and not reattempt the F1 Setup with the same gNB-DU-ID value.

Questions 1: Do companies agree with the approach based on a new cause value to indicate the conflict on gNB-DU-ID to the gNB-DU?

| Company | Comment |
| --- | --- |
| Nokia | Yes, we see this as a simple solution to address this issue. Similarly, the cause value is simply to identify the issue, and it is still an implementation option whether the gNB-DU does any further action to address this fault (e.g., raise an alarm, or assign a different value). |
| Samsung | No. This cannot solve the problem since gNB-DU cannot avoid using another duplicate gNB-DU ID with such cause value. The gNB-DU has 36 bits. It is difficult to be conflicted. We can rely on OAM. |
| Huawei | We are still not sure whether this is a practical issue or not. gNB-DU ID should be allocated in a coordinated way, how this could happen that a gNB-DU ID is allocated twice to different gNB-DU? |

Questions 2: Do companies have a different proposal on how to deal with this issue? Please indicate if so.

| Company | Comment |
| --- | --- |
| Nokia | During the online discussion it was mentioned OAM can solve this issue. However, in multivendor deployments there is no guarantee that the OAM system is the same for all vendors in the deployments. Hence, we see such solution as not fault proof for all deployments. |
| Samsung | This is not an issue. |
| Huawei | As commented above, firstly we are not sure if this is a practical issue; then if there happens such a case, we have to rely on OAM, since node identifier is not generated by node itself. |

# 4	Conclusion, recommendations [if needed]

[To be Updated]

# 5	References

- R3-213484, Duplicate gNB-DU ID failure, Nokia, Nokia Shanghai Bell