3GPP TSG-RAN WG3 #113-e	R3-214187

16 – 26 Aug 2021

Agenda Item:	21.3

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on New QoS Related Parameters

Document for:	Approval

# Introduction

CB: # 116_NotifiationControl

- Rewording the text with procedures rather than sub-IEs?

- No need for new sub section

- Check details

(HW - moderator)

Rev in R3-214272

This paper is just to collect company views, and possibly no need to upload.

# For the Chairman’s Notes

WA: carefully crafted text…

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

Based on the comments online, the CR for TS 29.413 is updated as follows (also uploaded in the Inbox / Drafts / CB # 116_NotifiationControl).

## 5.3 Exceptions for NGAP message contents and information element coding when used for non-3GPP access For the NGAP messages transferred between the Non-3GPP access network node and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied: PDU SESSION RESOURCE SETUP REQUEST message: - the following IEs shall be ignored, when received: - RAN Paging Priority IE - UE Aggregate Maximum Bit Rate IE (except for non-trusted non-3GPP access, trusted non-3GPP access and trusted WLAN access as specified in TS 23.501 [3]). - Notification Control IE included in the QoS Flow Level QoS Parameters IE - Alternative QoS Parameters Set List IE included in the QoS Flow Level QoS Parameters IE PDU SESSION RESOURCE RELEASE COMMAND message: - the following IEs shall be ignored, when received: - RAN Paging Priority IE PDU SESSION RESOURCE MODIFY REQUEST message: - the following IEs shall be ignored, when received: - RAN Paging Priority IE - Notification Control IE included in the QoS Flow Level QoS Parameters IE - Alternative QoS Parameters Set List IE included in the QoS Flow Level QoS Parameters IE INITIAL CONTEXT SETUP REQUEST message: - the following IEs shall be ignored, when received: <Skip the irrelevant> - RG Level Wireline Access Characteristics IE: the information given within this IE between the W-AGF and the AMF shall be stored in the UE context by the W-AGF as specified in TS 23.316 [6]. - Notification Control IE included in the QoS Flow Level QoS Parameters IE - Alternative QoS Parameters Set List IE included in the QoS Flow Level QoS Parameters IE

Question: Do you agree to the revision of the NGAP CR? If not, please provide your revision suggestions, or reasons.

Please provide any view / comments on this topic and the proposal in bold below:

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-213753 | Ignoring the notification Control for WWC (Huawei, BT) | CR0011r, TS 29.413 v16.2.0, Rel-16, Cat. F ** Nok: Remove new section, just add some texts. Sub IE description is not preferred. E///: Not needed, SA2 spec already has such kind of description. If needed, prefer to have a high level text description. HW: But Alternative QoS Parameters Set List IE is not there in SA2 spec CB: # 116_NotifiationControl - Rewording the text with procedures rather than sub-IEs? - No need for new sub section - Check details (HW - moderator) Rev in R3-214272 |
| --- | --- | --- |