3GPP TSG-RAN WG3 Meeting #113-e 	R3-214273

- E-Meeting, August 16-27, 2021

Source:	Nokia (moderator)

Title:	Summary of Offline Discusion - CB: # 117_ CellOperation

Agenda Item:	31.2.4

Document for:	Approval

# 1	Introduction

CB: # 117_CellOperation

-  Clarify on the scenario and issue

(Nok - moderator)

Summay of offline disc in R3-214273

# 2	For the Chairman Nodes

[To be Updated]

# 3	Discussion

The scenario under discussion is that of a cell that has been configured with Standalone and Non-Standalone support simultaneously.

Cell configuration is carried out at gNB-DU. Likewise Served Cell Information IE in F1AP includes the following IEs:

- 5GS TAC

- Configured EPS TAC

With the above IEs, the gNB-CU can identify whether the cell has been configured with support toward the 5GC (for Standalone) and toward the EPS (for EN-DC Non-Standalone). In our understanding there is no restriction in the cell configuration to support only one core type at a time (5GC or EPS). Hence, it is possible for a cell to operate in both modes simultaneously.

However, the gNB-CU controlled procedures for cell handling (cell activation/deactivation) do not make an explicit distinction on whether the command is applicable to NSA, SA or both NSA+SA. Therefore, if an issue is encountered in one of the core network interfaces (S1 or NG), it will lead to deactivation of the whole cell and drop all the UEs in the cell.

In order to avoid dropping all users in the cell, it is proposed to introduce changes that would allow the gNB-CU to signal to the gNB-DU on core network connectivity issues. Then, based on this information the gNB-DU can limit its operation only to the mode that was not affected by the core network interface issue.

Questions 1: Do companies agree that existing F1AP specification supports configuring a cell for both Standalone and Non-Standalone operation simultaneously?

| Company | Comment |
| --- | --- |
| Nokia | Yes, in our understanding Served Cell Information IE includes optional IEs 5GS TAC IE that indicates support for Standalone operation, and Configured EPS TAC IE which indicates support for EN-DC Non-Standalone operation. Both IEs can be set simultaneously. |
| Samsung | No. We are not sure if this is a real case. The EN-DC NR Cell cannot be accessed by the UE directly since some broadcast message is not provided, e.g., SIB1, while SA NR Cell should provide all the information for the UE access. So, how to support both in one cell is a question. |

Questions 2: For a cell configured with support for both Standalone and Non-Standalone operation, do companies see a need to support to allow the cell to remain operational if there is an issue toward the core network on only S1 or NG interfaces?

| Company | Comment |
| --- | --- |
| Nokia | Yes, if a cell is operational for both SA and NSA modes, and there is an issue only toward one of the core networks (e.g., issue only on S1), it would be beneficial to avoid dropping the users that were not impacted by the core network interface failure. |
| Samsung | No |

# 4	Conclusion, recommendations [if needed]

[To be Updated]

# 5	References

- R3-213478, Discussion on NSA-SA Cell Operation, Nokia, Nokia Shanghai Bell