3GPP TSG-RAN WG3 #113-e	R3-214144

Online, 16th - 26th August 2021

Agenda Item:	8.3.1

Source:	Ericsson (moderator)

Title:	CB: # 12_ RACSCapDetection - Summary of email discussion

Document for:	Approval

# Introduction

This is the Summary of offline disc document concerning RACS capability detection during NG based handover for which the document number R3-214144 was allocated

# For the Chairman’s Notes

To be added

# Discussion

## Shall there be a signaling-based solution or should we rely on OAM or even a “do-nothing”-approach?

Please provide your view

| Company | Comment |
| --- | --- |
| Qualcomm | As discussed in our paper, “do-nothing” essentially negates the RACS feature on N2/S1/inter-system handover (either because capability upload may be needed from the UE in some scenarios, or capabilities are always sent regardless). OAM is of course possible, but SA2’s LS specifically asked for an inter-operable plug-and-play methodology. So it seems difficult to escape the logic for some kind of signalling solution. |

## If there is a signaling-based solution agreed shall it rely on explicit node capability exchange or make use of criticality diagnostics information?

Please provide your view

| Company | Comment |
| --- | --- |
| Qualcomm | The two options are not as distant as it may seem. For RACS, both options require the introduction of new IEs at least in the forward message. In that sense the criticality option is also sending a capability (from the source). Then one criterion to consider is the impact on nodes. The criticality option would logically make use of failure transparent container, which does not exist in S1 specs, this would at least impact the MME. The IE handshake option (we do not need to think of it as capability) can be used in successful handovers (for example in a first handover, the source acts in a sub-optimal way e.g. by sending full capabilities so failure does not happen). Hence no CN impact. So, the above aspects should be taken into account, and have probably not been discussed yet. |

## In case explicit capability exchange is agreed

Any comments on documents [2], [3] (CRs)

| Company | Comment |
| --- | --- |
| Qualcomm | Release should be discussed |

## In case utilization criticality diagnostics is agreed

Any comments on documents [15], [16] (CRs, Huawei) and [9], [10] (CRs Ericsson)?

| Company | Comment |
| --- | --- |
| Qualcomm | Release should be discussed |

## Additional corrections:

Any comments on documents [5], [6], [11] (Ericsson) and [17], [18] (Huawei)?

| Company | Comment |
| --- | --- |
| Qualcomm | [11] is a good catch and should be corrected for sure. For the others, suggest merging / converging, but should also be cleaned up independently of above. |

## LS out

depends on outcome of above items, should be straight forward.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-213260	Further discussion on RACS Capability Detection for S1 and NG handover (Qualcomm Incorporated, Vodafone), discussion

- R3-213261	Detection of RACS support at target during N2/S1 handover (Qualcomm Incorporated, Vodafone)	CR 36.413 Rel-16

- R3-213262	Detection of RACS support at target during N2/S1 handover (Qualcomm Incorporated, Vodafone)	CR 38.413 Rel-16

- R3-213263	[DRAFT] Reply LS on RACS capability detection with S1 and NG handover (Qualcomm Incorporated) LS out

- R3-213364	Correcting assigned criticality for IEs in transparent handover containers (Ericsson), CR 38.413 Rel-16

- R3-213365	Correcting assigned criticality for IEs in transparent handover containers (Ericsson) CR 36.413 Rel-16

- R3-213400	[DRAFT] Reply LS on RACS capability detection with S1 and NG handover (Ericsson) LS out Rel-16

- R3-213580	On communication between source and target RAN node at CN handover for non-supported features (Ericsson) discussion Rel-17

- R3-213581	Introducing target Criticality Diagnostics in target to source failure container (Ericsson) other, 38.413 Rel-17

- R3-213582	Introducing target Criticality Diagnostics in target to source failure container (Ericsson) other, 36.413 Rel-17

- R3-213583	Correcting Presence of the Cell CAG Information IE in ASN.1 (Ericsson) CR 38.413 Rel-16

- R3-213665	Discussion on RACS capability detection with S1 and NG handover (CATT) discussion

- R3-213666	[Draft] Reply LS on RACS capability detection with S1 and NG handover (CATT) LS out

- R3-213730	On RACS capability detection for non-direct-connected nodes (Huawei) discussion

- R3-213731	On RACS capability detection for non-direct-connected nodes (Huawei) CR 38.413 Rel-17

- R3-213732	On RACS capability detection for non-direct-connected nodes (Huawei) CR 36.413 Rel-17

- R3-213733	On IEs with reject criticality in the source and target transparent container (Huawei) CR 38.413 Rel-16

- R3-213734	On IEs with reject criticality in the source and target transparent container (Huawei) CR 36.413 Rel-16

- R3-213735	[Draft] Reply LS on RACS capability detection with S1 and NG handover (Huawei) LS out