3GPP TSG-RAN WG3 #113-e	R3-214191

Online, 16 – 26 Aug, 2021

Agenda Item:	8.1

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on CB: # 29_RRCRejectDU

Document for:	Approval

# Introduction

CB: # 29_RRCRejectDU

- Evaluate the necessity of the scenario, whether the enhancements based on RRC Reject templates are worth the impact on the system?

- Stick to current mechanism RRC Reject message generated by gNB-CU or by gNB-DU?

- The NETWORK ACCESS RATE REDUCTION message is extended with an RRC container where the CU may provide a template of the RRCReject message that the DU may use while it is overloaded?

[NWM] (HW - moderator)

Summary of offline disc in R3-214191

# For the Chairman’s Notes

Propose the following:

# Discussion [if needed]

This was discussed in previous meeting, and was raised again in this meeting [1][2][3][4][5][6]. From moderator’s understanding, the main issues are, if gNB-DU is allowed to reject a RRC connection request, if yes, what’s the additional benefits that gNB-DU is allowed to generate a RRC connection reject message directly, comparing current mechanism; or, if there are any drawbacks for this proposal.

## Is gNB-DU allowed to reject UE access request based on current spec?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | As indicated in the discussion paper, the spec clearly specifies that: If the DU to CU RRC Container IE is not included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU should reject the UE under the assumption that the gNB-DU is not able to serve such UE. Here the only intention is that gNB-CU will make the final decision. In addition, gNB-CU also learns the overload situation of the gNB-DU. |

## Are there any potential benefits/drawbacks of this proposal?

Moderator’s note: Let’s make analysis on the potential benefits/drawbacks if gNB-DU is allowed to generate RRC connection reject message directly (with template provided by gNB-CU), especially comparing with current mechanism.

| Company | Comment |
| --- | --- |
| Huawei | In general we don’t think it will bring additional benefits. As also pointed in the discussion paper, e.g. If gNB-DU is allowed to autonomously reject a UE’s RRC connection request, it would further introduce additional issues, e.g. rejection criteria handling, how to stop, how reject with redirection could work, etc; and, even gNB-DU is allowed to reject a RRC connection request, such even should be reported to gNB-CU, this means that anyway additional F1 procedure is not avoidable. |

## Is there a need to introduce such a mechanism?

Moderators’ note: Companies are invited to share further thoughts if there is a need to introduce such a mechanism or current spec could already achieve the intention.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | No. | Taking the above comments into account, we think there is no need to introduce such a mechanism since no additional benefits are foreseen. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-213419, RRC Reject Template for the gNB-DU (Ericsson)

- R3-213474, Discussion on RRC Reject Template (Nokia, Nokia Shanghai Bell)

- R3-213475, RRC Reject Template [REJECT_TEM] (Nokia, Nokia Shanghai Bell)

- R3-214082, Further discussions on RRC reject template for the gNB-DU (Huawei)

- R3-214088, Consideration on Template RRC Reject for gNB-DU (ZTE)