3GPP TSG-RAN WG3 #113-e	draft R3-214213

Online, 16-26 August 2021

Agenda Item:	22.2.3

Source:	Samsung (moderator)

Title:	Summary of Offline Discussion on CB # MBS3_PTPMSwitch

Document for:	Approval

# Introduction

CB: # MBS3_PTPMSwitch

- Further discussion on the open issues and check RAN2 progress

- The gNB-CU or the gNB-DU decides the PTP/PTM switch?

- For the split MRB bearer with a common PDCP, either the shared F1-U tunnel is used for PTP and PTM transmission, or the shared F1-U tunnel is used for PTM and the dedicated F1-U tunnel used for PTP respectively?

- Is the shared F1-U tunnel used within per cell or per DU?

- Stage2/stage3 TPs if agreeable and check details, split work, if needed

- Capture agreements and open issues

(Samsung - moderator)

Summary of offline disc in R3-214213

# For the Chairman’s Notes

Propose to capture the following:

For the RAN2 agreed split MRB bearer with a common PDCP: the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU

Shared tunnel is established between gNB-CU and gNB-DU, to transmit initial data for a multicast service.

If data forwarding/PDCP retransmission is agreed, per-UE unicast F1-U tunnel is needed to transfer forwarded data or retransmitted data.

PTP/PTM switch decision should not be notified from gNB-DU to gNB-CU, if a shared F1-U tunnel for initial transmission is agreed.

FFS the shared tunnel is per DU or per Cell.

To discuss if the TP in below is agreeable in the second round.

R3-213577  (TP to TS 38.401 BL CR) Support of PTP and PTM switch (Huawei, CBN, China Unicom, China Telecom)

# Discussion

## PtP/PtM Decision should be notified to gNB-CU?

In last RAN3 meeting, the below working assumption was achieved.

WA: For the RAN2 agreed split MRB bearer with a common PDCP: the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU

In this meeting, many contributions propose to change this WA into agreement. Based on the proposals, moderator thinks we can change the WA into agreement.

Proposal 1:     Working assumption for PTP or PTM decision made by gNB-DU can be changed into agreement.

Does company agree proposal 1? If not, please indicates your objection and reason in the below table.

| Company | Comment |
| --- | --- |
| Samsung | Can turn it into agreement, if there is no major issue in the flow control. |
| Nokia | OK |
| Lenovo, Motorola Mobility | Agree. |
| ZTE | Agree. |
| Huawei | Agree |
| CATT | The WA is related to PTP transmission over F1-U. If gNB-DU makes decision but gNB-CU is not notified, there may be some questions with this proposal. E.g, how to implement UE specific data rate management over shared F1-U tunnel. So we prefer to keep the WA. |
| LGE | We can agree if it is possible that the gNB-CU notifies the gNB-DU of the switch decision. |
| CMCC | Agree |
| Ericsson | The WA quoted above can be turned into an agreement. |
| Qualcomm | Agree |

A related issue is some contributions propose gNB DU should notify the decision to gNB-CU. Does company agree the PTP/PTM switch decision should be notified from gNB-DU to gNB-CU?

| Company | Comment |
| --- | --- |
| Samsung | We think it is not needed. |
| Nokia | NOK. We also think it is not needed as long as split MRB is kept. |
| Lenovo, Motorola Mobility | It depends on the F1-U tunnel for PTP leg. If a UE dedicated F1-U tunnel is used for PTP leg, it may be useful to inform gNB-CU so that the gNB-CU can start/stop send the data over the UE dedicated F1-U tunnel. |
| ZTE | No. In RAN2 the dynamic mode switch for split MRB is a lower layer scheduling decision. |
| Huawei | Disagree, this is quite inefficient. |
| CATT | This question is associated with how to organize data over F1-U. Our preference is to notify. |
| LGE | Considering PTP transmission over F1-U, we think it is needed, for example, when the gNB-DU determines switching to PTP for a given UE. |
| CMCC | We think it is needed, same view with LGE. The switch decision signaling may carry the information about which UE switches to PTP mode. |
| Ericsson | Not ok, this is not needed, as it would contradict in a way the WA we want to turn into an agreement. The switch is a pure DU business, no CU function is associated to it. |
| Qualcomm | This depends on the F1 tunnel configuration decision. For split MRB, if PTP leg and PTM leg share same F1-U tunnel, the DU to CU notification is not needed. Otherwise, it is needed. |

Moderator’s summary:

All commented companies agree to change the WA into agreement.

4 companies think DU needn’t notify the decision to CU while 3 companies think notification is needed. 2 companies prefer to wait for the F1 tunnel conclusion. It is reasonable to have conclusion on F1 tunnel first. We can re-check if the notification issue can be concluded.

Proposal 1: Agree:

For the RAN2 agreed split MRB bearer with a common PDCP: the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU

PTP/PTM switch decision should not be notified from gNB-DU to gNB-CU, if a shared F1-U tunnel for initial transmission is agreed.

## F1-U tunnel

In F1-U interface, there are two alternatives for MBS multicast data transmission.

- Data is transmitted via one shared tunnel to gNB-DU. gNB-DU sends the received data via PTP leg or PTM leg to UE. The left figure in below shows one shared F1-U tunnel connecting with PTP leg and PTM leg.

- Data is transmitted via two types of tunnel to gNB-DU, per-UE unicast F1-U tunnel connected to PTP RLC and one shared F-U tunnel connected to PTM RLC. The right figure in below shows two types of tunnel in F1-U are used for multicast data transmission.

One opinion from the contributions is gNB-CU transmits PDCP PDUs to gNB-DU via a shared F1-U tunnel established for this MRB, and then gNB-DU makes the decision on whether PTP leg or/and PTM leg to use. On the other hand, another opinion from the contributions is initial transmission of PTP through unicast F1-U tunnel can benefit UE-specific DL flow control and PTP retransmission/forwarded data should be delivered through unicast F1-U tunnel. There is some difference between the initial transmission and forwarded/retransmitted data transmission. So in below, we separate the initial transmission case and forwarding/retransmission case.

Which option do you prefer for the initial data transmission for MBS multicast service? Option A or option B? Please indicates your preference and reason in the below table.

| Company | Comment |
| --- | --- |
| Samsung | For initial transmission, we prefer Option A. The main benefit of gNB-DU makes decision is gNB-DU can make quicker decision. If gNB-CU switches the transmission between two tunnels for PTP/PTM transmission based on the notification from gNB-DU, there is delay between the gNB-DU’s decision and the gNB-UP’s action. The benefit of quicker decision is losing. So we think for the initial transmission, the gNB-DU receives the initial transmission data via a shared tunnel and can decide transmit data via PTP leg or PTM leg, without notifying the decision to the gNB-CU. This is based on the assumption that CU-CP decides to setup common PDCP with two legs. If CU-CP decides to setup PTP leg only, the initial transmission is of course via unicast F1-U tunnel. |
| Nokia | Option A. same reasons as Samsung. |
| Lenovo, Motorola Mobility | We would prefer option A. |
| ZTE | Option A is preferred. - A shared tunnel for initial transmission (of PTP or PTM) offers the scalability other options can not. Scalability is why we pursue MBS. - As Samsung pointed out, option A fits into the gNB-DU making decision agreements. |
| Huawei | Option A |
| CATT | We prefer Option B as it unifies the structure and easier for flow control. In addition, for Option A, UE's specific data buffer is located in gNB-DU, according to RAN2 discussion, PDCP SR will be introduced to trigger PDU retransmission in downlink, while PDCP entity is located in gNB-CU. Thus the Option A would lead to buffer inconsistency issue. |
| LGE | We prefer Option B. |
| CMCC | Option A is preferred, same view with Samsung. |
| Ericsson | with the agreement on the WA and the current status of RAN2 discussions, there is no need to discuss any further. b) simply doesn’t exist (yet). |
| Qualcomm | Option A is preferred for initial transmission. |

Is per UE unicast F1-U tunnel also needed in order to transmit the forwarded data or PDCP retransmitted data, if assuming there is forwarded data or PDCP retransmitted data?

| Company | Comment |
| --- | --- |
| Samsung | For forwarded data, or retransmission data, unicast F1-U tunnel is needed. |
| Nokia | For forwarded data, or retransmission data, per UE unicast F1-U tunnel is needed. |
| Lenovo, Motorola Mobility | We support to have a unicast F1-U tunnel for PDPC retransmission. |
| ZTE | Yes, per UE F1-U tunnel for per UE packet data is a neat solution. |
| Huawei | Agree. For forwarded data or retransmitted data, the UE-dedicated F1-U tunnel is needed. |
| CATT | For forwarded data, or retransmission data, unicast F1-U tunnel is needed. |
| LGE | For forwarded data, or retransmission data, per UE unicast F1-U tunnel is needed. |
| CMCC | We think a per UE F1-U tunnel is needed for retransmitted data. Please clarify the scenario for forwarded data. |
| Ericsson | we should first discuss data forwarding and then have this discussion round. RAN2 didn’t even agree on SN status report for MRBs. |
| Qualcomm | Yes, per UE F1-U tunnel is needed for retransmission and forwarded data. |

Moderator’s summary:

For initial transmission, majority companies (8 vs 2) prefer option A, i.e. PTP/TPM data is transmitted via one shared tunnel to gNB-DU. So it is proposed to agree it.

If there is forwarded data, or retransmission data, all companies agree unicast F1-U tunnel is needed.

Proposal 2: Agree:

Shared tunnel is established between gNB-CU and gNB-DU, to transmit initial data for a multicast service.

If data forwarding/PDCP retransmission is agreed, per-UE unicast F1-U tunnel is needed to transfer forwarded data or retransmitted data.

## Shared F1-U tunnel is per DU or per cell?

For the shared F1-U tunnel, is it per DU or is it per cell?

| Company | Comment |
| --- | --- |
| Samsung | If the MRB is per DU, then the F1-U tunnel is per DU. Otherwise, it is per-cell. |
| Nokia | shared F1-U tunnel per cell. |
| Lenovo, Motorola Mobility | We prefer per DU F1-U tunnel for saving transmission resource in F1-U. |
| ZTE | Per DU. Common PDCP SN at least in the scope of one CU is pursued for better mobility among cells who belong to the same DU or even CU. Therefore a single tunnel is good enough for such MRB if the packet data is exactly the same. |
| Huawei | We suggest that adopt per cell shared F1-U tunnel. |
| CATT | Agree with Huawei. |
| LGE | We prefer per cell for the shared F1-U tunnel. |
| CMCC | Agree with Huawei. |
| Ericsson | This is part of the E1/F1 discussion and shall be discussed in AI 22.2.4. But if you want to hear our opinion: we want flexibility. |
| Qualcomm | Per cell |

Moderator’s summary:

There is no consensus or majority view. So keep this issue open.

Proposal 3: FFS:

FFS the shared tunnel is per DU or per Cell.

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-213549 | Discussion on dynamic change between PTM and PTP (CATT) |
| --- | --- |
| R3-213577 | (TP to TS 38.401 BL CR) Support of PTP and PTM switch (Huawei, CBN, China Unicom, China Telecom) |
| R3-213706 | F1 tunnel and flow control for MBS (Samsung) |
| R3-213771 | Discussion on switching decision between PTP and PTM (LG Electronics) |
| R3-213832 | Mode switching for NR MBS (ZTE) |
| R3-214094 | Discussion on dynamic change between PTP and PTM (CMCC) |