3GPP TSG-RAN WG3 Meeting #113-e	R3-214182

E-meeting, 16-26 Aug 2021

Agenda Item:	14.3

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on MRDC3_CPAC

Document for:	Approval

# Introduction

CB: # MRDC3_CPAC

- Further discussion on the open issues left in R3-212785 and check RAN2 progress

- How to provide the data forwarding address? How to support the Early data forwarding and Late data forwarding?

- Whether to introduce “List of Prepared PSCell IDs” in SN Change Confirm?

- How to support CPAC replace/cancel?

- Impact on F1 and E1 interfaces?

- Capture agreements as stage2/stage3 CRs and check details, split work, if needed

- List open issues for next meeting in the summary

(HW - moderator)

Summary of offline disc in R3-214182

# For the Chairman’s Notes

//to be added

# Discussion

## Preparation of multiple T-SN

It was FFS on whether it is allowed and how to prepare multiple Target SNs in SN initiated inter-SN CPC, by parallel or single SN Change Required procedure(s)? several options were listed last meeting:

1) If one SN change procedure can only prepare one target SN, and if parallel preparation is not supported, current SN change required/confirm procedures can be reused.

2) If one SN change procedure can only prepare one target SN, and if parallel preparation is supported, Target S-NG-RAN node ID IE needs to be introduced in SN Change confirm and SN change Refuse.

3) If one SN change procedure can prepare multiple target SNs, a list of Target S-NG-RAN node ID needs to be introduced in SN change required/confirm/refuse, and need to include data forwarding address per Target SN to support early data forwarding.

Question 1: Selection among the 3 options for multiple T-SN preparation.

- Option 1: One SN change procedure can only prepare one target SN, Parallel preparation is not supported

- Option 2: One SN change procedure can only prepare one target SN, Parallel preparation is supported

- Option 3: One SN change procedure can prepare multiple target SNs

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 2 | Option 2 supports multiple T-SN preparation with minimum standard impact. |

Question 2: If option 2 is selected, do you agree to introduce Target SN ID in SN Change Confirm and SN change Refuse?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 3: If option 3 is selected, do you agree to introduce a list of Target S-NG-RAN node ID in SN change required/confirm/refuse, and include data forwarding address per Target SN to support early data forwarding?

| Company | Answer | Comment |
| --- | --- | --- |

## Signalling design of Preparation of multiple PSCells

In RAN3#112-e meeting, the following agreement and working assumption were achieved:

- Initiating node provides upper limit for the number of PSCells to be prepared (i.e. maximum number of PSCells).

- WA: initiating node provides suggested number of PSCells to be prepared.

Considering that the suggested number can help the target node to make decision on how many PSCells to be prepared, it is better to turn the WA to agreement.

Question 4: do you agree to turn the WA “initiating node provides suggested number of PSCells to be prepared.” to agreement?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes | The initiating node is aware of the current transmission load of the UE, and it is helpful if it provide a suggested number of the PSCells to assist the T-SN to make the final decision. |

In [10], it is mentioned that the node initiating the CPAC does not know the situation in the target node, a feedback mechanism shall be defined to enable the target SN to indicate higher quota is beneficial. Therefore it is proposed that the node responding to the CPAC request may provide, in addition to the list of prepared PSCells, a number of additional cells that may be prepared.

Question 5: do you agree to include the “a number of additional cells that may be prepared” by the target SN, to enable the target SN to indicate higher quota?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | No | How the T-SN can know whether a higher quota is possible? |

## Signalling design of SN initiated inter-SN CPC

There is a RAN2 agreement for SN initiated inter-SN CPC:

  - SN initiated Inter-SN CPC, upon reception of ‎RRCReconfiguration/RRCConnectionReconfiguration message with CPAC configuration, UE responds with RRCReconfigurationComplete / RRCConnectionReconfigurationComplete message to MN. This message can include an embedded RRC complete message for source SN.

In [12], it is mentioned that it is FFS how to provide from the MN to the source SN, the embedded RRC complete message for source SN, there are two possibilities:

- Solution 1: introduce the RRC complete message for source SN in the SN Change Confirm message.

- Solution 2: add a new step from MN to the source SN, reusing SgNB Reconfiguration Complete message to transmit the RRC complete message for source SN

Question 6: In SN initiated inter-SN CPC, how to provide from the MN to the source SN, the embedded RRC complete message for source SN?

| Company | Answer | Comment |
| --- | --- | --- |

Last RAN3#112e has reached the agreement and FFS that:

- Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK.

- FFS whether to introduce “List of Prepared PSCell IDs” in SN Change Confirm.

In the submitted papers of this meeting, many companies would like to introduce “List of Prepared PSCell IDs” in SN Change Confirm. In [16], it mentions that RAN2#114e has an agreement that S-SN is informed about which candidates were accepted/ rejected by T-SN. And [3] considers that it may have a benefit that when the S-SN at least need to update the measurement configuration, or want to update execution condition corresponding to the prepared PScells that are chosen by the T-SN. [15] would like to align with CPA and MN initiated inter-SN CPC. On the other hand, [19] thinks that it seems meaningless to the S-NG-RAN node.

Based on the RAN2 agreement, and the provided views in these companies, it seems it is better to and has to introduce “List of Prepared PSCell IDs” in the SN Change Confirm message.

Question 7: Do you agree to introduce “List of Prepared PSCell IDs” in SN Change Confirm.

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

In [3], it is considered that for the SN initiated CPC, even if we will not allow the S-SN to initiate the preparation of the multiple T-SNs, it still need to have a list of PSCell IDs in the SN Change Required message, in order for the MN to explicitly know the target PSCell ID so can avoid the MN to dig into the RRC container, especially for EN-DC, this will help MeNB to avoid digging into NR RRC ASN.1. By the explicit PSCell ID in the SN Change Required message, the MN can know the T-SN for CPC.

Question 8: do you agree to include the “List of Prepared PSCell IDs” in SN Change Required?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | No | Seems not needed…as the PSCell ID is included in the RRC container, and the MN needs to dig into the RRC container, for example, if a bearer is changed from SN terminated bearer to MN terminated bearer, the MN needs to dig into the RRC container to get the SCG-RB-Config. |

## CPC triggered and CPC executed

In RAN3#111-e, the following WAs were achieved:

WA: in case of MN initiated inter-SN CPC, to support early data forwarding, the MN needs to inform source SN about CPC triggered (i.e. the successful reconfiguration of CPC at UE), details FFS.

WA: in case of both MN and SN initiated inter-SN CPC, to support late data forwarding, it is needed to inform the source SN about the successful CPC execution and UE accesses to the target SN, details FFS. RAN3 waits for RAN2 progress before discussing further details.

In RAN3#112-e, we got the following agreements and FFSs:

In case of MN initiated inter-SN CPC, introduce new X2AP class 2 procedure from MN to inform the source SN about “CPC triggered”.

FFS on providing the data forwarding address. For Xn, FFS new XnAP class2 procedure or reuse Xn-U Address Indication procedure.

In case of SN initiated inter-SN CPC, using a class 2 procedure in both X2AP and XnAP to indicate “CPC executed”. For X2, a new class2 procedure is introduced.

For Xn, it is FFS on introducing the new class2 procedure or resuing address indication procedure.

FFS if this new procedure can be reused to indicate “CPC triggered” in early data forwarding.

In the XnAP baseline CR, there is a FFS on which procedure to be used to indicate “CPC trigger”:

In case of CPC, upon receiving the MN RRC reconfiguration complete message from the UE, the MN informs the SN that the CPC has been triggered via [FFS new XnAP class2 procedure or Xn-U Address Indication] procedure, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

In [2] and [19], it is suggested to reuse existing XN-U Address Indication procedure to indicate “CPC trigger” and provide the data forwarding address. And in [19], it is also proposed to reuse this procedure to inform about "CPC triggered” or "CPC executed” to the source S-NG-RAN node and it is not needed to introduce a new class 2 procedure and increase the specification complexity.

Question 9: In MR-DC, how to indicate “CPC triggered” and “CPC executed”?

Option 1: introduce new XnAP class2 procedure

Option 2: reuse Xn-U Address Indication procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 2 |  |

In the baseline CR, MN informs “CPC triggered” to the source SN upon receiving the MN RRC reconfiguration complete message from the UE, in [2], it is mentioned that “in case of CHO with MR-DC, XN-U Address Indication message is executed after step 2, i.e., early data forwarding can be triggered after Handover. So that, the “CPC trigger” sending occasion for MN initiated CPC can be simply aligned with that for CHO with MR-DC.”

Question 10: In MR-DC, for MN initiated inter-SN CPC, when to inform CPC triggered from MN to the source SN?

Option 1: upon receiving the MN RRC reconfiguration complete message from the UE

Option 2: upon receiving SN addition request ACK from the T-SN

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 1 |  |

Based on the achieved agreements, a new X2AP: Conditional PSCell Change Notification procedure was introduced in the X2 related BL CRs. However, in [2], it is proposed to revert the agreement and reuse existing X2AP class 2 Data Address Indication procedure from MN to inform the source SN about “CPC triggered”.

Question 11: in EN-DC, whether to revert the existing agreement of using new X2AP procedure to inform “CPC triggered”, by reusing existing X2AP class 2 Data Address Indication procedure?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Keep the previous agreement | We assume that new X2APmessage could be used to indicate both CPC triggered, CPC execute, and CPAC cancel (there are proposals to introduce new Class2 procedure to support CPAC cancel, see section 3.5). Different with XnAP, in the X2AP: DATA FORWARDING ADDRESS INDICATION message, the DL GTP Tunnel Endpoint IE is a mandatory IE. If early data forwarding is ongoing, the forwarding address should not be provided in the procedure to indicate CPC execute, as if different forwarding address is provided, it is unclear how to handle the data from two forwarding tunnels. |

## CPAC replace and cancel

There are several papers [16] [15] [19] [22] [10] [8][17] [13] submitted on how to support CPAC replace and cancel, one paper [3] propose to further hold on the discussion of CPAC updating/ Cancel /replacement, until a RAN2/RAN3 reach a certain level on the basic CPAC procedures.

In order to discuss this topic clearly, we will split the discussion to CPA and CPC part separately, and discuss the way to modify the prepared PSCells, add some new prepared PSCells, cancel part of the prepared PSCells, and cancel all PSCells.

### Replace and cancel for CPA

Question 12: do you agree that:

- both the MN and the T-SN can trigger replace and cancel for CPA

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 13: for CPA, do you agree to:

- use MN/SN initiated SN modification procedure to update the configuration of the prepared PSCells

- use SN initiated SN modification procedure to add some PSCells as prepared

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 14: for CPA, to cancel part of the prepared PSCells, which option do you preferred?

Option 1: MN/SN initiated SN Release procedures

Option 2: MN/SN initiated SN Modification procedures

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 2 | As the node may trigger to update some of the prepared PSCells, or (SN only) to add some PSCells as prepared, and in the meanwhile ask to release some of other prepared PSCells. |

Question 15: for CPA, to cancel all of the prepared PSCells in the T-SN, do you agree to:

- use the MN/SN initiated SN Release procedures

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

### Replace and cancel for MN initiated inter-SN CPC

Question 16: do you agree that

- both the MN and the T-SN can trigger replace and cancel for MN initiated inter-SN CPC

- S-SN cannot trigger replace and cancel for MN initiated inter-SN CPC

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 17: for MN initiated inter-SN CPC, to update the configuration of the prepared PSCells, do you agree to:

- use MN/SN initiated SN modification procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 18: for MN initiated inter-SN CPC, to add some PSCells as prepared, do you agree to:

- use T-SN initiated SN modification procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 19: for MN initiated inter-SN CPC, to cancel part of the prepared PSCells, which option do you preferred?

Option 1: MN/SN initiated SN Release procedures

Option 2: MN/SN initiated SN Modification procedures

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 2 | Similar comment as CPA part, it is better to use the modification procedure to do the update, add new, and partial release. |

Question 20: for MN initiated inter-SN CPC, to cancel all of the prepared PSCells, do you agree to:

- use the MN/SN initiated SN Release procedures

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 21: for MN initiated inter-SN CPC, to inform from MN to the source SN about CPC cancel, do you agree to:

- introduce a new class2 XnAP procedure to inform CPC cancel

- enhance the agreed class2 X2AP CPAC notification procedure to inform CPC cancel

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes | Seems needed for the MN/T-SN initiated CPC cancel scenarios. |

### Replace and cancel for SN initiated inter-SN CPC

Question 22: do you agree that:

- the MN, the S-SN and the T-SN can trigger replace and cancel for SN initiated inter-SN CPC

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 23: for SN initiated inter-SN CPC, to update the configuration of the prepared PSCells, do you agree to use the following procedures:

- S-SN initiated: SN Change procedure + MN initiated SN Modification procedure

- MN initiated: MN initiated SN Modification procedure

- T-SN initiated: SN initiated SN Modification procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 24: for SN initiated inter-SN CPC, to add some PSCells as prepared by the T-SN, do you agree to:

- use T-SN initiated SN modification procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 25: for SN initiated inter-SN CPC, to cancel part of the prepared PSCells in a T-SN, triggered, which option do you preferred?

Option 1: SN release solution

- MN triggered: MN initiated SN Release procedure

- T-SN triggered: SN initiated SN Release procedure

- S-SN triggered: SN Change procedure + MN initiated SN Release procedure

Option 2: SN modification solution

- MN triggered: MN initiated SN Modification procedure

- T-SN triggered: SN initiated SN Modification procedure

- S-SN triggered: SN Change procedure + MN initiated SN Modification procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 2 | Similar comment as CPA part, it is better to use the modification procedure to do the update, add new, and partial release. |

Question 26: for SN initiated inter-SN CPC, to cancel a T-SN and in the meanwhile there are other prepared T-SNs not canceled, do you agree to use the following procedures?

- S-SN initiated: SN Change procedure + MN initiated SN Release procedure

- MN initiated: MN initiated SN Release procedure

- T-SN initiated: SN initiated SN Release procedure

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 27: in case of S-SN initiated CPC, to inform from MN to the source SN about CPC cancel, do you agree to:

- introduce a new class2 XnAP procedure to inform CPC cancel

- enhance the agreed class2 X2AP CPAC notification procedure to inform CPC cancel

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes | Seems needed for the MN/T-SN initiated CPC cancel scenarios. |

### Signalling design details

Question 28: do you agree to introduce a new indicator to indicate about the CPAC replace/cancel in the corresponding messages for CPAC replace/cancel.

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

Question 29: which option do you prefer to indicate that the SN release is due to cancellation of all PSCells in the SN Release procedures?

Option 1: CPAC cancel indicator

Option 2: introduce new cause value

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 1 | Seems option 1 is enough, no strong view on a new cause. |

## Data forwarding aspects

### Early Data Forwarding

The following agreement and WA were achieved in last RAN3 meeting:

- Support both PDCP SDU data forwarding and PDCP PDU data forwarding in early data forwarding.

- WA: Use the Early Status Transfer message to inform the discarding of forwarded PDCP PDU for both PDCP PDU data forwarding and PDCP SDU data forwarding.

Question 30: for PDCP SDU Forwarding and discarding, do you agree to:

- reusing the IEs within the First DL COUNT branch in the EARLY STATUS TRANSFER message.

- reusing the existing IEs in the DL Discarding branch in the EARLY STATUS TRANSFER message.

- extending the EARLY STATUS TRANSFER message to the following cases: from the source SN to the MN, and from the MN to the candidate SNs.

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Agree |  |

Question 31: for PDCP PDU Forwarding, do you agree that:

- the node hosting PDCP entity does not need to send the first DL count to the corresponding node.

- the node receiving the forwarded DL PDCP SDUs can forward the DL PDCP PDUs to other nodes in early data forwarding.

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Agree |  |

As discussed in [13], in R15, RAN3 introduced the discarded PDCP PDU SNs in the user plane for the flow control. Therefore there are two options to inform the discarding of PDCP PDU SNs.

- Option 1: user plane solution, i.e. reuse the DL USER DATA frame

- Option 2: control plane solution, i.e. the early status transfer message

In option 1, the current DL USER DATA frame can indicate the all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. Therefore it is only needed to add some descriptions in TS 37.340.

In option 2, the early data forwarding of several DRBs will be performed in CPAC. For some DRBs, the PDCP PDUs are forwarded. For some other DRBs, the PDCP SDUs are forwarded. Therefore option 2 can use the unified message to inform the discarding of all these forwarded DL data. But in MR-DC, the early status transfer message will first be sent to the CU-CP of the corresponding node, then the CU-CP of the corresponding node need forward the message to the DU. Currently there is no early status transfer message in F1. In this solution it is needed to introduce the Early Status Transfer procedure over F1 interface.

Question 32: for PDCP PDU Forwarding, select the solution to inform the discarding of DL PDCP PDU SNs:

-	Option 1: user plane solution, i.e. reuse the DL USER DATA frame

-	Option 2: control plane solution, i.e. the early status transfer message

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 1/2 | Both ok, slightly prefer Opt2 to align with DL PDCP SDU handling. |

### Late Data Forwarding

In [9], for CPA, the company prefers the option where MN initiates Late Data Forwarding upon receiving an Xn/X2 message from the target SN indicating that UE has successfully accessed the target PSCell. Upon UE successfully completing RACH on the target PSCell, target SN transmits an Xn/X2 message to MN (Class 2 procedure) indicating that UE has accessed the target PSCell.

Question 33: In MR-DC, for CPA, when to start late data forwarding.

- Option 1: upon receiving the RRC reconfiguration complete message from the UE

- Option 2: after the UE has successfully accessed the target PSCell, introduce a new class 2 procedure from T-SN to the MN to indicate that UE has accessed the target PSCell

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Option 1 |  |

## F1/E1 aspects

Two WAs were achieved in last RAN3 meeting:

- WA: Prepare one candidate PSCell in one CPAC procedure over F1 interface, same F1AP pair can be reused to prepare different candidate PScell for CPAC, reuse the existing IEs of R16 CHO and CPC. RAN3 only need to modify the procedure description.

- WA: For E1AP in all the CPAC cases, reuse the existing IEs and procedures of R16 CHO and CPC. RAN3 only need to modify the procedure description.

Question 34: do you agree to turn the WAs above to agreements？

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

In case control plane solution is selected in Question 32, there will be a need to introduce an F1AP: Early Status Transfer procedure to inform the discarding of DL PDCP PDU SNs.

Question 35: if control plane solution is selected in Question 32, do you agree to introduce an F1AP: Early Status Transfer procedure to inform the discarding of DL PDCP PDU SNs?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes |  |

## Others

### SCG activation/deactivation in CPAC

[16]: In Rel-17, SCG activation and deactivation is supported, and in CPA and MN initiated CPC, it’s seems straight forward that the MN can set the SCG related to the candidate PSCell activated or deactivated. In case of SN initiated inter-SN CPC, it could be worth clarifying the activation/deactivation state of SCGs related to the candidate target PSCell is determined by the source SN or by MN.

In SCG activation/deactivation discussion, RAN2 agreed that “the work will focus on the single deactivated SCG”, it could be understood that SCG deactivation with CPAC scenario is deprioritized.

Question 36: do you support to discuss SCG deactivation during CPAC procedures?

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | No | Seems not necessary to discuss CPAC with SCG deactivation, anyway, SCG activation/deactivation discussion is ongoing, it is preferred to postpone the discussion, to be discussed in the future if needed. |

### Others

If there is other topics need to be discussed, please elaborate below:

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-213238 (TP for CPAC BLCR to TS36.423) Data forwarding for EN-DC (ZTE)

- R3-213239 (TP for CPAC BLCR to TS38.423) Data forwarding for MR-DC with 5GC (ZTE)

- R3-213284 (TP for CPAC BL CR to TS 36.423 ) Discussion on CPAC (Conditional PSCell Addition and PSCell Change) (NEC)

- R3-213285 (TP for CPAC BL CR to TS 38.423 ) Discussion on CPAC (Conditional PSCell Addition and PSCell Change) (NEC)

- R3-213304 Discussion on Conditional PScell Addition/Change procedures (China Telecommunication)

- R3-213305 TP on 36.423 for introducing CPAC information in SgNB Addition procedure (China Telecommunication)

- R3-213306 TP on 38.423 for introducing CPAC information in SN Addition procedure (China Telecommunication)

- R3-213366 CPAC replace procedures (Qualcomm Incorporated)

- R3-213367 CPA and Inter-SN CPC: preparation, execution, and data forwarding (Qualcomm Incorporated)

- R3-213383 (TP to CPAC BL CR to 38.423, LTE_NR_DC_enh2-Core) Max number of PSCells, cancellation and modification of a CPAC (Nokia, Nokia Shanghai Bell)

- R3-213384 (TP to CPAC BL CR to 36.423, LTE_NR_DC_enh2-Core) Max number of PSCells, cancellation and modification of a CPAC (Nokia, Nokia Shanghai Bell)

- R3-213565 (TP to 37.340 CPAC BL CR) Support of inter-SN CPC (Huawei, CATT, Lenovo, Motorola Mobility)

- R3-213566 (TP to 38.423/36.423 CPAC BL CR) Support of Early data forwarding (Huawei)

- R3-213567 (TP to 38.473 38.463 38.401 CPAC BL CRs) CPAC in disaggregated architecture (Huawei)

- R3-213649 Open issues on conditional PScell Change/Addition (LG Electronics)

- R3-213718 Remaining issues on CPAC (Lenovo, Motorola Mobility)

- R3-213870 (TP for CPAC BL CR to TS 38.423) Support of SN-initiated CPC (Ericsson)

- R3-213871 (TP for CPAC BL CR to TS 38.423) CPC/CPA Cancellation (Ericsson)

- R3-213942 Remaining issues of Conditional PSCell Change/Addition (CATT)

- R3-213992 (TP to CPAC BLCR to 36.423) RRC Container content in Conditional PSCell Addition Information Acknowledge (Google Inc.)

- R3-213996 (TP to CPAC BLCR to 38.423) RRC Container content in Conditional PSCell Addition Information Acknowledge (Google Inc.)

- R3-214127 (TP for CPAC BL CR to TS 38.423) CPAC Cancellation (Samsung)

- R3-214128 (TP for CPAC BL CR to TS 36.423) CPAC Cancellation (Samsung)

- R3-213212 (TP for CPAC BL CR to TS36.423) Enhance Early Status Transfer message for CPA (ZTE)

- R3-213213 (TP for CPAC BL CR to TS38.423) Enhance Early Status Transfer message for CPA (ZTE)

- R3-213214 (TP for CPAC BL CR to TS 37.340) Clarification on Early Data Forwarding (ZTE)