3GPP TSG-RAN WG3 #114-e							                                              R3-215686

1-11 Nov 2021

Online

Agenda item:	10.2.4

Source:	CMCC

Title:	TP to SON BLCR 38.413 for inter-system load balancing

Document for:	Discussion

- Introduction

This contribution provides load reporting description for inter-system load balancing in TS 38.413.

# 2	Text proposal for TS 38.413

--------------------------------------------------Start of the change---------------------------------------------------

<Skip unchanged>

#### 9.3.3.34	Inter-system SON Information

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information | M |  |  |  |
| >Inter-system SON Information Report |  |  |  |  |
| >>Inter-system SON Information Report | M |  | 9.3.3.36 |  |
| >Inter-system SON Information Request |  |  |  |  |
| >>Inter-system SON Information Request | M |  | 9.3.3.xx |  |
| >Inter-system SON Information Reply |  |  |  |  |
| >>Inter-system SON Information Reply | M |  | 9.3.3.yy |  |

<Skip unchanged>

#### 9.3.3.36	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >>Inter-system HO Report | M |  | 9.3.3.40 |  |
| >Failure Indication Information |  |  |  |  |
| >>Inter-system Failure Indication | M |  | 9.3.3.38 |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication | M |  | 9.3.3.aa |  |
| >Inter-system Load Reporting Update Information |  |  |  |  |
| >>Inter-system Load Reporting Update | M |  | 9.3.3.a1 |  |

<Skip unchanged>

#### 9.3.3.xx 	Inter-system SON Information Request

This IE contains the request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Request | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Request | M |  | 9.3.3.zz |  |
| >Inter-system Load Reporting |  |  |  |  |
| >>Inter-system Load Reporting Request | M |  | 9.3.3.z1 |  |

<Skip unchanged>

#### 9.3.3.yy 	Inter-system SON Information Reply

This IE contains the reply information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Response | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Reply | M |  | 9.3.3.bb |  |
| >Inter-system Load Reporting |  |  |  |  |
| >>Inter-system Load Reporting Response | M |  | 9.3.3.b1 |  |

#### 9.3.3.z1 Inter-system Load Reporting Request

This IE contains the inter-system load reporting request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting RAT |  |  |  |  |
| > E-UTRAN |  |  |  |  |
| >> Cell To Report Item |  | 1 .. < maxnoofCellsineNB> |  |  |
| >>> Cell ID | M |  | 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| > NG-RAN |  |  |  |  |
| >> Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |
| >>> Cell ID | M |  | 9.3.1.73 | Contains the NG-RAN CGI IE. |
| CHOICE load reporting type | M |  |  |  |
| >Event based reporting |  |  |  |  |
| >>Number Of Measurement Reporting Levels | M |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale between the starting point and the reporting node's threshold for overload. The reporting node sends a report each time the cell load changes from one reporting level to another, when the cell load enters and exits overload state. |
| >Periodic Reporting |  |  |  |  |
| >>Reporting Periodicity | M |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting. |

#### 9.3.3.b1 Inter-system Load Reporting Response

This IE contains the inter-system load reporting response information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting RAT |  |  |  |  |
| > NG-RAN |  |  |  |  |
| >> Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |
| >>> Cell ID | M |  | 9.3.1.73 | Contains the NG-RAN CGI IE. |
| >>> Transmission Bandwidth | M |  |  | Contains the NR Transmission Bandwidth IE as defined in TS 38.423 [24]. |

#### 9.3.3.a1 Inter-system Load Reporting Update

This IE contains the inter-system load reporting update information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting RAT |  |  |  |  |
| >E-UTRAN |  |  |  |  |
| >> Cell to Report Item |  | 1 .. < maxnoofCellsineNB> |  |  |
| >>> Cell ID | M |  | 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| >>> Composite Available Capacity Group | O |  |  | Contains the Composite Available Capacity Group IE as defined in TS 36.423 [40]. |
| >>> Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 36.423 [40]. |
| >>> RRC Connections | O |  |  | Contains the RRC Connections IE as defined in TS 38.423 [24]. |
| >>> Number of Active UEs | O |  | INTEGER(0..16777215, ...) | As defined in TS 36.314. Value "1" is equivalent to 1 Active UEs, value "2" is equivalent to 2 Active UEs, value n is equivalent to n Active UEs. |
| >>> Number of NR capable Active UEs | O |  | INTEGER(0..16777215, ...) | Indicates the mean number of active UEs which also supports NR. Value "1" is equivalent to 1 NR capable Active UEs, value "2" is equivalent to 2 NR capable Active UEs, value n is equivalent to n NR capable Active UEs. |
| >NG-RAN |  |  |  |  |
| >> Cell to Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |
| >>> Cell ID | M |  | 9.3.1.73 | Contains the NG-RAN CGI IE. |
| >>> Composite Available Capacity Group | O |  |  | Contains the Composite Available Capacity Group IE as defined in TS 38.423 [24]. |
| >>> Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 38.423 [24]. |
| >>> RRC Connections | O |  |  | Contains the RRC Connections IE as defined in TS 38.423 [24]. |
| >>> Number of Active UEs | O |  | INTEGER(0..16777215, ...) | As defined in TS 38.314. Value "1" is equivalent to 0.1 Active UEs, value "2" is equivalent to 0.2 Active UEs, value n is equivalent to n/10 Active UEs. |

--------------------------------------------------End of the change-----------------------------------------------------