3GPP TSG-RAN WG3 #114-e							                                    R3-215974

1-11 Nov 2021

Agenda item:	22.2.2.

Source:			Samsung

Title:				(TP for BL CR for 38.473) Multicast Group Paging procedure in F1

Document for:	Discussions & Approval

# 1	Introduction

The discussion part is showed in R3-215508 with the below proposals.

Proposal 1: 	It is proposed RAN3 to agree UE Identity Index value to be used as UE identify for paging in NG group paging message for multicast session. The NG group paging message contains a list of UE Identity Index value, a common DRX and TMGI.

Proposal 2: 	It is proposed RAN3 to agree RAN group paging message includes a list of UE Identity Index value.a common DRX and TMGI.

Proposal 3: 	It is proposed RAN3 to agree the F1 group paging message is applied to RRC inactive mode and RRC idle mode UEs by including a list of UE Identity Index value, a common DRX and TMGI.

In this document, text proposal for F1 TS38.473 is proposed in section 2 against the BL CR for 38.473

# 2	Text Proposal

/*--------------------------------Start of the First Change----------------------------*/

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |

/*--------------------------------Omit unchanged part---------------------------*/

## 8.x	NR MBS Procedures

- 8.x.y	Multicast Group Paging

- 8.x.y.1	General

The purpose of the Multicast Group Paging procedure is used to provide the paging information to enable the gNB-DU to all involved UEs of the activation of an MBS Session. The procedure uses non-UE associated signalling.

Editor’s Note:	 procedure text is FFS

- 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node.

At the reception of the MULTICAST GROUP PAGING message, the he gNB-DU shall perform paging of the MBS Session identified by the MBS Session ID IE in cells which belong to the indicated Paging Cell List IE.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE according to TS 38.304 [24].

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

Editor’s Note:	 procedure text is FFS

- 8.x.y.3	Abnormal Conditions

Void.

/*--------------------------------Omit unchanged part---------------------------*/

#### 9.2.13.x	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | FFS |  | YES | reject |
| UE Identity List for Paging |  | 1 |  |  | YES | reject |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >> UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| Paging DRX (FFS) | M |  | 9.3.1.40 |  | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for paging. Value is FFS. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

/*--------------------------------End of the Changes-----------------------------*/