3GPP TSG-RAN WG3 #114-e							 R3-215811

1-11 Nov 2021

Online

Agenda Item:	8.1

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # 8_MBSPaging

Document for:	Approval

# Introduction

CB: # 8_MBSPaging

- RAN3 confirm RAN2 understanding? Stage3 TP?

- Reply LS to RAN2

(ZTE - moderator)

[NWM] Summary of offline disc R3-215811

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion- Second round [if needed]

<TBD>

# Discussion-First round

## General

RAN2 has sent an LS R3-214692 [1] to RAN3.

1. Overall Description: RAN2 considered two options for paging for multicast session activation notification for RRC_IDLE/ RRC_INACTIVE UEs as - Option 1: Paging for multicast session activation notification is used in all legacy Paging Occasions (POs). - Option 2: Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s). Further, RAN2 understands that option 2 is paging resource efficient and has made agreement for option 2, subject to RAN3 confirmation. 2. Actions: To RAN3, SA2 RAN2 respectfully asks RAN3 and SA2 to take above information into account in future work and provide feedback if needed.

We also received reply LS [2] from SA2, as below

SA2 would like to thank RAN2 for the LS on paging for multicast session activation notification and would like to provide the following feedback. RAN2 considered two options for paging for multicast session activation notification for RRC_IDLE/ RRC_INACTIVE UEs as - Option 1: Paging for multicast session activation notification is used in all legacy Paging Occasions (POs). - Option 2: Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s). RAN2 respectfully asks RAN3 and SA2 to take above information into account in future work and provide feedback if needed.. SA2 confirms that both Option 1 and Option 2 are feasible from SA2 perspective.

In this email discussion, RAN3 shall discuss and then decide:

- Whether option 1 and/or option 2 are feasible from RAN3 perspective

- If option 2 is feasible and agreed to satisfy RAN2 preference, RAN3 shall further provide interface signaling correspondingly.

## R3-215192: On RAN2 Group Paging Assumptions for NR MBS

From all of contributions related to this issue, only the contribution [7] provides different understanding from others, so we shall firstly discuss it.

Observation 1: For each MBS Session Activation occasion, the respective processes would need to create Group Paging Information (basically list of UEs or derived lists) by examining a potentially large data base more than once (AMFs and SMFs, for RRC_INACTIVE also within the gNB). The creation of such information is not always possible to be performed in advance to MBS Session Activation. Observation 2: The computational effort grows with the size of the related data bases (PDU Sessions/UE Contexts). Observation 3: The significance of the computational effort and the related timing aspect is given by the fact that the current scheme does not - always - allow to create and maintain Group Paging Information in advance to MBS Session Activation. Observation 4: Paging Resource efficiency, assumed by RAN2 to be an important design criterion for the design of the overall MBS Group Paging function may require Paging Optimisations to be taken into account, at least in their most basic form, e.g. first paging attempt only within the last serving cell. Observation 5: An obvious consequence of aiming at paging resource efficiency is the overall 5GS responsiveness, i.e. the delay between the (application) trigger at Session Activation and all of the joined UEs able to start receiving MC user data. Observation 6: There is obviously a relation between the computational effort spent for creating Group Paging Information, the way how Paging Optimisation schemes are applied to achieve paging resource efficiency and the responsiveness of MBS Group Paging. Observation 7: There seems to be a contradiction between statements received from RAN2 (R2-2104655) and the LS received for this meeting in R2-2108914. Option 2 will end up in option 1 if the number of UEs exceeds a certain limit. Observation 8: Minimise paging resource usage for IDLE and INACTIVE UEs and combining paging information from several connected AMFs adds further complexity/challenges to the gNB. RAN3 could consider to combine Session Activation and Group Paging on NG.

Observation r1: The RAN2 decision to rather utilise UE specific POs result in a non-negligible processing effort and paging resource usage on the network side which contradicts the aim to enable paging resource efficiency, as communicated by RAN2. Observation r2: Current distribution of information in the 5GS system (RAN/AMF/SMF) requires examining (PDU/UE) context databases several times (i.e. more than once) w/o the possibility to pre-collect and maintain Group Paging Related information for immediate usage at MBS Session Activation. Observation r3: Processing Group Paging Information associated with a list of UE identifiers Current distribution should be limited in case the number of UE IDs contains is “uncomfortably” large. Escalation strategies in terms of Paging Area or PO usage are expected to be meet a well-balanced trade-off between processing effort and resource usage. Observation r4: Applying Paging Optimisations introduced for UE individual paging may also be applied for multicast group paging, if the size of the group is kept within a reasonable limit.

In above observations, if this/these observations are related to RAN2/SA2, the problems/issues raised by them seem to not make sense, because both RAN2 and SA2 have discussed them then send LS to RAN3 including positive result.

Question1: Do companies agree with the following view?

- Which of above observations are RAN3 related observations, do you agree with it/them?

- The other observations related to RAN2/SA2 seems to not make sense.

| Company | Observation 1/2/… | Comment |
| --- | --- | --- |

Proposal 1: Communicate Observations r1 and r2 to RAN2 and SA2. Proposal 2: Allow several strategies to optimise Multicast Group Paging along different aspects (mainly responsiveness as a result of signalling/processing effort versus paging resource usage). Proposal 3: Overall, limit the size of the NG/Xn Multicast Group Paging message if the Group Paging Information is associated a list of UE (Paging) Identities. Proposal 4: Reconsider combining Group Paging and session activation on NG to allow combining RRC_IDLE and RRC_INACTIVE paging. Final Proposal: Agree on the draft Reply LS in R3-215193

Question 2: Do companies agree with the following view?

- Which of above proposals are RAN3 related observations, do you agree with it/them?

- The other proposals related to RAN2/SA2 seems to not make sense

| Company | Proposal 1/2… | Comment |
| --- | --- | --- |

## Which option is feasible and preferred in RAN3?

According to the RAN2 LS, two options have been discussed and Option 2 is preferred by RAN2. However, it is RAN3 to decide which option shall be used for Multicast group paging.

Question 3: From RAN3 point of view, which option is feasible and preferred? Can RAN3 follow RAN2’s preference as option2?

| Option 1: Paging for multicast session activation notification is used in all legacy Paging Occasions (POs). Option 2: Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s). Option 3: Other. If option 1/2 are not feasible, please input your suggestion |  |  |
| --- | --- | --- |
| Company | Option 1/2/3 | Comment |

## Assume to agree with Option 2

Option 2: Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s).

### UE identify within multicast group paging message

Some papers suggest to introduce “UE Identity Index value” as UE identify to save NG signaling overhead.

In [9], it states that according to TS38.304, PO calculation for paging notification uses 5G-S-TMSI mod 1024. Therefore, 5G-S-TMSI mod 1024 used as UE identify in NG group paging message is sufficient. After modular operation, UE identify can be same for the different UEs. It is 10 bits long identify. Compare to 5G-S-TMSI (48 bits) for each UE, the message size is reduced a lot.

Question 4: Which kind of UE identify do companies prefer in the Multicast group paging message?

Option 1：UE Paging Identity (i.e. 5G-S-TMSI, defined in 9.3.3.18 in TS38.413)

Option 2：UE Identity Index value (i.e., 5G-S-TMSI mod 1024 defined in 9.3.3.23 in TS38.413)

Option 3:   Other. If any, please input your suggestion

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |

### Paging DRX

In legacy, paging DRX is UE specific for unicast paging. However, in [9], it suggests to use common DRX value for multicast group paging, which can be configured to the gNB and all the relevant UEs can receive the group paging message on the POs calculated from this common DRX.

Question 5: Which kind of Paging DRX do companies prefer in the Multicast group paging message?

Option 1：Common Paging DRX

Option 2：UE specific Paging DRX, as legacy

Option 3:   Other. If any, please input your suggestion

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |

### Paging Area

In legacy, paging area (defined as TA list) is UE specific for unicast paging. However, in some papers, it is suggested to use a common paging area (i.e., multicast paging area, also defined as TA list) for multicast group.

Question 6: Which kind of Paging Area do companies prefer in the Multicast group paging message?

Option 1：UE specific Paging Area, as legacy, defined as TA list

Option 2：Multicast Paging Area, for all paging UEs, defined as TA list

Option 3:   Other. If any, please input your suggestion

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |

### Other IEs in Multicast group paging message

Many papers suggest to include MBS Service Area. It may be defined as TA list and/or cell list, however, the definition of MBS Service Area will be decided in other group.

Meanwhile, another IE, i.e., Paging Priority is also suggested to be included.

Question 7: Do companies agree with the following IEs in the Multicast group paging message?

- MBS Service Area, its definition is FFS

- Paging Priority, defined in 9.3.1.78, as legacy

| Company | 1: MBS Service Area 2: Paging Priority | Comment |
| --- | --- | --- |

### Left issues in RAN Multicast Group Paging

In [4], the “UE RAN Paging Identity” (i.e., I-RNTI full) can also be used as legacy for RAN multicast group paging.

#### 9.2.3.43 UE RAN Paging Identity The IE defines the UE Identity for RAN paging a UE in RRC_INACTIVE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE UE RAN Paging Identity; M. >I-RNTI full. >>I-RNTI full; M; BIT STRING (SIZE (40)).

Question 8: Do companies agree with “UE RAN Paging Identity” in the RAN Multicast group paging message?

| Company | YES/NO | Comment |
| --- | --- | --- |

### F1AP impact for Multicast Group Paging

It can be aligned with NGAP and XnAP, and be discussed in the second round.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-214692	LS on paging for multicast session activation notification (RAN2)	LS in

- R3-215798	Reply LS on paging for multicast session activation notification (SA2)	LS in

- R3-214753	(TP for BL CR for 38.413) Multicast group paging (ZTE)	other

- R3-214754	(TP for BL CR for 38.423) Multicast group paging (ZTE)	other

- R3-214755	[DRAFT] Reply LS on paging for multicast session activation notification (ZTE)	LS out To: RAN2 CC: SA2

- R3-214969	Response LS on paging for multicast session activation notification  (Nokia, Nokia Shanghai Bell)	LS out To: RAN2 CC:

- R3-215192	On RAN2 Group Paging Assumptions for NR MBS (Ericsson)	discussion

- R3-215193	[DRAFT] Reply LS on paging for multicast session activation notification (Ericsson)	LS out To: RAN2, SA2 CC:

- R3-215508	Discussion on paging for multicast session activation notification (Samsung)	discussion

- R3-215509	[Draft] Reply LS on paging for multicast session activation notification (Samsung)	LS out To: RAN2 CC: SA2

- R3-215571	Consideration on paging for multicast session activation notification (Huawei)	discussion

- R3-215572	[DRAFT] Reply LS on paging for multicast session activation notification (Huawei)	LS out To: RAN2, SA2, CC:

- R3-214787	(TP for 38.413) Stage 3 for MBS Group Paging  (Nokia, Nokia Shanghai Bell)	Other

- R3-215271	(TP to TS 38.423 BL CR) RAN Multicast Group Paging (Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated)	Other

- R3-215294	Discussion on multicast group paging (LG Electronics)	Discussion

- R3-215295	(TP for NR_MBS BL CR for TS 38.413) Discussion on multicast group paging (LG Electronics)	Other

- R3-215296	(TP for NR_MBS BL CR for TS 38.470) Discussion on multicast group paging (LG Electronics)	Other

- R3-215300	(TP for NR_MBS BL CR for TS 38.473) Discussion on multicast group paging (LG Electronics)	Other

- R3-215512	(TP for BL CR for 38.413) Multicast Group Paging procedure in NG (Samsung)	Other

- R3-215513	(TP for BL CR for 38.423) Multicast Group Paging procedure in Xn (Samsung)	Other

- R3-215514	(TP for BL CR for 38.473) Multicast Group Paging procedure in F1 (Samsung)	Other