3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220425

E-meeting, 17-26 Jan 2022

Title: 	(TPs to RA-SDT BL CRs of TS 38.423, 38.425) RACH based SDT without anchor relocation

Source: 	Huawei

Agenda item:	24.2

Document Type:	other

# 1. Introduction

Based on the discussion in [1], in this contribution, we provide the corresponding TPs to RA-SDT BL CRs of TS 38.423, 38.425 in section 2 and 3.

[1] R3-220424 (TPs to RA-SDT BL CRs of TS 38.300, 38.420) RACH based SDT without anchor relocation, Huawei

# 2. TP to TS 38.423 BL CR

---------Start of the First Change---------

## 8.1	Elementary procedures

//skip unchanged part

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SDT Context Transfer | SDT CONTEXT TRANSFER |

---------Start of the Next Change---------

### 8.2.4	Retrieve UE Context

//skip unchanged part

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

If the XnAP UE association was established, the old NG-RAN node shall, if supported, include the Old NG-RAN node UE XnAP ID reference IE in the RETRIEVE UE CONTEXT FAILURE message, to inform the new NG-RAN node to release the UE-associated logical Xn-connection.

---------Start of the Next Change---------

### 8.2.6	XN-U Address Indication

#### 8.2.6.1	General

For the retrieval of a UE context, the Xn-U Address Indication procedure is used to provide forwarding addresses from the new NG-RAN node to the old NG-RAN node for all PDU session resources successfully established at the new NG-RAN node for which forwarding was requested.

For MR-DC with 5GC, the Xn-U Address Indication procedure is used to provide data forwarding related information, and Xn-U bearer address information for completion of setup of SN terminated bearers from the M-NG-RAN node to the S-NG-RAN node as specified in TS 37.340 [8],

For RACH based SDT without anchor relocation case, the Xn-U Address Indication procedure is used to provide data forwarding address from the new NG-RAN node to the old NG-RAN node for SDT DRBs for which forwarding was requested.

The procedure uses UE-associated signalling.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Xn-U Address Indication, successful operation for UE context retrieval

Figure 8.2.6.2-2: Xn-U Address Indication, successful operation for MR-DC with 5GC

UE Context Retrieval

The Xn-U Address Indication procedure is initiated by the new NG-RAN node. Sending the XN-U ADDRESS INDICATION message, the new NG-RAN node informs the old NG-RAN node of successfully established PDU Session Resource contexts to which user data pending at the old NG-RAN node can be forwarded.

The new NG-RAN node may include Secondary Data Forwarding Info from target NG-RAN node List IE for an additional Xn-U tunnel for data forwarding.

Upon reception of the XN-U ADDRESS INDICATION message, the old NG-RAN node should forward pending user data to the indicated TNL addresses.

MR-DC with 5GC

The Xn-U Address Indication procedure is initiated by the M-NG-RAN node.

Upon reception of the XN-U ADDRESS INDICATION message, in case of data forwarding, the S-NG-RAN node should forward pending DL user data to the indicated TNL addresses; in case of completion of Xn-U bearer establishment for SN terminated bearers, the S-NG-RAN node may start delivery of user data to the indicated TNL address, and shall, if supported, use the received QoS Mapping Information IE within the DRBs to Be Setup List IE in the PDU Session Resource Setup Complete Info – SN terminated IE to set DSCP and/or flow label fields for the delivery of user data to the indicated TNL address.

If the XN-U ADDRESS INDICATION message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

If the XN-U ADDRESS INDICATION message includes the CHO MR-DC Indicator IE, the S-NG-RAN node shall, if supported, consider that the XN-U ADDRESS INDICATION message concerns a Conditional Handover, and act as specified in TS 37.340 [8].

If the XN-U ADDRESS INDICATION message includes the CHO MR-DC Early Data Forwarding Indicator IE set to "stop", the S-NG-RAN node shall, if supported and if already initiated, stop early data forwarding for the provided Data Forwarding Address information.

RACH based SDT without anchor relocation

The Xn-U Address Indication procedure is initiated by the new NG-RAN node. Sending the XN-U ADDRESS INDICATION message, the new NG-RAN node informs the old NG-RAN node the SDT DRBs to which downlink user data at the old NG-RAN node can be forwarded.

#### 8.2.6.3	Unsuccessful Operation

Not applicable.

#### 8.2.6.4	Abnormal Conditions

Void.

---------Start of the Next Change---------

### 8.2.xx	SDT Context Transfer

#### 8.2.xx.1	General

The purpose of the SDT Context Transfer procedure is for the old NG-RAN node to send the SDT context information and the UL data forwarding information to the new NG-RAN node in case of RACH based SDT without anchor relocation.

The procedure uses UE-associated signalling.

#### 8.2.xx.2	Successful Operation

Figure 8.2.xx.2-1: UE Information Transfer

The old NG-RAN node initiates the procedure by sending a SDT CONTEXT TRANSFER message to the new NG-RAN node.

If the UE-associated logical NG-connection is not established, the old NG-RAN node shall allocate a unique Old NG-RAN node UE XnAP ID reference to be used for the UE and include that in the SDT CONTEXT TRANSFER message; by receiving the Old NG-RAN node UE XnAP ID reference IE in the SDT CONTEXT TRANSFER message, the new NG-RAN node establishes the UE-associated logical Xn-connection.

If the SDT CONTEXT TRANSFER message includes the DL Forwarding IE in the SDT Data Forwarding Info from Last Serving NG-RAN node IE, the new NG-RAN node shall, if supported, act as specified in TS38.300 [9].

#### 8.2.xx.3	Abnormal Conditions

Void.

---------Start of the Next Change---------

### 8.3.9	RRC Transfer

#### 8.3.9.1	General

The purpose of the RRC Transfer procedure is to deliver a PDCP-C PDU encapsulating an LTE RRC message or NR RRC message to the S-NG-RAN-NODE that it may then be forwarded to the UE, or from the S-NG-RAN-NODE, if it was received from the UE. The delivery status may also be provided from the S-NG-RAN-NODE to the M-NG-RAN-NODE using the RRC Transfer.

The procedure is also used to enable transfer one of the following messages from the M-NG-RAN-NODE to the S-NG-RAN-NODE, when received from the UE:

-	the NR RRC message container with the NR measurements;

-	the E-UTRA RRC message container with the E-UTRA measurements;

-	the NR RRC message container with the NR failure information;

-	the NR RRC message container with the RRCReconfigurationComplete message;

-	the NR RRC message container with the UE assistance information.

The procedure is also used to enable transfer one of the following messages between the new NG-RAN node and the old NG-RAN node:

-	the NR RRC message container with the ULInformationTransfer message;

-	the NR RRC message container with the DLInformationTransfer message;

The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: RRC Transfer procedure, successful operation.

Figure 8.3.9.2-x: RRC Transfer procedure in SDT, successful operation.

In case of DC

The M-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the S-NG-RAN-NODE or the S-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the M-NG-RAN-NODE.

If the S-NG-RAN-NODE receives an RRC TRANSFER message which does not include the RRC Container IE in the Split SRB IE, or the RRC Container IE in the NR UE Report IE, or the the RRC Container IE in the Fast MCG Recovery via SRB3 from MN to SN IE, or the the RRC Container IE in the Fast MCG Recovery via SRB3 from SN to MN IE, it shall ignore the message. If the S-NG-RAN-NODE receives an RRC TRANSFER message with the Delivery Status IE in the Split SRB IE, it shall ignore the message. If the S-NG-RAN-NODE receives the RRC Container IE in the Split SRB IE, it shall deliver the contained PDCP-C PDU encapsulating an RRC message to the UE. If the S-NG-RAN-NODE receives the RRC Container IE in the Fast MCG Recovery via SRB3 from MN to SN IE, the S-NG-RAN-NODE shall deliver the contained RRC container encapsulating an RRC message to the UE.

If the M-NG-RAN-NODE receives the Delivery Status IE in the Split SRB IE, the M-NG-RAN-NODE shall consider RRC messages up to the indicated NR PDCP SN as having been successfully delivered to UE by S-NG-RAN-NODE. If the M-NG-RAN-NODE receives the RRC Container IE in the Fast MCG Recovery via SRB3 from SN to MN IE, the M-NG-RAN-NODE shall consider MCG link failure detected at the UE as specified in TS 37.340 [8].

In case of RACH based SDT without anchor relocation

The new NG-RAN node initiates the procedure by sending the RRC TRANSFER message to the old NG-RAN node or the old NG-RAN node initiates the procedure by sending the RRC TRANSFER message to the new NG-RAN node.

If the new NG-RAN node receives the RRC Container IE in the SDT via SRB between New NG-RAN node and Old NG-RAN node IE, it shall deliver the contained PDCP-C PDU encapsulating an RRC message to the UE. If the old NG-RAN-NODE receives the RRC Container IE in the SDT via SRB between New NG-RAN node and Old NG-RAN node IE, it shall consider the contained PDCP-C PDU encapsulating an RRC message including NAS message from the UE.

---------Start of the Next Change---------

#### 9.1.1.10	RETRIEVE UE CONTEXT FAILURE

This message is sent by the old NG-RAN node to inform the new NG-RAN node that the Retrieve UE Context procedure has failed.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node To New NG-RAN node Resume Container | O |  | OCTET STRING | Includes either the RRCRelease message as defined in TS 38.331 [10], or the RRCConnectionRelease message as defined in TS 36.331 [14], encapsulated in a PDCP-C PDU. | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node, in case of RACH based SDT without anchor relocation. | YES | ignore |

#### 9.1.1.11	XN-U ADDRESS INDICATION

This message is either sent by the new NG-RAN node to transfer data forwarding information to the old NG-RAN node, or by the M-NG-RAN node to provide either data forwarding or Xn-U bearer address related information for SN terminated bearers to the S-NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node, M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| Xn-U Address Information per PDU Session Resources List |  | 1 |  |  | YES | reject |
| >Xn-U Address Information per PDU Session Resources Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Data Forwarding Info from target NG-RAN node | O |  | Data Forwarding Info from target NG-RAN node 9.2.1.16 |  | – |  |
| >>Secondary Data Forwarding Info from target NG-RAN node List | O |  | 9.2.1.31 | This IE would be present only when the target M-NG-RAN node decide to split a PDU session between MN and SN | YES | ignore |
| >>PDU Session Resource Setup Complete Info – SN terminated | O |  | 9.2.1.30 |  | – |  |
| >>DRB IDs taken into use | O |  | DRB List 9.2.1.29 | Indicating the DRB IDs taken into use by the target NG-RAN node, as specified in TS 37.340 [8]. | YES | reject |
| CHO MR-DC Indicator | O |  | ENUMERATED (true, ...) | Indicating that the XN-U ADDRESS INDICATION message is for Conditional Handover, as specified in TS 37.340 [8]. | YES | reject |
| CHO MR-DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| SDT Data Forwarding Info from New NG-RAN node | O |  | SDT Data Forwarding Info from New NG-RAN node 9.2.3.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSsessions | Maximum no. of PDU sessions. Value is 256 |

---------Start of the Next Change---------

#### 9.1.1.xx	SDT CONTEXT TRANSFER

This message is sent by the old NG-RAN node to transfer SDT Context of the UE, and data forwarding information to the new NG-RAN node.

Direction: Old NG-RAN node  New NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| RLC Bearer Configuration | M |  | OCTET STRING | includes the RLC-BearerConfig IE as defined in subclause 6.3.2 of TS 38.331 [10] | YES | ignore |
| SDT DRBs To Be Setup List |  | 1 |  |  | Yes | ignore |
| > SDT DRBs To Be Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>DRB ID | M |  | 9.2.3.33 |  | - | - |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | - | - |
| >>S-NSSAI | M |  | 9.2.3.21 |  | - | - |
| >>RLC Mode | M |  | 9.2.3.28 |  | - | - |
| >>PDCP SN Length | M |  | 9.2.3.63 |  | - | - |
| >>UL Xn UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 |  | - | - |
| >>DL Forwarding | O |  | DL Forwarding 9.2.3.34 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

---------Start of the Next Change---------

#### 9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message or from the S-NG-RAN-NODE to the M-NG-RAN-NODE to report the DL RRC message delivery status.

For RACH based SDT without anchor relocation, this message is sent by the new NG-RAN node to the old NG-RAN node or by the old NG-RAN node to the new NG-RAN node, to transfer an RRC message for the NAS SDT.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node.

Direction: New NG-RAN node  Old NG-RAN node or Old NG-RAN node  New NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| Fast MCG Recovery via SRB3 from MN to SN |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | For NR-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the RRCReconfiguration message, or the RRCRelease message, or the MobilityFromNRCommand message. For NGEN-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the RRCConnectionReconfiguration message, or the RRCConnectionRelease message, or the MobilityFromEUTRACommand message. | – |  |
| SDT via SRB between New NG-RAN node and Old NG-RAN node |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | For SDT without anchor relocation, includes the UL-DCCH-Message containing the ULInformationTransfer message or the DL-DCCH-Message containing the DLInformationTransfer. Message as defined in subclause 6.2.1 of TS 38.331 [10]. | – | - |

---------Start of the Next Change---------

#### 9.2.3.61	RRC Resume Cause

The purpose of the RRC Resume Cause IE is to indicate to the old NG-RAN node the reason for the RRC Connection Resume as received from the UE in the ResumeCause defined in TS 36.331 [14] and TS 38.331 [10]. In this version of the specification, this is limited to the case of RNA update, and SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Resume Cause | M |  | ENUMERATED (rna-Update, mo-data, mo-signalling, ...) |  |

---------Start of the Next Change---------

#### 9.2.3.xxx	SDT Data Forwarding Info from New NG-RAN node

This IE indicates the SDT Data Forwarding Info from the new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SDT DRBs To Be Setup List |  | 1 |  |  |
| > SDT DRBs To Be Setup Item |  | 1..<maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>DL Xn UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

---------Start of the Next Change---------

//Asn.1 part to be added during the meeting if solution 4 is adopted by the group.

---------End of the Changes---------

# 3. TP to TS 38.425 BL CR

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the NR PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

| Bits |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |  |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. |  | Assistance Info. Ind. |  | UL Delay Ind. |  | DL Delay Ind. |  | 1 |  |
| Spare |  |  |  |  |  |  |  |  | UL Volume Ind. | PDCP Duplication Activation Suggestion |  | 1 |  |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |  |
| Assistance Information Type |  |  |  |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |  |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  |  |  |  | 0 or 4 |  |
| DL Delay DU Result |  |  |  |  |  |  |  |  |  |  |  | 0 or 4 |  |
| UL Available Data Volume |  |  |  |  |  |  |  |  |  |  |  | 0 or 4 |  |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

********************************

Skip to the next change

********************************

#### 5.5.3.xx	UL Volume Indicator

Description: This parameter indicates the presence of the UL available Data Volume.

Value range: {0= UL Available Data Volume not present, 1= UL Available Data Volume present}.

Field length: 1 bit.

#### 5.5.3.yy	UL Available Data Volume

Description: This field indicates UL available data volume as whole in the Buffer Status Report from UE over Uu interface. It is encoded as an Unsigned32 binary integer value. Value 0 indicates empty Buffer Status Report with buffer size set to 0 and can be used for terminating the SDT procedure.

Value range: {0..232-1}.

Field length: 4 octets.

********************************

Skip to the next change

********************************