3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220453

Online, 17th -26th  January 2022

Agenda item:	19.2.1

Source: 	Ericsson

Title: 	Discussion on signalling UE TX TEG association

Document for:	Discussion and Approval

# 1. Introduction

RAN3 received a LS from RAN1 on the UE Transmission Timing Error Group (Tx TEG) reporting [1], which is a new Rel-17 feature agreed by RAN1 for mitigating the impact of UE TX timing errors for UL TDOA/Multi-RTT positioning techniques. The LS in [1] mentions the following agreements:

Agreement: Confirm and modify the working assumption with the following modifications: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - Note: For mitigating UE Tx timing errors when both UL-TDOA and Multi-RTT, or UL-TDOA and DL-TDOA are used, the UE should provide the association information of UL SRS resources for positioning with Tx TEGs, subject to UE capability (in the bullets above): - to the serving gNB if a request to provide the association information is received from the gNB - to the LMF if a request to provide the association information is received from the LMF. - FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.

The RAN1 LS on agreed PHY layer parameters [2] also presents an exhaustive list of RAN1 agreements related to Tx TEG reporting. We list below the ones related to RAN3:

| ueTxTEG | New | A UE Tx TEG is associated with the transmissions of one or more UL positioning SRS resources. ueTxTEG may be sent from UE to LMF for supporting UL-TDOA or multi-RTT. | N/A | FFS for RAN2/RAN3 | Agreement Confirm and modify the working assumption with the following modifications: • For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. o The serving gNB should forward the association information provided by the UE to the LMF. o UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. • For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. o UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. • Note: For mitigating UE Tx timing errors when both UL-TDOA and Multi-RTT, or UL-TDOA and DL-TDOA are used, the UE should provide the association information of UL SRS resources for positioning with Tx TEGs, subject to UE capability (in the bullets above): o to the serving gNB if a request to provide the association information is received from the gNB to the LMF if a request to provide the association information is received from the LMF |
| --- | --- | --- | --- | --- | --- |
| ueTxTEG-ID | New | The ID of a UE Tx timing error group. One UE Tx TEG ID can be associated with one or more UL positioning SRS resource IDs. | FFS | FFS for RAN2/RAN3 |  |
| [srs-PosResourceSetId] | Existing | FFS: whether there is a need to include the positioning SRS resource set ID in ueTxTEG |  | FFS for RAN2/RAN3 |  |
| [srs-PosResourceId] | Existing | Positioning SRS resource ID |  | FFS for RAN2/RAN3 | FFS: the maximum number of positioning SRS Resources |
| trpRxTEG-ID | New | The ID of a TRP Rx timing error group, which is sent with RTOA measurements from gNB to LMF. | FFS | FFS for RAN3 |  |
| trpTxTEG | New | A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources. trpTxTEG may be sent from gNB to LMF for supporting DL-TDOA or multi-RTT. |  | FFS for RAN3 |  |
| trpTxTEG-ID | New | The ID of a TRP Tx timing error group. One TRP Tx TEG ID can be associated with one or more DL PRS resources | FFS | FFS for RAN3 |  |
| NR-DL-PRS-ResourceSetID | Existing | NR DL PRS ResourceSetID |  | FFS for RAN3 |  |
| NR-DL-PRS-ResourceID | Existing | •NR DL PRS ResourceID |  | FFS for RAN3 |  |
| trpRxTxTEG-ID-group | New | A gNB may report a trpRxTxTEG-ID-group with a TRP Rx-Tx measurement to LMF. The trpRxTxTEG-ID-group can be one of the following combinations of the TEG IDs: • An TRP RxTx TEG ID • A pair of TRP {RxTx TEG ID, Tx TEG ID} • A pair of TRP {Rx TEG ID, Tx TEG ID} • FFS: A triplet of TRP {RxTx TEG ID, Rx TEG ID, Tx TEG ID} | FFS | FS for RAN3 | Assuming the similar agreement as UE side will be made in the next meetAgreement: • For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB can support either or both of the following options: • Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID • Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID • Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement. • If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the gNB also reports the association of the TRP Tx TEG ID to the DL PRS resource(s) to the LMF under the condition that the TRP has more than one DL PRS resource. • FFS: how the association of the Tx TEG ID to the DL PRS resource(s) is determined by the TRP and how the association is reported to the LMF. • FFS: details of the signallinging |
| trpRxTxTEG-ID | New | The ID of the TRP RxTx timing error group. | FFS | FFS for RAN3 |  |
| trpTxTEG-ID | New | The ID of a TRP Tx timing error group. | FFS | FFS for RAN3 |  |
| trpRxTEG-ID | New | The ID of a TRP Rx timing error group. | FFS | FFS for RAN3 |  |
| [srs-PosResourceSetId] | New | The ID of a positioning SRS resource set. FFS: whether there is a need to include positioning SRS resource set ID. | FFS | FFS for RAN3 | Agreement: • Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF |
| srs-PosResourceId | New | The ID of a positioning SRS resource reported with RTOA measurement | FFS | FFS for RAN3 | Agreement: • Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF |
| [maxNumOfTRPRxTEG] | New | The maximum number of TRP-RxTEG per TRP | FFS | FFS for RAN3 |  |
| [maxNumOfTRPTxTEG ] | New | The maximum number of TRP-TxTEG per TRP | FFS | FFS for RAN3 |  |
| [maxNumOfTRPRxTxTEG] | New | The Max number of TRP RxTxTEG per TRP | FFS | FFS for RAN3 |  |
| numOfTRPRxTEG-PerPRSResource_RTOA | New | The number of different TRP Rx TEGs that the LMF requests a TRP to measure the same UL positioning SRS resource of a UE for RTOA measurements | [2, 3, 4, 6, 8] | FFS for RAN3 | Agreement: • Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. o M = [2, 3, 4, 6, 8] (FFS: other values) applies to all configured SRS resources for positioning o Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP Rx TEGs to measure the same SRS resources for positioning o FFS: details of the signalling, procedures o The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different |
| numOfTRPRxTEG-PerPRSResource_gNBRxTx | New | The number of different TRP Rx TEGs that the LMF requests a TRP to measure the same UL positioning SRS resource of a UE for gNB Rx-Tx measurements | [2, 3, 4, 6, 8] | FFS for RAN3 | Agreement: • Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. o M = [2, 3, 4, 6, 8] (FFS: other values) applies to all configured SRS resources for positioning o Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP Rx TEGs to measure the same SRS resources for positioning o FFS: details of the signalling, procedures o The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different |
| numOfTRPRxTxTEG-PerPRSResource_gNBRxTx | New | The number of different TRP RxTx TEGs that the LMF requests a TRP to measure the same UL positioning SRS resource of a UE for gNB Rx-Tx measurements | [2, 3, 4, 6, 8] | FFS for RAN3 | Agreement: Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP RxTx TEGs with the same TRP Tx TEG and report the corresponding multiple gNB Rx-Tx time difference measurements. M = [2, 3, 4, 6, 8] applies to all configured SRS resources. Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP RxTx TEGs to measure the same SRS resources FFS: details of the signalling, procedures The timestamps of the multiple gNB Rx-Tx time difference measurements in the same measurement report can be the same or different. |
| TRPTxTEG_Request_DL-TDOA | New | The parameter is used by a LMF to request a TRP to provide TRP Tx TEG association for DL-TDOA | FFS | FFS for RAN3 | Agreement: Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA • Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs • Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs |
| TRPRxTEG_Request_UL-TDOA | New | The parameter is used by a LMF to request a TRP to provide TRP Rx TEG association for UL-TDOA | FFS | FFS for RAN3 | Agreement: Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA • Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs |
| TRPRxTxTEG-ID-Request | New | The parameter is used by a LMF to request a gNB to provide TRP RxTxTEG-ID-group information for DL+UL positioning. | FFS | FFS for RAN3 |  |
| MeasPosSRSwithDiffRxTEGs_Request | New | The parameter is used by a LMF to request a TRP to measure the same UL positioning SRS resource with different UE Rx TEGs | FFS | FFS for RAN3 | Agreement: Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements • M = [2, 3, 4, 6, 8] (FFS: other values) • FFS: details of the signalling, procedures |
| Timestamp of a TRP measurement instance | New | The timestamp of a measurement instance. One TRP measurement report may contain multiple measurement instances of the same or different types of the measurements. | FFS | FFS for RAN3 | Agreement: Support enabling • A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and • A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and • Each measurement instance is reported with its own timestamp o FFS: The measurement instances are within a [configured] measurement time window • FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set o FFS: N (including N=1) • FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions o FFS: M (including M=1) • FFS: details of signalling, procedures, and UE capability if any • FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric • Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). • Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either. |
| numOfSRSMeasTimeOccasions-perMeasurementInstance | New | Each measurement instance in a TRP measurement report can be configured by LMF with either N=1 or 4 SRS measurement time occasions. | [1, 4] | FFS RAN3 | Agreement Each measurement instance in a TRP measurement report can be configured by LMF with either N=1 or 4 SRS measurement time occasions. |
| maxNumOf-RTOA-perSRSperRxTEG | New | The maximum number of reported RTOA measurements obtained from different UL SRS resources for positioning per TRP Rx TEG for a UE | 4 | FFS: RAN3 | Agreement The maximum number of reported RTOA measurements obtained from different UL SRS resources for positioning per TRP Rx TEG for a UE is 4. |
| maxNumOf-RxTxMeas-perSRSperTRPRxTEG | New | The maximum number of reported gNB Rx-Tx time difference measurements obtained from different UL SRS resources per TRP Rx TEG for a UE | 4 | FFS: RAN3 | Agreement The maximum number of reported UE Rx-Tx time difference measurements obtained from different DL PRS resources per UE Rx TEG for a TRP is 4. |
| maxNumOf-RxTxMeas-perSRSperTRPRxTxTEG | New | The maximum number of reported gNB Rx-Tx time difference measurements obtained from different UL SRS resources per TRP RxTx TEG for a UE | 4 | FFS: RAN3 | Agreement The maximum number of reported gNB Rx-Tx time difference measurements obtained from different UL SRS resources per TRP RxTx TEG for a UE is 4 |

# Discussion

In Release 17, accuracy enhancements for time-based methods (multi-RTT, DL TDOA, UL TDOA) have been proposed to mitigate timing errors, which have resulted in the introduction by RAN1 of the timing error group (TEG) at both the UE and TRP.  These include the following:

- UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin.

- TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin.

- UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin.

- TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin.

First thing we can understand from the LS in [1] is that to mitigate UE Tx TEG for UL TDOA, the UE sends the association information of UL SRS resources for positioning with UE Tx TEGs to the serving gNB, and the serving gNB should forward the association information provided by the UE to the LMF. Since this is done when the gNB configures UL SRS for the UE, we understand this can be signalled as part of the F1AP and NRPPa UL messages from gNB(DU) to LMF, i.e., the POSITIONING INFORMATION RESPONSE message.

Observation 1: Signalling of the UE Tx TEG association information with UL SRS resources is reported during the UL F1AP and NRPPa messages

Proposal 1: Add a UE Tx TEG Association IE in the NRPPa and F1AP POSITIONING INFORMATION RESPONSE messages

The serving gNB-DU sends the UE TX TEG association information to the serving gNB-CU every time it receives a periodic update from UE.  As mentioned in the RAN1 agreements above, the UE TX TEG association information may include one or more SRS resource set IDs and one or more SRS resource IDs associated with a UE TX TEG ID.

Observation 2: The UE TX TEG association information sent by the serving gNB-DU to the serving gNB-CU may include one or more SRS resource set IDs and one or more SRS resource IDs associated with a UE TX TEG ID

Since this is a report done periodically, instead of reporting all UE Tx TEG to UL SRS associations at every periodicity, the UE may only report the UE Tx TEG to UL SRS associations that have changed when compared to the previous reporting instance.  This constitutes a delta association report.  It should then also be noted that the serving gNB-DU may also report such delta association report to the serving gNB-CU.

For example, it can be assumed that the UE Tx TEG ID associated with SRS resource IDs #1 and #2 in SRS resource set ID #2 has changed since the previous reporting instance. Then, in the next UL association reporting, the serving gNB-DU can only report the UE Tx TEG ID association for SRS resource IDs #1 and #2 in SRS resource set ID #2 to the serving gNB-CU in a delta association report to save on signalling overhead.  Such delta report can be included in the F1AP and NRPPA POSITIONING INFORMATION UPDATE messageS.

Observation 3: Since the UE Tx TEG report is done periodically from UE, a delta association must be considered in the F1AP and NRPPa signalling to save on signalling overhead, where only the changed associated is reported

Proposal 2: Add the UE Tx TEG Association IE in the NRPPa and F1AP POSITIONING INFORMATION UPDATE messages with appropriate text.

On the other hand, RAN1 have also agreed that the TRP provides the association of TRP Tx TEG ID of one or more DL PRS resources and is sent from gNB to LMF for supporting DL-TDOA or multi-RTT. This can be part of the TRP Information IE in the TRP INFORMATION RESPONSE message based on LMF or gNB-CU request to TRPs to provide their DL PRS Tx TEG association information, as agreed by RAN1.

Observation 4: The TRPs should provide their DL PRS resource TEG association information Tx TEGs ID to the LMF.

Proposal 3: Add the TRP Tx TEG Association IE the TRP Information IE in the TRP INFORMATION RESPONSE message and corresponding codepoint in the TRP INFORMATION REQUEST message.

When the LMF receives the UE Tx TEG association from the serving gNB, it can instruct the NG-RAN in the MEASUREMENT REQUEST message to measure, e.g., UL-RTOA, using only the subset of UL-SRS associated with the received UE Tx TEG ID. This improves the UL-RTOA measurements from the TRPs and at the same time reduces network latency, since the gNBs/TRPs will only measure a subset of prioritized SRS resources rather than utilizing the complete set of resources.

Regarding the measurement results, RAN1 have agreed that LMF indicates a number M (Enumerated (2,3,4,6,8)) of  the TRP Rx TEGs that can be used by a TRP to measure the same UL positioning SRS resource of a UE for RTOA and/or gNB Rx-Tx measurements.  For gNB Rx-Tx measurements, LMF may also indicate a number M (also Enumerated (2,3,4,6,8)) of  TRP RxTx TEGs that can be used by a TRP to measure the same UL positioning SRS resource of a UE:

> The number of different TRP Rx TEGs that the LMF requests a TRP to measure the same UL positioning SRS resource of a UE for RTOA measurements [2, 3, 4, 6, 8] > The number of different TRP Rx TEGs that the LMF requests a TRP to measure the same UL positioning SRS resource of a UE for gNB Rx-Tx measurements [2, 3, 4, 6, 8] > The number of different TRP RxTx TEGs that the LMF requests a TRP to measure the same UL positioning SRS resource of a UE for gNB Rx-Tx measurements [2, 3, 4, 6, 8] FFS for RAN3

Proposal 4: Add the number of TRP Rx TEGs and TRP TxRx TEGs a TRP should use to measure the same SRS resource with in the MEASUREMENT REQUEST message

A draft TP to NRPPa is provided in the Annex below. A TP to F1AP mirroring the NRPPa new additions can be provided during the meeting

Proposal 5: Discuss and agree TP to NRPPa BL CR in Annex

# 3. Conclusion

Observation 1: Signalling of the UE Tx TEG association information with UL SRS resources is reported during the UL F1AP and NRPPa messages

Proposal 1: Add a UE Tx TEG Association IE in the NRPPa and F1AP POSITIONING INFORMATION RESPONSE messages

Observation 2: The UE TX TEG association information sent by the serving gNB-DU to the serving gNB-CU may include one or more SRS resource set IDs and one or more SRS resource IDs associated with a UE TX TEG ID

Observation 3: Since the UE Tx TEG report is done periodically from UE, a delta association must be considered in the F1AP and NRPPa signalling to save on signalling overhead, where only the changed associated is reported

Proposal 2: Add the UE Tx TEG Association IE in the NRPPa and F1AP POSITIONING INFORMATION UPDATE messages with appropriate text.

Observation 4: The TRPs should provide their DL PRS resource TEG association information Tx TEGs ID to the LMF.

Proposal 3: Add the TRP Tx TEG Association IE the TRP Information IE in the TRP INFORMATION RESPONSE message and corresponding codepoint in the TRP INFORMATION REQUEST message.

Proposal 4: Add the number of TRP Rx TEGs and TRP TxRx TEGs a TRP should use to measure the same SRS resource with in the MEASUREMENT REQUEST message

Proposal 5: Discuss and agree TP to NRPPa BL CR in Annex

# 4. References

- R3-220093, LS on the reporting of the Tx TEG association information, RAN1

- R1-2112976, Consolidated higher layers parameter list for Rel-17 NR, "Positioning" sheet

# 5. Annex: TP to NRPPa BL CR

<<START OF CHANGES>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

LMF	Location Management Function

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SRS	Sounding Reference Signal

TEG	Timing Error Group

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

<<NEXT CHANGE>>

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the UE Tx TEG Association IE is included in the POSITIONING INFORMATION RESPONSE message, the LMF shall, if supported, consider it as the UE TEG association for the SRS resources.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the NG-RAN node is unable to configure any SRS transmissions for the UE, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.6.4	Abnormal Conditions

Void.

### 8.2.7	Positioning Information Update

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall, if supported, consider it as the UE TEG association for the SRS resources that have changed their TEG association since the last update.

<<NEXT CHANGE>>

#### 9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| UE Tx TEG Association | O |  | 9.2.X |  | YES | ignore |

<<NEXT CHANGE>>

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by NG-RAN node to indicate that a change in the SRS configuration has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association | O |  | 9.2.X |  | YES | ignore |

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type List |  | 1 |  |  |  |  |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, trp Tx teg) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

<<NEXT CHANGE>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

<<NEXT CHANGE>>

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>NR CGI | M |  | 9.2.9 |  | - |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |
| >>SSB Information | M |  | 9.2.54 |  | - |  |
| >>SFN Initialisation Time | M |  | Relative Time 1900 9.2.36 |  | - |  |
| >>Spatial Direction Information | M |  | 9.2.45 |  | - |  |
| >>Geographical Coordinates | M |  | 9.2.46 |  | - |  |
| >>TRP type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [18] | YES | reject |
| >>TRP Tx TEG Association | M |  | 9.2.Y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

<<NEXT CHANGE>>

### 9.2.X UE Tx TEG Association

This information element contains the UE Tx TEG association.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Tx TEG Association item | 1 .. <maxnoUETEGs> |  |  |  |
| >UE Tx TEG ID | M |  | INTEGER (1..FFS) |  |
| >SRS Resource Set ID | M |  | INTEGER (0..15) |  |
| >SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoUETEGs | Maximum no of reported UE Tx TEG association. Value is FFS. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

### 9.2.Y TRP Tx TEG Association

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP TEG item | 1 .. <maxnoTRPTEGs> |  |  |  |
| >TRP Tx TEG ID | M |  | INTEGER (1..8, …FFS) |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >DL-PRS Resource ID List |  | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPTEGs | Maximum no of reported TRP Tx TEG association. Value is FFS. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

<<End of CHANGES>>