3GPP TSG-RAN WG3 #114bis-e	R3-220575

Electronic meeting, January 16th – 27th 2022

Agenda Item:	10.2.1.4

Source:	Ericsson

Title:	(TP for SON BL CR for TS 36.413): UE History Information for Secondary Node

Document for:	Discussion, Decision

# Introduction

In RAN3#114-e, the following agreements and FFS were agreed:

MN can initiate SN modification procedures to retrieve SN UHI before handover without delaying HO, MN may also subscribe to PSCell changes from SN. (Option 4)

Correlated MN and SN UHI using a nested structure shall be included in the handover request message from MN.

Correlated MN and SN UHI using a nested structure is transferred from MN to SN.

Only SN UHI is transferred from SN to MN.

UE History Information IE shall be included in the S-NODE ADDITION REQUEST and SGNB ADDITION REQUEST messages.

Include SN UHI, e.g. SCG UE History Information IE, in the following messages.

- S-NODE CHANGE REQUIRED and SGNB CHANGE REQUIRED messages

- S-NODE MODIFICATION REQUEST ACKNOWLEDGE and SGNB MODIFICATION REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUEST ACKNOWLEDGE and SGNB RELEASE REQUEST ACKNOWLEDGE messages

- S-NODE RELEASE REQUIRED and SGNB RELEASE REQUIRED messages

- S-NODE MODIFICATION REQUIRED and SGNB MODIFICATION REQUIRED messages

MN shall correlate MN and SN UHI.

Open issue 1: Whether/how to include Time spent without SCG or Time stamp in SN UHI.

Open issue 2: FFS how MN subscribes to PSCell changes from SN.

Open issue 3: FFS on the stage 2 description for SN UHI.

Open issue 4: FFS how to correlate MN and SN UHI.

To summarize the current landscape of UE history information discussions:

Usage of SN UHI for target node: (RAN3#113-e)

- SN node ping pong issue

- Assist target MN to select the appropriate SN

- Assist target MN in requirement of dual connectivity

Collection of UE history information:

- SN is responsible for collecting SN UHI. (RAN3#112-e WA).

Correlation of MN UE history information and SN UE history information:

- Correlation of MN UHI and SN UHI is beneficial (RAN3#109-e),

- Realized via two-dimensional structure for UHI (RAN3#112-e WA).

- Correlation is done at the MN (RAN3#114-e).

Transfer of correlated UE history information between MN and SN:

- SN UE history information is collected at SN and transferred to MN during Handover (RAN3#114-e).

- Correlated UE history information is sent from MN to SN (RAN3#114-e).

UE history information is transferred from SN and MN in the following messages: (RAN3#114-e).

- SN Change procedure (S-NODE CHANGE REQUIRED, SGNB CHANGE REQUIRED)

- SN Modification procedure

  - MN-initiated: S-NODE MODIFICATION REQUEST ACKNOWLEDGE, SGNB MODIFICATION REQUEST ACKNOWLEDGE

  - SN-initiated: S-NODE MODIFICATION REQUIRED and SGNB MODIFICATION REQUIRED messages

- SN Release procedure

  - MN-initiated: S-NODE RELEASE REQUEST ACKNOWLEDGE, SGNB RELEASE REQUEST ACKNOWLEDGE

  - SN-initiated: S-NODE RELEASE REQUIRED, SGNB RELEASE REQUIRED

Content of SN UE history information:

- includes PSCell list, time UE stayed in cell. (RAN3#109-e).

- does not include HO clause. (RAN3#110-e).

- No enough benefit to introduce Cell type. (RAN3#113-e)

- Including originating node of PSCell change is FFS. (RAN3#112-e WA)

- Whether to use timestamp or time spent without SCG (RAN3#114-e)

MN subscription to PSCell changes:

- FFS how MN subscribes to PSCell changes from SN (RAN3#114-e)

In this contribution, we discuss the open issues and give proposals on the way forward. A complete solution considering the proposals in the paper is also included before the conclusion for clarity.

# Discussion

## Subscription of SN UE history information

- Add a flag indicating subscription of PSCell changes to the SN addition request and/or SN modification request messages sent from the master node to the secondary node.

The information sent from the secondary node to the master node in case of PSCell changes should include the Global Cell ID of the new PSCell, and other feasible information of the previous PSCell. Three alternative solutions for how to send this could be considered:

- Introduce a new optional IE PSCell change information containing Global Cell ID of the new PSCell and other available SN UHI content in the SN MODIFICATION REQUIRED message.

- Introduce a dedicated, mandatory class 2 procedure message PSCELL CHANGE INFORMATION including the Global Cell ID of the new PSCell.

- Send the full SN UHI during each PSCell change from the SN to the MN. This can be sent through the SN MODIFICATION REQUIRED message which has already been agreed to contain SN UHI leading to reduced specification impact at the cost of extra signalling

Which of these solutions to use could be further discussed in RAN3.

RAN3 to discuss and conclude on one of the three options to
a) Introduce a new optional IE PSCell change information containing Global Cell ID of the new PSCell and other available SN UHI content in the SN modification required message. b) Introduce a dedicated, mandatory class 2 procedure message PSCELL CHANGE INFORMATION including the Global Cell ID of the new PSCell.  c) Use the existing SN MODIFICATION REQUIRED message to send the complete SN UHI
for signaling the PSCell changes to the master node subscribing for such information.

## Correlated UE history information between MN and SN

According to the current discussions, the MN collects PCell related UHI (legacy UHI) and the SN collects PSCell related UHI (SN UHI). The SN sends SN UHI to the MN for correlation. During correlation, the MN puts each PSCell entry under the appropriate PCell entry during which both cells were active. This can be accomplished through the use of the IE ”time UE stayed in cell” [4] present in legacy UHI and (expected to be agreed for) the SN UHI.

The IE ”time UE stayed in cell” can take values from 0 to 4095, upon exceeding this time it is set to 4095. This can cause problems with correlation when this IE has overflowed as the MN cannot determine the exact time and thus cannot perform correlation. Consider the figure below that illustrates the different Pcells and PSCells the UE connects from 10:00 to 13:00. The top and bottom rows correpond to the current PCell and PSCell repectively.

PSCell A

PCell A

PCell B

10:00:00

11:00:00

12:00:00

13:00:00

PCell C

PCell B

PSCell B

PSCell C

PSCell A

Consider the scenario where the UE has connected to a series of PCells for the following times:

- PCell A: From time 10:00:00 to 10:50:00

- PCell B: From time 10:50:00 to 12:00:00

- PCell C: From time 12:00:00 to 12:40:00

- PCell B: From time 12:40:00 to time of correlation (12:50:00)

During the same time, the UE is connected to the following PSCells:

- PSCell A: From time 10:00:00 to 10:02:00

- PSCell B: From time 10:02:00 to 10:42:00

- PSCell C: From time 10:42:00 to 12:10:00

- PSCell A: From time 12:10:00 to SN release at time 12:40:00

The (simplified) PCell UHI (left) at the MN and (simplified) PSCell UHI (right) look as shown below,

| PCell ID | Time UE stayed in PCell(s) |
| --- | --- |
| PCell A | 3000 |
| PCell B | 4095* |
| PCell C | 2400 |
| PCell B | 600 |

| PSCell ID | Time UE stayed in PSCell(s) |
| --- | --- |
| PSCell A | 100 |
| PSCell B | 2000 |
| PSCell C | 4095* |
| PSCell A | 1200 |

The entries for PCell B and PSCell C exceed the limits and thus make accurate correlation of the two UHIs impossible.

Correlation of PCell and PSCell entries is not possible using the current standardized IEs in the legacy UHI

### Correlation of PCell and PSCell entries using optional timestamp

| PSCell ID | Time UE stayed in PSCell(s) | Optional time stamp |
| --- | --- | --- |
| PSCell A | 100 |  |
| PSCell B | 2000 |  |
| PSCell C | 4095* | 10:42:00 |
| PSCell A | 1200 |  |

The above problem can be solved through the use of an optional timestamp in the UHI that is only used when the IEs ”time UE stayed in the cell” and ”time UE stayed in secondary cell” exceed their limits. In the above example, the UHIs will contain the following information,

| PCell ID | Time UE stayed in PCell(s) | Optional time stamp |
| --- | --- | --- |
| PCell A | 3000 |  |
| PCell B | 4095* | 10:50:00 |
| PCell C | 2400 |  |
| PCell B | 600 |  |

The timestamps can then be used to perform correlation. This would however require the clocks in MN and SN to be synchronized. This can be avoided by also sending the timestamp of SN release. Then MN can then use  the timestamps in the SN UHI together with SN release time and correlate it with MN UHI and the current time at MN.

Thus we propose

- Introduce an optional timestamp indicating the time of successful PSCell connection in UE History Information that is used when the IE ”time UE stayed in cell” exceeds the limit.

- Introduce an optional timestamp indicating SN release time in S-NODE RELEASE REQUEST ACKNOWLEDGE and S-NODE RELEASE REQUIRED messages to facilitate UHI correlation.

### Information content of SN UE History Information to facilitate correlation

UE history information should include the amount of time spent with no PSCell to correctly interpret the correlated information. This information can be used to enhance understanding of previous UE choices in terms of DC. For example, A UE may not be connected to SCGs for a certain period as it is moving at a high speed. A node can then use this information to deduce that the UE is travelling too fast to configure a reliable DC connection. The MN determines the feasibility of initiating DC for a UE. It can decide to initiate SN addition due to obtained coverage, or due to additional bandwidth requirements of the UE. Thus we propose the following

- Add the following IEs to the (correlated) UE history information under relevant PCell entry.

  - Time spent with no PSCell.

  - Reason for no PSCell.

## Transfer of mobility history information (MHI) from MN to SN

Mobility history information (MHI) is mobility related information that is collected and stored by the UE. This information is a nested structure that consists of serving PCell and PSCell information. Current discussions in RAN2 consider transfer of the nested MHI only to the MN. In the current 3GPP landscape, MHI from the UE is transferred during HO (only PCell changes) through the IE “UE History Information from the UE”.

Mobility history information is collected by the UE and then transferred during MN handover through the IE “UE history information from the UE”.

“UE history information from the UE” contains PSCell information which is required at the Secondary node for mobility optimizations.

The IE ”UE History information from the UE” containing the mobility report will contain PSCell information in Dual-connectivity mode. Once this information is retrieved by the MN, it needs to be forwarded to the SN as RAN2 concluded against sending the mobility report directly to the SN. This requires two solutions depending on when the MN retrieved the mobility report last.

The MN can forward the report to the SN in the SN ADDITION REQUEST message if it has already retrieved the report from the UE before the start of DC. If the MN has however not retrieved the report before SN addition, then the same can be forwarded to the SN optionally in the SN MODIFICATION REQUEST message.

Thus we propose sending MHI to the SN in the following messages

- Add the IE “UE history information from the UE” in the following messages

  - Mandatory IE in the SN ADDITION REQUEST message

  - Optional IE in the SN MODIFICATION REQUEST message

## Transfer of UHI during conditional handovers

No discussions have taken place yet on transfer of UHI during conditional handovers (CHO). As HO requests are sent from a source RAN node to multiple target RAN nodes well in advance of the actual HO, the UHI sent to the target RAN nodes will not be up-to date. There are two solutions to prevent this scenario of wrong UHI.

The first is to instruct all the candidate target nodes to start timers to keep track of the time spent by the UE in the latest PCell assigned by the source RAN node. This timer can then be used by the target to which the UE connects successfully to update the “time spent in cell” for the latest PCell. The other target RAN nodes will discard the timer.

The second solution is to introduce a new message after the HANDOVER SUCCESS to update the target RAN node with the updates UHI. This prevents the creation of multiple timers in different candidate nodes at the cost of an additional message after the successful completion of CHO.

RAN3 to decide on one of the two solutions for transfer of latest UHI during conditional handovers. A) Instruct all candidate RAN nodes to start a timer to update UHI upon successful HO. B) A new message to update UHI after successful HO.

# Conclusion

In the above we have made the following observations:

Observation 1	Correlation of PCell and PSCell entries is not possible using the current standardized IEs in the legacy UHI

Observation 2	Mobility history information is collected by the UE and then transferred during MN handover through the IE “UE history information from the UE”.

Observation 3	“UE history information from the UE” contains PSCell information which is required at the Secondary node for mobility optimizations.

We propose the following:

Proposal 1	Add a flag indicating subscription of PSCell changes to the SN addition request and/or SN modification request messages sent from the master node to the secondary node.

Proposal 2	RAN3 to discuss and conclude on one of the three options to a) Introduce a new optional IE PSCell change information containing Global Cell ID of the new PSCell and other available SN UHI content in the SN modification required message. b) Introduce a dedicated, mandatory class 2 procedure message PSCELL CHANGE INFORMATION including the Global Cell ID of the new PSCell.  c) Use the existing SN MODIFICATION REQUIRED message to send the complete SN UHI for signaling the PSCell changes to the master node subscribing for such information.

Proposal 3	Introduce an optional timestamp indicating the time of successful PSCell connection in UE History Information that is used when the IE ”time UE stayed in cell” exceeds the limit.

Proposal 4	Introduce an optional timestamp indicating SN release time in S-NODE RELEASE REQUEST ACKNOWLEDGE and S-NODE RELEASE REQUIRED messages to facilitate UHI correlation.

Proposal 5	Add the following IEs to the (correlated) UE history information under relevant PCell entry.

a.	Time spent with no PSCell.

b.	Reason for no PSCell.

Proposal 6	Add the IE “UE history information from the UE” in the following messages

a.	Mandatory IE in the SN ADDITION REQUEST message

b.	Optional IE in the SN MODIFICATION REQUEST message

Proposal 7	RAN3 to decide on one of the two solutions for transfer of latest UHI during conditional handovers. A) Instruct all candidate RAN nodes to start a timer to update UHI upon successful HO. B) A new message to update UHI after successful HO.

# References

- R3-205659, Summary of Offline for MR-DC UHI, ZTE (moderator), 3GPP TSG-RAN WG3 #109-e, 17-28 August 2020, Online.

- R3-212660, Offline Discussion on UE History Information in EN-DC, CATT (moderator), 3GPP TSG-RAN WG3 #112-e, Online, 17 – 27 May 2021.

- 3GPP TS 37.340 V16.5.0 (2021-03), Technical Specification, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2 (Release 16)

- 3GPP TS 38.423 V16.7.0 (2021-10), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NG-RAN; Xn application protocol (XnAP) (Release 16)

# Appendix A: Text Proposal to 38.423

#### NOTE: If the TP is agreeable, a revision will ASN.1 will be provided during the meeting

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

-- TEXT OMITTED –

If the M-NG-RAN node receives in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message within the PDU Session Resource Modification Response Info –MN terminated IE a DRBs Admitted to be Setup or Modified Item with DRB ID(s) that it has not requested to be setup or modified, the M-NG-RAN node shall ignore the contained information.

If the S-NODE MODIFICATION REQUEST message contains the PSCell change subscription IE, the S-NG-RAN node shall, if supported, signal the Global Cell ID of the new PSCell upon each PSCell change, to the M-NG-RAN node.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

-- TEXT OMITTED –

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| Pcell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB Ids | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| PSCell change subscription | O |  | ENUMERATED (start, ...) |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

-- TEXT OMITTED –

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node |  | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. |  | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 |  |  | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  |  | – |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  |  | – |  |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  | 9.2.1.9 |  |  | – |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  | 9.2.1.11 |  |  | – |  |
| >>>S-NSSAI | O |  | 9.2.3.21 |  |  | YES | reject |
| >PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. |  | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. |  | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  |  | YES | ignore |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 |  | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) |  | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 |  | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  |  | YES | reject |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) |  | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) |  | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  |  | YES | ignore |
| PSCell change subscription | O |  | ENUMERATED (start, ...) |  |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

# Appendix B: Text Proposal to 38.413

#### NOTE: If the TP is agreeable, a revision will ASN.1 will be provided during the meeting

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| Cell Type | M |  | 9.3.1.98 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 |

#### 9.3.1.X	No Secondary Cell Information

This IE contains information on the time spent by the UE with no secondary cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time UE Spent with no Secondary Cell | M |  | INTEGER (0..4095) | If the duration is more than 4095s, this IE is set to 4095. |
| Time UE Spent with no Secondary Cell Enhanced Granularity | O |  | INTEGER (0..40950) | If the duration is more than 4095s, this IE is set to 40950. |
| Reason for no secondary cell | O |  | OCTET STRING | Contains information on why secondary cell was not configured. E.g. No coverage, not required due to low data usage, etc ... |

# Appendix  C: Text Proposal to 36.413

#### NOTE: If the TP is agreeable, a revision will ASN.1 will be provided during the meeting

#### 9.2.1.43a	Last Visited E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Cell Type | M |  | 9.2.1.66 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |
| HO Cause Value | O |  | 9.2.1.3 | The cause for the handover from the E-UTRAN cell. | YES | ignore |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 | YES | ignore |