3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220602

Online, 17th -26th  January 2022

Title:	TP to TS 38.470 BL CR: Support of reuse the F1-C/F1-U tunnel

Source:	Ericsson

Agenda Item:	24.3

Document for:	Other

# TP to F1 stage 2 BL CR

START OF CHANGES

### 5.2.3	F1 UE context management function

The F1 UE context management function supports the establishment and modification of the necessary overall UE context.

The establishment of the F1 UE context is initiated by the gNB-CU and accepted or rejected by the gNB-DU based on admission control criteria (e.g., resource not available).

The modification of the F1 UE context can be initiated by either gNB-CU or gNB-DU. The receiving node can accept or reject the modification. The F1 UE context management function also supports the release of the context previously established in the gNB-DU. The release of the context is triggered by the gNB-CU either directly or following a request received from the gNB-DU. The gNB-CU request the gNB-DU to release the UE Context when the UE enters RRC_IDLE or RRC_INACTIVE.  The gNB-CU can also request the gNB-DU to retrieve previous F1-C and F1-U tunnel information for the next SDT SRB and DRB transmissions.

END OF CHANGE