3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220693

E-meeting, 17-26 Jan 2022

Title: 	(TPs to TS 38.410, 38.413 BL CRs) Multicast Session Activation Deactivation and Update

Source: 	Huawei, CBN, China Unicom, China Telecom, Lenovo, Motorola Mobility

Agenda item:	22.2.2

Document Type:	Other

# 1. Introduction

- In the RAN3#114e meeting, there were several agreements about multicast session management achieved:

Multicast Session Management - About providing mapped QoS flow and associated QoS flow information from CN to RAN, update UE associated NGAP: PDU Session Resource Modify Request Transfer IE and PDU Session Resource Setup Request Transfer IE. - Introduce a non-UE associated NGAP Class1 Multicast Session Update procedure triggered by MB-SMF to support multicast session update in case the change of some of QoS parameters and/or service area. - Introduce non-UE associated NGAP Class1 Multicast Session Activation procedure and Multicast Session Deactivation procedure, FFS on whether a single procedure or separate procedures should be used for activation/update/deactivation. - WA: introduce new MB-SMF containers in TS 38.413, subject to SA2/CT4. Applicable for both MC and BC. - Introduce one or two non-UE associated Class1 NGAP procedure(s), triggered by the gNB to implement Multicast Distribution Setup function, and Multicast Distribution Release function. - For location dependent Multicast MBS service, the MBS Distribution Setup/Release procedure is used to setup/release the NG-U tunnel for an area Session.

- In this paper, we provide the text proposal to TS 38.410 BL CR and TS 38.413 BL CR in Section 4 and Section 5.

# 2. Discussion

In the last meeting, there was agreement that using non-UE associated NGAP Class1 procedure to support Multicast Session Activation/Deactivation, while whether a single procedure or two separate procedures should be used for Multicast Session activation/deactivation is FFS. From RAN3 point of view, we will further discuss both alternatives as follow:

Alt 1: Separate procedures, i.e. Multicast Session Activation procedure, Multicast Session Deactivation procedure, and Multicast Session Update procedure

For this alternative, the involved procedures and messages could include be:

Multicast Session Activation Procedure

- MULTICAST SESSION ACTIVATION REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION ACTIVATION RESPONSE message (from the NG-RAN to the AMF)

- MULTICAST SESSION ACTIVATION FAILURE message (from the NG-RAN to the AMF)

Multicast Session Deactivation Procedure

- MULTICAST SESSION DEACTIVATION REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION DEACTIVATION RESPONSE message (from the NG-RAN to the AMF)

Multicast Session Update Procedure

- MULTICAST SESSION UPDATE REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION UPDATE RESPONSE message (from the NG-RAN to the AMF)

- MULTICAST SESSION UPDATE FAILURE message (from the NG-RAN to the AMF)

To activate the session, the AMF will initiates a MULTICAST SESSION ACTIVATION REQUEST message to the NG-RAN node to activate the MBS resources of one MBS session and the corresponding MBS QoS flows. Upon receipt of this message, the NG-RAN node shall send the MULTICAST SESSION ACTIVATION RESPONSE message to indicate that the request was successfully fulfilled, or send the MULTICAST SESSION ACTIVATION FAILURE message with an appropriate cause value to indicate failure.

To deactivate the session, the AMF will initiates a MULTICAST SESSION DEACTIVATION REQUEST message to the NG-RAN node to deactivate the MBS resources of one MBS session and the corresponding MBS QoS flows. Upon receipt of this message, the NG-RAN node shall indicate in the MULTICAST SESSION DEACTIVATION RESPONSE message, there is no failure case foreseen.

Alt 2: Single procedure, i.e. Multicast Session Update procedure

For this alternative, the involved messages could include be:

- MULTICAST SESSION UPDATE REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION UPDATE RESPONSE message (from the NG-RAN to the AMF)

As there is no failure case foreseen for Multicast Session Deactivation, in this solution, the procedure only includes two messages, one update request, and one update response. To change the session status, the AMF will send a MULTICAST SESSION UPDATE REQUEST message to the NG-RAN node to change the MBS resources of one MBS session and the corresponding MBS QoS flows. Upon receipt of this message, the NG-RAN node shall response with the MULTICAST SESSION STATUS UPDATE RESPONSE message.

Comparison of Alternative 1 and Alternative 2

Different from alternative 1, if the NG-RAN node cannot activate/update the MBS resources, it shall explicitly indicate failure to activate/update in MULTICAST SESSION STATUS UPDATE RESPONSE message with an appropriate cause value.

To align with TS 23.247, during the session update procedure, the service area or the MBS QoS flow(s) are allowed to be updated. During session activation/deactivation procedure, only the session status can be changed.

For session deactivation, the NG-RAN node will not indicate the failure with cause, only response successful operation.

For the session activation scenario, the resource may not able to fulfill the QoS requirement, since the radio resources were released during session deactivation, the admission control should be performed during the session activation procedure because the MBS session to be activated is in the competition with other session, the QoS flow(s) successfully active and/or failure to be activated should be included in the MB-SMF container in response message.

Considering the parameters for those messages are quite different, to support the same functions of Alt 1, CHOICE type will be used in the REQUEST and RESPONSE messages in Alt 2, Stage3 design will be quite complex for Alt 2.

Although both alternative are workable, we prefer separate procedures, because each procedure is introduced for a single purpose, and the extension of each procedure can be easily done in the future if needed.

Proposal 1: Introduce the following separate non-UE associated class1 NGAP procedures with the corresponding messages:

Multicast Session Activation Procedure

- MULTICAST SESSION ACTIVATION REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION ACTIVATION RESPONSE message (from the NG-RAN to the AMF)

- MULTICAST SESSION ACTIVATION FAILURE message (from the NG-RAN to the AMF)

Multicast Session Deactivation Procedure

- MULTICAST SESSION DEACTIVATION REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION DEACTIVATION RESPONSE message (from the NG-RAN to the AMF)

Multicast Session Update Procedure

- MULTICAST SESSION UPDATE REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION UPDATE RESPONSE message (from the NG-RAN to the AMF)

- MULTICAST SESSION UPDATE FAILURE message (from the NG-RAN to the AMF)

# 3. Conclusion

In this contribution, we discussed how to support Multicast Session Activation, Deactivation and Update, get the following proposal:

Proposal 1: Introduce the following separate non-UE associated class1 NGAP procedures with the corresponding messages:

Multicast Session Activation Procedure

- MULTICAST SESSION ACTIVATION REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION ACTIVATION RESPONSE message (from the NG-RAN to the AMF)

- MULTICAST SESSION ACTIVATION FAILURE message (from the NG-RAN to the AMF)

Multicast Session Deactivation Procedure

- MULTICAST SESSION DEACTIVATION REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION DEACTIVATION RESPONSE message (from the NG-RAN to the AMF)

Multicast Session Update Procedure

- MULTICAST SESSION UPDATE REQUEST message (from the AMF to the NG-RAN)

- MULTICAST SESSION UPDATE RESPONSE message (from the NG-RAN to the AMF)

- MULTICAST SESSION UPDATE FAILURE message (from the NG-RAN to the AMF)

# 4. Text Proposal to BL CR of TS 38.410

# 6	Signalling procedures of the NG interface

//skip unchanged part

## 6.xx	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

-	Multicast Session Update;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

-	Distribution Setup;

-	Distribution Release;

Editor’s Note 2: FFS whether the Distribution Setup and Distribution Release procedures are used for Broadcast and Multicast.

Editor’s Note 3: FFS on combination of Multicast Session Update and Broadcast Session Update procedures.

Editor’s Note 4: FFS on NG-RAN triggered release procedures for both, BC and MC session resources.

# 5. Text Proposal to BL CR of TS 38.413

## *****************First change*******************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[11]	3GPP TS 32.422: "Trace control and configuration management".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC inactive state".

[13]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[14]	3GPP TS 38.414: "NG-RAN; NG data transport".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 38.331: "NG-RAN; Radio Resource Control (RRC) Protocol Specification".

[19]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[20]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[25]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[26]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[30]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[32]	3GPP TS 37.340: " Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[33]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[34]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[35]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[36]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[37]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[38]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[39]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[40]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP) ".

[41]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[42]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2”.

## *****************Next changes*******************

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Multicast Session Activation | MULTICAT SESSION ACTIVATION REQUEST | MULTICAT SESSION ACTIVATION RESPONSE | MULTICAT SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAT SESSION DEACTIVATION REQUEST | MULTICAT SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION FAILURE |

## *****************Next changes*******************

## 8.xx	MBS Session Management Procedures

### 8.xx.a	Multicast Session Activation

#### 8.xx.a.1	General

The purpose of the Multicast Session Activation procedure is to request a NG-RAN node to activate the MBS resources of one MBS session and the corresponding MBS QoS flows.

The procedure uses non-UE-associated signalling.

#### 8.xx.a.2	Successful Operation

Figure 8.xx.a.2-1: Multicast Session Activation, successful operation

The AMF initiates the procedure by sending a MULTICAST SESSION ACTIVATION REQUEST message to the NG-RAN node.

Upon receipt of this message, the NG-RAN node should activate the MBS resources indicated in the MULTICAST SESSION ACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION ACTIVATION RESPONSE message for which MBS Session the request was fulfilled.

#### 8.xx.a.3	Unsuccessful Operation

Figure 8.xx.a.3-1: Multicast Session Activation, unsuccessful operation

If the NG-RAN node cannot activate the MBS resources indicated in the MULTICAST SESSION ACTIVATION REQUEST message, it shall respond with a MULTICAST SESSION ACTIVATION FAILURE message with an appropriate cause value.

#### 8.xx.a.4	Abnormal Conditions

Not applicable.

### 8.xx.b	Multicast Session Deactivation

#### 8.xx.b.1	General

The purpose of the Multicast Session Deactivation procedure is to request a NG-RAN node to deactivate the MBS resources of one MBS session and the corresponding MBS QoS flows.

The procedure uses non-UE-associated signalling.

#### 8.xx.b.2	Successful Operation

Figure 8.xx.b.2-1: Multicast Session Deactivation, successful operation

The AMF initiates the procedure by sending a MULTICAST SESSION DEACTIVATION REQUEST message to the NG-RAN node.

Upon receipt of this message, the NG-RAN node should deactivate the MBS resources indicated in the MULTICAST SESSION DEACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION DEACTIVATION RESPONSE message for which MBS Session the request was fulfilled.

#### 8.xx.b.3	Unsuccessful Operation

Not applicable.

#### 8.xx.b.4	Abnormal Conditions

Not applicable.

### 8.x.c	Multicast Session Update

#### 8.x.c.1	General

The purpose of the Multicast Session Update procedure is to request NG-RAN node to update the MBS service area or the MBS information related to a previously established MBS session.

The procedure uses non-UE associated signalling.

#### 8.x.c.2	Successful Operation

Figure 8.x.c.2-1. Multicast Session Update procedure. Successful operation.

The AMF initiates the procedure by sending a MULTICAST SESSION UPDATE REQUEST message to the NG-RAN node.

#### 8.x.c.3	Unsuccessful Operation

Figure 8.x.c.3-1. Multicast Session Update procedure. Unsuccessful operation.

If the NG-RAN node fails to update any requested modification, the NG-RAN node shall send MULTICAST SESSION UPDATE FAILURE message.

#### 8.x.2.4	Abnormal Conditions

Not applicable.

## *****************Next changes*******************

### 9.2.x	MBS Session Management Messages

#### 9.2.x.a1	MULTICAST SESSION ACTIVATION REQUEST

This message is sent by the AMF to a NG-RAN node to request previously deactivated MBS resources to be re-activated.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Session Activation Request Transfer | M |  | OCTET STRING | Containing the Multicast Session Activation Request Transfer IE specified in subclause 9.3.A.a1 | YES | reject |

#### 9.2.x.a2	MULTICAST SESSION ACTIVATION RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources previously deactivated have been activated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Session Activation Response Transfer | M |  | OCTET STRING | Containing the Multicast Session Activation Response Transfer IE specified in subclause 9.3.A.a2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.a3	MULTICAST SESSION ACTIVATION FAILURE

This message is sent by the NG-RAN node to the AMF to indicate multicast session activation failure.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Session Activation Failure Transfer | M |  | OCTET STRING | Containing the Multicast Session Activation Failure Transfer IE specified in subclause 9.3.A.a3 | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.b1	MULTICAST SESSION DEACTIVATION REQUEST

This message is sent by the AMF to a NG-RAN node to request to deactivate the MBS resources of a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Session Deactivation Request Transfer | M |  | OCTET STRING | Containing the Multicast Session Deactivation Request Transfer IE specified in subclause 9.3.A.b1 | YES | reject |

#### 9.2.x.b2	MULTICAST SESSION DEACTIVATION RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources have been deactivated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Session Deactivation Response Transfer | M |  | OCTET STRING | Containing the Multicast Session Deactivation Response Transfer IE specified in subclause 9.3.A.b2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.c1	MULTICAST SESSION UPDATE REQUEST

This message is sent by the AMF to a NG-RAN node to update the MBS context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| Multicast Session Update Request Transfer | O |  | OCTET STRING | Containing the Multicast Session Update Request Transfer IE specified in subclause 9.3.A.c1 | YES | reject |

#### 9.2.x.a2	MULTICAST SESSION UPDATE RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources previously deactivated have been activated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| Multicast Session Update Response Transfer | M |  | OCTET STRING | Containing the Multicast Session Update Response Transfer IE specified in subclause 9.3.A.c2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.a3	MULTICAST SESSION UPDATE FAILURE

This message is sent by the NG-RAN node to the AMF to indicate multicast session activation failure.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| Multicast Session Update Failure Transfer | M |  | OCTET STRING | Containing the Multicast Session Update Failure Transfer IE specified in subclause 9.3.A.c3 | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

## *****************Next changes*******************

#### 9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session or a MBS QoS flow within a MBS Session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

## *****************Next changes*******************

#### 9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

#### 9.3.1.z	Area Session ID

This IE identifies a Area Session. The definition and use of the Area Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area Session ID | M |  | FFS |  |

#### 9.3.1.s	MBS Service Area information

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |
| --- | --- | --- | --- | --- | --- |
| MBS service Area Cell Item |  | 0..<maxnoofCellsforMBS> |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |  |
| MBS service Area TAI Item |  | 0..<maxnoofTAIforMBS> |  |  |  |
| >>TAI | M |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is FFS. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is FFS. |

## *****************Next changes*******************

### 9.3.A	MB-SMF Related IEs

#### 9.3.A.a1	Multicast Session Activation Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |

#### 9.3.A.a2	Multicast Session Activation Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Successful Cell List |  | 1 .. <maxnoofCellsingNB> |  |  | YES | reject |
| >NR CGI | M |  | 9.3.1.7 |  | - | - |
| >QoS Flows Failed to Activate List | O |  | 9.3.1.13 |  | - | - |
| Unsuccessful Cell List |  | 0 .. <maxnoofCellsingNB> |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.7 |  | - | - |
| >Cause | O |  | 9.3.1.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsingNB | Maximum no. of cells that can be serviced by a gNB. Value is 16384. |

#### 9.3.A.a3	Multicast Session Activation Failure Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |

#### 9.3.A.b1	Multicast Session Deactivation Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |

#### 9.3.A.b2	Multicast Session Deactivation Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |

#### 9.3.A.c1	Multicast Session Update Request Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.z |  |
| MBS Service Area information | O |  | 9.3.1.s |  |
| MBS QoS Flows To Be Modify List |  | O |  |  |
| >MBS QoS Flows To Be Modify Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSQoSFlows | Maximum no. of QoS Flows allowed within one MBS session. Value is 64. |

#### 9.3.A.c2	Multicast Session Update Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| Successful Cell List |  | 1 .. <maxnoofCellsingNB> |  |  | YES | reject |
| >NR CGI | M |  | 9.3.1.7 |  | - | - |
| >QoS Flows Failed to Modify List | O |  | 9.3.1.13 |  | - | - |
| Unsuccessful Cell List |  | 0 .. <maxnoofCellsingNB> |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.7 |  | - | - |
| >Cause | O |  | 9.3.1.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsingNB | Maximum no. of cells that can be serviced by a gNB. Value is 16384. |

#### 9.3.A.c3	Multicast Session Update Failure Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |

## *****************Next changes*******************

//asn.1 part to be added

## *****************End of the changes*******************