3GPP TSG-RAN WG3 Meeting #114bis-e 	   R3-220720

Online, 17 – 26 January, 2022

Title: 	Discussion on open issues of RA-SDT without anchor relocation

Source: 	CATT

Agenda item:	24.2

Document for:	Discussion and Decision

1. Introduction

In the previous RAN3 e-meetings, on RA-based SDT, the following agreements, working assumptions and open issues are summarized in [1]:

Common for RA-SDT

For CG based SDT, RAN3 will further discuss impacts and mainly consider split-gNB case.

Subsequent UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported for SDT

The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements. Details will be discussed later.

UL data for SDT is buffered at the receiving node in the successful context retrieval procedure. For other cases, the common understanding is that UL data may need to be buffered as well, details are pending.

The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of assistance information are pending future discussion

For RA-SDT, “SDT Indicator” is introduced in RETRIEVE UE CONTEXT REQUEST message, and the message may include other SDT Assistance Information.

For split CU/DU case, gNB-DU sends “SDT indicator”to gNB-CU in the INITIAL UL RRC MESSAGE TRANSFER message.

During SDT procedure, SRB PDCP PDU (FFS on the first SDT payload) shall be transferred between new gNB and anchor gNB, either via extending the XnAP RRC TRANSFER message or via defining a new XnAP class-2 procedure.

Specific for RA-SDT without anchor relocation

For RA-SDT without anchor relocation case, SDT related UE context may be retrieved from anchor gNB to the receiving gNB.

For RA-SDT without anchor relocation case, for SDT DRB transfer, its stage2 overall procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. For SDT SRB transfer is FFS.

The receiving gNB forwards PDCP PDUs (RLC SDUs) via GTP-U tunnel per SDT DRB to the anchor gNB.

In case of no anchor relocation, the anchor gNB is not allowed to transfer its security related IEs included in “UE Context Information – Retrieve UE Context Response” IE to the receiving gNB. (refer to SA3 LS: R2-2109065)

SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB.

How to include those information is FFS.

Enhance RNA update without UE context relocation procedure for RA-SDT without anchor relocation case

FFS: How to transfer SDT SRB PDCP PDU

Extending the XnAP: RRC TRANSFER message or defining a new XnAP class-2 procedure.

Carrying the first SRB PDCP PDU in the RETRIEVE UE CONTEXT REQUEST message.

FFS: The detail information included in SDT related RLC bearer configuration to be transferred from anchor gNB to receiving gNB in case of SDT without anchor relocation.

FFS: If agreed to enhance Periodic RNA update without UE context relocation procedure, whether it is acceptable to use the Retrieve UE context failure message to send SDT related UE context information

FFS: whether to allow optional transport of first DRB payload in the RETRIEVE UE CONTEXT REQUEST message.

To be continued...

In this contribution, we will further discuss the open issues to support RA-based SDT and provide the observations and proposals accordingly.

2. Discussion

2.1 Handling of SDT SRB

On handlinf of SDT SRB signallings, we had some discussion in the last meeting, some initial agreements and FFSes could be found in the chairman notes [1]:

During SDT procedure, SRB PDCP PDU (FFS on the first SDT payload) shall be transferred between new gNB and anchor gNB, either via extending the XnAP RRC TRANSFER message or via defining a new XnAP class-2 procedure. For RA-SDT without anchor relocation case, for SDT DRB transfer, its stage2 overall procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. For SDT SRB transfer is FFS. SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB. How to include those information is FFS. FFS: How to transfer SDT SRB PDCP PDU

In case RA-SDT without anchor relocation, the handling of SDT SRB signallings should be similar as the SDT DRB data. The RLC handling should be done in the receiving gNB, and then the PDCP PDU should be forwarded to the anchor gNB for further proceeding.

On handling of the first SDT SRB payload, two options:

Option 1: Handle it using the same principle as it for other SDT SRB signallings and SDT DRB data.

Option 2: Include it in the Retrieval UE Context Request message.

Fot the option 1, RLC proceeding for the SDT SRB signalling should be done in the receiving node, and then the PDCP PDU is transferred to the anchor gNB. The transfer of the PDCP PDU should be done after the SDT related RLC bearer configuration for SDT DRB/SRB is provided to the receiving gNB from the anchor gNB.

For the option 2, the handling of the 1st SDT signalling is different with the subsequent ones, the signalling is included in the Retrieval UE Context Request message. As the RLC context for SDT SRB has not been provided to the receiving node, the RLC PDU is included in the Retrieval UE Context Request message.

We do not see big gains to go for the option 2. Firstly, SDT feature does not have strict latency requirement. Secondly, it requires the anchor gNB to handle the 1st SDT signalling even if it decides to relocate the anchor for SDT. Thirdly, in case of CU-DU split architecture, the impact to the anchor gNB is even worse, as the anchor gNB-CU have to select one gNB-DU to proceed with RLC handling for the forwarded RLC PDU, and then send back the PDCP PDU to the anchor gNB-CU.

Above all, the option 1 is preferred, i.e. RLC handling is proceed in the receiving node for all the SDT signallings and data.

Proposal 1: For the first SDT SRB signalling, RLC handling should be done in the receiving node after SDT related RLC bearer configuration is obtained.

Following the discussion above, the 1st SDT SRB should not be transferred in the Retrieval UE Context Request message. On how to transfer the SDT SRB signallings between the receiving gNB and the anchor gNB, either reusing the XnAP RRC TRANSFER message or defining a new XnAP class 2 message is feasible.

However, the existing RRC TRANSFER message is designed for dual connection. It seems easier to reuse the RRC TRANSFER message with necessary enhancement/extension.

Proposal 2: RRC TRANSFER message could be reused with necessary extension to transfer the UL/DL SRB PDCP PDU (including the first one) during SDT procedure between new gNB and anchor gNB.

2.2 Define and Transfer SDT RLC Conext

Following the agreements of the last meeting, SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB.

In Xn interface, the RRC Context in In Retrieval UE Context Response message is encapsulated as HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331. The RLC-BearerConfig is included in the CellGroupConfig as part of the RRC Context. The content of the RLC-BearerConfig is pasted in the Annex A as the reference.

In the last meeting, several options were discussed on how to transfer the SDT RLC Context between the receiving gNB and the anchor gNB, e.g. reuse the Retrieval UE Context Response/Failure, define new Class 1 or class 2 messages.

No matter which message is adopted, the SDT related RLC configuration need to be transferred for SRB/DRB(s). Such kind of info should not be exposed in the Xn interface, more appropriate way is to define a message in RAN2 to carry the SDT related RLC configuration.

Proposal 3: RRC Container should be used to transfer the necessary RLC configuration for SDT.

The SDT related configuration should include at least the RLC-BearerConfig, whether need to transfer the other configuration to the receiving gNB is pending to RAN2. RAN2 could further check if the current RRC context in Retrieval UE Context Response message could be reused for SDT.

Observation 1： At least the RLC-BearerConfig should be transferred from the anchor gNB to the receiving gNB for SDT handling, whether need to transfer the other configuration is pending to RAN2.

Therefore, we should send an LS to RAN2 to ask them to further work on the definition of RRC Container totransfer the necessary RRC context for SDT handling.

Proposal 4: Discuss and agree the LS to RAN2 [2] to request for defining of RRC Container for RLC-BearerConfig and or the other information.

3. Conclusion

In this contribution, we mainly discussed how to handle the SDT SRB signallings especially for the first one, and also discussed how to define and transfer SDT RLC context between gNBs. Based on the discussion we provide the following observations and proposals:

Proposal 1: For the first SDT SRB signalling, RLC handling should be done in the receiving node after SDT related RLC bearer configuration is obtained.

Proposal 2: RRC TRANSFER message could be reused with necessary extension to transfer the UL/DL SRB PDCP PDU (including the first one) during SDT procedure between new gNB and anchor gNB.

Proposal 3: RRC Container should be used to transfer the necessary RLC configuration for SDT.

Observation 1： At least the RLC-BearerConfig should be transferred from the anchor gNB to the receiving gNB for SDT handling, whether need to transfer the other configuration is pending to RAN2.

Proposal 4: Discuss and agree the LS to RAN2 [2] to request for defining of RRC Container for RLC-BearerConfig and or the other information.

4. Reference

- RAN3#114e Chairman Notes

- R3-220721 Draft LS on defining a RRC container for SDT related RLC bearer config, CATT

5. Annex A RLC configure

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

#### –	RLC-BearerConfig

The IE RLC-BearerConfig is used to configure an RLC entity, a corresponding logical channel in MAC and the linking to a PDCP entity (served radio bearer).

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v1610                            RLC-Config-v1610                                    OPTIONAL    -- Need R

]]

}

-- TAG-RLC-BEARERCONFIG-STOP

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

UL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-PollRetransmit                    T-PollRetransmit,

pollPDU                             PollPDU,

pollByte                            PollByte,

maxRetxThreshold                    ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly,

t-StatusProhibit                    T-StatusProhibit

}

UL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL    -- Cond Reestab

}

DL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly

}

T-PollRetransmit ::=                ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610,

ms4-v1610, spare1}

PollPDU ::=                         ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=                        ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=                    ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=                ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=                ENUMERATED {size6, size12}

SN-FieldLengthAM ::=                ENUMERATED {size12, size18}

RLC-Config-v1610 ::=                SEQUENCE {

dl-AM-RLC-v1610                     DL-AM-RLC-v1610

}

DL-AM-RLC-v1610 ::=                 SEQUENCE {

t-StatusProhibit-v1610              T-StatusProhibit-v1610                               OPTIONAL,   -- Need N

...

}

T-StatusProhibit-v1610 ::=          ENUMERATED { ms1, ms2, ms3, ms4, spare4, spare3, spare2, spare1}

-- TAG-RLC-CONFIG-STOP