3GPP TSG-RAN WG3 #114bis-e                                                                         R3-220884

Online, 17th – 26th January 2022

Agenda Item:	22.2.2

Source:	CMCC, Huawei

Title:	(TP to TS 38.300) MBS session management over NG

Document for:	Approval

1	Introduction

This is a text proposal for TS 38.300 BLCR on the MBS session management over NG based on the discussion paper.

- Text proposal to TS 38.300 BLCR

--------------------------------------------------------------Start of the change---------------------------------------------------------

16.x	NR MBS

16.x.1	General

Editor’s Note: General aspects to be covered here.

16.x.2	Architecture

Editor’s Note: Architecture aspects to be covered here.

The overall NG-RAN architecture specified in section 4 applies for NR MBS.

16.x.3	Session Management

Editor’s Note: Session Management aspects to be covered here.

#### 16.x.3.1	QoS Model

The following QoS model applies to both multicast and broadcast:

- An MBS Session Resource may be associated with one or more MBS QoS flows.

- Each MB QoS flow is associated with a QoS profile.

Editor’s Note: whether 5GC sends MBS Session AMBR to NG-RAN and how NG-RAN node would handle it is FFS.

Editor’s Note: specification of applicability of QoS flow QoS parameters and PDU Session parameters to an MBS Session Resources is expected to be specified. How to reference to TS 23.501 (by SA2) is FFS.

#### 16.x.3.2		MBS Session Resources for Multicast

The 5GC sets up MBS Session Resources and the linkage between MBS session and associated PDU session in the NG-RAN node using the PDU Session Resource Modify procedure.

MBS Session Resources consist of an MBS context in the NG-RAN node, control plane resources, NG-U resources and radio resources.

There are two types of delivery modes as defined in TS 23.501:

-	MBS shared delivery mode

-	MBS individual delivery mode

In MBS shared delivery mode, the MBS context is setup in the NG-RAN node when the first UE joins the MBS session in the NG-RAN node and released when the last UE leaves the MBS session in this NG-RAN node.

In MBS shared delivery mode, shared NG-U resource is used. The NG-RAN node triggers the setup of the shared NG-U resources when the MBS context is setup and triggers the release of the shared NG-U resources when the MBS context is removed using the MBS distribution procedure.

The shared NG-U resources consist of one shared NG-U tunnel per MBS session which can be of the following types:

-	one shared GTP-U tunnel per NG-RAN node if unicast transport is used.

-	one shared GTP-U tunnel across all involved NG-RAN nodes if multicast transport is used. The multicast transport is identified by one multicast IP address and one source IP address.

### 16.x.3.2		MBS Session Resources for Broadcast

--------------------------------------------------------------End of the change---------------------------------------------------------