3GPP TSG-RAN WG3 Meeting #114b-e	R3-220910

E-meeting, 17-26 Jan 2022

Title: 	Stage 2 TPs to 38.300 on QoE measurement

Source: 	Huawei

Agenda item:	15.2.1.2

Document Type:	Discussion and Approval

Introduction

This document provides text proposal to TS38.300 for NR QoE based on our contributions [1] [2] [3].

References

[1]		R3-220912,	Further discussions on RAN visible QoE metrics, Huawei

[2]		R3-220911,	Further discussions on mobility support of QoE measurement, Huawei

[3]		R3-220913,	Further discussions on alignment between QoE measurement and MDT measurement, Huawei

Text Proposal to TS38.300

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MCE	Measurement Collection Entity

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TCE	Trace Collection Entity

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

<Next modification>

# X	 QoE Measurement Collection

  - X.1	 Overview

The QoE Measurement Collection function enables collection of application layer measurements from the UE.

The measurements are supported for the following service types:

-	Streaming services;

-	MTSI services.

-	VR services.

Both signalling based and management based QoE measurement collection are supported.

  - X.2	 QoE Measurement Configuration

### X.2.1	 QoE Measurement Collection Activation and Reporting

The feature is activated in the NG-RAN either by direct configuration from the OAM system (management based activation), or by signalling from the OAM via the CN (signalling based activation), using UE-associated siganlling. One or more QoE measurement collection jobs can be activated at a UE per service type. The UE reports the QoE measurement results to the NG-RAN node in uplink RRC message(s), and the NG-RAN node transfers the received QoE measurement results as report(s) to the MCE.

For signalling-based QoE, the OAM initiates the QoE measurement activation for a specific UE via the Core Network, and the NG-RAN node receives the QoE measurement configuration by means of UE-associated signalling. The QoE measurement configuration for signalling-based activation includes application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The NG-RAN node forwards the corresponding QoE measurement configuration(s) to the UE in a downlink RRC message.

For management-based activation, the OAM sends the QoE measurement configurations to the NG-RAN node. The QoE measurement configuration for management-based activation includes application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The NG-RAN node selects the UE(s) that meet the required QoE measurement capability, Area Scope and Slice Scope.

Editor’s Note: The detailed content of QoE configuration needs to be revisited based on the outcome of each subtopic in RAN2 and RAN3.

The UE reports QoE measurement results to the NG-RAN node in an uplink RRC message. The NG-RAN node transmits the QoE report and the corresponding QoE Reference ID to the MCE.

The application layer measurement configuration and measurement reporting are supported in RRC_CONNECTED state only. Application layer measurement configuration received by the gNB from OAM or CN is encapsulated in a transparent container, which is forwarded to a UE in a downlink RRC message (there can be multiple configurations in the same message). Application layer measurement report received from UE's higher layer are encapsulated in a transparent container and sent to the network in an uplink RRC message, as specified in TS 38.331 [12]. An RRC identifier conveyed in the RRC signalling is used to identify the QoE configuration and report between the gNB and the UE. The RRC identifier is mapped to the QoE Reference in the gNB. The QoE measurement report is forwarded to OAM together with the QoE Reference.  gNB can release multiple application layer measurement configurations from the UE in one RRC message at any time.

Upon reception of QoE release message, the UE discards any unsent QoE reports corresponding to the released application layer configuration. The UE discards the reports received from application layer when it has no associated QoE configuration configured.

### X.2.2	 QoE Measurement Collection Deactivation

QoE Measurement Collection deactivation permanently stops all or some of QoE measurement collection jobs towards a UE, resulting in the release of the corresponding QoE measurement configuration(s) in the UE. The deactivation of QoE measurement collection is supported by using UE-associated siganlling. A list of QoE Reference is used to deactivate the corresponding QoE measurement collection job(s).

Upon reception of QoE release message, the UE discards any unsent QoE reports corresponding to the released application layer configuration. The UE discards the reports received from application layer when it has no associated QoE configuration configured.

### X.2.3 RAN overload QoE handling

QoE Measurement Collection pause/resume procedure is used to pause/resume the reporting for all QoE reports or to pause/resume QoE reporting per QoE configuration in a UE in case of RAN overload.

Application layer measurements can be paused by the gNB using a downlink RRC message that is used to temporarily stop application layer measurement reports from being sent from the UE to the network.

### X.2.4 QoE Measurement handling in RRC_IDLE and RRC_INACTIVE

f the UE enters RRC_INACTIVE, the UE AS configuration for the QoE is stored in the UE Inactive AS context.

If the UE enters RRC_IDLE state, the UE releases all the QoE measurement configurations.

Editor’s Note: The structure of TS 38.300 BLCR needs further discussion.

### X.2.5	 Per-slice QoE Measurement

When a service is provided within a configured slice, the QoE Measurement for this service type could also be configured together with the corresponding slice scope, so that the user experience of this service could also be evaluated on a per-slice basis. The UE includes the slice ID inside the QoE report container when reporting QoE measurement result (pending SA4 decision).

  - X.3	 QoE Measurement Continuity for Mobility

The QoE Measurement Collection continuity for intra-system intra-RAT mobility is supported, with the area scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the area. For RRC_CONNECTED state mobility, the source NG-RAN node may transmit the signaling based QoE Measurement configuration(s) of a specific UE to the target NG-RAN node via XnAP. The source NG-RAN node also may transmit the QoE measurement information of a specific UE to the target NG-RAN node via XnAP or NGAP. For RRC_INACTIVE state mobility, signaling-based (FFS) QoE measurement configuration(s) of a specific UE can be restored from the node hosting the UE context when it resumes to RRC_CONNECTED state. Multiple sets of QoE measurement configurations should be supported during mobility.

For signalling based QoE, at handover to a target gNB which supports QoE, the target gNB decides which QoE configurations to keep and which to release, e.g. based on QoE configuration information received from the source gNB in Xn/NG signalling.

When the UE resumes the connection in a gNB supporting QoE, the UE keeps QoE measurement configurations indicated by the target gNB and releases the QoE measurement configurations which are not indicated by the target gNB for restoration. When the UE resumes the connection in a gNB not supporting QoE, the UE releases all QoE measurement configurations.

Editor’s Note: Further descriptions pending the work progress in RAN2.

  - X.4	 RAN Visible QoE Measurement

AN visible QoE measurement is configured by the NG-RAN node, where a subset of QoE metrics are reported from the UE. A set of QoE values could also be used for RAN visible QoE measurement. Both QoE metrics and QoE values could be utilized by the NG-RAN node for optimization. RAN visible QoE measurements are supported for the DASH streaming and VR services. In the RAN visible QoE measurement, the UE also reports the PDU session ID and the QoS flow IDs corresponding to the service that is measured outside the QoE report container.

ditor’s Note: This section is pending work in other WGs, e.g., FFS on whether QoE value(s) can be reported from the UE or derived by the NG-RAN node, and the specific definition of RAN visible QoE values (if agreed, pending SA4 decision).

  - X.5	 Alignment of MDT and QoE Measurements

Radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The immediate MDT can be configured before the QoE measurement is configured or the two could be configured simultaneously. The TCE/MCE is responsible for the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. The NG-RANincludes time stamp information and related ID information for the MCE/TCE to enable the correlation of corresponding measurement results of MDT and QoE. The C-RNTIs of UE in each cell and UE mobility history info might also be needed.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>