3GPP TSG-RAN3 Meeting #114-bis-e	R3-220925

Electronic meeting, 17 - 26 January 2022

Agenda item:	17.2

Source: 	Samsung

Title: 	(TP for BL CR TS 38.300) Support of Service Continuity

Document for:	Other & Agreement

Introduction

In RAN3 114e Meeting, RAN3 had discussed Multi-Carrier Resource Sharing Solution and Configuration based on Re-partitioning Solutions, RAN3 made some agreements and had below open issues.

To be continued:

- Pre-emption aspects (existing tools enough?, is clarification needed for the interaction between the existing tools QoS framework (ARP) and RRMPolicyRatio ?)

- Whether a slice resource change indication should be included in the handover request acknowledge message?

In this contribution, we’d like further discuss the open issues above.

Discussion

Configuration based solution

For configuration based solution, in [2], the following are summarized in study phase:

But the following needs to be further studied, e.g., for the S-NSSAI 1,

-	it can explicitly use resources belonging to which S-NSSAIs;

-	it can use the dedicated but not used resources of other S-NSSAIs;

-	it can preempt the used prioritized and/or shared resources from other S-NSSAIs.

In last RAN3 meeting, pre-emption was discussed and had below open issues need to be discussed.

- Pre-emption aspects (existing tools enough?, is clarification needed for the interaction between the existing tools QoS framework (ARP) and RRMPolicyRatio ?)

Considering whether the un-used dedicated resources can be used for other slices had not been discussed, so we’d like further discuss the below two aspects,

- Slice resource pre-emption

- Un-used dedicated resources usage

Pre-emption

According to the RRMPolicy defined in TS 28.541, there’re three types of resources for different slice(s) group member, the details are in the following.

-	Shared resources: means the resources that are shared with other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The shared resources are not guaranteed for use by the associated rRMPolicyMemberList. The shared resources quota is represented by [rRMPolicyMaxRatio-rRMPolicyMinRatio].

-	Priortized resources: means the resources are preferentially used by the associated RRMPolicyMemberList. These resources are guaranteed for use by the associated RRMPolicyMemberList when it needs to use them. When not used, these resources may be used by other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The prioritized resources quota is represented by [rRMPolicyMinRatio-rRMPolicyDedicatedRatio]

-	Dedicated resources: means the resources are dedicated for use by the associated RRMPolicyMemberList. These resources cannot be shared even if the associated RRMPolicyMember does not use them. The Dedicated resources quota is represented by [rRMPolicyDedicatedRatio].

In the last meeting, below stage 2 descriptions had been agreed, but after the further check, these descriptions have already been supported by current specification, as a slice can be configured to the multiple rRMPlolicyMemberLists, which means e.g. slice 1 can be associated with dedicated resources, Prioritized resources and shared resources at the same time, if the dedicated resources for slice 1are not available, slice 1 can use the resources from the shared pool or/and prioritized pool.

“The Resource Repartitioning allows a slice to use resources from the shared pool or/and prioritized pool when its own dedicated or prioritized resources are not available as specified in TS 28.451 [xx].”

Observation 1, the stage 2 descriptions relating to Resource Repartitioning have already been supported by current specifications.

On the other hand, if the resources from Shared resources and the un-used Prioritized resource) a be pre-empted for other slices with high priority in the same rRMPlolicyMemberList and the dedicated resource cannot be pre-empted for any slices in the associated rRMPlolicyMemberList, the SLA of the slices associated for prioritized and dedicated resources will not be impacted by the slice resources pre-emption.

Observation 2, there will have no impact on the SLA of the slices associated for un-used prioritized and dedicated resources, if the Shared Resources and the un-used Prioritized Resources can be pre-empted for slice resource pre-emption.

Proposal 1, RAN3 agrees a slice can pre-empt the resources in the shared pool or/and prioritized pool when the pools are overloaded.

Regarding how to realize the slice resource pre-emption, using QoS framework (ARP) for the slice resources pre-emption was discussed in the last meeting, we would like to further discuss it in this meeting.

In current specification, the QoS framework (ARP) is per QoS flow, but the slice resource pre-emption is per PDU session. If we apply the ARP parameters in PDU session level pre-emption, it may work as below.

| Allocation/Retention Priority | How it may work for a PDU session associated with a slice |
| --- | --- |
| >Priority Level | Using the highest priority among all QoS flows for slice resource pre-emption comparison. |
| >Pre-emption Capability | If at least one of the QoS flows can trigger pre-emption, the PDU session may trigger pre-emption. |
| >Pre-emption Vulnerability | If all the QoS flows in the PDU session can be pre-emptable, the PDU session can be pre-emptable. |

If using the above operation for PDU session level pre-emption, some QoS flows with low ARP priority may get a free ride with the QoS flows with high ARP priority in the same PDU session, which seems not appropriate and unfair for all QoS flows. So the per slice pre-emption mechanism is needed for slice resource pre-emption.

Observation 3, it is not appropriate to use current QoS framework (ARP) for PDU session level pre-emption as low priority QoS flows may get a free ride along with the high priority QoS flows in the same PDU session associated with the same slice (i.e. ARP and slice resource pre-emption are from different dimensions).

Proposal 2, RAN3 agrees to introduce a per-slice (or per-PDU session) pre-emption mechanism.

To realize per-slice (or per-PDU session) pre-emption, the PDU session level priority, Pre-emption capability and Pre-emption Vulnerability need to be introduced when PDU session setup or modified over NGAP, XnAP, F1AP and E1AP.

Proposal 3, RAN3 agrees to introduce per-slice (per-PDU session) priority level, Pre-emption capability and Pre-emption Vulnerability in PDU session setup and modified related messages over NGAP, XnAP, F1AP and E1AP.

Unused dedicated resources

We understand the dedicated resources is used for better isolation among different slices, however, if the dedicated resources are not used for a while, it would be flexible if the un-used resources can be used for other slices. If un-used dedicated resources can be used for other slices, some conditions for using this kind of resources are needed to keep the SLA of the slices associated with the dedicated resources, such as

- The un-used dedicated resource can only be used for other slices during a specific time.

- If the un-used dedicated resource is used for other slices, the corresponding PDU sessions/QoS flows can be pre-empted anytime when the PDU session associated with dedicated resources are requested.

- Only some critical PDU sessions but not associated with the slices of dedicated resources can use the un-used dedicated resources.

The using conditions can be realized by OAM implementation or some assistance information (e.g. whether the PDU session is allowed to use the un-used dedicated resources).

Observation 4, it would be beneficial if the un-used dedicated resources can be used in a more flexible way without impact the SLA of the slices associated with the dedicated resources.

Proposal 5, the un-used dedicated resources can be used for other slices under some conditions without impact the SLA of the slices associated with the dedicated resources

Proposal 5bis, if the un-used dedicated resources are agreed to be used for other slices, RAN3 further discuss whether the conditions are up to implementation or assistance information from CN.

Resource change report

As we discussed in the last meeting, if the slice resource change (e.g. pre-emption or un-used dedicated resources are used) is performed during mobility, the change of slice resource indication should be notified to the source gNB, so that the source gNB can consider this change to make handover decision in case of CHO (e.g. the source gNB may not select the gNB with slice resource change indication as a candidate for the UE), or the source gNB can consider this change to make better handover decision for the following handover decision (e.g. the source gNB may not select the gNB with slice resource change indication as a target for the following handover UEs).

Companies commented that the slice resource change indication in the handover request acknowledge message needs GAP analysis with the Resource status exchange in SON, and RAN3 had below open issue for further discussion,

- Whether a slice resource change indication should be included in the handover request acknowledge message?

So we did below GAP analysis on this issue.

| Slice resource change items | Slice related Resource status exchange |
| --- | --- |
| According to TS 28.541, the Resourcetype assigned for a slice can be - PRB (for NRCellDU) - RRC connected users (for NRCellCU) - DRB (for GNBCUUPFunction) | According to current BL CR in SON WI, the slice resource is - Slice PRB usage - Slice capacity |

According to the gap analysis above, if the slice resource shortage is due to lack of PRB resources, the changed resource can be represented implicitly by slice PRB usage in Resource status exchange related messages over Xn. But if the slice resource shortage is due to lack of RRC Connected users and/or DRB, there’s no corresponding slice related resource status exchange over Xn.

Observation 4, current slice related resource status exchange over Xn cannot represent the slice resource change due to lack of RRC connected users and DRB usage for a requested PDU session associated with a specific slice.

Proposal 6, the slice resource change indication including resource type (at least RRC connected users, DRB) should be included in handover request acknowledge message to notify the source gNB for making better handover decision.

Stage 2 update

Based on the above discussion, we propose a TP in Annex for BL CR TS 38.00.

Proposal 7, RAN3 agree the TP in the Appendix.

Conclusion

In this paper, we discuss the pre-emption issue and un-used dedicated resource usage, and also discussed why and how to report slice resource change for better handover decisions. The following are observations and proposals.

Observation 1, the stage 2 descriptions relating to Resource Repartitioning have already been supported by current specifications.

Observation 2, there will have no impact on the SLA of the slices associated for un-used prioritized and dedicated resources, if the Shared Resources and the un-used Prioritized Resources can be pre-empted for slice resource pre-emption.

Proposal 1, RAN3 agrees a slice can pre-empt the resources in the shared pool or/and prioritized pool when the pools are overloaded.

Observation 3, it is not appropriate to use current QoS framework (ARP) for PDU session level pre-emption as low priority QoS flows may get a free ride along with the high priority QoS flows in the same PDU session associated with the same slice (i.e. ARP and slice resource pre-emption are from different dimensions).

Proposal 2, RAN3 agrees to introduce a per-slice (or per-PDU session) pre-emption mechanism.

Proposal 3, RAN3 agrees to introduce per-slice (per-PDU session) priority level, Pre-emption capability and Pre-emption Vulnerability in PDU session setup and modified related messages over NGAP, XnAP, F1AP and E1AP.

Observation 4, it would be beneficial if the un-used dedicated resources can be used in a more flexible way without impact the SLA of the slices associated with the dedicated resources.

Proposal 5, the un-used dedicated resources can be used for other slices under some conditions without impact the SLA of the slices associated with the dedicated resources

Proposal 5bis, if the un-used dedicated resources are agreed to be used for other slices, RAN3 further discuss whether the conditions are up to implementation or assistance information from CN.

Observation 4, current slice related resource status exchange over Xn cannot represent the slice resource change due to lack of RRC connected users and DRB usage for a requested PDU session associated with a specific slice.

Proposal 6, the slice resource change indication including resource type (at least RRC connected users, DRB) should be included in handover request acknowledge message to notify the source gNB for making better handover decision.

Proposal 7, RAN3 agree the TP in the Appendix.

References

Appendix TP for BL CR TS 38.300

#### 16.3.3.1	Handling of Slice Resources

The NG-RAN node may use Multi-Carrier Resource Sharing or Resource Repartitioning to allocate resources to a slice.

In Multi-Carrier Resource Sharing the RAN node can setup the dual connectivity or carrier aggregation with different frequency and overlapping coverage where the same slice is available.

Editor’s Note: whether the “new cell” can be outside the RA of the UE is FFS.

The Resource Repartitioning allows a slice to pre-empt the resources in the shared pool or/and prioritized pool when the pools are overloaded.

The Resource Repartitioning allows a slice to use the un-used dedicated resources under conditions without impact the SLA of the slice associated with the dedicated resources when the shared pool or/and prioritized pool are overloaded.