3GPP TSG-RAN WG3 #115-e	R3-222566

21st February – 3rd March 2022

E-Meeting

Agenda Item:	30.2

Source:	Ericsson - Moderator

Title:	Summary of Offline Discussion – MultiSIM

Document for:	Approval

# Introduction

This is the SOD document for the following come back:

CB: # 19_MultiSIM

- Continue the discussion on remaining open issues

- Capture agreements and provide TPs/CRs if agreeable

(E/// - moderator)

Summary of offline disc R3-222411

Deadline by 23rd, Wednesday 12h UTC (before the online session)

# For the Chairman’s Notes (Second round)

Agreements (from the second round):

About the Paging Cause Coding: It is agreed to use A) “Define the IE directly in the tabular”, as in the example of XnAP/ F1AP BL CRs. It is agreed that all the BL CRs are aligned with this coding. S1AP and NGAP BL CRs need to apply the change. FFS are removed from all the BL CRs related to this.

Revise the Draft 38.300 CR to resolve FFS.

Agree the below CRs:

- R3-222699

- R3-222664

- R3-222705

- R3-222706

- R3-222707

# Two issues for further discussion after the Second round

## Related to Draft 38.300 CR comments received

Two comments received:

- Instead of using “neighboring NG-RAN nodes”, proposed to use “other NG-RAN nodes in RNA;

- DRX related text seems not relevant.

Any comments?

| - Company | - Comments |
| --- | --- |
| - Ericsson | - Neighboring NG-RAN nodes seems to be the term used in the other similar context. Otherwise no strong view. - Agree. |

## On the MUSIM Gap topic

There are basically two major views:

- gNB-CU receives the preferred MUSIM gap from the UE, gNB-CU picks up the one to be used and sends the chosen MUSIM gap to gNB-DU.

- gNB-DU generates the MUSIM gap and sends it to gNB-CU.

- Proposal 3.2.1: F1AP would need to be updated so that the MUSIM Gap is sent from gNB-CU to gNB-DU

- Any comments?

| - Company | - Comments |
| --- | --- |
| - Ericsson | - Yes, we agree. |

- Proposal 3.2.2: F1AP would need to be updated so that the MUSIM Gap is sent from gNB-DU to gNB-CU

- Any comments?

| - Company | - Comments |
| --- | --- |
| - Ericsson | - We do not think gNB-DU needs to generate the MUSIM Gap. We think gNB-CU picks one MUSIM Gap from the preferred MUSIM Gap and uses it. - However we also know there is another view that gNB-DU generates MUSIM-Gap. - As A way forward on this topic, the protocol could support both cases: - gNB-CU to pick up one from UAI - gNB-CU to request gNB-DU to generate MUSIM, gNB-DU signals back the MUSIM Gap. |

# Second round discussion:

## “Paging Cause” coding implementation of Option 1

There are two ways to implement Option 1 and both ways are used in the specification. Usually when the IE is used only once, it is defined directly in the message; if the IE is used multiple times, it is defined as a new IE.

- Define the IE directly in the tabular: Taking XnAP/ F1AP BL CRs as an example:

- 9.1.1.7 RAN PAGING This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE. Direction: NG-RAN node1  NG-RAN node2. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. Unchanged IEs not included …. Paging Cause; O; ENUMERATED(voice, …); YES; ignore.

- Define a new IE, Taking NGAP BL CR as an example:

- 9.2.4.1 PAGING This message is sent by the AMF and is used to page a UE in one or several tracking areas. Direction: AMF  NG-RAN node IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. Unchanged IEs not included …. Paging Cause; O; 9.3.1.xx; YES; ignore. - 9.3.1.xx Paging Cause This IE is used to notify about a paging cause for paging a UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Paging Cause; M; ENUMERATED (voice, …).

Question 2.1: Please indicate you preferred One choice only.

| - Company | - Or B) | - Comments |
| --- | --- | --- |
| - Ericsson | - A) | - Choice A) is easy to read as all the information is in one line. - The IE is used only once, and it will not be used in any other messages. |
| - ZTE | - A) | - Same view as E/// |
| - Huawei | - A | - Agree with Ericsson and ZTE |
| - Radisys |  | - No Strong opinion. Both are ok. |
| - vivo | - A |  |
| - Samsung |  | - Both are fine. |
| - CATT | - A |  |
| - Nokia | - A |  |

Question 2.2:	Should we adapt the 2.1 decision to all the BL CRs?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson |  | - No strong view, as long as the IE naming is the same. |
| - ZTE |  | - The IE naming shall be the same, others shall try to align as much as possible. |
| - Huawei | - Yes | - Prefer align all the BL CRs |
| - Radisys |  | - No strong view |
| - vivo | - No strong view |  |
| - Samsung | - Yes | - Prefer align. |
| - CATT | - No strong view, prefer align |  |
| - Nokia | - Yes |  |

- Moderator Summary 2.1:

- Proposal 2.1: It is agreed to use A)	Define the IE directly in the tabular, as in the example of XnAP/ F1AP BL CRs. It is agreed that all the BL CRs are aligned with this coding.

## Endorsement of Draft CR R3-221930 on TS 38.300

At the last meeting, it is agreed to produce a draft CR on TS 38.300.

The input document is R3-221930  Introduction of Multi-USIM Support for TS 38.300 captured two changes. The first change is in “9.2.2.1 Overview”:

The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes the registration area configured for the UE, the Periodic Registration Update timer, and the UE Identity Index value, and may include the UE specific DRX, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, the Expected UE Behaviour, the UE Radio Capability for Paging and Paging Cause Support Indication (naming FFS). The UE registration area is taken into account by the NG-RAN node when configuring the RNA. The UE specific DRX and UE Identity Index value are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RNA Update timer. The NG-RAN node takes into account the Expected UE Behaviour to assist the UE RRC state transition decision. The NG-RAN node may use the UE Radio Capability for Paging during RAN Paging. The NG-RAN node takes into consideration the Paging Cause Support Indication (naming FFS) to include the Paging Cause in RAN Paging for a UE in RRC_INACTIVE state. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the NR Paging eDRX Information, the Paging Cause may be included.

Question 2.3: Do you agree on the first change on “9.2.2.1 Overview”? Please answer with Yes/No with comment.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - Yes | - Please fix the “FFS” |
| - ZTE | - Yes | - Agree with E/// |
| - Huawei | - Yes |  |
| - Radisys | - Yes | - FFS will be removed and name will be changed to “Paging Cause Indication for Voice Service” in the revision |
| - vivo | - Yes |  |
| - Samsung | - Yes | - Fix “FFS”. |
| - CATT | - Yes | - One comments, whether the wording “neighbour NG-RAN node(s)” is corrected? - We may change to “When sending XnAP RAN Paging to other NG-RAN node(s) in the RNA,” |
| - Nokia | - OK but | - Need to remove FFS, and also the NR eDRX part seems not relevant here. |

The second change in “9.2.5 Paging”:

For operation with shared spectrum channel access, a UE can be configured for an additional number of PDCCH monitoring occasions in its PO to monitor for paging. However, when the UE detects a PDCCH transmission within the UE's PO addressed with P-RNTI, the UE is not required to monitor the subsequent PDCCH monitoring occasions within this PO. If Paging Cause is included in the Paging message, a UE in RRC_IDLE or RRC_INACTIVE state may use the Paging Cause as per TS 23.501[3].

Question 2.4: Do you agree on the second change on “9.2.5 Paging”? Please answer with Yes/No with comment.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - Yes | - No comment. |
| - ZTE | - Yes |  |
| - Huawei | - Yes |  |
| - Radisys | - Yes |  |
| - vivo | - Yes |  |
| - Samsung | - Yes |  |
| - CATT | - Yes |  |
| - Nokia | - OK |  |

- Moderator Summary 2.2:

- Proposal 2.2: Revise the Draft 38.300 CR to resolve FFS, and also discuss the comments received and implement the comments when agreed:

- Instead of using “neighboring NG-RAN nodes”, proposed to use “other NG-RAN nodes in RNA

- DRX related text seems not relevant.

## MUSIM Gap from gNB-DU to gNB-CU

- It is proposed to Introduce a MUSIM GapConfig IE in the DU to CU RRC Information in F1AP to specify the MUSIM gap configuration.

- In the first round, companies had expressed their choice. Further technique discussion is needed.

Question 2.5:  Should gNB-DU signal MUSIM Gap to gNB-CU via F1AP? Please state the technique aspects.

| - Company | - Comments |
| --- | --- |
| - Ericsson | - Our understanding of MUSIM gap is that: - UE signal to gNB the preferred MUSIM gap; - gNB pick up one or adjust; - gNB confirm to UE via RRC. - This is very different than the existing MeasGap, in which gNB-DU is supposed to generate the Gap. - With this, we do not see the need for gNB-DU to generate one and send to gNB-CU. |
| - ZTE | - No strong view. |
| - Huawei | - Yes. We acknowledge the understanding of MUSIM gap by Ericsson, but this is not contradicted to that in a CU-DU split structure, it is DU that decide the gap. Step 2 is decided by DU, it is DU that pick up one/adjust. Note that, as we already mentioned, if we look at the details of the captured MUSIM-GapConfig-r17, it includes many DU-controlled lower layer information such as frame number/subframe number, which clearly indicates that DU should be responsible for this. Then in a CU-DU structure, we can expand the whole procedure as: - Before switching from Network A, UE sends the UE assistance information (UAI) to the CU indicating the preferred MUSIM gap (suggested gapoffset, gapLength etc) in Network A while temporarily switching to network B; - The CU sends the UAI to the DU in the CU to DU RRC information - Based on that, gNB-DU picks up one or adjust and signals the determined MUSIM gap configurations to the CU (this is where we need MUSIM GapConfig IE in the DU to CU RRC Information in F1AP), which can be sent to the UE via RRC Hope this clarifies.  |
| - Radisys | - Agree with HW. - MeasGapConfig is available in DU to CU container. Since MeasGapConfig does not include the MUSIM GapConfig IE, it needs to be introduced. |
| - vivo | - No strong view. |
| - Samsung | - Agree with Huawei. - And in our understanding, the DU has to know which configuration for MUSM gap is used. The UAI doesn’t carry the selected MUSIM gap configuration. |
| - CATT | - Technically agree with Huawei |
| - Nokia | - Same understanding as Huawei. |

- Moderator Summary 2.3:

- On the MUSIM Gap topic, two major views:

- gNB-CU receives the preferred MUSIM gap from the UE, gNB-CU needs to send the chosen MUSIM gap to gNB-DU.

- gNB-DU generates the MUSIM gap and sends it to gNB-CU.

- Proposal 2.3: It is proposed to discuss this topic a bit further.

## Summary of the BL CR updates needed

Moderator summarize the updates needed at the end of this meeting, in order to complete MUSIM WI.

- Revision of R3-221573	Introduction of MultiSIM support over S1, ZTE;

- Revision of R3-221574	Introduction of MultiSIM support over NG, Nokia,

- Revision of R3-221575	Introduction of MultiSIM support over Xn, Ericsson

- Revision of R3-221576	Introduction of MultiSIM support over F1, Huawei

- Depends on the comments on Q2.2 and Q2.3, we may need to make a revision of;

R3-221930	Introduction of Multi-USIM Support for TS 38.300, RadiSys

- Withdraw R3-221577	Introduction of MultiSIM support over E1

Question 2.6: What is your view on the above summary, please answer Yes/No with comment

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - Yes | - The revisions shall implement all the agreements. |
| - ZTE | - Yes |  |
| - Huawei | - Yes |  |
| - Radisys | - Yes |  |
| - vivo | - Yes |  |
| - Samsung | - Yes |  |
| - CATT | - Yes |  |
| - Nokia | - Yes |  |

- Moderator Summary 2.4:

- Proposal 2.4: The revisions are requested.

# First Round Conclusion (on the 23rd of February)

Agreements (from the First round):

- It is agreed to use naming “Paging Cause Indication for Voice Service”, and the coding is “supported, …”. This is align with the SA2 specification.

- RAN3 understands that the presence of the “RAN2 list” indicates the gNB/eNB supports the transmission of paging cause for service indication. If the service is "voice", the list will include the Paging cause "voice". If the service is other than IMS voice, the list will be empty.

- It is proposed to remove the Paging cause from DL DATA NOTIGICATION in E1AP. Then the E1AP BL is not needed.

- It is agreed NOT to introduce the new NGAP cause value “Release due to MUSIM”.

- Agree Option1 in this meeting, if needed, extending the cause value in future. Stage3 details need to be decided in the 2nd Round.

- Moderator Note: Issue 2 was not discussed online and will be further discussed at the second round.

- Issue 2:  MUSIM gap configuration from DU to CU

- It is proposed to discuss this further if the MUSIM gap is generated in DU, thus it is needed to be sent from gNB-DU to gNB-CU.

# First Round SOD Summary (For the Chairman’s Notes)

Agreements (from the First round):

- It is agreed to use naming “Paging Cause Indication for Voice Service”, and the coding is “supported, …”. This is align with the SA2 specification.

- RAN3 understands that the presence of the “RAN2 list” indicates the gNB/eNB supports the transmission of paging cause for service indication. If the service is "voice", the list will include the Paging cause "voice". If the service is other than IMS voice, the list will be empty.

- It is proposed to remove the Paging cause from DL DATA NOTIGICATION in E1AP. Then the E1AP BL is not needed.

- It is agreed NOT to introduce the new NGAP cause value “Release due to MUSIM”.

To be discussed online:

Issue 1: Paging Cause coding

Option 1: Ericsson, Huawei, CATT, Radisys, Nokia, Samsung, Qualcomm

Option 2: ZTE, Vivo, Samsung

Option 3: ZTE, Vivo, Nokia, Samsung

Technique questions/discussions from SOD:

- How can CN indicate gNB that one UE is paged for non-voice?

- RAN2 does not include anything like “non-IMS voice” in the coding.

- How can gNB differentiate these two cases? Case 1: if UE and/or AMF doesn’t support the feature, the AMF will not include the Paging Cause IE.  Case 2: If instead the UE and AMF support the feature and the service is non IMS voice, the AMF will not include the Paging Cause IE.

- We can simply perform that if gNB/eNB support the transmission of paging cause for service indication, then if CN pages the UE with “voice” as paging cause, RRC will send Paging Cause “voice”; else RRC will send “empty list”

Issue 2:  MUSIM gap configuration from DU to CU

It is proposed to discuss this further if the MUSIM gap is generated in DU, thus it is needed to be sent from gNB-DU to gNB-CU.

# First round SOD discussion:

## Remaining Issue 1: “Paging Cause support indication” IE: naming and Coding

By input document [1] ZTE, [3] Nokia, [5] Huawei, [9] Ericsson, [10] Samsung,

It is proposed to use the same naming and coding as in SA2 TS 23.501, naming “Paging Cause Indication for Voice Service”, coding “supported, …”

By the input document [12] CATT:

It is proposed to use naming “Paging Cause support indication”, coding “voice …”

Question 1: It is proposed to use naming “Paging Cause Indication for Voice Service”, coding “supported, …” Please indicate your view.

| - Company | - Comments |
| --- | --- |
| - Ericsson | - Support the proposal. - It is much easier to read the specification when SA2 TS and RAN TS using the same terminology. |
| - Huawei | - Agree. Align with SA2 is the easiest way. For the good of the topic progress, it is suggested to take this proposal unless companies have strong opposed views. |
| - ZTE | - Yes, agree. |
| - CATT | - Yes, if majority companies agree, I can accept this proposal. My original thinking is for future proof |
| - vivo | - Agree |
| - Samsung | - Support the proposal. |
| - Radisys | - Same view as CATT. Prefer future proof. |
| - Nokia | - OK. Good to align with SA2. |
| - Qualcomm | - Agree |

- Moderator Summary 1:

- Proposal 1: It is proposed to use naming “Paging Cause Indication for Voice Service”, coding “supported, …”

## Remaining Issue 2: “Paging Cause” IE: Coding

There are two Options presented at the RAN3#114bis-e:

- Paging Cause IE Coding.  Choose from the below two Options:

  - Option 1: “voice, …”

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | M |  | ENUMERATED (voice, …) |  |

  - Option 2: A new IE structure as in below: This IE indicates the paging cause for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present, it implies that the corresponding paging entry is for a service other than IMS voice. |

Option 1: By input document [5] Huawei, [9] Ericsson, [12] CATT:

It is proposed that the coding of ‘Paging Cause IE’ should be ‘ENUMERATED (voice, …).

Option 2: By input document [1] ZTE, [10] Samsung,

It is proposed to use the “Paging Record” proposal.

Option 3: By input document [3] Nokia,

It is proposed for the paging cause in Paging message, use the name “Paging Cause” and encode in an ENUMERATED (IMS voice, non IMS voice).

The discussion in RAN3 is related to that RAN2 has agreed “to provide the paging cause value “voice” to UEs by extending the Paging message. The presence of the extension, which can be empty list if none of the UEs have paging cause “voice”, indicates that network supports transmission of paging cause for service indication”,

It is clear from RAN2 that the empty list is used to indicate if the network supports transmission of paging cause, to differentiate from the Rel 16 network, which would not support the transmission.

Question 2: The presence of the “RAN2 list” indicates the gNB/eNB supports the transmission of paging cause for service indication. If the service is "voice", the list will include the Paging cause "voice". If the service is other than IMS voice, the list will be empty.

Do you agree? Answer in Yes/No with comment.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - Yes | - We support the above understanding. - gNB/eNB knows if it supports the transmission of the paging cause for service indication. CN only needs to indicate if the paging cause is for “voice”. |
| - Huawei | - Yes | - We also acknowledge this. |
| - ZTE |  | - My understanding is that when gNB/eNB sends Paging message to UE including the empty “RAN2 list”, it means that the service is non-IMS voice service. - My concern is that how the gNB/eNB knows this kind of service? I think it shall be indicated by CN. I.e., CN shall indicate it to gNB/eNB via NG/S1 paging message. |
| - CATT | - Yes |  |
| - vivo | - Yes |  |
| - Samsung | - Yes, but | - The extension for paging cause in RRC paging message shall be present to indicate the network support of paging cause even though the extension includes empty list. - “The presence of the extension, which can be empty list if none of the UEs have paging cause “voice”, indicates that network supports transmission of paging cause for service indication.” - So it’s better to avoid including the extension when gNB/eNB sends a Paging message only to non-MUSIM UEs. With option 1, gNB/eNB can’t determine whether to include the extension IE with empty list or not in Paging message. |
| - Radisys | - Yes |  |
| - Nokia | - Yes |  |
| - Qualcomm | - Yes |  |

Three Options are listed here for clarity:

Option 1: (if to introduce a new IE, otherwise it is defined directly as ENUMERATED)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | M |  | ENUMERATED (voice, …) |  |

Option 2: A new IE is defined. The Semantics describes more details.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present, it implies that the corresponding paging entry is for a service other than IMS voice. |

Option 3: (if to introduce a new IE, otherwise it is defined directly as ENUMERATED)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | M |  | ENUMERATED (iMS voice, non IMS voice) |  |

Question 3: In the above three options, what is your preferred option? Answer in Choice with comments.

| - Company | - Choice | - Comments |
| --- | --- | --- |
| - Ericsson | - Option 1 | - The empty list in RAN2 specification is related to if gNB/eNB support the transmission of paging cause, there is no need to involve CN. - The solution that CN only needs to indicate “voice” in the paging cause is also beneficial in the RAN sharing scenario. |
| - Huawei | - Option 1 | - Option 2 seems best matches RAN2 conclusion, but not really needed. As pointed out in our paper, gNB/eNB anyway has the knowledge of whether itself supports the Paging Cause feature, and whether the MME/AMF supports this feature. Option 3 is thus also not needed, and RAN2 does not include anything like “non-IMS voice” in the coding.. Therefore, with option 1, gNB/eNB has enough knowledge to meet RAN2’s requirements, and we prefer option 1. |
| - ZTE | - Option 2 and Option 3 are fine | - For Option 1, if the service is non-IMS voice, how can CN transfer this service type to gNB/eNB who is supporting Multi-SIM feature? |
| - CATT | - Option 1 | - Option 1 is simple |
| - vivo | - Option2 or option3 | - We have the same question as ZTE, with option1, how can CN indicates gNB that one UE is paged for non-voice. |
| - Samsung | - Option 2 or Option 3 | - If majority companies think the overhead of the extension presence in RRC paging message is negligible, option 1 is also acceptable to us. |
| - Radisys | - Option 1 | - Agree with E/// and HW |
| - Nokia | - Option 1 or option 3 | - Option 1 is the simplest. - The question with option 1 though is how can gNB differentiate these two cases? - Case 1: if UE and/or AMF doesn’t support the feature, the AMF will not include the Paging Cause IE. - Case 2: If instead the UE and AMF support the feature and the service is non IMS voice, the AMF will not include the Paging Cause IE. - But I assume gNB should in case 1 not send the extension over the radio, whereas in case 2 the gNB should send the extension with empty. - This is why tdoc [3] wonders if a second codepoint is needed like option 3? -> need to clarify case 1 and case 2. |
| - Qualcomm | - Option 1 | - Option 1 is simpler. We don’t need this IE to carry UE/AMF/RAN capability information. |

- Moderator Summary 2:

- Proposal 2: RAN3 understands that the presence of the “RAN2 list” indicates the gNB/eNB supports the transmission of paging cause for service indication. If the service is "voice", the list will include the Paging cause "voice". If the service is other than IMS voice, the list will be empty.

- Proposal 3: to discuss further the option, taking into account of the offline questions/discussion already have in the SOD.

## Remaining Issue 3: New Cause value over NGAP “Release due to MUSIM”,

By input document [3] Nokia discussed Cause for switching:

In case of NAS Leave, the AMF will send NG Release Command to gNB, If we don’t have a new dedicated cause value, this could hit KPI i.e. this is a specific intentional release, not a failure release.

It is proposed to introduce a new specific cause value in TS 38.413 for release due to MUSIM NAS Leave.

By input document [9] Ericsson, the benefit to introduce a new cause value is not seen.

Question 4:  Do you see the need to differentiate the Context Release due to MUSIM NAS leave? Answer in Yes/No with comments.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - No | - NAS leave is a NAS based solution. If AMF makes a decision to release UE context, it is said to use a legacy release. We have not differentiated all the other causes for AMF release, it is thus difficult to motive this new release cause. |
| - Huawei | - Slightly No |  |
| - ZTE | - Yes | - It is useful for KPI. |
| - CATT |  | - No strong opnion |
| - vivo |  | - No strong view |
| - Samsung | - Slightly Yes | - It might be used for KPI. |
| - Radisys |  | - Agree with E/// |
| - Nokia | - Slightly Yes | - We prefer but no strong view. |
| - Qualcomm | - No | - Don’t think AMF initiated NG release downgrades KPI. |

- Moderator Summary 3:

- Proposal 4: It is agreed not to introduce the new NGAP cause value.

## MUSIM gap configuration from DU to CU

By input document [5] Huawei,

- It is proposed to Introduce a MUSIM GapConfig IE in the DU to CU RRC Information in F1AP to specify the MUSIM gap configuration.

Question 5:  Should gNB-DU signal MUSIM Gap to gNB-CU via F1AP? Answer in Yes/No with comments.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Huawei | - Yes | - By checking the elements of MUSIM-GapConfig-r17, it is mainly used by the DU to determine when not schedule the UE so that the UE can access another network. Since the gap configuration is configured at DU, a new MUSIM GapConfig IE should be included in the DU to CU RRC Information IE. |
| - ZTE | - Prefer No | - We are fine to follow majority company’s view. |
| - Samsung | - Yes | - We have the similar view as Huawei. |
| - Radisys | - Yes | - MeasConfig does not contain the new IE MUSIM-GapConfig-r17. Hence agree to add it in DU to CU RRC Container. |
| - Nokia | - Yes | - Also same view as Huawei and Samsung. |
| - Qualcomm | - Yes | - Agree with Huawei. |
| - Ericsson |  | - The difference with the MeasGap is that it is generated in DU. - MUSIM Gap is proposed by UE, and gNB choose one and confirms to the UE. - Therefor not sure if it is needed to be generated by DU. |

- Moderator Summary 4:

- Proposal 4: It is proposed to discuss this further.

  - Remove the paging cause from DL DATA NOTIFICATION in E1AP

By the input documents [5] Huawei and [12] CATT: we already agree add the Paging Cause in DL DATA NOTIFICATION in E1AP. But from SA2 specification TS23.501 [2] the paging cause should interpret from Paging Priority Indicator (PPI) which already include in the message DL DATA NOTIFICATION. So the CU-CP also can derive the paging cause from Paging Priority Indicator (PPI).

For a UE in RRC-Inactive, the NG-RAN should provide the Voice Service Indication in the RAN Paging message only when there is Paging Cause Indication for Voice Service indication in the UE context and detects the downlink data which triggers the RAN Paging message is related to voice service based on the Paging Policy Indicator, in the header of the received downlink data, as specified in clause 5.4.3.2

It is proposed to remove/rediscuss the paging cause from DL DATA NOTIFICATION in E1AP

Question 6: Should the Paging cause from DL DATA NOTIGICATION in E1AP be removed? (Please provide Yes/No and comments)

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Huawei | - Yes | - CU-CP can deduce whether the paging is originated due to voice service with receiving the PPI. |
| - ZTE | - Prefer No | - We are fine to follow majority company’s view |
| - CATT | - Yes | - We should avoid introducing duplication information. |
| - vivo | - Yes | - Duplication information is not needed. |
| - Samsung |  | - We’re fine with removing it depending on majority company’s view. |
| - Radisys | - No Strong view | Text from 23.501 Clause 5.4.3.2 - NOTE 1: This PPD feature may be used to determine the Paging Cause Indication for Voice Service, as described in clause 5.38.3. |
| - Nokia | - Yes | - Support removal. |
| - Qualcomm | - Agree | - CU-CP should rely on PPI. |

- Moderator Summary 5:

- Proposal 5: It is proposed to remove the Paging cause from DL DATA NOTIGICATION in E1AP.

# References

- R3-221792	(TP for BLCR Introduction of MultiSIM support over S1) Support of Multi-SIM	ZTE

- R3-221793	(TP for BLCR Introduction of MultiSIM support over NG/Xn/F1) Support of Multi-SIM	ZTE

- R3-221820	(TP for TS 38.413) Conclusions on Multi-USIM devices	Nokia, Nokia Shanghai Bell

- R3-221821	(TP for TS 38.473) Conclusions on Multi-USIM devices	Nokia, NOkia Shanghai Bell

- R3-221926	Discussion on MUSIM leftover issues 	Huawei

- R3-221927	(TP to BL CR 38.473) MUSIM gap configuration	Huawei

- R3-221930	Introduction of Multi-USIM Support for TS 38.300 (RadiSys, Reliance JIO, Ericsson)	draftCR

- R3-222035	TP on Introduction of MultiSIM support over Xn	Ericsson

- R3-222036	MUSIM reaming issues	Ericsson

- R3-222117	(TP for LTE_NR_MUSIM CR for TS 38.413) Discussion on open issues for MUSIM UE 	Samsung

- R3-222118	(TP for LTE_NR_MUSIM CR for TS 38.473) Discussion on supporting MUSIM gap configuration		Samsung

- R3-222215	Discussion on Multi-SIM	CATT

- R3-222216	(TP to BLCR for TS 38.463) Multi-SIM CATT