3GPP TSG-RAN WG3 #115-e																	      R3-22xxxx

21th Feb – 3rd Mar 2022

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Draft | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RA-SDT BLCR to TS 38.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-01-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Support Rel-17 SDT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the agreed TP R3-216070 including a bit typo. Capture the agreed TP R3-221204 and R3-221346 Remove some editor’s note according to the progress and fix some typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SDT cannot be supported in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | XX.1, XX2, XX3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-216274 Rev1: R3-220076 Rev2: R3-221562 Rev3: R3-222591 |  |  |  |  |  |  |  |  |

Change Begins

- XX	Small Data Transmission

<Omit content from RAN2>

- XX.1	Support of RACH based SDT

For RACH based SDT, the UE in RRC_INACTIVE initiates RACH and requests RRC resume together with UL SDT data. If the UE accesses a gNB other than the last serving gNB, the UL SDT data is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (i.e., single packet, multiple packets) may also be provided by the receiving gNB to help the decision.

If the last serving gNB decides not to relocate the UE context, at least SDT RLC context transfer information necessary for the receiving gNB to handle SDT is retrieved to the receiving gNB.

If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, SRB PDCP PDUs  is transferred between the receiving gNB and the last serving gNB via XnAP RRC Transfer procedure.

When the receiving gNB receives DL non SDT data or DL non-SDT signalling from core network during RACH based SDT with anchor relocation procedure, the receiving gNB may send the UE to RRC_CONNECTED state directly by sending the RRC Resume message.

When the last serving gNB receives DL non SDT-data or DL non-SDT signalling from core network during RACH based SDT without anchor relocation procedure, it could move the UE back to RRC_INACTIVE by generating RRCRelease message.

- XX.2	RA-based SDT with UE context relocation

The overall procedure for RACH based small data transmission with UE context relocation is illustrated in the figure XX.2-1.

Figure XX.2-1. RA-based SDT with UE context relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provide SDT assistance information.

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT, and initiates NG-AP Path Switch Request procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the UL SDT data is delivered to the UPF, the UL NAS PDU, if any, is delivered to AMF.

7. After SDT transmission is completed, the receiving gNB generates and sends RRCRelease message including the Suspend indication to the UE to move the UE back to RRC_INACTIVE.

NOTE: In case DL non-SDT data or DL non-SDT signalling arrives, the receiving gNB may decide to send the UE to RRC_CONNECTED state by sending RRCResume message.

8. The receiving gNB indicates the last serving gNB to remove the UE context by sending UE CONTEXT RELEASE message. The UE CONTEXT RELEASE message can be sent after step 6.

- XX.3	RA-based SDT without UE context relocation

The overall procedure for RACH based small data transmission without UE context relocation is illustrated in the figure XX.3-1.

Figure XX.3-1. RA-based SDT without UE context relocation

1.  	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provide SDT assistance information.

3.  	The last serving gNB decides not to relocate UE context for SDT.

4.  	The last serving gNB transfers the SDT related RLC context.

5. 	The receiving gNB acknowledges receiving SDT related RLC context and forwards associated DL TNL address, if needed. After Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF.

6. 	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes Suspend Indication.

7.	The receiving gNB forwards the RRCRelease message to the UE.

8. 	The UE moves to RRC_INACTIVE mode.

Change Ends