3GPP TSG-RAN WG3 #115-e	R3-222419

Online, 21.02 – 3.03 2022

Agenda Item:	31.2.5

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on MLB enhancements 
(CB #SONMDT3)

Document for:	Approval

# Introduction

CB: # SONMDT3_LoadBalance

- Whether to introduce per-slice MSC?

- Check the details on per-SSB MSC

- Remove the Editor’s note, capture agreements and provide the TPs if agreeable

(Nok - moderator)

Summary of offline disc R3-222419

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion (2nd round)

## Information about the serving beam

Question 8: At the online discussion, it was questioned if an gNB may be able to apply special HO criteria to a UE served in a selected (overloaded) beam, because the CU is not aware of a beam that its own UEs are served in. Please comment.

| Company | Comment |
| --- | --- |
| Nokia | According to our understanding, the problem is here: an gNB can’t apply special HO handling to UEs from one beam only. In the measurement report, a UE reports measured beam (possible target), but not the own one. |

Question 9: If you acknowledge the issue, how do you prefer to resolve it: (a) remove per-beam MSC, or (b) enable providing serving beam information from the DU?

| Company | Comment |
| --- | --- |
| Nokia | We are fine with either of the approaches. What is not acceptable, is leaving the solution with negotiations of the target beam only – is the node initiating it can’t modify mobility for own UEs, modifying HO criteria at the neighbour gNBs will not help much, while may create huge HO discrepancy. |

## Per-MIMO PRB reporting

Question 10: Can TPs in [8] and [9] be agreed?

| Company | Comment |
| --- | --- |
| Nokia | They are all right and match the XnAP TP agreed at RAN3 #114-bis. However, perhaps the reporting can be enclosed in a separate block (like e.g. “PRB usage for MIMO”), which will then be included as a mandatory IE in the Radio Resource Status. This will help legibility of the standard. |

## Stop mechanism

Question 11: Please, indicate if you consider adding a stage-2 clarification for the ‘stop’ mechanism in NR. If yes, please, indicate what change and where could be added.

| Company | Comment |
| --- | --- |
| Nokia | We do not think such clarification is needed at this meeting, but we are fine to have it, if majority prefers so. |

# Discussion (1st round)

## Per-slice mobility setting change

Two companies propose to discuss further and enable per-slice MSC ([1],[2]). Other two comment that it shall not be considered ([6],[7]). This has not changed much since the last meeting. However, in [2], it is suggested that operators shall provide their view.

Question 1: A question to operators mainly: considering the limitations discussed in [2], is the per-slice MSC possibly valuable?

| Company | Answer (Yes/No) | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| CMCC | Yes, but | As far as we know, the discussion on slice priority in RAN2 and SA2 is mainly focused on per UE priority, which means that different UEs may have different slice priority order list. And per slice MSC in RAN3 is actually discussing on a per cell slice priority order list, for which in our opinion is of little use when different UEs have different slice priorities. So the discussion on per cell slice priority order list, including potential new requirement on OAM/CN is not needed from our understanding. Moreover, since the HO is performed on a per UE basis, if NG-RAN node is able to obtain the slice priority for each UE, then the source node is able to perform HO request based on the per UE slice priority information; and under such condition, the negotiation of per slice HO trigger offset will be valuable. However, considering this is the last meeting in R17, and we may have little chance to guarantee that the NG-RAN node has been able to obtain the slice priority for each UE (for which in our opinion is still under discussion in SA2) in the remaining time of R17, we would like to re-open the discussion in R18 when more progress in RAN2 and SA2 is achieved. |
| Deutsche Telekom | Tends to No | For 1:1 mapping between slice and service/QoS flow, it may bring benefits, but we also note that service-related MSC wasn’t introduced in 3G/4G systems. For cases where slices acting as logical network for business customers support different services/QoS flow types we don’t see any benefits. Furthermore, if a UE is simultaneously connected to several slices, the need of slice prioritization is additionally required which requires clarification by SA2 and/or RAN2. |
| BT | Yes | We believe it is beneficial to enable per-slice MSC. Although the solution does not differentiate between services inside the slice, we believe it is still beneficial to enable a HO trigger on a slice level while still being able to maintain the hysteresis between cells. Operators have some flexibility of which services will be included within the slice, and only a subset of the slices may need to be handled differently with MLB. |

## Per-beam mobility setting change

The solution has been agreed at the last meeting, but details remain still undecided. The first issue is the parameter to negotiate. In [2],[4] and [7], it is propose to change the current definition in the BL CR to the HO Trigger.

Question 2: Considering support, can RAN3 agree to use the HO Trigger for the per-beam MSC, instead of elected parameters currently listed in the BL CR?

| Company | Answer (Yes/No) | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes | There may be an issue with the own HO Trigger change – please, see Q3 below. |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |
| CMCC | Yes |  |
| Deutsche Telekom | Yes |  |
| CATT | Yes |  |
| Huawei | Yes |  |
| BT | Yes |  |
| ZTE | Yes |  |

In [4], there is a possible problem mentioned: in disaggregated CU-DU architecture, the CU is not aware of serving beam for own UEs. It can’t therefore apply a modified HO triggering to own UEs in the overloaded beam. To address this issue, the serving beam information is proposed in [5] to be added to F1. Alternatively, a beam may be switched off completely.

Question 3: Do you acknowledge the issue mentioned in [4]?

| Company | Answer (Yes/No) | If the answer is ‘yes’, do you agree for the solution proposed in [5], or do you prefer switching off the overloaded beam? If the answer is ‘no’, how the overloaded node can select UEs to be applied beam-specific HO trigger? |
| --- | --- | --- |
| Nokia | Yes | We propose both, the information on serving beam or the beam switching off. The latter is more “total” solution, so the information on serving beam, as proposed in [5], may be better. |
| Ericsson | No | It is still unclear why per-SSB MSC needs to be connected to individual UEs. For beam-level MSC we should follow the same principle adopted for cell-specific MSC, that is the proposed SSB offset should not be associated to any specific UE but the changes negotiated among nodes should relate to RAN information (in this case SSB offset). Therefore, the negotiation of per SSB offsets should be disconnected from specific UEs served by the CU/DU. Our understanding is that the CU has enough knowledge and therefore has all the information it needs |
| Qualcomm | No | Same view as Ericsson. |
| Samsung |  | We don’t understand why a beam may be switch completely. |
| CMCC |  | We are also not sure why we need additional per UE MSC on top of the existing per cell/per SSB MSC. The serving beam information itself may be useful, but it might not be in the scope of MSC. |
| Deutsche Telekom | No | We share the same view as Ericsson and other companies. |
| CATT | No | This can be covered (although not very precise maybe) by the measurement report from the UE. Considering the “neighbour beam” used is also from the measurement report, making the “current beam” from it as well makes sense anyhow. So no need for a separate indication as it is quite costly. |
| Huawei | No | Same view as Ericsson. Alternatively, we can even simplify the negotiation to be between source cell and target cell/beams not considering source beams. |
| BT | No | Same view as Ericsson. |
| ZTE | No | Same view as Ericsson. |

Also in [4], it is further proposed to enable a flag indicating that a beam is to be switched off. This can help to handle CHO UEs (but in [6], it is proposed not to consider CHO), but also to handle beam overload, if it is not known which UEs use the overloaded beam.

Question 4: Do you agree to enable a flag or value indicating that a beam is proposed to be switched off to address the issues listed above?

| Company | Answer (Yes/No) | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes | It does not need to be used, while it offers bigger robustness and helps address CHO users. |
| Ericsson | No | Beam deactivation is not a way to balance load, but to save energy when a beam is not loaded, or to change beam shape when necessary for CCO. We do not see the link to use MOBILITY CHANGE REQUEST and MOBILITY CHANGE ACKNOLWEDGE to negotiate beam deactivation. This is part of CCO at best. |
| Qualcomm | No | Same view as Ericsson |
| Samsung | No | Same view as Ericsson |
| CMCC |  | Same view as Ericsson |
| Deutsche Telekom | No | We don’t see the benefit for this indicator for a beam to be switched off. As already stated by Ericsson this may be a topic for CCO. |
| CATT | No | Same view as Ericsson |
| Huawei | No | Could be in CCO |
| BT | No | Same view as Ericsson |
| ZTE | No | Same view as Ericsson |

## Completion of the reporting of the load in cells that may be aggregated with the reporting cell

At RAN3 #114-bis, a solution for reporting load (CAC) in cells that may be aggregated with the reporting cell was agreed. Some details were still marked as “editor’s note”. At this meeting, there are proposals for removing it:

- In [3] and [6], the note is removed. In [3], ale some other edits are made.

- In [7], it is also proposed to enable indicating a minimum level of CAC for a cell to be included.

Question 5: Do you agree for the above changes that will enable completing the solution?

| Company | Suported solution | Comments, if any is not acceptable |
| --- | --- | --- |
| Nokia | (1) | Regarding (2), the thresholds do not seem cell-specific. Therefore, it seems more like something that an operator may set globally in its network – signalling is not needed. |
| Ericsson | (1) partially, (2) | (1) In order to limit the signalling, the number of reported cells which can be aggregated should be limited. One possibility would be to lower the maximum number of items, which is actually too high in the TP (2) This information will help the reporting node to select the PSCells/SCells which fulfil the load requirements from the requesting node. This is not static (e.g. depends on the requesting node load and overload requirements) and therefore cannot be configured by OAM |
| Qualcomm | - – OK - – No strong view | For (1), also please correct typo: Neithbour  Neighbour NOTE: In order to avoid duplication, the eNB2 may include only one copy of NR Neithbour Cell Measurement Result Item per RESOURCE STATUS UPDATE messages per NR neighbour cell, even if this NR cell neighbours to multiple E-UTRA cell served by eNB2. The eNB1 should deduce which NR cell is capable of performing EN-DC with the corresponding served E-UTRA cell by using the NR Neighbour Information IE received during the X2 Setup procedure or the eNB Configuration Update procedure. The eNB2 should only include NR Neithbour Cell Measurement Result Item for NR cells that neighbour to at least one cell served by eNB1. |
| Samsung |  | For (2), we share the same view as Nokia. |
| CMCC | (1)-OK, (2)-No strong view | We share view with QC. |
| Deutsche Telekom | (1) Yes (2) Yes, but … | (1) QC’s comment to be considered. (2) The proposal to have a measure to limit the overall number of PSCells/SCells to be exchanged is fine for us, but only one of the proposed approaches should be applied. Our preference would be a threshold to be given by the requesting node. |
| CATT | (1)-Agree, (2)-Not quite good | We prefer the neighbour to report a CAC for multiple purposes rather than a Boolean of whether the cell is above or below a threshold. When making the decision of handover, many inputs should be considered. A smart method is to use a smooth abstract function and compares the output of this function to a given abstract threshold. The input should be as smooth as possible to achieve the best performance so a 100-grade CAC is much better. |
| Huawei | (1) | We share the opinions with Nokia. |
| BT | (1), (2) maybe | For (2) agree that minimum level of CAC would not be static and dependent on the requesting node load. But question if this extra filtering would be required if the number reported cells was limited as per Ericsson's suggestion. |
| ZTE | (1) | For (1), the editorial change by Qualcomm should be merged. For (2), share the view with Nokia |

## Other proposals

There are some further proposals:

- In [7], it is proposed to enable partial stop of resource reporting in NR.

- In [8] and [9], it is proposed to add the per-MIMO PRB reporting to X2 and F1.

Question 6: Do you agree for the above proposals?

| Company | Suported solution | Comments, if any is not acceptable |
| --- | --- | --- |
| Nokia | (2), possibly (1) | We understand that (2) is just completion of the agreement made at RAN3 #114-bis. Regarding (1), it is all right, as a copy of the LTE solution – but the TP in (1) does not seem to include the proposal – so how is it supposed to look like? |
| Ericsson | (1) and (2) partially | (1) Solution is to add Cell Reporting Indicator IE into RESOURCE STATUS UPDATE message, at cell level. Please check the revised TP in R3-222508 in the inbox. (2) Per-S-NSSAI PRB usage is optional in the Radio Resource Status IE. Shouldn’t be the case for all rel-17 PRB usage enhancement (e.g. PRB usage for MIMO)? Also, in order to clarify that the PRB usage for MIMO is per cell, we propose to change the name to “Cell DL GBR PRB usage” keeping the definition in the semantics description only |
| Samsung | In general ok for (1) and (2). | Will check the detail of the TP |
| CMCC | Support (2); No strong view on (1) | To reply Ericsson’s comments on (2), we do not think the addition of per cell PRB usage is just an enhancement in R17. Please recall that the reason we only adopt per SSB PRB usage was because all of us thought at that time that the receiving node was able to calculate per cell PRB usage according to per SSB PRB usage, so it was not needed for the reporting node to additionally report per cell PRB usage. However, as observed by us during last meeting, the receiving node is actually not able to calculate the exact per cell PRB usage merely by the received per SSB PRB usage, so it is necessary for the reporting node to report per cell PRB usage. In addition, in operators’ current network, per cell PRB usage is sometimes more important for making load balancing decisions at the receiving node than per SSB PRB usage. So the per cell PRB usage (i.e. PRB usage for MIMO) shall be reported mandatorily. Moreover, since we’ve already enabled the measurements of PRB usage for MIMO for L2 measurement, meaning that the measurements has already been there for R17 base stations, then why not reporting these measurements also for load reporting, considering the fact that per cell PRB usage is deemed to be important for LB decision making. |
| Deutsche Telekom | (1) and (2) | Both proposals are fine with us. |
| Huawei | (2) yes (1) no - see comments | (1) As stated before we prefer implicit method. But the problem with (1) is that this ties the reporting problem to cell resources. If we assume that the HW resources are more pooled in NG-RAN compared to LTE it could make sense to allow the requesting node to make a request not limited to removing cells but rather reduce the general scope (reduce the types of measurements or measurements in other cells handled by this node) Also, when the requesting node sends the updated request, the reporting node will be able to reject if the requesting node does not adjust the measurement scope in a suitable way. So we prefer the implicit method without limitations. Otherwise we have a system where the reporting node more or less decides what measurements should be done. |
| BT | (1) and (2) |  |
| ZTE | Only (2) | For (1), share the view with Huawei. |

Since RAN3 is closing the discussion on Rel.17 MLB enhancement, is there anything else that requires attention at this meeting?

Question 7: Please, provide any comments, if there are any issues that require attention before the work on Rel.17 MLB enhancements is closed?

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-221777, Per-Slice Mobility Settings Change (NEC)

- R3-221831, (TP for SON BLCR for 38.423) Load Balancing Enhancements (Huawei)

- R3-221913, (TP for 36.423) Support of PSCell MLB (CATT)

- R3-221935, [TP to SON BL CR for TS 38.423]  Per-beam mobility setting change (Nokia, Nokia Shanghai Bell)

- R3-221936, [TP to SON BL CR for TS 38.473]  Per-beam mobility setting change (Nokia, Nokia Shanghai Bell)

- R3-222066, Further Discussion on Load Balancing Enhancements in NR (ZTE, China Telecom)

- R3-222068, (TP for SON BL CR for TS 38.423, TS 36.423) MLB enhancements (Ericsson)

- R3-222258, (TP for SON BLCR to 38.473) PRB usage for MIMO (CMCC)

- R3-222259, (TP for SON BLCR to 36.423) PRB usage for MIMO (CMCC)