3GPP TSG-RAN WG3 Meeting #116-e	R3-223494

Electronic Meeting, May 9th – 19th, 2022

Agenda item:	9.1.3.1

Source: 	Intel Corporation

Title: 	Corrections for Rel-17 SCG

Document for:	Discussion and Decision

# Introduction

For Rel-17 SCG, we spotted the following three issues that needs to be corrected:

- Weird interaction descriptions in X2/XnAP?

- Correction for E1AP

- Correction for TS 38.401

In this contribution, we explain these issues and propose how to fix them.

# Discussion

## Interaction descriptions in X2/XnAP?

When SN requests SCG (de)activation via SN-initiated SN modification procedure, currently there are interaction descriptions with SN Addition procedure:

In TS 32.423:

Interaction with the SgNB Addition Preparation procedure:

If the SCG Activation Request IE was included in the SGNB ADDITION REQUEST message, the en-gNB may use the SCG Activation Request IE in the SgNB initiated SgNB Modification procedure.

In TS 38.423:

Interaction with the S-NG-RAN node Addition Preparation procedure:

If the SCG Activation Request IE was included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use the SCG Activation Request IE in the S-NG-RAN node initiated S-NG-RAN node Modification procedure.

for which, the purpose of these interaction descriptions (with the SCG Activation Request IE during SN Addition procedure) is not clear..

Moreover, they don't provide meaningful context as all they say is that SN "may" use the IE included in the SN ADDITION REQUEST message. From this "may" statement, we are not sure what information these interaction descriptions are trying to convey.

Looking up the history, those interaction descriptions were inherited from the first TPs (R3-212924, R3-212909) agreed at the RAN3#112-e meeting into SCG BLCRs for X2/XnAP where no context was found in the summary of discussions (R3-212784).

Also such interactions seems to have never been captured in stage-2 SCG BLCR for TS 37.340 (R2-2203690).

Given this situation, the best way seems to remove those unclear descriptions (unless we are mistaken).

Proposal 1: For SCG (de)activation request via SN-initiated SN modification procedure, remove the interaction descriptions with SN Addition procedure in X2/XnAP.

## Correction for E1AP

For SCG activation/deactivation, RAN3 adopted the following principles over X2/Xn/F1/E1.

- Over X2/Xn

  - During SN addition, partial rejection is allowed only for "deactivation" request from MN (i.e. SN is allowed to reject and feedback "activation" without failing the whole procedure). For "activation" request, if cannot be obeyed, SN shall fail the whole procedure with the appropriate cause value.

  - For MN-initiated SN modification, partial rejection is allowed by SN for both "activation" and "deactivation" request from MN.

  - For SN-initiated SN modification, partial rejection is not allowed by MN for both "activation" and "deactivation" request from SN.

- Over F1

  - F1 UE context setup follows the SN addition principles over X2/Xn.

  - F1 UE context modification follows the MN-initiated SN modification principles over X2/Xn.

- Over E1

  - CU-CP (decision maker) just informs the SCG activation/deactivation status over Bearer context setup/modification procedure for which CU-UP shall follow. That is, no partial rejection is allowed by CU-UP (i.e. if cannot be obeyed, CU-UP shall fail the whole procedure).

Based on the above principles (which are quite straightforward), SCG BLCRs for X2/Xn/F1/E1 are quite aligned (including cause values). But for E1AP, we spotted one thing.

Since CU-UP has to obey CU-CP, the following has been added for the failure operation of Bearer Context Setup procedure (Section 8.3.1.3 in TS 37.483):

If the gNB-CU-UP cannot establish the requested bearer context, or cannot even establish one bearer, or cannot treat SCG with the indicated activation status it shall consider the procedure as failed and respond with a BEARER CONTEXT SETUP FAILURE message and appropriate cause value.

However, the similar treatment is missing for the failure operation of Bearer Context Modification procedure where the principle is the same. Therefore, we propose to add the similar treatment for Section 8.3.2.3 in TS 37.483 as follows:

Proposal 2: For E1AP, add the failure case for Bearer Context Modification procedure when CU-UP cannot obey the request from CU-CP, similarly to Bearer Context Setup procedure.

If the gNB-CU-UP cannot successfully perform any of the requested bearer context modifications, or cannot treat SCG with the indicated activation status, it shall respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

## Corrections for stage-2 TS 38.401

RAN3 added three call flows into TS 38.401 from this Rel-17 SCG works, but unfortunately, some updates are necessary to be accurate.

### Section 8.4.4

### 8.4.4 SCG Deactivation and Activation This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

MR-DC (from definition-wise) includes EN-DC together with NGEN-DC, NE-DC, and NR-DC. To be precise, it should be described as just "MR-DC with gNB in SN". And en-gNB/SgNB is interchangeably used in X2AP to indicate gNB in secondary node in case of EN-DC. We don't have to mention them separately. It is better to be re-worded as "MR-DC with gNB in SN that consists of.."

### Section 8.4.4.1

#### 8.4.4.1 SN Addition with SCG Activation or Deactivation Figure 8.4.4.1-1: SCG Activation or Deactivation in SN Addition procedure 1. The MN sends the SN Addition Request message to the SN, indicates the request of SCG activation or deactivation. 2. The SN-CU-CP sends the BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the activation or deactivation of the SCG. 3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP. 4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation or deactivation. 5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates the SCG status. In case SCG activation is requested, the SN-DU shall indicate SCG activated in the message. 6. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP as described in Section 8.9.2 and may notify the SCG status if needed. 7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP. 8. The SN sends the SN Addition Request Acknowledge message to the MN, indicates the SCG status. In case SCG activation is requested, the SN shall indicate SCG activated in the message.

For SN Addition with SCG Activation or Deactivation, in the last RAN3#115-e meeting, we agreed the following:

Modify the signalling procedures of the SN addition with SCG (de)activation in the TS 38.401 BL CR as presented in the SOD. In this case, it is optional to include the SCG (de)activation indicator in the Bearer Context Modification Request message.

But somehow this was not properly reflected onto steps 6/7 and still they are not dashed.

Moreover, for the case of SN Addition, partial rejection is not allowed for "activation" request from MN. If SN-DU cannot accept "activation" request during UE context setup (i.e. steps 4/5), SN-DU would fail the whole procedure, and in this case, SN-CU-CP has to tear down the bearer context established during steps 2/3 and inform the failure to the MN. This has not been properly reflected.

Proposal 3: Fix 38.401 as explained above.

# Conclusion

For Rel-17 SCG, we have identified the following three issues:

1)  Weird interaction descriptions in X2/XnAP?

Proposal 1: For SCG (de)activation request via SN-initiated SN modification procedure, remove the interaction descriptions with SN Addition procedure in X2/XnAP.

2)  Correction for E1AP

Proposal 2: For E1AP, add the failure case for Bearer Context Modification procedure when CU-UP cannot obey the request from CU-CP, similarly to Bearer Context Setup procedure.

3)  Correction for TS 38.401

Proposal 3: Fix 38.401 as explained in Section 2.3.

Due to the quota limit, please excuse us to provide the proposed changes as TP format in Sections 5-8.

# Reference

[1] R3-222903, SCG BLCR for TS 36.423

[2] R2-2203690, (RAN2) MR-DC BLCR for TS 37.340

[3] R3-222904, SCG BLCR for TS 38.401

[4] R3-222905, SCG BLCR for TS 38.423

[5] R3-222906, SCG BLCR for TS 38.463

[6] R3-222975, SCG BLCR for TS 38.473

# Change for TS 36.423

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

### 8.7.7	SgNB initiated SgNB Modification

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

#### 8.7.7.2	Successful Operation

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

If the CG-CandidateList is included in the SgNB to MeNB Container IE in the SGNB MODIFICATION REQUIRED message, the MeNB shall, if supported, use it for the purpose of CPAC.

# Change for TS 38.423

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

### 8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

If the SCG Activation Request IE is included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall consider that the S-NG-RAN node is about to reconfigure the SCG resources as specified in TS 37.340 [8].

If the Conditional PSCell Change Information Required IE is included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the request provides the configuration update for the list of PSCells prepared at the target SN, as described in TS 37.340 [8].

If the CG-CandidateList is included in the S-NG-RAN node to M-NG-RAN node Container IE in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, use it for the purpose of CPAC.

Interaction with the M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure:

If applicable, as specified in TS 37.340 [8], the S-NG-RAN node may receive, after having initiated the S-NG-RAN node initiated S-NG-RAN node Modification procedure, the S-NODE MODIFICATION REQUEST message including the measGapConfig IE as defined in TS 38.331 [10] within the M-NG-RAN node to S-NG-RAN node Container IE.

If applicable, the S-NG-RAN node may receive, after having initiated the S-NG-RAN node initiated S-NG-RAN node Modification procedure, the S-NODE MODIFICATION REUEST message including the SN triggered IE.

# Change for TS 37.483

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

#### 8.3.2.3	Unsuccessful Operation

Figure 8.3.2.3-1: Bearer Context Modification procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested bearer context modifications, or cannot treat SCG with the indicated activation status, it shall respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing the Security Indication Modify IE in the PDU Session Resource To Modify List IE for a PDU session that may result in the change of security status that has been applied but the DRBs that have been established for that PDU session are not requested to be released via the DRB To Remove List IEs as specified in TS 38.331 [10], then the gNB-CU-UP shall respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

# Change for TS 38.401

/////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

### 8.4.4	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in MR-DC with gNB in SN that consists of a gNB-CU and gNB-DU(s).

#### 8.4.4.1	SN Addition with SCG Activation or Deactivation

Figure 8.4.4.1-1: SCG Activation or Deactivation in SN Addition procedure

1. The MN sends the SN Addition Request message to the SN, indicates the request of SCG activation or deactivation.

2. The SN-CU-CP sends the BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the activation or deactivation of the SCG.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation or deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates the SCG status. In case SCG activation is requested, the SN-DU shall indicate SCG activated in the message.

NOTE 1:	In case SCG cannot be activated at the SN-DU when requested, the SN-CU-CP initiates the Bearer Context Release procedure to tear down the bearer context in the SN-CU-UP instead of steps 6 and 7, then inform SCG activation failure by sending the SN Addition Request Reject message to the MN instead of step 8.

6. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP as described in Section 8.9.2 and may notify the SCG status if needed.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE 2: Steps 7 and 8 may be skipped in case the SN-DU accepted the SCG activation or deactivation request.

8. The SN sends the SN Addition Request Acknowledge message to the MN, indicates the SCG status. In case SCG activation is requested, the SN shall indicate SCG activated in the message.

#### 8.4.4.2	MN initiated SN Modification with SCG Activation or Deactivation

Figure 8.4.4.2-1: SCG Activation or Deactivation in MN initiated SN Modification procedure

1. SCG is deactivated or activated.

2. The MN sends the SN Modification Request message to the SN, indicates the request of SCG activation or deactivation.

3. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG activation or deactivation.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

5. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation or deactivation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates the SCG status.

7. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG status.

8. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE: Steps 7 and 8 may be skipped in case the SN-DU accepted the SCG activation or deactivation request.

9. The SN sends the SN Modification Request Acknowledge message towards the MN, indicates the SCG status.

#### 8.4.4.3	SN initiated SN Modification with SCG Activation or Deactivation

Figure 8.4.4.3-1: SCG Activation or Deactivation in SN initiated SN Modification procedure

1. SCG is deactivated or activated.

2. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG activation or deactivation.

3. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation or deactivation.

5. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates the SCG status.

6. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG status.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE: Steps 6 and 7 may be skipped in case the SN-DU accepted the SCG activation or deactivation request.

8. The SN sends the SN Modification Required message to the MN, indicates the request of SCG activation or deactivation.

9. The MN sends the SN Modification Confirm message to the SN.

NOTE: Steps 8 and 9 will be skipped in case the SN-DU rejected the SCG activation or deactivation request.