3GPP TSG-RAN WG3 Meeting #117-e                                                      	R3-224609

E-Meeting: Aug 15th – Aug 24th, 2022

Agenda item:	10.4

Source:	Qualcomm Inc.

Title:	Signaling based logged MDT override protection

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the following objective from Rel-18 SON/MDT WID:

Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when UE reselects to NR after logged measurements are collected and before uploading the logged MDT report

# Discussion

In Rel-17, a signaling based logged MDT configured in LTE for which UE has unretrieved MDT results is lost upon the configuring another signaling based logged MDT in NR, and vice-versa.

Observation 1: In Rel-17, a signaling based logged MDT configured in LTE for which UE has unretrieved MDT results is lost upon the configuring another signaling based logged MDT in NR, and vice-versa

Rel-18 WID objective only mentions to provide support for override protection when UE reselects to NR after collecting logged MDT measurements in LTE and before uploading the LTE logged MDT report. The reverse scenario is not mentioned in the WID, hence should be clarified.

Proposal 1: It should be clarified whether to support signaling based logged MDT override protection in the reverse scenario to what is mentioned in the WID i.e., to address the scenario where the signaling based MDT is configured in gNB when UE reselects to LTE after logged measurements are collected and before uploading the NR logged MDT report

In Rel-17, a UE indicates the availability of logged MDT results to the gNB upon reconnection/resume/reconfiguration/setup only if it is for the logged MDT configuration of the same RAT i.e., NR in this case. Therefore, even if a UE has logged MDT results, a gNB RAN might end up configuring a NR logged MDT and the unretrieved NR logged MDT would be lost.

Observation 2: In Rel-17, a UE indicates the availability of logged MDT results to the gNB (or eNB) upon reconnection/resume/reconfiguration/setup only if it is for the logged MDT configuration of the same RAT

To avoid this, a UE can also indicate whether it is already configured with logged MDT of a different RAT and network can then make sure it doesn’t configure a logged MDT of another RAT.

Proposal 2: UE can indicate to gNB whether it is configured with a signaling based LTE logged MDT configuration (and vice-versa) so that the gNB can make sure to retrieve the LTE logged MDT report from UE before configuring a signaling based NR logged MDT configuration.

Alternatively, we could modify UE behavior in Rel-17 and mention in the specifications that UE doesn’t discard already collected logged MDT results upon receiving another logged MDT configuration of another RAT if the signaling based logged MDT override protection is needed

Proposal 3: If the signaling based logged MDT override protection is needed, a UE should not discard already collected logged MDT results upon receiving another logged MDT configuration of another RAT

# Conclusion

Observation 1: In Rel-17, a signaling based logged MDT configured in LTE for which UE has unretrieved MDT results is lost upon the configuring another signaling based logged MDT in NR, and vice-versa

Proposal 1: It should be clarified whether to support signaling based logged MDT override protection in the reverse scenario to what is mentioned in the WID i.e., to address the scenario where the signaling based MDT is configured in gNB when UE reselects to LTE after logged measurements are collected and before uploading the NR logged MDT report

Observation 2: In Rel-17, a UE indicates the availability of logged MDT results to the gNB (or eNB) upon reconnection/resume/reconfiguration/setup only if it is for the logged MDT configuration of the same RAT

Proposal 2: UE can indicate to gNB whether it is configured with a signaling based LTE logged MDT configuration (and vice-versa) so that the gNB can make sure to retrieve the LTE logged MDT report from UE before configuring a signaling based NR logged MDT configuration.

Proposal 3: If the signaling based logged MDT override protection is needed, a UE should not discard already collected logged MDT results upon receiving another logged MDT configuration of another RAT

# 4. References