3GPP TSG-RAN WG3 #117-e	R3-225032

Online, 15th -24th August, 2022

Agenda Item:	13.4

Source:	ZTE (moderator)

Title:	        Summary of CB # IAB4_IntMit

Document for:	Approval

# Introduction

This paper is for the following offline discussion:

CB: # IAB4_IntMit - Should specific solutions for PCI collision avoidance/resolution be adopted on top of existing ones? - Should specific solutions for RACH collision avoidance/resolution be adopted on top of existing ones? - Should specific solutions for resource coordination be adopted on top of existing ones? (ZTE - moderator) Summary of offline disc R3-225032

This discussion has two phases:

Phase 1: Converge on potential proposals. Please give your feedback before Thursday, August 18th at 23.59 UTC

Phase 2: TBD

The following contributions will be discussed in this CB:

| R3-224357 | Discussion on the interference mitigation (Huawei) | Discussion |
| --- | --- | --- |
| R3-224379 | Interference mitigation for mobile IAB (Nokia, Nokia Shanghai Bell) | Discussion |
| R3-224432 | Interference mitigation of mobile IAB-node mobility (Lenovo) | Discussion |
| R3-224499 | Mitigation of Interference and Reference Signal Collisions for Mobile IAB-Nodes (Ericsson) | Discussion |
| R3-224506 | PCI collision avoidance for mobile IAB (Qualcomm Inc.) | Discussion |
| R3-224707 | Avoidance of resource collisions due to IAB-node mobility (Fujitsu) | Discussion |
| R3-224714 | Discussion on mitigation of interference in mobile IAB scenario (ZTE) | Discussion |
| R3-224769 | Discussion on mitigation of interference (Xiaomi) | Discussion |
| R3-224828 | Discussion on mitigation of interference (samsung) | Discussion |

# For the Chair’s Notes

Propose to capture the following Agreements:

…

# PHASE 1: Discussion

## Handling of PCI collision

Based on the above mentioned contributions, all contributions agree that PCI of mobile IAB-DU could be configured via OAM as in current specification. The scenario of PCI collision includes PCI collision between mobile IAB-DU cell and fixed cell, and between mobile IAB-DU cells.

Avoidance of PCI collision via OAM/PCI partitioning

Based on contributions R3-224499 (Ericsson), R3-224432(Lenovo), R3-224379 (Nokia), R3-224769(Xiaomi), PCI partitioning (i.e. dedicated PCI space) could be used for mobile IAB to avoid PCI collision between mobile IAB-DU cell and fixed cells when the trajectory of mobile IAB node is predictable. R3-224506(Qualcomm) observes that with only 1008 PCI values available, PCI partitioning to avoid PCI collision for mobile IAB is not practical in many scenarios. And R3-224379 (Nokia) observes that the probability of PCI collision between mobile IAB-nodes depends on the use case and deployment scenario.

Q1: Do you agree that PCI collision in mobile IAB scenario could be avoided by OAM//PCI partitioning?

| Company | Yes/No | Comments |
| --- | --- | --- |

Detection of PCI collision

Assume that PCI collision occurs, the first issue is how to detect the PCI collision. Based on R3-224506(Qualcomm), enhancements to measurements in support of PCI collision detection are in RAN2 scope. Based on R3-224828(samsung), four cases should be considered for PCI conflict detection: Case 1: PCI conflict prediction before full migration; Case 2: PCI conflict detection during full migration; Case 3: PCI conflict detection after full migration; Case 4: PCI conflict between different mobile IAB nodes. Based on R3-224499 (Ericsson), detection of PCI collision could be already supported by existing PCI Optimisation Function. Based on R3-224714(ZTE), after receiving PCI of mobile IAB-DU cells via F1 setup procedure, the IAB-donor can detect whether the PCI of mobile IAB-DU cell collide with neighbour cells. And based on R3-224707(Fujitsu), some enhancements to F1AP or XnAP signaling are proposed so that F1-terminating donor-CU can determine whether PCI/RACH reconfiguration is needed.

Q2: Is there any enhancement needed for detection of PCI collision? If yes, what enhancement is needed? Is it in RAN2 or RAN3 scope?

| Company | Yes/No | Comments |
| --- | --- | --- |

How to reconfigure PCI

After PCI collision is detected, PCI could be reconfigured for mobile IAB-DU cell. Based on R3-224357(Huawei), R3-224499 (Ericsson), R3-224432(Lenovo), R3-224714(ZTE), legacy PCI Optimisation Function could be reused, i.e. donor CU can reconfigure the PCI of mobile IAB-DU cell. Based on R3-224707(Fujitsu), the F1-terminating donor-CU could reconfigure the PCI after detection of collision. Based on R3-224769(Xiaomi), the following two scenarios needs to be considered: scenario1, IAB-donor is changed, i.e. during full migration; scenario2, IAB-donor is unchanged, i.e. not in full migration. And mobile IAB-node, serving IAB-donor or target IAB-donor is responsible for the PCI/RACH (re)configuration.

Regarding how to determine the PCI change, R3-224506(Qualcomm) proposes that the decision on network-controlled PCI change is conducted by the IAB-donor-CU-CP, and it is up to implementation. R3-224769(Xiaomi) proposes that some additional information should be considered for dynamic PCI/RACH (re)configuration. R3-224707(Fujitsu) proposes that the dynamic resource configuration could be performed according to the actual location.

Q3: Is there any enhancement needed for reconfiguration of PCI of mobile IAB-DU cell? If yes, what enhancement is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |

How to avoid UE service interruption due to PCI change

Based on R3-224379 (Nokia), when PCI collision occurs, the PCI of one of the colliding mobile IAB cells needs to be reconfigured, which would have impact to UEs connected to the mobile IAB-node that has to change the PCI. Based on R3-224357(Huawei), NW use handover procedure to switch the UEs to the new cell with this updated PCI. Based on R3-224506(Qualcomm), network-controlled PCI change should be supported without inter-donor full migration which use inter-cell UE handover while the mobile IAB-DU remains connected to the same CU.

Q4: Is there any enhancement needed to avoid UE service interruption due to PCI change? If yes, what enhancement is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |

Besides, the issue on PCI configuration of logical IAB-DU cells (e.g. whether the same or different PCIs are used between two logical DUs) mentioned in R3-224379 (Nokia) and R3-224432(Lenovo) is not covered in this CB since it will be discussed in CB # IAB2_Mobility.

## Handling of RACH collision

Regarding handling of RACH collision, it seems that two issues are mentioned in R3-224357(Huawei), R3-224432(Lenovo), R3-224499 (Ericsson), R3-224828(samsung), R3-224769(Xiaomi), R3-224707(Fujitsu), and R3-224714(ZTE). based on R3-224499 (Ericsson), no enhancement is needed

- Issue 1: conflict of the RACH resource configuration

For issue 1, R3-224357(Huawei) proposes to ask RAN1 whether enhancement is needed for solving the potential confliction of the RACH resource configuration or any other physical resource configurations.

R3-224769(Xiaomi) and R3-224707(Fujitsu) propose to use dynamic resource configuration (e.g. PCI/RACH, etc) for mobile IAB-DU.

- Issue 2: collision of RACH attempts from UEs due to IAB node migration

For issue 2, R3-224432(Lenovo) observes that RACH-less handover for the served UEs can avoid the potential RACH collision due to mobile IAB-node mobility. And R3-224432(Lenovo) proposes to ask RAN1 to firstly evaluate whether the legacy RACH configurations are sufficient for the case that legacy handover including RACH needs to be performed. Based on R3-224828(samsung), some enhancements should be proposed to resolve RACH issues. Based on R3-224714(ZTE), the RACH collision issue may only exist in the full migration procedure without inter-topology transport, and the RACH collision issue could be discussed after we have conclusion on whether inter-topology transport is supported in full migration.

Q5-1: What’s the issue to be discussed regarding RACH collision in R18 mobile IAB, issue 1 and/or issue 2?

Q5-2: For issue 1, is there any enhancement needed to resolve the issue? If yes, what enhancement is needed? Do we need to send an LS to RAN1 to ask RAN1 whether enhancement is needed for solving the potential conflict of the RACH or any other physical resource configurations?

Q5-3: For issue 2, is there any enhancement needed to resolve the issue? If yes, what enhancement is needed? Do we need to discuss this issue at the beginning of R18 IAB?

| Company | Yes/No | Comments |
| --- | --- | --- |

## Resource coordination

Based on R3-224379 (Nokia), Rel.17 XnAP provides means can be reused to share relevant information about the cell configurations to anticipate potential interference issues while the IAB-node is moving, and RAN3 can wait conclusions of the other WGs whether Rel.17 provides sufficient information sharing between the nodes. Based on R3-224499 (Ericsson), resource coordination between the mIAB and its parent nodes is enabled by the existing XnAP and F1AP procedures for the coordination of semi-static cell-level configurations, and hence no further enhancement is needed.

Q6: Is there any enhancement needed for resource coordination for interference mitigation in mobile IAB scenario? If yes, what enhancement is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |

## Others

Q7: Is there any other issue to be discussed for interference mitigation in mobile IAB scenario?

| Company | Comments |
| --- | --- |

# PHASE II: Convergence of PH1

# References

- R3-224357	Discussion on the interference mitigation (Huawei)	Discussion

- R3-224379	Interference mitigation for mobile IAB (Nokia, Nokia Shanghai Bell)	Discussion

- R3-224432	Interference mitigation of mobile IAB-node mobility (Lenovo)	Discussion

- R3-224499	Mitigation of Interference and Reference Signal Collisions for Mobile IAB-Nodes (Ericsson)	Discussion

- R3-224506	PCI collision avoidance for mobile IAB (Qualcomm Inc.)	Discussion

- R3-224707	Avoidance of resource collisions due to IAB-node mobility (Fujitsu)	Discussion

- R3-224714	Discussion on mitigation of interference in mobile IAB scenario (ZTE)	Discussion

- R3-224769	Discussion on mitigation of interference (Xiaomi)	Discussion

- R3-224828	Discussion on mitigation of interference (samsung)	Discussion