3GPP TSG-RAN WG3 #117-e	R3-225024

15th – 24th Aug 2022, Online

Agenda Item:	17.3

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # NTN2_UELocation

Document for:	Approval

# Introduction

CB: # NTN2_UELocation

- Whether the verification shall be performed in NG-RAN or 5GC?

- Discussion on positioning methods, existing method or enhanced method?

- Behavior of NG-RAN if the reported UE location is not reliable? (e.g., trigger the UE context release procedure or send the verified result to 5GC)

- Capture agreements and open issues

(Nok - moderator)

Summary of offline disc R3-225024

The discussion has two phases:

Phase 1: Identify potentially achievable agreements for online discussion.

Phase 2: based on Phase 1 discussion, discuss the Stage-2/3 TP.

The deadline for Phase 1 is Tuesday, Aug 16th, 23:59:59 UTC. This allows the moderator to prepare the proposals for Wednesday online session.

# For the Chairman’s Notes

Propose to capture the following Agreement:

…

# Phase 1 Discussion

## Issue 1: Whether the verification shall be performed in NG-RAN or 5GC

Almost all contributions propose to consider the following conclusion from RAN discussion:

-	Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning

-	Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded

-	When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded.

-	Solutions using existing NG-RAN architecture and procedures shall be considered

Contribution ([3][4]) propose the existing NG-RAN UE Position Procedure (TS38.305 Figure 5.2-1, copied as below) can be reused.

Figure 5.2-1: Location Service Support by NG-RAN

In existing position architecture, the CN node (i.e. LMF) request the UE and/or the gNB(s) to perform the necessary measurement, and the LMF is responsible for handling the measurement report collected from the UE and/or the gNB(s).  Even the position procedure may need to be updated once RAN1/2 decide the position method used for verification, but the existing RAN3 related position procedure(s) is reused in general.

Contribution ([3]) also propose Stage-2 TP. Moderator believe the TP can be discussed once RAN3 make a decision on this issue.

Q1: Please share your view on following proposals

- Q1-1: The verification is performed in the CN

- Q1-2: The existing RAN3 related position procedure(s) is reused for verification

| Company | Comment |
| --- | --- |
| Nokia | Q1-1: agree. As concluded by RAN, “When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline”, it is preferred to reuse the LMF to handle the measurement report collected from the UE and/or gNB. Q1-2: agree. |
| CATT | Q1-1: Neutral. The target of the verification is to verify whether the reported UE location is reliable or not. Currently, the reported UE location is not provided to CN. If verification is done in 5GC, we should clarify the target is to verify the TAC/Mapped CGI reported from NG-RAN, or the reported UE location? In Rel-17, verification of the TAC/Mapped CGI reported from NG-RAN is already supported by LCS service. The delta is we consider the RAT-dependent Positioning methods for verification in Rel-18. If verification is done in NG-RAN, to adopt the existing positioning architecture and procedures as much as possible, it may need to allow the NG-RAN node to be an LCS client. Q1-2:Agree. |
| Qualcomm | Q1-1: Agree Q1-2: We would prefer to wait for SA2 outcome on this before making any agreements in RAN3 |

Summary:

Suggest following proposal:

Proposal

## Issue 2: Behavior of gNB if the reported UE location is not correct? (e.g., trigger the UE context release procedure or send the verified result to 5GC)

Contribution ([5]) propose:

Proposal 1: If the gNB detects the network verified UE location is different from the reported UE location, it shall initiate the UE context release procedure or perform the NG based handover.

Contribution ([6]) propose Stage-3 CR. Moderator believe the CR can be discussed once RAN3 make a decision on this issue.

Contribution ([3]) propose “if the verified UE location is not consistent with the area the AMF is serving, the AMF triggers UE detach procedure toward the NG-RAN.”

Moderator believe this is related to Q1. In case the CN performs the verification, the CN may detach the UE and initiate UE context release. Otherwise, the gNB may initiate the UE context release procedure. Since the AMF behavior should be defined in SA2, it is suggested that RAN3 only consider the gNB behavior.

Q2: Please share your view on the behavior of gNB if the reported UE location is not correct, e.g.  initiate UE context release or perform NG based handover.

| Company | Comment |
| --- | --- |
| Nokia | This may be not needed. We prefer the CN performs the verification. In case the verification result is negative (i.e. UE’s reported location is not correct), CN can initiate the de-register procedure. The gNB just follow the normal AMF initiated UE Context Release procedure. In addition, the verification procedure is different to normal position procedure. The Verification procedure may be completed without determining the UE’s actual location, e.g. verification procedure can be completed once detecting the actual measurement report does not match with the UE’s reported location. So the gNB may not know UE’s actual action, and it cannot initiate NG-based HO. |
| CATT | We understand this is linked to Q1-1. If the verification is done in CN, the NG-RAN does not necessary to know if the reported UE location is correct or not. NG-RAN should not perform the UE context release or NG based handover upon reported UE location is received, because it does not know whether UE location is verified. Which means the Rel-17 behaviors defined in NG-RAN may need to be adjusted, e.g. the actions to be decided in 5GC after UE location is verified as reliable or not reliable. If the verification is done in NG-RAN, NG-RAN should perform the UE context release or NG based handover after the UE location is verified. And in case NG-RAN identifies the reported UE location is not reliable, it may need to indicate the verification result the 5GC for further action(s). |
| Qualcomm | We prefer CN verification of the UE location. Based on the verification, CN can initiate appropriate actions to the UE. RAN is need not be involved here. For CN actions upon detecting incorrect UE location, we prefer to wait for SA2 outcome. |

Summary:

Suggest following proposal:

Proposal

## Issue 3: positioning method

Contribution ([2]) propose the detailed measurement/position method depends on the progress in RAN2/RAN1.

Contribution ([4]) propose “Proposal 2: Multi-RTT positioning method should be studied for verification of the UE reported location.”

Contribution ([7]) propose:

Proposal 2: Exchange of TRP info between NG-RAN and LMF may need to be adjusted due to the moving of the TRPs, the details could be left to WI phase.

Proposal 4: For LEO case, whether and how to support the positioning procedure during inter-cell, inter-satellite handover should be further discussed.

Proposal 6: Existing positioning methods, e.g. ECID, UL/DL TDOA, UL/DL AOA, could be considered for UE location verification. The details should be evaluated in RAN1 firstly.

Moderator believe the specific position method used for verification should be discussed in RAN1/2, and the impact to RAN3 can be discussed once RAN1/2 make a decision.

Q3: Please share your view on following proposals

- RAN3 wait for RAN1/2 progress on the specific position method to be used for verification

| Company | Comment |
| --- | --- |
| Nokia | Agree. Before RAN1/2 make a decision on the position method to be used for verification, it is premature for RAN3 to discuss the impact. |
| CATT | Agree. The selection/evaluation of the verification methods should be done in RAN1/RAN2 firstly. We do not need to dig the details for now. |
| Qualcomm | Agree. It is premature to discuss the positioning methods in RAN3 |

Summary:

Suggest following proposal:

Proposal

## Any other issues

| Company | Comment |
| --- | --- |

# Phase 2 Discussion

# Conclusion, Recommendations

# References

- R3-224234, Network verified UE location aspects (THALES)

- R3-224384, Discussion on the network verified UE location	 (Nokia, Nokia Shanghai Bell)

- R3-224578, Initial Observations on Network Verified UE Location (Ericsson LM)

- R3-224595, UE location verification (Huawei)

- R3-224623, Initial consideration on Network verified UE location for NR NTN (ZTE)

- R3-224624, Cause value on Network verified UE location for NR NTN (ZTE)

- R3-224635, Discussion on UE location verification for NR NTN (CATT)