3GPP TSG-RAN WG3 #117-e	R3-22xxxx

15th – 24th Aug 2022

Online

Agenda Item:	16.3

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on SL Relay Service Continuity

Document for:	Approval

# Introduction

CB: # SLRelay2_ServiceContinuity

- Source gNB or target gNB to select the target relay UE for four scenarios? Potential impacts on HO procedures, information related with target Relay UE, UE RRC states, etc.

- For other scenarios, e.g. inter-gNB indirect-to-direct/ inter-gNB indirect-to-indirect path switching, potential impact on RAN3.

- For path switching, potential impact on HO procedure.

- How to support CU/DU split architecture and potential impact on F1, e.g. intra-gNB-DU and inter-gNB-DU path switching for intra-gNB indirect-to-indirect path switching scenario.

- Whether to support path switching from direct path with MR-DC to indirect, path switching from indirect to direct path with MR-DC?

- Focus on the high-level agreements, capture agreements and open issues.

(E/// - moderator)

Summary of offline disc R3-225021

# For the Chairman’s Notes

Propose the following:

First Round

Proposal 1: Reuse the existing network procedures to support single-hop L2 U2N Relay in Rel-18.

Proposal 2: Source gNB decides to trigger path switching for the L2N remote UE.

Proposal 3: Working assumption: source gNB decides which path to be switch to under target gNB, either indirect or direct.

Proposal 4: Current signaling can support Scenario A, i.e., inter-gNB indirect to direct path switch. If any problematic scenario is identified by RAN2, then RAN3 continues discussing any enhancements.

Proposal 5: Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch.

Proposal 6: To support Scenario B and D, i.e., inter-gNB indirect/direct to direct path switch, RAN3 focuses on the following options on how to select the target Relay UE.

- Option 1: source gNB selects on target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

Proposal 7: RAN3 focuses on the XnAP and possible F1AP impacts to support the basic scenarios.

Second Round

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion – second round

# Discussion – first round

### Using Rel-17 SL Relay procedures as the baseline

In [4][6][10], it is proposed that basically Rel-17 Sidelink Relay procedures can be reused as a foundation for Rel-18 discussion. These will include Rel-17 SL relay and legacy handover procedures. Other than Xn handover signaling, two papers [7][10] mentioned about NG handover (without Xn interface). The moderator would suggest checking its necessity after XnAP discussion is stable.

Some enhancements are required case by case. Details will be discussed in the following sections.

Q1. Companies are invited to indicate their view on reusing the existing procedures for single-hop Layer-2 UE-to-Network relay in Rel-18.

| Company | Y/N | Comment |
| --- | --- | --- |
| E/// | Yes | As elaborated in [4], in R17 it is the remote UE who triggers the relay UE to RRC_CONNECTED state via PC5 signaling. The same principle should be applied in R18. |
| CATT | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes | Rel-17 Sidelink Relay procedures can be reused as a baseline. |
| Samsung | Yes | We support to reuse the existing procedures as much as possible. |
| China Telecom | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| Huawei | Yes | Solution for R18 service continuity can take the R17 procedures as the baseline. |
| Nokia | Yes |  |
| LGE | Yes |  |
| Xiaomi | Yes |  |

Summary:

All the companies say yes to reusing existing procedures for L2 U2N reply in R18.

Proposal 1:

Reuse the existing network procedures to support single-hop L2 U2N Relay in Rel-18.

### Node to decide path switching

In general, either from the proposals or the proposed signaling flows, we can see companies naturally would see the source gNB to make decision triggering the path switching, considering it has the Uu measurement configuration and measurement report from the Remote UE.

Q2. Companies are invited to indicate their view on the source gNB making decision on triggering path switching.

| Company | Y/N | Comment |
| --- | --- | --- |
| E/// | Yes |  |
| CATT | Yes |  |
| Qualcomm | Yes | Source gNB should select the target relay UE and inform target gNB |
| ZTE | Yes | It shall be clarified that this question is for source gNB making decision on triggering path switching, not related to target relay UE selection. |
| Samsung | Yes | It seems to be natural. On top of that, we can further check and discuss, Which node to decide to use direct/indirect path Which node to decide the target relay UE |
| China Telecom | Yes | Source gNB can decide to use direct/indirect path. |
| vivo | Yes |  |
| CMCC | Yes |  |
| Huawei | Yes | Source gNB can decide to trigger the path switch, and if source gNB decides to path switch to a indirect path, it can select the target relay UE, which is discussed in 4.2.2/4,2,3. |
| Nokia | Yes |  |
| LGE | Yes |  |
| Xiaomi | Yes | Decision for triggering should be source gNB, agree this questions is not related to decision regarding final target Relay selection |

Summary:

All the companies agree that it is the source gNB who decides to trigger path switching for L2N relay.

Proposal 2:

Source gNB decides to trigger path switching for the L2N remote UE.

One paper [10] provided the analysis on which node should decide the path switching type, i.e., either direct or indirect. The reason for having target gNB as decision maker is that it knows the Uu measurement between RRC_CONNECTED state target relay UE. Also, the paper proposed that the source gNB may inform measurement results of Uu link signal strength (between remote UE and target gNB) and sidelink signal strength (between remote UE and the selected relay UE).

Q3. Companies are invited to indicate their view on target gNB making decision on which path (i.e., indirect or direct) to select for the Remote UE.

| Company | Y/N | Comment |
| --- | --- | --- |
| E/// | No | The source gNB should be able to decide either switching to indirect or direct path. Basically it can select based on the reported measurements from the Remote UE, including PC5 measurements to the candidate target Relay UE(s), and the Uu measurements to the candidate target cell(s). This will avoid transferring measurement results over XnAP which is not following the legacy HO procedure. |
| CATT | No | Note that a relay UE only can be a relay UE when Uu between relay UE and cell is good. If the Uu is not good, it will not be a relay UE. Besides, if target gNB decides path switching type, source CU will only trigger a handover request to target gNB when Uu between remote UE and target cell is good and regardless PC5 measurements report because source gNB does not make a decision about whether direct or indirect path is used in target path. In this case, target gNB will never decide to add a relay UE for remote UE, expect the Uu between remote UE and target cell is not good. |
| Qualcomm | No | Uu link quality should NOT be the main criteria while selecting a target relay and source cell measurements should not be provided to target gNB during HO preparation. Besides, source gNB can certainly determine the target path type based on the reported measurements. |
| ZTE | No | A RRC_connected UE can act as a relay UE and transmit discovery message only when its Uu link quality is good enough/satisfy Uu RSRP threshold if configured. It is not necessary for target gNB to consider the Uu link quality of target relay UE to decide path switch type. |
| Samsung | Yes | There are two options on the table on which node to decide, source or target. And there are three types of measurements that can be used for decision: - Uu measurement between potential target node and remote UE - PC5 measurement between potential target relay UE and remote UE - Uu measurement between potential target node and potential target relay UE The pro for target node to decide, is that target node is able to obtain (3), which could be beneficial to decide a proper target relay UE together with (1) and (2); while the con for target node to decide, is that it may additionally transfer (2) over Xn. Vice versa, the pro for source node to decide is (2) is not needed to be additionally transferred over Xn; while the source node is unable to obtain (3) without enhancement. Compared with two options, we prefer target node to decide since it has more knowledge for making a proper direct/indirect decision with limited spec impact. |
| China Telecom | No | The Uu measurement result of the source gNB in the legacy HO procedure will not be notified to the target gNB. If the link quality between target gNB and (remote) UE meets the requirements, the source gNB will trigger the traditional handover procedure. This is independent of whether the relay UE is introduced. |
| vivo | No |  |
| CMCC | No |  |
| Huawei | No | Source gNB selecting the path is more align with the legacy design and can avoid too much spec impact in Xn signaling. |
| Nokia |  | Further study is needed, with considering RAN2 progress. There are pros/cons for source gNB or target gNB to make the decision. It may be simple that source make the decision, but target gNB may have better knowledge. We suggest study both options, but not make decision in the 1st meeting. |
| LGE | Yes | Same view with Samsung If the candidate relay UE is RRC_CONNECTED, the target gNB can know the Uu measurement result between the candidate relay UE and the target gNB. Thus, when the source gNB requests HO to the target gNB, the target gNB can select either direct path with the remote UE or indirect path via the relay UE. Also, RAN2 has same discussion on this topic. So, at this time, we prefer not to filter out this solution. |
| Xiaomi |  | We agree with comments by Samsung above that target gNB is privy to specific information that will improve the success of the HO. There may also be other factors that need consideration including target Relay UE loading. This needs further evaluation before any conclusion can be drawn. |

Summary:

In total 12 companies provided feedback to this question.

8 companies gave an affirmative answer.

2 companies prefer to have the target gNB as the decision maker on which path to select.

2 companies are neutral and would see further study.

Considering majority of the companies would go for source gNB, and some companies have different view. Moderator would propose a WA, which can be reviewed later.

Proposal 3:

Working assumption: source gNB decides which path to be switch to under target gNB, either indirect or direct.

## Supported scenarios for service continuity

### Scenario A: Inter-gNB indirect-to-direct path switch

In most of the papers, the understanding is that current stage-3 should be able to support this scenario. One concern is brought in [1] about lossless delivery for DL data forwarding. The paper describes that the delivery status of DL data might be mismatched between relay UE and remote UE. If PC5 RLF failures happens, some DL data being buffered in relay UE may be not transmitted to remote UE during path switch.

Q4. Companies are invited to indicate their view whether current stage-3 signalings can support Scenario A. Indicate if any enhancements are foreseen.

| Company | Y/N | Comment |
| --- | --- | --- |
| E/// | Yes | Current mechanism is able to support this scenario. Regarding DL data loss, we are not seeing the possibility of such scenario. PC5 RLF is triggered either by RLC retransmissions or HARQ DTX. Theoretically it could occur during path switching. But, if the PC5 channel quality becomes worse before RLF, the UE should be triggered relay reselection before switching the path. Thus it seems no benefit to consider and standardize data lossless for PC5 RLF during path switching. |
| CATT | Yes | Stage 3 is enough, while stage 2 needs to capture the RRC reconfiguration for relay UE to remove the remote UE related context. For DL data loss, it similar as IAB. There is no suitable solution after a lot of discussion in R17 for IAB hence we achieved the following agreement: In Rel-17, the following aspects aiming at avoiding unnecessary DL transmissions will not be specified: - the avoidance of unnecessary DL data transmission over the source path between IAB donor CU and migrating IAB node - the transmission of in-flight DL packets buffered at migrating IAB node and its descendant node(s), after the migration |
| Qualcomm | Yes | PC5 RLF during path switching is indeed a corner case and further enhancements for lossless delivery is not needed |
| ZTE | See comments | Naturally current stage 3 signallings can be used as baseline. But the lossless delivery for DL data forwarding shall be considered. Since hop by hop RLC ARQ is supported in U2N relay, when the S-gNB forward to target gNB all DL data which have not been acknowledged by the relay UE instead of remote UE, remote UE may suffer from data loss. For example, S-gNB transmits DL packet {1,2,3,4,5,6} to relay UE and obtains ack of {1,3,4,6} from relay UE. Relay UE also sends packet {1,3,4,6} to remote UE but only receive ack of {1,3,6} from remote UE. Then the remote UE receives the path switch command and switch to the T-gNB for connection establishment. According to the legacy DL data forwarding procedure, where only the unacknowledged data packets are forwarded (i.e. S-gNB forward {2, 5} to T-gNB), the remote UE will suffer from data loss, i.e., packet 4 is lost. - for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE. |
| Samsung |  | We are open to further discuss the case when PC5 link failure happens during inter-gNB path switching. However, for normal case, i.e. PC5 RLF does not happen during path switching, the current stg-3 signaling seems enough. |
| China Telecom | Yes | Current mechanism is able to support this scenario. The enhancement of PC5 RLF can be considered a lower priority. |
| vivo |  | Open to discuss for further enhancement such as PC5 link failure handling |
| CMCC | Yes | Current mechanism is sufficient for scenario A. If more concern is raised by RAN2, we can discuss it further. |
| Huawei | Yes with comments | The current stage-3 signaling is enough if PC5 link is good during the indirect-to-direct path switch procedure. PC5 RLF happens along with the path switch is a corner case. |
| Nokia | Yes |  |
| LGE | Yes, but | Agree that current stage-3 signalings can support Scenario A. About lossless delivery for DL data forwarding, I was told that this issue is also raised in RAN2. So, we prefer not to exclude the issue related to lossless delivery for DL data forwarding at this meeting. |
| Xiaomi | Yes | We believe the current stage 3 supports the majority of cases wherein the PC5 RLF does not occur during path switch. However we believe RAN3 can wait for RAN2 feedback as to the need to solve the PC5 RLF during HO case for R18 |

Summary:

In total 12 companies provided feedback to this question.

9 companies gave an affirmative answer.

Some companies agree that PC5 RLF during path switch is a RAN2-initiated discussion. Some companies think this should not happen, or is only a corner case.

Moderator would propose to agree that existing procedures can support Scenario A. Any further enhancements will be pending in RAN2.

Proposal 4:

Current signaling can support Scenario A, i.e., inter-gNB indirect to direct path switch. If any problematic scenario is identified by RAN2, then RAN3 continues discussing any enhancements.

### Scenario C: Intra-gNB indirect-to-indirect path switch

[3] mentioned about that both intra-DU and inter-DU should be supported for Scenario B. [11] describes the possible behavior updates over F1. The moderator think the F1 impacts are about network behavior and might be resolved in stage-2. Majority companies see there is no or limited changes considering when the inter-gNB indirect/direct to indirect path switching is supported.

Q5. Companies are invited to indicate their view that current RAN3 signalings are able to

support Scenario C. Indicate if any potential impacts are foreseen.

| Company | Y/N | Comment |
| --- | --- | --- |
| E/// | Yes |  |
| CATT | Yes | Stage 3 is enough, while stage 2 needs to capture the RRC reconfiguration for relay UE to remove the remote UE related context. |
| Qualcomm | Yes | We can decide stage-2 impacts later. |
| ZTE | Yes | Current RAN3 signallings are used as baseline. For inter-gNB indirect-to-indirect path switch (Scenario D), the lossless delivery for DL data forwarding as in Scenario A shall be considered. |
| Samsung | Yes | Current stg3 seems sufficient for both intra-DU and inter-DU case. |
| China Telecom | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| Huawei | Yes | The current stage-3 signaling is enough |
| Nokia | Yes |  |
| LGE | Yes | We can decide stage-2 impacts later. |
| Xiaomi | Yes |  |

Summary:

All say yes.

Proposal 5:

Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch.

### Scenario B: Inter-gNB direct-to-indirect path switch

### Scenario D: Inter-gNB indirect-to-indirect path switch

Considering the new path is indirect in Scenario B and D, the network behaviors would be the same for both the source gNB and target gNB, e.g., how to select the target Relay UE. The moderator combines these two scenarios for easy discussion.

In the cases of inter-gNB indirect/direct to indirect path switching, the network node has to select the corresponding target Relay UE based on certain criteria. To RAN3, the question would be who selects the candidate relay UE(s) and who has the power to decide. Either source gNB or target gNB can be the choice.

The papers [1]~[10] proposed that the source gNB provides the candidate target Relay UE information to the target gNB. However, there are differences in details. One paper [2] proposes to wait for RAN2 if multiple target Relay UE(s) will be involved.

In summary, three possible options are given below.

- Option 1: source gNB selects on target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

- Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

Q6. Companies are invited to indicate their view on the preference within above options.

| Company | Option | Comment |
| --- | --- | --- |
| E/// | Option 2 | Option 2 follows the legacy handover principle, since the target gNB should be the final decision maker considering it understands Uu radio signal strength of the potential Relay UE and so on. In Option 1, the target gNB still can reject the path switching request procedure by sending the Handover Preparation Failure message. This may cause redundant signaling exchange between network nodes. Thus, Option 2 offers more flexibility and could reduce the signaling. Option 3 is not aligned with current HO principle, i.e., not providing UE measurements from the source to the target. |
| CATT | Option 1 and potion 2 | In each handover request message, it only has one target CGI in current spec. For stage 3 change, we can introduce one target relay UE ID in handover request which follows the current handover principle. If we want to have some candidate target relay UEs, multiple handover request messages can be sent to target gNB. |
| Qualcomm | Option 1 | Option 3 should not be considered as E/// pointed out Option 2 is more like an optimization. Even in Option 2, there is a possibility that all the candidate relay UEs proposed by source gNB are rejected by target gNB. There is not much benefit, instead more overhead on Xn and burden on target gNB to down select among the candidates. Option 1 is sufficient. |
| ZTE | Option 1 and 2 | Either the source UE or target UE can select the target relay UE. Both options can be supported. Source gNB could decide to select a target relay UE or leave to target gNB selection. If source gNB identifies that there are multiple candidate relay UEs served by a same neighbour/target gNB based on the measurement report received from remote UE (NCGI), source gNB may request the neighbour/target gNB to select a target relay UE by including the multiple candidate relay UEs in HO request. |
| Samsung | Option3 | We still prefer to follow similar principle as for handover, namely, the target node makes the final decision. And our understanding is that Option2 and Option3 actually do not contradict with each other, and our opinion is that current RRC spec has provided Uu measurement in inter-node RRC message (HandoverPreparationInformation) during handover preparation procedure. All we need to do is to additionally transfer PC5 measurement between potential relay UE and remote UE for inter-node signaling. So Option3 could be a starting point since anyway candidate target relay UE information such as relay UE identity needs to be transferred together with PC5 measurement. |
| China Telecom | Option 1 or Opotion 2 | We note that option 2 is beneficial to increase the handover success rate of the remote UE, signaling details may need further discussion. Option 1 or 2 are both feasible for us. |
| vivo | Option 2 | This option 2 is more align with legacy hanover execution |
| CMCC | Option 2 | Option 2 is mostly close to legacy HO procedure. For option 3, source gNB provides more details of candidate UE to target gNB for decision, it looks feasible, we are open for option 3. |
| huawei | Option 1 | Option 1 is better, source gNB can select a proper target relay UE based on the measurement report from the remote UE. When receiveing the HO request from source gNB, target gNB will make the HO decision whether the remote UE can be connected via the target relay UE. Target gNB can response HO response or HO preparation failure based on its admission control decision. |
| Nokia | Option 1 or Option 2 | Need further study on both Option 1 and Option 2, but not make the decision in the 1st meeting. |
| LGE | Option 2 or Option 3 | Agree with Samsung. Current specification already supports the source gNB behavior to transfer Uu measurement to the target gNB. Therefore, in order to support Option 3, our understanding is that this can be solved by including PC5 measurement into the inter-node RRC message (i.e., HandoverPreparationInformation) in RAN2. |
| Xiaomi | Option 2 | Is the preferred option as it would potentially enable a higher chance of a successful HO in our opinion. |

Summary:

In total 12 companies provided feedback to this question.

9 companies support Option 2.

6 companies support Option 1. Among them 2 companies would like to see the combination of Option 1 and 2.

2 companies support Option 3.

Considering following the legacy HO similar mechanism, Option 1 and/or 2 have majority support. As one company proposed, RAN3 can make down-selection based on the opinions, i.e., opt out Option 3.

Proposal 6:

To support Scenario B and D, i.e., inter-gNB indirect/direct to direct path switching, RAN3 focuses on the following options on how to select the target Relay UE.

- Option 1: source gNB selects on target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

## Other aspects

In [4], scope down-prioritization was mentioned. For instance, whether path switching with MR-DC should be supported or not. From moderator’s point of view, this can be discussed in RAN2 first considering they have concluded for Rel-17 that no MR-DC is considered for SL. Furthermore, there are some dependencies with RAN2’s progress on signaling design details. In this proposal, we would focus on RAN3 identified impacts.

Q7. Companies are invited to indicate their view RAN3 focuses on identified XnAP and potential F1AP impacts. Please raise any other point here.

| Company | Y/N | Comment |
| --- | --- | --- |
| E/// | Yes |  |
| CATT | Yes |  |
| Qualcomm |  | It is our understanding that relay UE can still be in MR-DC. The limitation is only for the remote UE i.e., remote UE can’t be in MR-DC. But agree to study RAN3 impacts in path switching with MR-DC |
| ZTE | Yes |  |
| Samsung | Yes | We can start with basic scenarios, and may update depending on RAN2 progress. |
| China Telecom | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| Huawei | Yes | Agree with Samsung, we can focus on the baseline procedure first, and de-prioritize this issue. |
| Nokia |  | Agree with QC, but ok for Samsung proposal. |
| LGE | Yes |  |

Summary:

All say yes, expect one company clearly states the interest to study RAN3 impacts with MR-DC. 3 companies would prefer to study the basic scenarios first and wait for RAN2 progress.

Proposal 7:

RAN3 focuses on the XnAP and possible F1AP impacts to support the basic scenarios.

With regard to the detailed stage-2 and stage-3 changes, e.g., target Relay UE ID, signaling flows, and so on, we will keep in the second round once some agreements for the above questions can be made.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-224371, Discussion on Service continuity enhancement for U2N relay (ZTE)

- R3-224381, Discussion on Support Service Continuity Enhancements  (Nokia, Nokia Shanghai Bell)

- R3-224398, Service continuity for L2 U2N relay (China Telecommunication)

- R3-224522, Service Continuity Enhancements for Layer-2 UE-to-Network Relays (Ericsson)

- R3-224523, RAN decision on selection of the target Relay UE (Ericsson)

- R3-224552, SL relay: Inter-gNB mobility (Huawei)

- R3-224601, Service continuity enhancements for L2 relays (Qualcomm Incorporated)

- R3-224739, Support Service Continuity Enhancements for SL relay (CATT)

- R3-224740, Correction to 38.423 for SL relay (R18) (CATT)

- R3-224751, Service continuity enhancements for L2 U2N relay (LG Electronics)

- R3-224832, (TP to TS 38.401) Initial discussion on service continuity enhancement (Samsung)

- R3-224905, Service continuity for U2N relay (CMCC)