3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225773

Electronic Meeting, October 10th – 18th, 2022

Agenda item: 		10.2.2

Source:	Intel Corporation

Title:	MRO for fast MCG recovery

Document for:		Discussion and Decision

# Introduction

In RAN3#117-e meeting, the following agreements were achieved for MRO for fast MCG recovery [1]:

  - SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure)

  - the signalling delay is longer than the time the UE waits for the response (T316 expired);

  - other problem are not precluded if legacy MRO mechanism cannot cope with it.

The objective of this contribution is to analyze the fast MCG recovery failure issue.

# Discussion

Fast MCG recovery is a procedure that an MR-DC UE initiates in the event of RLF on PCell, instead of a RRC re-establishment procedure.

When MCG failure occurs, a UE will trigger an MCG Failure information to be sent to the MN via the SN either though split SRB1 or SRB3, and meanwhile the UE will start a timer T316 during which the UE will be waiting for more instructions from the MN. After receiving the MCGFailurelnformation message from the SN, MN will send a response message such as RRCReconfiguration or RRCRelease to guide the UE handover to either the current MN or another target MN or to transition to IDLE.

During the Fast MCG recovery procedure, the following two cases could happen and cause recovery failure:

- T316 expiry due to signalling transmission delay through network interface and/or Uu interface

- SCG failure due to SCG deactivated or SCG RLF

Observation: During the Fast MCG recovery procedure, T316 expiry or SCG failure could happen and cause recovery failure.

When the Fast MCG recovery failure happens, a UE will perform RRC re-establishment procedure which may lead to a longer interruption time. Therefore, in order to help with the network optimization to avoid the failure occurrence and improve the operating efficiency, it is beneficial to for the UE to report the Fast MCG recovery failure and also the failure type in the RLF report. And as in legacy, MN will collect the RLF report which includes the Fast MCG recovery failure related information and perform configuration optimizations.

Proposal: Fast MCG recovery failure and the failure type should be included in the RLF report.

If any failure happens after the successful fast MCG recovery, for example, RLF after RRC re-establishment, this case will follow the normal failure operation.

# Conclusion

In this contribution we analyze the fast MCG recovery failure issue, and the following observation and proposal were drawn from the analysis.

Observation: During the Fast MCG recovery procedure, T316 expiry or SCG failure could happen and cause recovery failure.

Proposal: Fast MCG recovery failure and the failure type should be included in the RLF report.

# References

- RAN3_117-e_agenda_20220825_EOM1