3GPP TSG-RAN WG3 Meeting #117b electronic                                                    R3-225810

10 October – 18 October, 2022

Agenda Item:	12.2.1

Source:	CMCC

Title:	Discussion on AI/ML Capability Exchange

Document for:	Discussion and Decision

1	Introduction

In last RAN3#117e-meeting, the exchange of AI/ML related capability was discussed, but no consensus achieved. On one hand, companies have different understanding of AI/ML related capability, such as supported AI/ML use cases, capability of providing predicted data, etc. On the other hand, companies have different views on whether transfer the capability over the Xn interference.

This contribution will further discussion on this issue.

2	Discussion

In some contributions submitted to last RAN3#117 E-meeting, it was proposed that before requesting the predicted data from neighboring NG-RAN node(s), the source NG-RAN node should first request AI/ML capability from a neighbouring NG-RAN node. The capability could be e.g., supporting energy efficiency prediction and resource status prediction. Also, it was proposed to signal over the Xn interface which use cases are supported with AI/ML by the NG-RAN node.

Then the issue was discussed in the CB: # AIRAN1_General_Stage2 [1], and companies provided their views:

The proponents deem that the exchange of AI/ML related capability is beneficial since requesting the AI/ML capability before the actual AI/ML model training and inference take place could help to reduce the signalling overhead of unnecessary data request, especially for predicted information as input information. Furthermore, the detailed capability information is diverse, including：

- enumerate information, i.e., true or false

- whether it hosts AI/ML training and inference

- supported AI/ML use cases

- capability of providing predicted data, rather than use case; since some predicted information as input are common for all three use cases, and the requesting NG-RAN node may use the prediction information for different purposes (i.e., NG-RAN node may use the predicted information for a different AI/ML use case from the neighbouring NG-RAN node)

And, some companies think that the supported models, model types, model details would not be transferred because it exposes sensitive information.

Therefore, it is observed that companies have different understanding of the AI/ML Capability, and further discussion is necessary to clarify the exact meaning of AI/ML capability.

Proposal 1: RAN3 is asked to discuss and converge on the meaning of AI/ML capability.

On the contrary, the opponents think that the explicit signaling is unnecessary since implicit way or error handling mechanism could work. Also, it can be done by OAM when configuring the NG-RAN nodes. Furthermore, if the node does not have the AI/ML model or cannot provide predicted data, it can reject the request. For instance, regarding to resource status prediction, when receiving the resource status prediction request, the node can reject the request if no resource status prediction model. Thus, there is no need to bring in additional capacity exchange.

In our understanding, the exchange of capability information is somehow helpful for the signalling overhead reduction when neighbouring nodes don’t support the requested use case or predicted data, especially when the capability changes dynamically. On the other hand, for the static or semi-static capability, the exchange is unnecessary since OAM configuration or error handling mechanism could help the requesting node to obtain the capability information.

Therefore, only for the capability information that changes dynamically, the capability exchange over Xn interference is meaningful.

Proposal 2: Exchange the dynamically changed AI/ML capability over Xn interface, and not to exchange the static or semi-static capability information.

To determine what kind of detailed capability information should be exchanged, the clarification on dynamic capability and static/semi-static capability is needed and more discussion is necessary.

Proposal 3: RAN3 is asked to discuss on the dynamic AI/ML capability and static/semi-static AI/ML capability.

3	Summary

In this contribution, we provide our views on the exchange of AI/ML related capability over Xn interference, and following proposals are made:

Proposal 1: RAN3 is asked to discuss and converge on the meaning of AI/ML capability.

Proposal 2: Exchange the dynamically changed AI/ML capability over Xn interface, and not to exchange the static or semi-static capability information.

Proposal 3: RAN3 is asked to discuss on the dynamic AI/ML capability and static/semi-static AI/ML capability.

4	Reference

[1] 	R3-225013, Summary of CB: # AIRAN1_General_Stage2, CMCC