3GPP TSG-RAN WG3 #117bis-e	R3-225924

10th – 18th Oct 2022

Online

Agenda Item:	14

Source:	Huawei (moderator)

Title:	CB: # MobilityEnh1_L1L2Mo

Document for:	Approval

# Introduction

This document contains the summary of the offline discussion for the following CB:

CB: # MobilityEnh1_L1L2Mo - Discuss on the message flow for L1/L2 based inter-cell mobility for intra-DU inter-CU and inter-DU intra-CU scenarios, e.g. HO preparation, HO execution, HO complete phases. - Discuss on whether gNB-DU can initiate the L1/L2 mobility configuration procedure, or update or modify the candidate cell(s) based on L1 measurements, etc. - Discuss on the issues, e.g. whether to support inter-DU data forwarding. - Capture the agreements and open issues (HW - moderator) Summary of offline disc R3-225924

# For the Chairman’s Notes

# Discussion

## Intra-DU L1/L2 mobility

Baseline message flow:

Quite lots of contributions (R3-225350, R3-225404, R3-225420, R3-225493, R3-225573, R3-225617, R3-225648, R3-225648, R3-225744, R3-225784, and R3-225796) propose a stage 2 overall procedure for intra-DU L1/L2 mobility.  And in R3-225645, a message flow is proposed based on the agreements of last RAN3 meeting. Some companies further propose RAN3 to adopt a message flow as baseline for further discussion. The moderator also thinks that a baseline message flow seems beneficial. The message flow in R3-225645 produced precisely based on the progress of last meeting are chosen by the moderator as the starting point to build the baseline.

The content in R3-225645 is cited here for your reference.

--------------------------------------------------------- the start of change ------------------------------------------------------------- - 8.2.1.X Intra-gNB-DU L1/L2 based Handover This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based handover. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based mobility procedure for intra-NR. - The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing the cell quality measurements of serving and neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU. FFS: the gNB-DU initiates the L1/L2 handover configuration to the gNB-CU containing a list of suggested candidate cells. - The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration. - WA: The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU containing the candidate cells list. - WA: If the request for configuring L1/L2 inter-cell mobility is accepted, the source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the candidate cells. FFS whether the DU can only accept some of the candidate cells suggested by the CU and provides a selected candidate cell list to the CU. - The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS). - FFS How L1 measurements for L1/L2 inter-cell mobility are configured e.g. Serving DU with/without Candidate DU involvement, CU, etc. - The source gNB-DU forwards the received RRCReconfiguration message to the UE. - The UE responds to the source gNB-DU with an RRCReconfigurationComplete message. - The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message. - WA: The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 handover. - WA: The gNB-DU sends the L1/L2 mobility command to the UE. FFS: how the gNB/gNB-DU detects the UE access and whether there is an F1 impact. FFS: For intra-DU L1/L2 handover, whether and how to release the source cell/prepared cells’ resources in the gNB DU is FFS. --------------------------------------------------------- the end of change -------------------------------------------------------------

The moderator would like to draw the following question:

Q1: Is above message flow reflecting the agreements/status of last meeting agreeable as the baseline for further discussion?

Please feedback your views and comments on the wording in below table.

| Company | Comment |
| --- | --- |
| vivo | Yes, except that the FFS in the first box after step 10 should be “how the gNB/gNB-DU detects the UE access and the F1 impact” to align with the agreement. |
| NEC | In order to facilitate and the discussion, it is good to have a baseline signalling flow. |
| E/// | In general the above steps are fine, together with WAs and FFS. Some of our comments are followed: - Since this is about intra-DU case, there is no concept of source DU. Suggest revising to the “the candidate gNB-DU”. - Update the wording “handover” to “L1/L2 inter-cell mobility”. We provided rewording in the above signaling flows. More would be required, which can be checked in the reviewing TP stage. |
| China Telecom | Agree to adopt the above message flow as baseline for further discussion. |
| Intel | Yes, except steps 3 and 4 where the parallel UE Context Modification procedure (like CHO/CPAC) is still on the table. We have agreed that the existing F1AP procedure (e.g., F1AP UE CONTEXT MODIFICATION) is reused for handover configuration for inter-cell mobility, but this is still working assumption and we didn't agree that only a single procedure is used to prepare multiple candidate cells simultaneously, which needs to be discussed. |
| Lenovo | Yes, it’s good to have the message flow as the baseline. |
| NTT DOCOMO | Agree. |
| LGE | The above message flow is agreeable. |
| Google | OK to have a baseline signalling flow for further discussion. |
| Qualcomm | The overall signalling flow looks fine. Some comments: Step 4: It may be added that a subset of the candidate cells suggested by the gNB-CU are accepted by the gNB-DU to be prepared for L1/L2 mobility, and the lower layer RRC configurations of these accepted cells are transmitted in the UE CONTEXT MODIFICATION RESPONSE. Step 9: The term “L1 measurement report” should be used to be consistent with Q5. |
| Nokia | OK, but image format needs to be editable (visio, or msc), otherwise revisions will be hard for other companies trying to provide input. Otherwise, signalling image to use for discussion moving forward OK. |
| CATT | Ok for most of the message flow, but as this is for intra-DU, no need to mention “source” |
| Samsung | Agree to discuss further based on the flow above. |
| ZTE | It is beneficial to have a baseline for further discussion. But we don’t quite agree with the description for step 4 since we have not discussed whether it is the CU or the DU decides the final candidate cell list, i.e. the CU informs the candidate cell list directly to the DU or the CU suggests the candidate cell list to the DU and the DU responds with the final candidate cell list (the DU may only accept some of the candidate cells suggested by the CU). So we added some extra contents in the description of step 4 to not preclude the case that the DU decides the final candidate cell list. Additionally, as for the FFS added by Ericsson for step 5, since we are talking about the procedure for the intra-DU case, we don’t understand why we need to mention Serving DU with/without Candidate DU involvement. So we would suggest removing “e.g. Serving DU with/without Candidate DU involvement, CU, etc.”. FFS How L1 measurements for L1/L2 inter-cell mobility are configured. |
| CMCC | We agree with above message flow. |
| Huawei | Agree to have a baseline message flow for further discussion. Agree to most of all comments above. Apart from: - The FFS after step 5 seems related to inter-DU case, better to remove it from the intra-DU here. For the comments on parallel UE Context modification, propose to add FFS in the text since we will discuss it later. “FFS: whether parallel UE Context Modification procedure should be used rather than a candidate list in a single message.” |

Moderator’s summary:

All companies agree to have a baseline mesasage flow for further discussion. And many companies provide rewording comments on the message flow.

Proposal 1: Continue refining on the intra-DU L1/L2 mobility message flow in R3-225645 by the end of Friday considering all comments received during the discussion.

Handover Configuration:

For gNB-CU initiated L1/L2 handover configuration, papers in R3-225350, R3-225646, R3-225420, R3-225493, R3-225574, R3-225617, R3-225796 R3-225784, and R3-225815 propose that the gNB-CU decides a suggested candidate cell list and sends the list to the gNB-DU in UE Context Modification Request message. The gNB-DU may decide whether to accept the candidate cells in the list as the target cell of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message.

While, in R3-225784, it is proposed to adopt a CHO/CPAC-like solution which is that parallel preparation is done for each candidate cell (indicated by SpCell ID IE) via the UE Context Modification procedure,

In summary, the following proposal reflecting majority view is made by the moderator:

Proposal: During L1/L2 handover configuration, the gNB-CU sends a suggested candidate cell list to the gNB-DU in UE Context Modification Request message. The gNB-DU decides whether to accept the candidate cells in the list as the target cells of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message.

Q2: Is above proposal agreeable? Any support to the parallel preparation per candidate cell?

Please feedback your views and comments on above proposal in below table.

| Company | Comment |
| --- | --- |
| vivo | We slightly prefer to agree the above proposal. From RAN3 point of view, both solutions are OK. But the proposed solution make it possible for further signaling optimization. If multiple candidate cells belongs to single DU, some configurations(e.g. RLC/MAC configurations) for UE to use in candidate cells can be shared by the candidate cells. This can reduce the signaling overhead on both F1 and Uu. |
| NEC | Yes, it is agreeable. For parallel preparation, can be FFS and align with RAN2 if parallel preparation can be configured to the UE, later can discuss how we design in F1AP e.g. suggested by R3-225784 is a way to do. |
| E/// | Yes to the first question, No to the second. For L1/L2 inter-cell mobility, parallel preparation per candidate cell will cause signaling overhead. Prefer to follow XnAP adopting per node level. |
| China Telecom | Agree with the proposal. |
| Intel | As analysed in our paper R3-225784, from NW preparation aspects, L1/L2 based mobility is the same with CHO/CPAC in terms of preparing multiple candidate cells and pre-configures those prepared cell configurations in advance to the UE. The difference lies on the HO execution part where CHO/CPAC is executed based on pre-configured conditions in the UE side, while here in L1/L2-based mobility, HO is explicitly commanded by NW based on L1 measurements. From this sense, it is better to design the signalling following CHO/CPAC approach that has been optimized for handling multiple candidate cell preparations with a DU and that has been already working well. |
| Lenovo | Agree with the first question. For the second question, it is better to wait for the RAN2 progress of RRC modelling. |
| NTT DOCOMO | Agree with the proposal. |
| LGE | The above proposal is agreeable. But, as NEC mentioned, it seems necessary to align with RAN2 whether the parallel preparation can be configured to the UE. |
| Google | Agree with the first question and for the second question we also think RAN2 progress should be checked first. |
| Qualcomm | Yes, this is agreeable for the gNB-CU initiated preparation procedure. We also think that gNB-DU may initiate the preparation procedure (please see our response to Q3 below). The parallel preparation proposal is probably motivated by the fact that much of the existing signalling can be reused. However, it can be simpler and better from the point of view of latency to prepare multiple candidate cells at once, and enhancements to the existing signalling can be done for this purpose. We therefore do not support parallel preparation of candidate cells. |
| Nokia | Agree wit the statement in the question. However, we assume there is a single message and not multiple messages per cell. |
| CATT | Yes, agree with the proposal. |
| Samsung | Not sure at this moment. The intention is to use one message to include candidate cell list, and use another message to reflect the access control result. However, different candidate cells may result in different access results, e.g., the admitted DRBs may be different for different candidate cells. How to reflect such difference? |
| ZTE | Is the first proposal precluding the case the DU may only accept some of the candidate cells suggested by the CU? If no, we are fine with this propose and we would suggest adding an extra FFS to further clarify, e.g. FFS DU can only accept some of the candidate cells suggested by the CU and provides a selected candidate cell list to the CU. For the second question, we agree that the it is better to wait for RAN 2 progress. |
| CMCC | Agree with the first question, FFS to the second. |
| Huawei | Agree to the proposal. For parallel handover preparation, we don't think its needed but ok to check with RAN2. Regarding the question from ZTE, we think that it’s apparent that the DU may accept some of the candidate cells. |

Moderator’s summary:

Majority view thinks that the proposal from moderator is agreeable.

1 company still thinks CHO/CPAC approach is their preference. 3 companies think that the parallel preparation approach is related to the RRC modelling whichi is discussing in RAN2 and propose to wait for RAN2 progress. 1 company raised a question that how to handle the case if the admitted DRBs is different for different candidate cells in a single response messsage?

Based on the status above, the moderator would like to propose:

Proposal 2: During L1/L2 handover configuration, the gNB-CU sends a suggested candidate cell list to the gNB-DU in UE Context Modification Request message. The gNB-DU may accept all, or part of the candidate cells in the list as the target cells of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message.

Further discuss in 2nd round:

How to handle the case if the admitted DRBs is different for different candidate cells in a single response messsage?

The following open issue was made at last meeting:

FFS on whether gNB-DU can also initiate the L1/L2 mobility configuration procedure.

In R3-225404, a gNB-DU initiated L1/L2 handover configuration procedure is proposed. In which, the gNB-DU suggests the candidate cells for L1/L2 mobility to the gNB-CU based on UE L1 measurements via a F1AP class 2 procedure. The gNB-CU sends the accepted the candidate cells including the cell configurations to the gNB-DU and to the UE.

In R3-225784,  it proposes that HO preparation with DU and HO configuration to the UE are separated as NW cannot anticipate when and to which cell to switch the UE at the time when configuring L1/L2 mobility to the UE (like CHO/CPAC).

On the other hand, majority views in R3-225646, R3-225573, R3-225735, R3-225762, R3-225815, R3-225796 and R3-225744 think that the gNB-DU should not initiate the L1/L2 handover configuration due to various drawbacks and reasons.

In summary, the moderator tries to draw a proposal based on the majority view.

Proposal: gNB-DU initiated L1/L2 handover configuration is not allowed.

Q3: Is above proposal agreeable?

Please feedback your views and comments on the above proposal in below table.

| Company | Comment |
| --- | --- |
| vivo | Yes, we agree the proposal. gNB-DU initiated L1/L2 handover configuration consists two steps: gNB-CU configurates UE with configuraion of L1 measurement based on L3 measurement result reported by UE; gNB-DU initiated L1/L2 handover configuration according to the L1 measurement result reported by UE; gNB-CU initiated L1/L2 handover configuration consists only one step: gNB-CU initiated L1/L2 handover configuration based on L3 measurement result reported by UE. Hence, gNB-CU initiated L1/L2 handover configuration leads to less latency for L1/L2 handover configuration. Due to the fast change of signal quality for FR2, it may be too late to initiate L1/L2 handover if candidate cells are configured after L1 measurement results are already good. |
| NEC | We also support that the gNB-CU to initiate L1/L2 handover configuration, based on UE’s measurement result. |
| E/// | Yes. The gNB-DU should just forward the measurement report to the gNB-CU, then CU decides whether to initiate the L1/L2 inter-cell mobility since RAN2 would make decision that measurement result for preparation could be L3. |
| China Telecom | Agree, it should be the gNB-CU to initiate L1/L2 handover configuration. |
| Intel | We are not sure how our analysis (cited above) that HO preparation with a DU and HO configuration to the UE should be separated is related to the proposal of DU-initiated configuration, but we are in the position that DU may only "suggest" some candidate cells for L1/L2 based mobility, but only this is when the majority agrees to do so, and definitely the final decision whether to initiate L1/L2 mobility and which candidate cells to prepare shall be by CU. |
| Lenovo | Agree with the proposal, it’s the CU to initiate the L1/L2 HO configuration, e.g., based on the L3 measurement result. |
| NTT DOCOMO | Agree. We think Rel-18 L1L2 based inter-cell mobility should support only gNB-CU initiated procedure. The gain of gNB-DUinitiated procedure is not clear, and in inter-DU case, the network signaling may be more complicated than gNB-CUinitiated procedure. On the other hand, a mechanism that gNB-DU can propose or update candidate cell(s) may be useful in some cases, and this is discussed in Q4. |
| LGE | The above proposal is agreeable because the gNB-CU should decide whether to initiate the L1/L2 inter-cell mobility based on the measurement result. |
| Google | Agree with the proposal for the CU to initiate the L1/L2 HO configuration |
| Qualcomm | We do not agree to the proposal and think that gNB-DU initiated preparation should also be supported besides gNB-CU initiated preparation. As mentioned in our contribution R3-225404, we think that the gNB-DU can suggest candidate cells for L1/L2 mobility to the gNB-CU based on L1 measurement results received from the UE. The gNB-DU may perform further processing of the L1 measurements to determine the candidate cells to suggest for L1/L2 mobility, a process that we think can be left up to gNB-DU implementation. In a message to the gNB-CU, the gNB-DU can provide (a) List of candidate cells for L1/L2 mobility, (b) Lower layer configuration associated with each candidate cell. The gNB-CU accepts a subset of the candidate cells suggested by the gNB-DU to be configured for L1/L2 mobility for a UE and provides the final configurations to be forwarded to the UE. The procedure may involve lower latency than the gNB-CU initiated procedure, as mentioned in our contribution. An argument that was brought up in the contributions not agreeing to gNB-DU initiated preparation is that the L1 measurements are too fluctuating to be reliable and hence the procedure should not be initiated based on them. As mentioned in our contribution, the gNB-DU can perform further processing of the L1 measurements (e.g., averaging) to determine the candidate cells for L1/L2 mobility. |
| Nokia | Agree with the proposal. DU should not be allowed to initiate the L1/L2 mobility procedure |
| CATT | Yes, before configuration the candidate cell, there no L1 measurement results. |
| Samsung | Yes, we think it should be the gNB-CU who initiate the configuration. |
| ZTE | Agree. Allow the DU to initiate the L1/L2 mobility configuration procedure would make the whole procedure more complicated and conflict with the CU initiated L1/L2 mobility configuration procedure. |
| CMCC | We agree the proposal. The node which initiates the procedure can only be gNB-CU. The L3 measurement including the cell measurement is reported through RRC signalling to CU, hence CU will be the first node acquiring the measurement results. |
| Huawei | The proposal is agreeable for us. |

Moderator’s summary:

Almost all companies agree with the proposal. And the proponent for DU initiated L1/L2 mobility configuration keeps their position.

The moderator tries to draw a proposal accoding to the majority view.

Proposal 3: gNB-DU initiated L1/L2 handover configuration is not allowed.

In R3-225350, it is proposed to allow the gNB-DU to suggest, if any some candidate cells to the gNB-CU in the UE Context Response message. In R3-225815 and R3-225784, it is proposed to allow the gNB-DU to update or modify the candidate cell(s) based on L1 measurements. But there is no any detail on how and in which step that the candidate cells are updated.

Q4: Can the gNB-DU modify or update the candidate cells suggested by the gNB-CU? If yes, in which step in the message flow in Q1?

Please feedback your views and comments on the above question in below table.

| Company | Comment |
| --- | --- |
| vivo | Yes. Based on the candidate cells’ load or L1 measurement results, gNB-DU can modify or update the candidate cells suggested by the gNB-CU in step 4. If one candidate cell‘s load is changed and too heavy to server the UE, gNB-DU can modify or update the candidate cells after step 4 and before step 10. |
| NEC | If this is NOT to ask if the gNB-DU can suggest new candidate cells, then we think the gNB-DU can update the configuration of candidate cells that was suggested by the gNB-CU. |
| E/// | The answer is Yes. Regarding in which step, please refer to our F1AP CR in R3-225351. The proposal is to add the suggested candidate cells in the UE Modification Required message from DU. The intention is to follow existing F1AP principle that DU is the one who sends request when it realizes updates need to be done. |
| China Telecom | We think the gNB-DU should have the ability to adjust the candidate cell set by refusing one/some of the requested candidate cell ID(s) through UE CONTEXT MODIFICATION RESPONSE message. |
| Intel | Again, we are in the position that the final decision on which candidate cells to prepare shall be by CU. DU may suggest some candidate cells for L1/L2 mobility (only when the majority agrees to do so, details FFS), but once CU decides which candidate cells to prepare and initiates the parallel UE Context Modification procedure for each candidate cell, DU shall not modify or update during such procedure - i.e. DU shall be only allowed to either accept or reject (as same to CHO/CPAC). Please see our Proposal 4 of R3-223784 which follows the CHO/CPAC approach that has been optimized for handling multiple candidate cell configurations with a DU: Proposal 4: For intra-DU L1/L2 mobility, in terms of preparing multiple candidate cell configurations with DU over F1, RAN3 to follow the CHO/CPAC approach that has been optimized for handling multiple candidate cell configurations with a DU, i.e. - HO preparation with DU and HO configuration to the UE are separated as NW cannot anticipate when and to which cell to switch the UE at the time when configuring L1/L2 mobility to the UE (like CHO/CPAC). - For HO preparation with DU, adopt parallel preparation signalling (like CHO/CPAC) each for each candidate cell (indicated by SpCell ID IE) via the UE Context Modification procedure, where DU is able to accept/reject the request for each candidate cell basis and also able to provide lower-layer configuration separately for each candidate cell. - Add an IE similar to "Conditional Intra-DU Mobility Information" IE in the UE CONTEXT MODIFICATION REQUEST message that handles L1/L2 mobility initiation/replace/cancellation triggered by CU-CP (like CHO/CPAC). - Extend the Candidate Cells To Be Cancelled List IE in the UE CONTEXT MODIFICATION REQURED message to handle cancellation request triggered by DU (like CHO/CPAC). |
| Lenovo | We think it is possible for the CU to update or release the suggested candidate cells. DU can accept or refuse some suggested candidate cells that were suggested by the CU. Of course, DU can update the configuration of candidate cells. |
| NTT DOCOMO | Yes. In step 4, between step7-step9, or after HO completion, gNB-DU can modify or update the candidate cells. This is because if the gNB-DU determines that some of the cells selected as candidate cells are inappropriate, or if it has a better cell, it cannot immediately reflect the candidate cell list update to the UE and other network nodes, it may result in HO failure or inefficient mobility. |
| LGE | Yes. As Ericsson mentioned, from the existing F1AP principle point of view, the gNB-DU may request the modification or update of the candidate cells suggested by the gNB-CU. Regarding step, if this request is based on L1 measurement, the message flow will be existed between step 9 and step 10. |
| Google | Yes for that the DU can accept or reject some candidate cells from CU. |
| Qualcomm | Yes, we think that the gNB-DU may modify or update the candidate cells suggested by the gNB-DU based on L1 measurements. The suitable step for this purpose in the message flow in Q1 is Step 4. |
| Nokia | At this point we should focus on CU proposing certain cell(s) and DU either accepting or rejecting them. Whether the DU can “propose” different cells than those indicated by CU is not our view and is a separate topic. |
| CATT | For DU modify or update the candidate cell, it can have two difference understandings: One is CU give candidate cell list to DU, and DU choose some of the cells in the list to generate configuration, such modification should be supposed (the modify only include access control, not include add new cell out of candidate cell list suggested by CU ).And this is what we discussed in Q2. Another is after CU configurde the candidate cells to UE, and UE start L1 measurement and send measurement results to the DU, in this time, whether enable DU to modify and update candidate cells. We think that is a rare case and depend on how many candidate celsl will configure to UE. If candidate cell is configured enough, not need such adjust. This issue can be discussed latter. |
| Samsung | Yes, with the UE CONTEXT MODIFICATION REQUIRED message. |
| ZTE | Yes. There are two kinds of cases. The first one is as we mentioned in the comments for Q1. In this case, the selected candidate cell list is provided to the CU via the UE context modification response message. The second case is that after the L1 measurements are configured, the DU can make the decision based on L1 measurements. In this case, this step shall be placed between step 9 and 10. UE Modification Required message can be reused. One thing we would like to clarify is that the DU can only consider to modify or update the candidate cells which are in the range of the original candidate cell list suggested by the CU. |
| CMCC | Yes, we think DU have the capability to modify the candidate cells. |
| Huawei | No, we think that the gNB-DU may accept some of candidate cells suggested by the gNB-CU, but it cannot suggest new cells to the gNB-CU. The gNB-CU should have full knowledge when sending the candidate cell list to the gNB-DU. |

Moderator’s summary:

First of all, sorry to make a confusing question. The intention of Q4 is to discuss whether the gNB-DU can suggest new candidate cells after the gNB-CU inidates the L1/L2 configuration.

Not sure if all companies understand the questions right, but quite few companies think that the gNB-DU should be able to suggest new candidate cells to the gNB-CU, e.g., in step 4, or between step 9 and 10.

The moderator therefore prospoe to further discuss this online with the following proposal:

Proposal 3: Discuss online about whether the gNB-DU can suggest new candidate cells after the gNB-CU initiates the L1/L2 handover configuration.

Handover Execution:

At last meeting, RAN3 made a WA on HO execution phase that:

"WA: RAN3 assumes that the UE sends the L1 measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell. All details are up to RAN1 and RAN2 discussion".

The following agreement was made in RAN2 at the same time.

- Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS)

Based on the above RAN2 agreement, papers in R3-225350, R3-225404, R3-225420, R3-225493, R3-225573, R3-225646, and R3-225815 propose to turn the WA into agreement.

Proposal: To turn the WA into an agreement: “The UE sends the L1 measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell.”

Q5: Is it agreeable to turn the WA into an agreement as above proposal?

Please feedback your views and comments on the above question in below table.

| Company | Comment |
| --- | --- |
| vivo | Yes. |
| NEC | yes |
| E/// | Partially yes, the wording “L1 measurement report” needs to be changed. RAN2 has not made any agreement on measurements yet, but uses the word “assume”, which implies the measurements may not be L1. The discussion is ongoing in both RAN1 and RAN2 whether the UE sends the L1 measurement report or via MAC. From RAN3 signalling flow point of view there would be no difference between L1 or MAC. Thus, we propose to change to “lower-layer measurement report” is sent from the UE to the serving DU. |
| China Telecom | Agree. Ericsson’s rewording is OK for us. |
| Intel | Yes. May be just re-word to "L1 and/or L2 measurement" |
| Lenovo | Yes. |
| NTT DOCOMO | Yes. |
| LGE | Yes. Regarding rewording the “L1 measurement report”, it seems necessary to align with RAN2 rather than the decision by RAN3. |
| Google | Yes and support the re-wording suggestions. |
| Qualcomm | Yes, this seems reasonable. |
| Nokia | Yes |
| CATT | Yes |
| Samsung | Yes |
| ZTE | Yes. We are fine with the rewording. |
| CMCC | Yes, the rewording is acceptable. |
| Huawei | Yes, OK to the wording from E///. |

Moderator’s summary:

Consensus achieved with one rewording.

The proposal is:

Proposal 5: To turn the WA into an agreement: “The UE sends the lower-layer measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell.”

There is an FFS on how the gNB-DU detects the UE access and whether there is F1 impact.

FFS on how the gNB/gNB-DU detects the UE access and whether there is an F1 impact.

Among the contributions in this meeting, papers in R3-225350 and R3-225617 propose to wait for RAN2 progress for all. While in R3-225646, R3-225420, R3-225573, R3-225735, R3-225815. R3-225493, R3-225762, R3-225796 and R3-225744, all propose that how the gNB-DU detects the UE access is up to RAN2. And once the gNB-DU detects the UE, it informs the gNB-CU by the Access Success message with the target cell ID.

And furthermore, in R3-225762 it thinks that the F1-U DDDS can be reused for this purpose. In R3-225404, it thinks that the RRCReconfigurationComplete in the target side can be used. And in R3-225573, it thinks that RRCReconfigurationComplete in the target cell is needed after RACH or RACH-less procedure.

Another different approach proposed in R3-225784 is DDDS + ACCESS SUCCESS as soon as DU confirms which cell the UE is going to access. This approach requires the gNB-DU sends out the Access Success once it receives the successful ACK for the L1/2 HO CMD.

Regarding the DDDS from gNB-DU to gNB-CU, R3-225646 points out that if the RLC is not re-established during handover, the DDDS message can be skipped. This is pending to RAN2 discussion. And the need of RRCReconfigurationComplete at target cell should be also discussed by RAN2.

Therefore, the moderator would like to check your views on following two options:

- Option 1: the gNB-DU sends the Access Success to the gNB-CU after it detects the UE appeared in the target cell.

- Option 2: the gNB-DU sends the Access Success to the gNB-CU once it receives the successful ACK for the L1/2 handover command.

The tentative proposal reflecting majority view is like:

Proposal: How the gNB-DU detects the UE access to the target cell is up to RAN2. Once the gNB-DU detects the UE, it informs the gNB-CU by the Access Success message with the target cell ID. The need of RRCReconfigurationComplete in the target cell is FFS and up to RAN2.

Q6: Is the proposal agreeable? Any views and comments to option 2?

Please feedback your views and comments on the above question in below table.

| Company | Comment |
| --- | --- |
| vivo | Agree with the proposal. How the gNB-DU detects the UE access depends on if RACH is used in L1/2 handover, which is in RAN2‘s scope. Similarly, if RRCReconfigurationComplete is needed in L1/2 handover is also up to RAN2 decision. For option 2, gNB-DU receives ACK from the L1/2 handover command does not mean that the L1/2 handover is succeed, so ACCESS SUCCESS shall not be sent to gNB-CU at this time. |
| E/// | We think it is premature to down-select any option for now. These two options could be under discussion for inter-DU also. For example, in Option 2, it should the source DU who sends the Access Success to CU. Our suggestion is opening inter-DU solution discussion in the second round. |
| China Telecom | Agree with the proposal. |
| Intel | Not sure what it means by "once DU detects the UE" but we support the option 2 because the candidate cells configuration has been successfully configured to the UE before and it is the DU who commands the UE to switch to one of candidate cells. The purpose of this L1/2 HO CMD is nothing more than to simply command the UE to handover to which cell among pre-configured. Once successful ACK, the commanding DU knows which cell the UE will be handed over to for 100% sure. Please note that this execution part is different to CHO/CPAC where NW has no idea which candidate cell (among pre-configured) the UE is going to access until the UE actually access the corresponding cell via RACH or whatever. Here in L1/L2 mobility, DU who sends L1/2 HO CMD knows which cell the UE will be handed over, because it is that DU that decided (based on L1 and/or L2 measurements). And as discussed in our paper R3-225784, we also need to consider CU-UP as well into our picture. Please read our R3-225784 for intra-DU case and R3-225785 for inter-DU case. |
| Lenovo | Agree with the proposal. |
| NTT DOCOMO | Agree with the Proposal. However, as some companies pointed out, to initiate data forwarding earlier, it may be effective to notify the CU of the target cell ID earlier, like in option 2. RAN3 should consider option 1 as a baseline, and option 2 should be discussed with data forwarding. |
| LGE | The proposal is agreeable. For Option 2, when the gNB-DU receives the successful ACK for the L1/L2 handover command, whether it means the L1/L2 handover is succeed is up to RAN2. |
| Google | Agree with Intel’s analysis that the DU decides and sends the L1/L2 HO CMD knows which cell the UE is going to access. |
| Qualcomm | The proposal is agreeable. Regarding Option 2, it might be the case that the UE fails to access the target cell. In that case, Option 2 would have been the wrong thing to do and is thus not agreeable. |
| Nokia | Selection is premature. We should both study whether to (a) inform CU after command for L1/2L2 handover and (b) after UE access the target cell. In that sense both should be marked as FFS. |
| CATT | Prefer the proposal as baseline and option 2 maybe an additional optimization for reducing latency. For the option 2, DU receive the ACK message only means UE get the HO command and will change the cell, not means change cell successful. |
| Samsung | It is too early to do the down-selection. There are two issues: - Issue 1: how to inform gNB-CU the success access of UE towards target cell - Issue 2: how to inform gNB-CU the target cell? For issue 1, the legacy DDDS can be reused as the legacy RRC based HO. For issue 2, the above two options can be considered as candidate solutions. For option 1, this can be done by either gNB-DU (F1 ACCESS SUCCESS message) or UE (depending on RAN2 discussion); for option 2, it relies on an existing or new F1AP message. In our understanding, option 2 may bring more benefit than option 1. For example, option 1 can avoid the triggering of L3 HO from gNB-CU side for the same target cell. So, our proposal is: How the gNB-DU detects the UE access to the target cell is up to RAN2. The source gNB-DU can inform the target cell once it receives the successful ACK for the L1/2 handover command |
| ZTE | We agree with NTT DOCOMO. We think these two options can coexist since option 2 would be beneficial, e.g. for data forwarding. To avoid the confusion with option 1, maybe we can use other F1 messages to notify the CU earlier instead of the access success message. |
| CMCC | Agree with the proposal. |
| Huawei | Agree with the proposal. We are not sure with option 2. Since the purpose of this indication is to indicate the gNB-Cu that the UE has accessed the target cell successfully. It should be triggered by the target cell. Option 2 seems triggered by the source cell. |

Moderator’s summary:

Views are quite diverse. The moderator proposes to keep this issue open:

Proposal 6: How the gNB-DU detects the UE access to the target cell is up to RAN2. FFS on when the gNB-DU should indicates the gNB-CU about the successful access to the target cell of the UE.

- Option 1: the gNB-DU indicates to the gNB-CU after it detects the UE appeared in the target cell.

- Option 2: the gNB-DU indicates to the gNB-CU once it receives the successful ACK for the L1/2 handover command.

Handover Complete:

Regarding the resource release of the source cell/other prepared cells after L1/L2 handover, it is still FFS:

For intra-DU L1/L2 handover, whether and how to release the source cell/prepared cells’ resources in the gNB DU is FFS.

Papers in R3-225350, R3-225735, and R3-225744 think that we should assume the source cell and other prepared cells are not released after L1/L2 handover to support the subsequent handover to those cells.

On the other hand, papers in R3-225573, R3-225646, R3-225404, R3-225420 and R3-225493 think that the gNB-CU shall have the capability to modify, and release the prepared cells resources (including the source cell) irrespective the subsequent handover. The reason is that the prepared cells may be not suitable anymore as target cells for future L1/L2 handovers along with the UE movement. And the UE Context Modification procedure can be reused in intra-DU case.

In R3-225744, it proposes to reuse the UE Context Release procedure (gNB-CU initiated) and UE Context Release Request procedure (gNB-DU initiated) to release the source cell/prepared cells’ resources.

The UE Context Release will release the whole UE context in the gNB-DU, so it seems not proper to be used to release the prepared cells resources in intra-DU case.

Based on above summary, the moderator proposes the following proposal:

Proposal: For intra-DU L1/L2 mobility, the gNB-CU uses the UE Context Modification procedure to modify or release the prepared cells resources at the gNB-DU (incl. the source cell).

Q7: Is the proposal agreeable?

Please feedback your views and comments on the above question in below table.

| Company | Comment |
| --- | --- |
| vivo | Yes, the UE Context Modification procedure can be used in intra-DU scenario to modify or release the prepared cells for L1/L2 handover. |
| NEC | OK for the moment. May be other options would be DU release source cell and other target cells (if can prepare multiple target cells) locally then inform gNB-CU.. |
| E/// | Prefer to wait. The need to discuss this depends on the RAN2 solution for the execution: i) UE releases candidates upon execution; or ii) UE keeps candidates upon execution. If RAN2 agrees that the UE releases RAN3 may discuss how to release / cancel candidates. At the moment, it needs to wait. |
| China Telecom | Agree with the proposal. |
| Intel | We are fine as long as we follow the F1AP signalling design for CHO/CPAC. |
| Lenovo | Agree with the proposal, using the existing UE Context Modification Request message. |
| NTT DOCOMO | Agree with the proposal. |
| LGE | We share a similar view as Ericsson. |
| Google | Agree with E/// and prefer waiting for RAN2. |
| Qualcomm | Upon handover completion, we think that the gNB-CU should decide whether to keep or release the source cell and/or the candidate target cells, and whether to modify the configurations or resources associated with the source cell and/or the candidate target cells, if it keeps them. The gNB-DU can accordingly indicate to the gNB-DU, e.g., by using the UE Context Modification procedure, and the UE. The benefit of not releasing is that the latency resulting from network procedures to reconfigure the UE for the next cell change is avoided. RAN2 may also need to be involved because current UE behaviour is that UE releases the source cell configuration upon accessing the target cell. |
| Nokia | Needs more study. However, as long as the procedure is optional for now, it is OK as it need more discussion on how to address this case (source becoming new target candidate cell) |
| CATT | Yes, the details can be FFS according to RAN2 |
| Samsung | Agree with the proposal |
| ZTE | We share the same view as Ericsson. This may depends on the solution design in RAN2. |
| CMCC | Agree with Ericsson. |
| Huawei | We agree with the proposal. Even RAN2 decides that the UE will keep the candidate cells configurations after handover execution, the gNB-Cu should be able to update or modify, release the configuration in the UE. |

Moderator’s summary:

Some companies are ok to the proposal. Some other companies think that it depends on UE behavior upon handover execution. 1 Comapany clarifies that even RAN2 decides that the UE will keep the candidate cells configurations after handover execution, the gNB-CU should be able to update or modify, release the configuration in the UE.

Based on the status above, the moderator would like to keep the release step open and optional in order to build a complete message flow.

Proposal 7: For intra-DU L1/L2 mobility, the gNB-CU may or may not use the UE Context Modification procedure to modify or release the prepared cells resources in the gNB-DU (incl. the source cell). Details are pending to RAN2.

User plane handling in intra-DU L1/L2 mobility:

Since this is the last question that this CB can discuss, the moderator collects all the proposals related to user plan handling for intra-DU case here and calls for comments.

- F1-U UL/DL TEID handling, proposed in R3-225646, R3-225783 and R3-225784.

- DDDS on F1-U, proposed in R3-225646, R3-225784 and R3-225762

- E1 impact, such as setup, update or remove resources at gNB-CU-UP, in R3-225574.

- Data forwarding, in R3-225574.

The moderator would like to check the companies’ views on those issues.

Q8: Do you agree to study above issues related to user plane handling in intra-DU L1/L2 mobility?

Please feedback your views and comments on above question in below table.

| Company | Comment |
| --- | --- |
| vivo | For a), UL/DL TEID will be changed if L1/2 handover happened, so the handling for F1-U UL/DL TEID is needed to be studied. For b) and d), if RLC is not re-established for intra-DU case, DDDS and data forwarding are not needed. So we propose to wait for RAN2 process before RAN3 starts to discuss b) and d). For c), we are open to study. |
| NEC | Yes, anyway this will need to discuss. But let us conclude some basic things, then can continue to discuss the user plane related aspect. |
| E/// | a) ok to discuss whether new TEIDs needs are needed. b) current agreement in RAN2 is not stable yet and will be continued. Prefer to wait for further discussion. c), d) open to discuss, not see detailed analysis on E1 at least. |
| China Telecom | We think we should focus on the basic procedure for L1/L2 mobility first and then evaluate the user plane handing. |
| Intel | a) Yes, change of F1-U UL/DL TEID handling shall be supported following the legacy intra-DU HO principle (as specified in TS 38.401) for DU to differentiate which data to be transmitted with old configuration or new configuration within the same DU. b) Yes, DDDS is also necessary considering CU-UP and also to support change of F1-U UL/DL TEID for subsequent L1/L2 handovers. c) There are two ways to handle bearer contexts that we have agreed during Rel-16 CHO. Multiple bearer contexts can be established in the same CU-UP for the same UE as if different UEs, or only one bearer context can be maintained in the same CU-UP. We believe R3-225574 is talking about the former case, while in our paper R3-225784, we considered the latter case and provided the whole procedure involving E1 and the same CU-UP based on this cell-agnostic "one" bearer context that we have agreed before. d) Early data forwarding - need clarifications on how early data forwarding worked in the past (e.g. Rel-16 intra-CU CHO) and also should be worked out together with considering the CU-UP entity. |
| Lenovo | We are open to discuss the user plane related aspects. However, it’s better to focus on the basic procedure for L1/L2 mobility first. |
| NTT DOCOMO | We did not mention about this point in Tdoc, however, we think discussion on user plane handling may be useful to interruption reduction. RAN3 should discuss a high-level procedure considering option 1 in Q6 as a baseline, thereafter, we should discuss a mechanism for data forwarding. |
| Google | Prefer focusing on signalling aspects first and then the user plane handling. |
| Qualcomm | Yes, the above issues should be studied. |
| Nokia | Trigger an FFS on this item and continue at further meetings. |
| CATT | Agree with Google, all proposal can be discussed one by one, but now we should have a baseline signal flow. |
| Samsung | One thing we would like to clarify is whether the inter-DU data forwarding is in the scope of this WI considering the restriction in WID, i.e., no new RAN interfaces are expected. In our understanding, the user plane data from between two DUs does not mean the interface should be established between them since the forwarding is only about IP packet forwarding. |
| ZTE | We are fine to discuss these issues. |
| CMCC | In our view, RAN3 should focus on the procedure first. FFS for issues related to user plane handling. |
| Huawei | We are fine to study those issues above in next meetings. |

Moderator’s summary:

If we leave the inter-DU data forwarding to the inter-DU case, the following is proposed based on the discussion above:

Proposal 8: The following open issues on user plane handling in intra-DU L1/L2 mobility are identified for further study:

a)	F1-U UL/DL TEID handling.

b)	DDDS on F1-U

c)	E1 impact, such as setup, update or remove resources at gNB-CU-UP

d)	Data forwarding

# Conclusion, Recommendations

TBD

# References

| R3-225315 | LS on L1/L2-based inter-cell mobility (RAN2) | LS in |
| --- | --- | --- |
| R3-225350 | Solutions for L1/L2 based inter-cell mobility (Ericsson) | discussion |
| R3-225351 | Additions for L1/L2 based inter-cell mobility (Ericsson) | CR1037r, TS 38.473 v17.2.0, Rel-18, Cat. B |
| R3-225404 | Signalling Support for L1/L2 based Inter-Cell Mobility (Qualcomm Incorporated) | discussion |
| R3-225420 | Consideration on support of intra-DU L1/L2 based Inter-Cell Mobility (China Telecommunication) | discussion |
| R3-225421 | Consideration on support of Intra-CU inter-DU L1/L2 based Inter-Cell Mobility (China Telecommunication) | discussion |
| R3-225493 | Discussion on L1/L2 based inter-cell mobility (Lenovo) | discussion |
| R3-225494 | (TP to TS 38.401 & TS 38.470) Support of L1/L2 based inter-cell mobility (Lenovo) | other |
| R3-225573 | Discussion on Intra-gNB-DU L1/L2 Inter-Cell Mobility Procedure (Nokia, Nokia Shanghai Bell) | discussion |
| R3-225574 | Discussion on Additional Considerations for L1/L2 Inter-Cell Mobility (Nokia, Nokia Shanghai Bell) | discussion |
| R3-225617 | Further discussion on L1/L2 mobility (CATT) | discussion |
| R3-225645 | Introduction of L1/L2 handover (Huawei) | CR0260r, TS 38.401 v17.2.0, Rel-18, Cat. B |
| R3-225646 | (TP for L1L2Mob BLCR for TS 38.401): Discussion on L1/L2 Mobility procedure (Huawei) | discussion |
| R3-225648 | Discussion of L1/L2 based Inter-Cell Mobility (NTT DOCOMO INC.) | discussion |
| R3-225735 | Open issues for L1/L2 based inter-cell mobility (LG Electronics Inc.) | discussion |
| R3-225743 | Discussion on collision between L1/L2 based mobility and L3 mobility (vivo) | discussion |
| R3-225744 | Discussion on L1/L2 based mobility procedures (vivo) | discussion |
| R3-225762 | Discussion on L1/L2 based Inter-cell Mobility (Samsung) | discussion |
| R3-225783 | High-level principles for L1/L2 based inter-cell mobility (Intel Corporation) | discussion |
| R3-225784 | Considerations for L1/L2 based "intra-DU" mobility (including TPs for L1/L2 Mob for TS 38.401 and TS 38.473) (Intel Corporation) | discussion |
| R3-225785 | Considerations on L1/L2 based "intra-CU inter-DU" mobility (including TPs for L1/L2 Mob for TS 38.401 and TS 38.473) (Intel Corporation) | discussion |
| R3-225796 | Discussion on L1/L2 based Inter-Cell Mobility (CMCC) | discussion |
| R3-225815 | Discussion on L1/L2 based mobility (ZTE) | discussion |
| R3-225816 | TP for L1L2Mob BL CR to TS 38.473 (ZTE) | other |