3GPP TSG-RAN WG3 #118-e                                                                                           R3-226284

France, 14th - 18th November, 2022

Agenda Item:	16.3

Source:	CATT

Title:	Discussion on Service Continuity Enhancements for SL relay

Document for:	Discussion

- Introduction

In last meeting, some agreements for service continuity enhancements for U2N relay were achieved [1]:

WA: Source gNB selects the target path type (direct or indirect)

For direct/indirect to indirect path switching, enhance Xn: HANDOVER REQUEST to include at least the Remote UE L2 ID and Relay UE L2 ID. FFS whether to include a single Target Relay L2 ID or a list of Target candidate Relay L2 IDs.

For inter-gNB path switching scenarios, RAN3 should specify mechanisms to support service continuity for L2 U2N relays in NG based handovers as well after supporting service continuity for L2 U2N relays in Xn based handovers, If there is some conclusion from SA2, and then to support NG based HO.

WA: During inter-gNB path switching, source gNB can signal the serving cell of the relay UE to target gNB via existing IE Target Cell Global ID.

In this contribution, we further discuss the service continuity and give us proposals.

- Discussion

a) Inter-gNB indirect-to-direct path switching

For the handover preparation/execution stage, it is same as the normal UE inter-CU handover. After switching, target CU sends a UE context release message to source CU. Source CU may send UE context modification request to gNB-DU and also send a RRC reconfiguration to source relay UE to release remote UE context e.g., release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE. Note that the source gNB can send the RRC reconfiguration message to the source relay UE to release the remote UE context at any time after the source gNB sends the RRC reconfiguration message to the remote UE.

Observation 1: Source gNB can send the RRC reconfiguration message to the source relay UE to release the remote UE context at any time after the source gNB sends the RRC reconfiguration message to the remote UE.

Observation 2: Target CU sends RRC reconfiguration conveyed in UE context modification request for relay UE and UE context release request for DU to release remote UE context.

Proposal 1: Take Figure 1 as baseline for inter-gNB indirect-to-direct path switching.

Figure 1 inter-gNB indirect-to-direct path switching

b) Inter-gNB direct-to-indirect path switching

In the last meeting, there are three options for selection of target Relay UE.

Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

In legacy, source gNB is responsible for selecting one target cell and target gNB make the final decision of target cell. Specifically, source gNB provides one target cell in Xn handover request message and a list of candidate target cells on each frequency in candidateCellInfoList in HandoverPreparationInformation.

candidateCellInfoList

A list of the best cells on each frequency for which measurement information was available

When coming to the selection of target relay UE, we can follow the same principle. Source gNB is able to select one target relay UE based on the PC5 measurement information. From RAN3 perspective, we can introduce one target Relay UE L2ID in Xn handover request message.

The Remote UE L2ID also need to be include in Xn handover request message for local ID allocation at target gNB but it is already conveyed in sidelinkUEInformationNR-r16 in HandoverPreparationInformation.

Observation 3: Remote UE L2ID is already included in sidelinkUEInformationNR-r16 in HandoverPreparationInformation in Xn Handover Request message.

Option 2 propose to send a list of candidate target relay UE in Xn Handover Request message without PC5 measurement information. It can reduce handover failure to some extend but it still likely none of target relay UE selected by target gNB. Option 3 may more efficient to reduce handover failure compared with Option 2.

Except reducing handover failure, the proponent of Option 3 also consider that target gNB has the information about RRC status of candidate relay UE. If both RRC_CONNECT relay UE1 and RRC_IDLE relay UE2 has similar measurement quantity, target gNB may select RRC_CONNET relay UE1 to save the time/resource during relay UE2 into RRC_CONNECT. In our view, RRC_CONNECT relay UE has higher priority does not have significant benefit. If the source link is near to RLF and have no time to wait for target relay UE into RRC_CONNECT, then source gNB should select direct path instead of indirect path. The weigh between how urgent the handover is and the performance of target Uu/PC5 link is up to source gNB’s implementation.

Observation 4: RRC status considered in target relay UE selection has no significant benefit. If the source link is near to RLF and source gNB has no time to wait for target relay UE into RRC_CONNECT, source gNB should select direct path instead of indirect path by implementation.

In our view, option 3 is an optimization to reduce handover failure and whether to support option 3 can be discussed in RAN2. Even if RAN2 agree to introduce measurement information of Remote UE, it should be included in HandoverPreparationInformation instead of explicitly include in XnAP.

Proposal 2: RAN3 supports Option 1 and takes Option 3 as an optimization. Whether to support such optimization i.e., source gNB provides the measurement information and candidate target Relay UE in HandoverPreparationInformation can be discussed in RAN2.

Figure 2 inter-gNB direct-to-indirect path switching

Proposal 3: Take Figure 2 as baseline for inter-gNB direct-to-indirect path switching.

c) Intra-gNB indirect-to-indirect path switching

GNB-CU decides switch remote UE to a target relay UE which is camped in the same gNB. GNB-CU triggers UE context setup/modification procedure for target relay UE, and also triggers UE context setup procedure for remote UE. After switching, gNB-CU sends UE context modification to convey RRC reconfiguration for source relay UE to release remote UE context, and also sends UE context release to release remote UE context from DU side.

Figure 3 intra-gNB indirect-to-indirect path switching

Proposal 4: Take Figure 3 as baseline for intra-gNB indirect-to-indirect path switching.

d) Inter-gNB indirect-to-indirect path switching

This procedure of inter-gNB indirect-to-indirect path switching is similar as the inter-gNB direct-to-indirect path switching. The difference is that source gNB should send RRC reconfiguration to source relay UE to release remote UE context after remote UE connects to target relay UE. This RRC reconfiguration message for relay UE can be sent any time after source gNB sends RRC reconfiguration message to remote UE based on gNB implementation.

Figure 4 inter-gNB indirect-to-indirect path switching

Observation 4: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE.

Proposal 5: Take Figure 4 as baseline for inter-gNB indirect-to-indirect path switching.

- Conclusion

Observation 1: Source gNB can send the RRC reconfiguration message to the source relay UE to release the remote UE context at any time after the source gNB sends the RRC reconfiguration message to the remote UE, without the target gNB indication

Observation 2: Target CU sends RRC reconfiguration conveyed in UE context modification request for relay UE and UE context release request for DU.

Proposal 1: Take Figure 1 as baseline for inter-gNB indirect-to-direct path switching.

Observation 3: Remote UE L2ID is already included in sidelinkUEInformationNR-r16 in HandoverPreparationInformation in Xn Handover Request message.

Observation 4: RRC status considered in target relay UE selection has no significant benefit. If the source link is near to RLF and source gNB has no time to wait for target relay UE into RRC_CONNECT, source gNB should select direct path instead of indirect path by implementation.

Proposal 2: RAN3 supports Option 1 and takes Option 3 as an optimization. Whether to support such optimization i.e., source gNB provides the measurement information and candidate target Relay UE in HandoverPreparationInformation can be discussed in RAN2.

Proposal 3: Take Figure 2 as baseline for inter-gNB direct-to-indirect path switching.

Proposal 4: Take Figure 3 as baseline for intra-gNB indirect-to-indirect path switching.

Observation 5: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE.

Proposal 5: Take Figure 4 as baseline for inter-gNB indirect-to-indirect path switching.

- Reference

[1] RAN3#117bis-e, Chair notes.

- Annex for 38.423

============ Start of Change ==============

8.2.1	Handover Preparation

8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

If the Target Relay UE L2ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure target Relay UE for subsequent path switching of Remote UE.

============ Next Change ==============

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Target Relay UE L2ID | O |  | BIT STRING (SIZE(24)) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

============ Next Change ==============

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

SRB-ID

TargetRelayUEL2ID

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

id-TargetRelayUEL2ID

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters							PRESENCE optional }|

{ ID id-MobilityInformation					CRITICALITY ignore	TYPE MobilityInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE			PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore	TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo						CRITICALITY ignore	TYPE QMCConfigInfo								PRESENCE optional }|

{ ID id-FiveGProSeAuthorized				CRITICALITY ignore	TYPE FiveGProSeAuthorized						PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters			CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters				PRESENCE optional },

{ ID id-TargetRelayUEL2ID					CRITICALITY ignore	TYPE TargetRelayUEL2ID							PRESENCE optional }|

...

}

============ Next Change ==============

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

TargetRelayUEL2ID ::=  BIT STRING(SIZE(24))

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

#### -- IEs

--

-- **************************************************************

#### id-ActivatedServedCells

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

id-TargetRelayUEL2ID													ProtocolIE-ID ::= xxx

END

-- ASN1STOP

============ End of Change ==============