3GPP TSG-RAN3 Meeting #118	R3-226486

Toulouse, France, 14 November - 18 November 2022

Agenda Item:	10.2.6

Source:	Ericsson

Title:	RAN3’s position on the network-based on the support of signalling based logged MDT configuration override protection in Rel-18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we reaffirm RAN3’s position on the complexity of network-based solution on the support of signalling based logged MDT override protection in Rel-18.

# 2	Discussion

Signalling based logged MDT protection over management-based MDT configuration has been extensively discussed in the past and the clear agreement was that a UE based solution is much more effective than any RAN3 based method which is far too complex. This agreement has a been captured and liaised to RAN2 already in [1].

These discussions were long and lasted over a number of Rel-17 meetings. It gradually became apparent during those discussions that a network-based solution incurs in high complexity. The main reasons for this can be summarized below:

- For an inactive UE resuming in a new serving RAN, the old serving RAN needs to store the last known logged MDT configuration and forward it to the new serving RAN. Moreover, the remaining logging duration are required to be signalled to the new serving RAN in order to inform it of when the UE would become available for a new configuration. This implies that the old serving RAN needs to maintain a timer running for the UE even when the UE has moved to inactive state

- For Idle UEs that move to connected mode in a new serving RAN, the last serving RAN needs to signal information to the last serving CN (such as the remaining logging duration and/or the time when the logged MDT configuration was activated). The CN needs to maintain a running timer counting the remaining logging duration for the UE. Such information stored in the last serving CN needs to be retrieved by the new serving RAN at the time the UE moves to Connected Mode, so to allow the new serving RAN to understand whether a UE is on a signalling based logged MDT configuration and how long is left before such configuration expires.

  - It is worth noticing that if the UE moves to RRC_Connected in an area served by a different AMF, the old serving AMF will need to signal the logged MDT configuration information to the new serving AMF.

- In case of inter RAT and inter system mobility in connected and Idle mode, coordination is needed between the different RATs/System, so that information about the last active MDT configuration is signalled from source RAT/System to target RAT/System to enable an educated decision on any new logged MDT configuration for the UE. For example, if a UE in RRC_Idle is configured with a logged MDT configuration for NR, and if the UE re-selects in E-UTRAN, the old serving AMF will need to transfer MDT configuration information to the new serving MME, so that if the UE returns to RRC_Connected in the NG-RAN, the MDT configuration information can be signalled from the MME to the new serving AMF and then be provided to the serving NG RAN.

The above network-based solutions impact multiple interfaces and therefore RAN3 concluded they are complex.

The discussions on handling of signalling based vs management based Logged MDT served the purpose of bringing to light that a network based solution would require a large amount of signalling and especially of state maintenance per UE in different parts of the system.

Conclusion 1 	Network based solutions revealed to be of high complexity and cost due to considerable signalling and due to maintenance of UE context information between RAN nodes, across CN nodes and across different systems

Indeed, it is the UE itself that manages the logged MDT process once this is configured by the RAN. The UE keeps track of the duration of the process, of whether the UE camping area allows for collection of logged MDT measurements, of when the logged MDT process should be stopped.

Correspondingly, the simplest possible way of fulfilling the requirement from RAN2 is to let the UE itself to reject a management based logged MDT configuration over a signalling based one.

Leaving the full implementation of the RAN2 requirements to the network would require a very complex UE-[RAN/CN] sync solution as the logging state of each of the UEs that are configured with logged MDT would need to be continuously monitored and tracked not only by the RAN nodes but also by CN nodes across different systems.

Furthermore, such solution would in addition, be weaker than a UE based method as the UE is in charge of handling the logged MDT process, hence the UE is always up to date about whether a logged MDT configuration is active. On the contrary, a network-based solution may be subject to failure due to failure of transferring information across different RATs and systems.

Correspondingly we suggest RAN3 should reaffirm this position by sending a new LS to RAN2:

- It is proposed to send an LS to RAN2 expressing RAN3’s position on the cross-RAT override protection of the signalling based logged MDT configuration

# 3	Conclusion

In this contribution, the following observations and proposals were derived:

Conclusion 1 	Network based solutions revealed to be of high complexity and cost due to considerable signalling and due to maintenance of UE context information between RAN nodes, across CN nodes and across different systems

- It is proposed to send an LS to RAN2 expressing RAN3’s position on the cross-RAT override protection of the signalling based logged MDT configuration

# 4. References

- R3-207176, LS on UE based solution related to Logged MDT, RAN3