3GPP TSG-RAN WG3 Meeting #118-e                                                      	R3-226519

Toulouse, Nov 14th – Nov 18th, 2022

Agenda item:	11.2

Source:	Qualcomm Incorporated

Title:	QoE enhancements for high mobility scenarios

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further on QoE enhancements for high mobility scenarios based on the agreements last meeting.

# Discussion

The following was agreed last meeting:

OAM should have the flexibility to collect QoE only in high mobility scenarios and/or in HSDN cells instead of collecting blindly. FFS whether enhancements are needed to support the requirement

  - QoE enhancements for HSDN cells

We propose to discuss the HSDN cells scenario first i.e., the potential enhancements needed for collecting QoE from only HSDN cells.

Observation 1: OAM knows whether a cell is a HSDN cell or non-HSDN cell

Observation 2: If OAM is interested in collecting QoE from all HSDN cells, currently it needs to provide the list of all HSDN cells in the Area Scope of QMC

In case of m-based QoE, OAM should be able to indicate that it is interested in collecting QoE only from only HSDN cells by providing a “HSDN only” indicator in Area Scope of QMC. This is similar to a discussion we are having currently in Rel-18 SON/MDT for NPN where a “CAG only” indicator can be provided in Area Scope of MDT to collect MDT only from CAG cells.

Proposal 1: In case of m-based QoE, OAM should be able to indicate that it is interested in collecting QoE only from HSDN cells by providing a “HSDN only” indicator  in Area Scope of QMC

Proposal 2: Upon UE mobility outside the area scope (from a HSDN cell to non-HSDN cell), gNB can release the QoE config (if there is no ongoing session) or pause the QoE config to ensure QoE is collected only in HSDN cells

  - QoE enhancements for UEs in high mobility state

Let’s now discuss the potential enhancements needed for collecting QoE from only UEs in high mobility state.

Observation 3: The gNB knows a UE’s mobility state after receiving the mobilityState in RRC Complete messages

Observation 4: OAM doesn’t know a UE’s mobility state directly and it might only know if provided in SON/MDT reports

Proposal 3: If OAM is interested in collecting QoE from only UEs in high mobility state, it can provide a “High Mobility State Only” indicator in m-based QoE configuration sent to gNB

Proposal 4: Upon receiving a “High Mobility State Only” indicator in m-based QoE configuration, a gNB can select only those UEs which are in high mobility state

Proposal 5: Upon UE changing mobility states (from a High Mobility state to Low/Medium mobility state), gNB can release/pause the QoE config to ensure QoE is collected only in high mobility state

# Conclusion

Observation 1: OAM knows whether a cell is a HSDN cell or non-HSDN cell

Observation 2: If OAM is interested in collecting QoE from all HSDN cells, currently it needs to provide the list of all HSDN cells in the Area Scope of QMC

Proposal 1: In case of m-based QoE, OAM should be able to indicate that it is interested in collecting QoE only from HSDN cells by providing a “HSDN only” indicator in Area Scope of QMC

Proposal 2: Upon UE mobility outside the area scope (from a HSDN cell to non-HSDN cell), gNB can release the QoE config (if there is no ongoing session) or pause the QoE config to ensure QoE is collected only in HSDN cells

Observation 3: The gNB knows a UE’s mobility state after receiving the mobilityState in RRC Complete messages

Observation 4: OAM doesn’t know a UE’s mobility state directly and it might only know if provided in SON/MDT reports

Proposal 3: If OAM is interested in collecting QoE from only UEs in high mobility state, it can provide a “High Mobility State Only” indicator in m-based QoE configuration sent to gNB

Proposal 4: Upon receiving a “High Mobility State Only” indicator in m-based QoE configuration, a gNB can select only those UEs which are in high mobility state

Proposal 5: Upon UE changing mobility states (from a High Mobility state to Low/Medium mobility state), gNB can release/pause the QoE config to ensure QoE is collected only in high mobility state

# 4. References