3GPP TSG-RAN3 Meeting #118	R3-226650

14 Nov - 18 Nov 2022,

Toulouse, France

Title:	Draft Reply LS on Time Synchronization Status notification towards UE(s)

Response to:	S2-2209876/R3-226168

Release:	Rel-18

# Work Item:	     FS_5TRS_URLLC

Source:	Qualcomm Incorporated [to be RAN3]

To:	SA2

cc:	RAN2, SA WG3

Contact Person:	Geetha Rajendran

E-mail Address:	geethapr (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on FS_5TRS_URLLC.

RAN3 would like to provide the following feedback on SA2 LS on Key Issue #1 - On 5GS time synchronization status report towards the UE(s)

Questions:

- One method proposes to use the Ciphered SIB approach that is used for broadcast of assistance data for positioning. SA2 would like to additionally get feedback on this from SA3 and RAN3 from security and NGAP impact perspective, respectively.

Answer: It is feasible for NG-RAN to receive Ciphered SIB from AMF for broadcast. This will have NGAP impacts to introduce a new procedure to receive Ciphered content from AMF and broadcast it in NG-RAN.

- SA2 would like to seek RAN3 input regarding the potential impacts in RAN3 specifications (e.g., NGAP, F1-AP) to enable RAN time synchronization status report control procedure

Answer: RAN3 acknowledges that RAN timing synchronization information can be obtained via OAM. However alternate CP solution of receiving the RAN timing synchronization information status is also feasible from RAN3 pov. There could be NGAP impact to report the RAN timing synchronization information status from NG-RAN to AMF when the RAN timing synchronization changes. For non-split architecture, there could be impact to F1AP and E1AP interface to receive the clock quality information at gNB-CU-CP before it is sent to AMF via NGAP. With multiple interface impacts and clock quality triggered reporting of RAN timing synchronization information, RAN3 does not see clear benefits in CP based solution to send the RAN timing synchronization information to AMF.

2. Actions:

To SA2

ACTION: 	RAN3 kindly asks SA2 to take the above information into consideration.

3. Date of Next RAN3 Meeting:

RAN3#119                   Feb 27- Mar 03, 2023                                    F2F

RAN3#119bis-e           April 17 – April 26, 2023                                 E-Meeting