3GPP TSG-RAN WG3 Meeting #118	R3-226675

Electronic Meeting, November 14th – 18th, 2022

Agenda item:	14.2

Source: 	Intel Corporation

Title: 	Discussions for L1/L2 based "intra-DU" mobility (including TPs for L1/L2 Mob for TS 38.401 and TS 38.473)

Document for:	Discussion and Decision

# Introduction

In the last RAN3#117bis-e meeting, RAN3 discussed L1/L2 based inter-cell mobility and agreed the following:

During L1/L2 handover configuration, the gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request procedure, FFS in one message or multiple messages.

The gNB-DU may accept the target cells of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message(s). gNB-DU may accept all or part of the target candidate cells.

gNB-DU initiated L1/L2 handover configuration is not allowed.

FFS on whether the gNB-DU can suggest candidate cells after the gNB-CU initiates the L1/L2 inter-cell mobility configuration. Three Options are left for further discussion: Opt 1. DU cannot suggest any candidate cells. Opt 2. DU suggest candidate cells within the list provided by CU. Opt 3. DU suggest candidate cells outside the list provided by CU.

The UE sends the lower-layer measurement report to the gNB-DU and the gNB-DU triggers UE mobility to a target candidate cell.

FFS on the need and when the gNB-DU indicates the gNB-CU about the initiation of L1/L2 handover command.

FFS For intra-DU L1/L2 mobility, the gNB-CU may use the UE Context Modification procedure to modify or release the prepared cells resources in the gNB-DU (incl. the source cell). Details are pending to RAN2.

The following open issues on user plane handling in intra-DU L1/L2 mobility as well as inter-DU case are raised for further study:

a)	F1-U UL/DL TEID handling as in intra-DU legacy HO.

b)	DDDS on F1-U

c)	E1 impact, such as setup, update or remove resources at gNB-CU-UP

d)	Data forwarding

The following previous agreements for intra-DU case are confirmed to be also applicable for inter-DU case:

1.Both intra- DU and intra-CU inter-DU scenarios are supported for L1/L2 mobility.

2.RAN3 will study the signaling impacts on below use cases following to RAN2 prioritization:

-	Stand alone

-	Carrier Aggregation (Change of PCell)

-	NR-DC (Change of PCell at MN, Change of PScell at SN)

3.RAN3 will aim for a single solution for network signaling design on L1/L2 based inter-cell mobility to support all agreed scenarios. The details of solution are FFS.

5.RAN3 focuses on the network-controlled procedure for L1/L2 based inter-cell mobility.

7.The configuration of candidate target cell(s) for L1/L2 mobility is initiated by the gNB-CU. Details are FFS.

For inter-DU inter-cell mobility, the UE Context Setup procedure is reused for handover configuration.

With the above progress, in our companion paper [1], we have provided several important principles for L1/L2-based mobility network signalling designs:

Proposal 1: For L1/L2 based inter-cell mobility, RAN3 to design overall signalling procedures for intra-DU and intra-CU inter-DU cases honouring the following "intra-CU HO" principles that have been specified from Rel-15: - The procedures should support the cases where security key is updated or retained (as specified in TS 33.501 Section 6.9.2.3.1) - For intra-DU case, the procedure should support the special F1-U UL/DL TEID handling (as specified in TS 38.401 Section 8.2.1.2.), for DU to differentiate which data to be transmitted with old configuration or new configuration within the same DU Proposal 2: For L1/L2 based inter-cell mobility, RAN3 to first focus and complete network signalling design for the scenario having only a single CU-UP entity, after then to extend the solution for the scenario where multiple CU-UPs are involved with the CU-CP. Proposal 3: For L1/L2 based inter-cell mobility, RAN3 to allow the consecutive HO decisions based on a single pre-configuration of candidate cells. Proposal 4: For L1/L2 based inter-cell mobility, with respect to network signalling design for HO execution phase, RAN3 to aim for common signalling designs for intra-DU case and inter-DU case, respectively, that can be applicable for any execution (i.e. regardless of whether initial HO or subsequent HO).

With these principles in mind, this contribution focuses on intra-DU mobility case and proposes signalling designs including DU, CU-CP, and CU-UP for NW preparation phase (for multiple candidate cell configurations) as well as HO execution phase.

# Discussion

## NW preparation for multiple candidate cell configurations

While RAN3 agreed that DU cannot initiate L1/L2 mobility, but we still have FFS on whether DU can suggest candidate cells based on L1 measurements and how to.

Regardless of whether DU suggests candidate cells before or after CU initiates L1/L2 mobility, in general we think that the final decision maker should be on CU-CP side. The suggested candidate cells from DU (if any) should be just for information and the final decision on whether to configure L1/L2 mobility to the UE and which candidate cells should be always by CU-CP.

Proposal 1: For intra-DU L1/L2 mobility, it should be the CU-CP who decides whether to configure L1/L2 mobility to the UE and which candidate cells to prepare for L1/L2 mobility configuration in the NW side, regardless of whether DU suggests candidate cells based on L1 measurements before or after CU initiates L1/L2 mobility.

Moreover, we don't think "suggestion" from DU is essential for this feature to work. Thus we propose to deprioritize DU's candidate cell suggestion.

Proposal 2: RAN3 to deprioritize the discussions of DU's candidate cell suggestion, given this is not essential for this feature to work.

Once CU-CP decides intra-DU L1/L2 mobility and which candidate cells to configure, following the legacy intra-DU HO principle [1][2], CU-UP first needs to retrieve new UL TNL per DRB from CU-UP. The existing New UL TNL required IE in BRR CTXT MOD REQ message can be re-used.

Proposal 3: Once CU-CP decides intra-DU L1/L2 mobility and which candidate cells to configure, following the legacy intra-DU HO principle, CU-UP retrieves new UL TNL per DRB from CU-UP. The existing New UL TNL required IE in BRR CTXT MOD REQ message can be re-used.

Having new UL TEID per each DRB, CU-CP now needs to establish candidate cell configurations with the DU. From NW point of view, this L1/L2-based mobility is like CHO/CPAC in a sense that NW needs to prepare multiple candidate cell configurations and configure them to the UE. The difference lies on the HO execution part where CHO/CPAC is executed based on configured conditions in the UE side, while here in L1/L2-based mobility, HO is explicitly commanded by DU based on L1 measurements.

From this sense, in terms of preparing L1/L2 mobility with DU over F1, it is beneficial to align F1 signalling design like what we already have for CHO/CPAC. During CHO/CPAC, we adopted parallel preparation each for each candidate cell (indicated by SpCell ID IE), where DU is able to accept/reject the request for each candidate cell basis and also able to provide lower-layer configuration separately for each candidate cell. On top of such parallel preparations over F1, CHO/CPAC initiation/replace/cancellation have been also worked out separately for intra-DU and intra-CU inter-DU cases. For L1/L2 mobility, our signalling design doesn't have to unnecessarily restricted to share the same lower layer configuration for multiple candidate cells by using the "single" procedure. Our design should be flexible and full fetched to support possibly different admission results and configurations per each candidate cell basis. From this sense, there is no reason not to follow our L1/L2 mobility design over F1 to that of CHO/CPAC that has been already optimized for multiple candidate cell configurations.

For intra-DU case, the parallel UE Context Modification procedures have been used for CHO/CPAC. For L1/L2-based mobility, we can enhance the UE Context Modification procedure following similarly by "Conditional Intra-DU Mobility Information" IE.

Proposal 4: For intra-DU L1/L2 mobility, in terms of preparing multiple candidate cell configurations with DU over F1, RAN3 to follow the CHO/CPAC approach that has been optimized for handling multiple candidate cell configurations with a DU, i.e.

- For HO preparation with DU, adopt parallel preparation signalling (like CHO/CPAC) each for each candidate cell (indicated by SpCell ID IE) via the UE Context Modification procedure, where DU is able to accept/reject the request for each candidate cell basis and also able to provide lower-layer configuration separately for each candidate cell.

- Our signalling design doesn't have to unnecessarily restricted to share the same lower layer configuration for multiple candidate cells by using the "single" procedure. Our design should be flexible and full fetched to support possibly different admission results and configurations per each candidate cell basis.

- Add an IE similar to "Conditional Intra-DU Mobility Information" IE in the UE CONTEXT MODIFICATION REQUEST message that handles L1/L2 mobility initiation/replace/cancellation triggered by CU-CP (like CHO/CPAC).

- Extend the Candidate Cells To Be Cancelled List IE in the UE CONTEXT MODIFICATION REQURED message to handle cancellation request triggered by DU (like CHO/CPAC).

While doing so, new UL TEID per DRB (retrieved from CU-UP by CU-CP) has to be delivered to the DU as well so that DU can meet the legacy intra-DU HO principle [1][2] and use it when "intra-DU" L1/L2 mobility is executed. This can be provided by DRB to Be Modified List in the UE CONTEXT MODIFICATION REQUEST message per DRB basis, where in, the UL UP TNL Information to be setup List IE is mandatorily included for each DRB to be modified. Only the first item on this list can be used to provide new UL TEID.

Once DU receives such UE CONTEXT MODIFICATION REQUEST message, from the current signalling design, DU shall also provide the corresponding new DL TEID for each DRB requested to be modified to the CU-CP as well (Note that the DL UP TNL Information to be setup List IE is also mandatory in DRB Modified List of the UE CONTEXT MODIFICATION RESPONSE message.) Then, thanks to parallel preparations for each candidate cell, CU-CP can obtain new DL TEID for each DRB for each candidate cell that has been successfully prepared with the DU, which later can be delivered to CU-UP to meet the legacy intra-DU principle [1][2] and use it when "intra-DU" L1/L2 mobility is executed.

Proposal 5: For intra-DU L1/L2 mobility, when CU-CP initiates parallel UE Context Modification procedures for each candidate cell, CU-CP includes new UL TEID (retrieved from CU-UP by CU-CP) for each parallel request via DRB to Be Modified List IE, so that DU can meet the legacy intra-DU HO principle and use it when "intra-DU" L1/L2 mobility is executed. This is already possible by the existing signalling, so no new signalling support is necessary.

Proposal 6: In return, DU provides the corresponding new DL TEID for each DRB requested to be modified to the CU-CP, so that CU-CP can obtain new DL TEID for each DRB for each candidate cell that has been successfully prepared with the DU, which later can be delivered to CU-UP to meet the legacy intra-DU HO principle and use it when "intra-DU" L1/L2 mobility is executed. This is already possible by the existing signalling, so no new signalling support is necessary.

The above proposals for NW preparation phase for multiple candidate cell configurations with DU and CU-UP can be summarized by the following figure:

Figure 1: The proposed NW preparation phase during intra-DU L1/L2 mobility, involving DU and CU-UP

## HO execution

Once configured to the UE, based on L1 measurements, DU will decide HO to one of candidate cells and send L1/2 HO CMD to the UE to initiate mobility toward the selected cell.

Then the next question is how to design signalling between DU, CU-CP, and CU-UP so that we can minimize interruption as well as meet the legacy intra-DU HO principles [1][2] as much as possible.

In the legacy intra-DU HO, two DDDS signalling from DU has been specified. Once DU stopped transmission before configuring HO CMD to the UE, the first DDDS was sent over the existing TNL to inform not successfully transmitted/delivered PDUs. Once the UE accesses to the target cell, initial DDDS was sent over the new TNL (identified by new UL/DL TEIDs) to inform CU-UP that the UE has accessed the target cell and CU-UP can start delivering packets.

And when it comes to Rel-17 CHO/CPAC, we have additionally adopted "ACCESS SUCCESS" message for DU to indicate the cell accessed by the UE promptly to CU-CP before receiving RRC Reconfiguration Complete message. This was not needed for the legacy intra-CU HO because anyway CU-CP knows which cell the UE is handed over to. But in case of CHO/CPAC, this was necessary because CU-CP needs to know which cell (among candidate cells pre-configured) the UE has accessed as early as possible to continue subsequent mobility procedure and thus to reduce latency.

Also, especially when there is only one bearer context maintained in the CU-UP (supported only for intra-CU CHO/CPAC case; for inter-CU CHO/CPAC, parallel bearer contexts over E1 for the same UE is also possible by implementation as captured in TS 38.401 [2]), there are mainly two reasons why CU-CP needs to know the accessed cell early:

- When security key is not retained, key update depends on the chosen cells' PCI [3], which cannot be updated/pre-configured to CU-UP until CU-CP knows which cell the UE accessed to. ACCESS SUCCESS was necessary for CU-CP to provide the right security key and initiate DL delivery as early as possible.

- There are multiple DL TEIDs provided by DU for each candidate cell during CHO/CPAC preparation phase, which cannot be pre-configured to the CU-UP until the UE accessed. ACCESS SUCCESS was necessary for CU-CP to provide the right DL TEIDs (and SDAP/PDCP configurations) to the CU-UP corresponding to the cell that the UE has accessed as early as possible.

Considering the above mechanisms that has been devised so far, we think the right approach is to incorporate all the above DDDS + ACCESS SUCCESS mechanisms.

Proposal 7: For intra-DU L1/L2 handover execution, RAN3 to adopt DDDS signalling and ACCESS SUCCESS message that have been devised so far to minimize latency during the legacy HO or CHO/CPAC, i.e. DU sends two DDDS to CU-UP and ACCESS SUCCESS message to CU-CP as soon as DU confirms the UE's success access on the target cell.

Proposal 8: Following the legacy, the first DDDS is sent to CU-UP to inform not successfully transmitted/delivered PDUs using the existing F1-U. The second (which is initial DDDS) is sent to CU-UP to inform the UE's successful access using new F1-U with DL TEID corresponding to the chosen cell by DU and new UL TEID provided by CU-CP.

Proposal 9: Following CHO/CPAC, ACCESS SUCCESS message includes the cell the UE successfully accessed, which may further trigger the E1AP Bearer Context Modification procedure to provide to CU-UP the right DL TEIDs for new F1-Us, updated SDAP/PDCP configurations (if needed), and updated security key (if needed).

Having these basic mechanisms, we also think that it is beneficial to allow DU to trigger two DDDSs and ACCESS SUCCESS before the UE successfully accesses to the target cell. That is, when DU decides receives successful ACK for L1/2 HO CMD sent to the UE.

For L1/L2 mobility, the candidate cells configuration has been configured to the UE before, so the purpose of this L1/2 HO CMD would be nothing more than to simply command the UE to handover to which cell among pre-configured. The ACK may not even be necessary for DU to confirm. Moreover, once DU confirms, we think it is unlikely that the UE fails the subsequent procedures ‒ RACH to the target cell (if configured) or sending RRC Reconfiguration Complete, where the former (RACH) could be skipped and the latter (RRC Reconfiguration Complete) may later be decided unnecessary pending RAN2 progress. Continuing the mobility procedure before the optional RACH or RRC Reconfiguration Complete would work fine for this L1/L2 mobility.

From this sense, for the sake of minimizing latency, we think we should not close door for DU to trigger two DDDSs and ACCESS SUCCESS message when DU receives successful ACK for L1/2 HO CMD sent to the UE. This can be left up to DU implementation.

Proposal 10: For the sake of minimizing latency, RAN3 to allow DU to trigger two DDDSs and ACCESS SUCCESS message when DU receives successful ACK for L1/2 HO CMD sent to the UE. This can be left up to DU implementation.

The proposed signalling flows for HO execution (e.g. to cell A) with DU and CU-UP can be summarized by the following figure:

Figure 2: The proposed HO execution phase during intra-DU L1/L2 mobility, involving DU and CU-UP

## Subsequent HO executions

Once the initial HO is performed, DU can further trigger HO (based on pre-configured candidate cells and L1 measurements) to any of candidate cells. For example, once the UE is handed over to cell A, it can further be handed over to cell B or back to the original cell.

Figure 3: The proposed subsequent HO execution phase during intra-DU L1/L2 mobility, involving DU and CU-UP

As proposed in our companion paper [1], network signalling design for HO execution should be aligned regardless of initial HO execution or subsequent HO executions. The only difference is on the usage of UL TEID for subsequent handovers.

To meet the legacy intra-DU principle [1][2], UL TEID used for new cell B HO should be different to what has been used for cell A HO before. Previously for cell A HO, "new" UL TEIDs (provided by CU-UP based on request from CU-CP) was used by DU for initial DDDS as well as by CU-UP for DL delivery. Given DU knows that another HO (to cell B) is being executed, all we need to do is simply to toggle between old UL TEIDs and new UL TEIDs at DU to use to meet the legacy intra-DU principle and let CU-UP use the UL TEID of the initial DDDS sent from DU for subsequent DL delivery.

Proposal 11: For intra-DU L1/L2 mobility, for subsequent HO executions, RAN3 to adopt the same network signalling design as for initial HO execution, with the difference that DU toggles between old UL TEIDs and new UL TEIDs for initial DDDS by DU to let CU-UP use the UL TEID of initial DDDS sent from DU for subsequent DL delivery, in order to meet the legacy intra-DU principle of the special F1-U UL/DL TEID handling (as specified in TS 38.401 Section 8.2.1.2.)

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

NW preparation for multiple candidate cell configurations

Proposal 1: For intra-DU L1/L2 mobility, it should be the CU-CP who decides whether to configure L1/L2 mobility to the UE and which candidate cells to prepare for L1/L2 mobility configuration in the NW side, regardless of whether DU suggests candidate cells based on L1 measurements before or after CU initiates L1/L2 mobility.

Proposal 2: RAN3 to deprioritize the discussions of DU's candidate cell suggestion, given this is not essential for this feature to work.

Proposal 3: Once CU-CP decides intra-DU L1/L2 mobility and which candidate cells to configure, following the legacy intra-DU HO principle, CU-UP retrieves new UL TNL per DRB from CU-UP. The existing New UL TNL required IE in BRR CTXT MOD REQ message can be re-used.

Proposal 4: For intra-DU L1/L2 mobility, in terms of preparing multiple candidate cell configurations with DU over F1, RAN3 to follow the CHO/CPAC approach that has been optimized for handling multiple candidate cell configurations with a DU, i.e.

- For HO preparation with DU, adopt parallel preparation signalling (like CHO/CPAC) each for each candidate cell (indicated by SpCell ID IE) via the UE Context Modification procedure, where DU is able to accept/reject the request for each candidate cell basis and also able to provide lower-layer configuration separately for each candidate cell.

- Our signalling design doesn't have to unnecessarily restricted to share the same lower layer configuration for multiple candidate cells by using the "single" procedure. Our design should be flexible and full fetched to support possibly different admission results and configurations per each candidate cell basis.

- Add an IE similar to "Conditional Intra-DU Mobility Information" IE in the UE CONTEXT MODIFICATION REQUEST message that handles L1/L2 mobility initiation/replace/cancellation triggered by CU-CP (like CHO/CPAC).

- Extend the Candidate Cells To Be Cancelled List IE in the UE CONTEXT MODIFICATION REQURED message to handle cancellation request triggered by DU (like CHO/CPAC).

Proposal 5: For intra-DU L1/L2 mobility, when CU-CP initiates parallel UE Context Modification procedures for each candidate cell, CU-CP includes new UL TEID (retrieved from CU-UP by CU-CP) for each parallel request via DRB to Be Modified List IE, so that DU can meet the legacy intra-DU HO principle and use it when "intra-DU" L1/L2 mobility is executed. This is already possible by the existing signalling, so no new signalling support is necessary.

Proposal 6: In return, DU provides the corresponding new DL TEID for each DRB requested to be modified to the CU-CP, so that CU-CP can obtain new DL TEID for each DRB for each candidate cell that has been successfully prepared with the DU, which later can be delivered to CU-UP to meet the legacy intra-DU HO principle and use it when "intra-DU" L1/L2 mobility is executed. This is already possible by the existing signalling, so no new signalling support is necessary.

HO execution

Proposal 7: For intra-DU L1/L2 handover execution, RAN3 to adopt DDDS signalling and ACCESS SUCCESS message that have been devised so far to minimize latency during the legacy HO or CHO/CPAC, i.e. DU sends two DDDS to CU-UP and ACCESS SUCCESS message to CU-CP as soon as DU confirms the UE's success access on the target cell.

Proposal 8: Following the legacy, the first DDDS is sent to CU-UP to inform not successfully transmitted/delivered PDUs using the existing F1-U. The second (which is initial DDDS) is sent to CU-UP to inform the UE's successful access using new F1-U with DL TEID corresponding to the chosen cell by DU and new UL TEID provided by CU-CP.

Proposal 9: Following CHO/CPAC, ACCESS SUCCESS message includes the cell the UE successfully accessed, which may further trigger the E1AP Bearer Context Modification procedure to provide to CU-UP the right DL TEIDs for new F1-Us, updated SDAP/PDCP configurations (if needed), and updated security key (if needed).

Proposal 10: For the sake of minimizing latency, RAN3 to allow DU to trigger two DDDSs and ACCESS SUCCESS message when DU receives successful ACK for L1/2 HO CMD sent to the UE. This can be left up to DU implementation.

Subsequent HO executions

Proposal 11: For intra-DU L1/L2 mobility, for subsequent HO executions, RAN3 to adopt the same network signalling design as for initial HO execution, with the difference that DU toggles between old UL TEIDs and new UL TEIDs for initial DDDS by DU to let CU-UP use the UL TEID of initial DDDS sent from DU for subsequent DL delivery, in order to meet the legacy intra-DU principle of the special F1-U UL/DL TEID handling (as specified in TS 38.401 Section 8.2.1.2.)

Based on the above proposals, the overall call flows proposed for intra-DU L1/L2 mobility is as follows.

Figure 4: Overall call flow for intra-DU L1/L2 mobility, involving DU and CU-UP

The corresponding TPs for TS 38.401 and TS 38.473 can also be found in Section 5 and Section 6, respectively.

# Reference

[1] R3-226674, "General principles for L1/L2 based inter-cell mobility", Intel Corporation

[2] TS 38.401, NG-RAN; Architecture description, RAN3

[3] TS 33.501, Security architecture and procedures for 5G system, SA3

# TP for NR feMob2 BL CR for TS 38.401

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.2.1	Intra-NR Mobility

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.2	Intra-gNB-DU handover

This procedure is used for the case that the UE moves from one cell to another cell within the same gNB-DU or for the case that intra-cell handover is performed during NR operation, and supported by the UE Context Modification (gNB-CU initiated) procedure as specified in TS 38.473 [4]. When the intra-gNB-DU handover is performed (either inter-cell or intra-cell) or the intra-gNB-DU L1/L2 based handover is prepared, the gNB-CU provides new UL GTP TEID to the gNB-DU and the gNB-DU provides new DL GTP TEID to the gNB-CU. During HO, the gNB-DU shall continue sending UL PDCP PDUs to the gNB-CU using the previous UL GTP TEID until it re-establishes the RLC, and after then start sending using the new UL GTP TEID. The gNB-CU shall continue sending DL PDCP PDUs to the gNB-DU using the previous DL GTP TEID until it performs PDCP re-establishment or PDCP data recovery, and after then start sending using the new DL GTP TEID.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.X	Intra-gNB-DU L1/L2 based inter-cell Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based inter-cell mobility. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based inter-cell mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 based inter-cell Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU. The UE CONTEXT MODIFICATION REQUST message is sent for each candidate cell.

- If the request for configuring L1/L2 inter-cell mobility is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the accepted target candidate cell. The response message is sent for each requested candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the L1/L2 inter-cell mobility command to the UE.

- FFS: How the gNB-DU detects the UE access to the target cell is up to RAN2.

- The gNB-DU sends an initial Downlink Data Delivery Status frame to inform the gNB-CU. The gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted, are sent from the gNB-CU to the gNB-DU. The gNB-DU also indicates the gNB-CU about the UE successful access to the target cell by Access Success message. The target cell ID is included.

NOTE:	The step 12 may happen as soon as the gNB-DU receives successful ACK for L1/2 inter-cell mobility command sent to the UE.

- FFS: The gNB-CU may send the UE Context Modification message to the gNB-DU to release the resources of prepared cells.

- FFS: The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

# TP for NR feMob2 BL CR for TS 38.473

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Estimated Arrival Probability IE is contained in the Conditional Intra-DU Mobility Information IE or contained in the L1L2 Intra-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the L1L2 Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the L1L2 Mobility Trigger is set to "L1L2HO-initiation", the gNB-DU shall consider that the request concerns an intra-DU L1/L2 based handover for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the L1L2 Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the L1L2 Mobility Trigger is set to "L1L2HO-replace", the gNB-DU shall replace the existing prepared conditional mobility identified by the gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the L1L2 Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the L1L2 Mobility Trigger is set to "L1L2HO-cancel", the gNB-DU shall consider that the gNB-CU is about to remove any reference to, and release any resources previously reserved for the candidate cells associated to the UE-associated signalling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE. If the Candidate Cells To Be Cancelled List IE is also included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that only the resources reserved for the cells identified by the included NR CGIs are about to be released by the gNB-CU.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. If the Conditional Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "CHO-initiation", or the L1L2 Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "L1L2HO-initiation", the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

If the Conditional Intra-DU Mobility Information IE was included and set to "CHO-initiation" or "CHO-replace" but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, or if the L1L2 Intra-DU Mobility Information IE was included and set to "L1L2HO-initiation" or "L1L2HO-replace" but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells.

In case of "CHO-replace" or "L1L2HO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT MODIFICATION REQUEST message was not prepared using the same UE-associated signalling connection, the gNB-DU shall ignore this candidate cell.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.5	UE Context Modification Required (gNB-DU initiated)

#### 8.3.5.1	General

The purpose of the UE Context Modification Required procedure is to modify the established UE Context, e.g., modifying and releasing radio bearer resources, or sidelink radio bearer resources or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change or intra-DU L1/L2 based mobility. The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall consider that only the resources reserved for the candidate cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-CU UE F1AP ID IE are about to be released by the gNB-DU.

If the PC5 RLC Channel Required to be Modified List IE or the PC5 RLC Channel Required to be Released List IE is included in the UE CONTEXT MODIFICATION REQUIRED message and the F1AP-IDs is associated with a U2N Relay UE, the PC5 RLC Channel Required to be Modified List IE or the PC5 RLC Channel Required to be Released List shall include the Remote UE Local ID and correspondingly, the PC5 RLC Channel Modified Item IEs in the UE CONTEXT MODIFICATION CONFIRM message shall include the Remote UE Local ID IE.

If the UE Multicast MRB Required to Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUIRED message containing for an MRB the MRB type reconfiguration IE set to "true" the gNB-CU shall take the MRB Reconfigured RLC mode IE into account to reconfigure the UE and to decide whether to request a PDCP status report as specified in TS 38.300 [6] and include the MBS PTP Retransmission Tunnel Required IE in the UE Multicast MRB Confirmed to Be Modified Item IEs IE.

If the L1L2 Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall consider that only the resources reserved for the candidate cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-CU UE F1AP ID IE are about to be released by the gNB-DU.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.5.3	Abnormal Conditions

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE or in the L1L2 Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUIRED message were not prepared using the same UE-associated signaling connection, the gNB-CU shall ignore those non-associated candidate cells.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.8	Access Success

#### 8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or L1/L2 based mobility. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Conditional handover or conditional PSCell addition or conditional PSCell change

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO preparations or conditional PSCell addition or conditional PSCell change preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled.

L1/L2 based mobility

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU.

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| L1L2 Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >L1L2 Mobility Trigger | M |  | ENUMERATED (L1L2HO-initiation, L1L2HO-replace, L1L2HO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifL1L2HOcancel | 0 .. <maxnoofCellsinL1L2intraDUmobility> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofCellsinL1L2intraDUmobility | Maximum no. cells that can be prepared for a L1/L2 intra-DU mobility. Value is 8 (FFS). |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |
| ifL1L2HOcancel | This IE may be present if the L1L2 Mobility Trigger IE is present and set to "L1L2HO-cancel". |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.10	UE CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| UE Multicast MRB Required to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Required to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| L1L2 Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinL1L2intraDUmobility> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 32. |
| maxnoofCellsinL1L2intraDUmobility | Maximum no. cells that can be prepared for a L1/L2 intra-DU mobility. Value is 8 (FFS). |

| Condition | Explanation |
| --- | --- |
| ifMRBTypeReconf | This IE shall be present if the MRB Type Reconfiguration IE is present. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or L1/L2 based mobility.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |