3GPP TSG-RAN WG3 #118                                                                                 R3-226863

France, 14th - 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0270 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38.401 for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung, Huawei, CMCC, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Remote UE initial access procedure, the gNB-DU does not know bear mapping for remote UE’s SRB0/1 before UE context setup procedure for remote UE. As a result, the gNB-DU does not know how to transfer the RRC setup for remote UE (step 14 in TS38.401 8.19.1). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include bearer mapping related description in step 13 in remote UE initial access procedure. Impact Analysis: This CR is backward compatible. This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SL relay function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remote UE initial access cannot be support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.473 CR1077 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1 Editorial changes and add co-sign company. |  |  |  |  |  |  |  |  |

============ Start of Change ==============

8.19.1	Remote UE initial access

The signalling flow for Remote UE Initial access is shown in Figure 8.19.1-1.

Figure 8.19.1-1: Remote UE Initial Access procedure

1.	The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 Relay RLC Channel.

3.	The U2N Relay UE withhold the received RRC message and send the SidelinkUEInformation message to the gNB-DU. Before that, if the U2N Relay UE is in RRC_IDLE/RRC_INACTIVE status, it should trigger the RRC establishment/resume procedure to enter RRC_CONNECTED status and gNB may configure the U2N Relay UE with Uu Relay RLC channel(s) for relaying of U2N Remote UE’s SRB0/1.

4.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the SidelinkUEInformation message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

5.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0/1 if not configured yet.

6.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Relay UE to gNB-CU.

7.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0/1.

8.	The gNB-DU sends the RRCReconfiguration message to the U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel configuration and bearer mapping for relaying of U2N Remote UE’s SRB0/1.

9.	The U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

11.	After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header

12.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE , the gNB-DU UE F1AP ID of the U2N Relay UE and the container of SL-PHY-MAC-RLC-Config for at least the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message .

13.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1. The DL RRC MESSAGE TRANSFER message shall also contain bearer mapping for relaying of U2N Remote UE’s SRB0.

14.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N Relay UE.

15.	The gNB-CU configures the U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of SRB1 towards gNB-DU if not configured yet.

NOTE 1: This step may be performed earlier, e.g., via steps 5~8.

16.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N Relay UE.

17.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

18.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

19.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

20.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

21.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via U2N Relay UE.

22.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

23.	The U2N Remote UE responds with the SecurityModeComplete message.

24.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

25.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N Relay UE.

27.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N Relay UE.

28.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

29.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

30.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Relay RLC channels for the U2N Relay UE for the relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N Relay UE.

NOTE 2: This step may be performed earlier.

============ End of Change ==============