3GPP TSG-RAN3 Meeting #119bis-e                                                             R3-231186

# E-Meeting : April 7th – 27th , 2023                                               Revision of R3-230082

Agenda Item:	20.2

Source: 	Qualcomm Incorporated

Title: 	Signaling enhancements to enable MT-SDT for RRC_INACTIVE UEs.

WID:                           Mobile Terminated-Small Data Transmission (MT-SDT) for NR (NR_MT_SDT)

Document for:	Discussion/Decision

# Introduction

Following are the agreements from RAN3#119.

- MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling.

- Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator.

- When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message.

- The encoding and the name of MT-SDT information IE is FFS.

- The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

- Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message.

- FFS on F1AP MT-SDT information

- FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

- FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication)

- FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message.

- FFS: whether receiving gNB-CU or receiving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receiving gNB to make the final decision on triggering MT-SDT Uu paging.

- FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

This contribution discusses various enhancements to support MT-SDT for R18 NR UEs in RRC_INACTIVE state.

# Enhancements to support MT-SDT for RRC_INACTIVE state UEs

## E1-AP, F1-AP and Xn-AP enhancements for MT-SDT trigger and decision making unit within gNB.

Similar to R17 MO-SDT, for the case of R18 MT-SDT, gNB can trigger MT-SDT for DL data or signaling transmission when MT data/signaling is available for MT SDT configured radio bearers and DL data volume is less than or equal certain threshold value. MT-SDT data threshold value can be for CU-CP based on implementation or OAM approach.

A typical MT-SDT call flow for DL data is shown below.

Call Flow 1 : Typical MT-SDT call flow for DL data.

During E1 AP Bearer Context setup/modification procedures, CU-CP has to provide SDT indication per DRB level to CU-UP. During RAN#121, following was agreed.

RBs configured for SDT are common for MO-SDT and MT-SDT

RAN2 agreed RBs configured for SDT are common for MO-SDT and MT-SDT

RAN2 Agreement means, each DRB can be configured to support both MO-SDT and MT-SDT instead of each DRB configured with either MO-SDT or MT-SDT or both. In existing E1-AP Bearer Context Setup/modification signaling, there is an existing SDT indication is already available per DRB level and the same SDT Indicator can be used for both MO-SDT and MT-SDT indication.

Reuse existing SDT indicator per DRB ID within E1 BEARER CONTEXT SETUP and MODIFICATION Signaling procedures for both MO-SDT and MT-SDT indication.

As shown in above MT-SDT call flow 1, when UE is in RRC_INACTIVE state and DL data arrives at CU-UP, it can provide notification to CU-CP about the DL data volume per DRB and optional MT-SDT flag if SDT is configured for radio bearers. CU-CP is expected to trigger DL-SDT only when available DL user plane data for transmission is less than or equal to CU-CP configured MT-SDT data threshold and all data belongs to the MT-SDT radio bearers or if there is DL Upper layer signalling is pending for transmission at CU-CP. In order to trigger MT-SDT, CU-CP has to send F1 Paging Message to DU and DU will send RAN Paging Message to UE.

In order to indicate to UE about the RRC paging message is intended for MT-SDT, RAN2#121 already agreed to include MT-SDT cause in RRC Paging Message. This means that CU-CP has to provide MT-SDT indication as part of F1 Paging Message.

Within given dis-aggregated gNB, CU-CP knows DL data volume, which bearers are configured to support MT-SDT, UE radio capabilities are known. In last meeting some companies were arguing to leave the final MT-SDT decision to DU. But DU does not have any additional information than CU-CP and argument of DU resource unavailability to trigger MT-SDT is not sufficient because MT-SDT is not intended to serve data volume and resources should not be concern. If DU uses normal paging for DL data delivery, requires UE to transition into RRC_CONNECTED state and it requires additional signalling and more radio resources. DU does not know any radio conditions of RRC_INACTIVE UE because UE does not transmit any type UL control channels or physical signaling in RRC_INACTIVE state.

CU-CP has all information required to make MT-SDT decision and trigger MT-SDT than DU.

Once RRC_INACTIVE UE received RRC Paging Message including MT-SDT indicator, UE is expected to respond to paging message either by using 2 step or 4 step RACH based or CG based configuration resources. If there is MO data is available for MO-SDT configured radio bearers, UE is expected to evaluate MO SDT trigger criteria and respond to incoming paging or UE may only evaluate MT-SDT trigger procedures. UE is expected to resume RRC connection procedure by transmitting RRC Resume Request (either by RA-SDT or CG based procedure) and it is upto UE implementation whether to use MO-SDT or MT-SDT Resume cause depending on whether MO-SDT conditions are satisfied or not.

Upon receipt of RRC Resume Request message, CU-CP triggers F1-AP UE context setup procedure as shown in call flow1 to create UE context in DU and F1-U tunnels will be established between the DU and CU-UP. CU-UP is expected to deliver DL data to DU (via F1-U Tunnels). In Msg 4, DU indicates contention resolution and also delivers DL data as MT-SDT data to UE. If there is additional data available at both DL and UL, data will be exchanged between UE and gNB. DL data can be scheduled by using dynamic grants and UL data can be transmitted by UE using either CG or dynamic UL grants mechanism. Once data transfer is completed, CU-CP is expected to send RRC Release message including suspend configuration to UE. This causes UE to transition back into the RRC_INACTIVE state and DU is going to release UE context.

For RAN Paging, CU-CP has to send RAN Paging via Xn interface to neighbour gNBs and Xn Paging Message includes MT-SDT indication and DL data volume so that neighbour gNB can make its own decision whether to trigger MT-SDT paging or normal paging.

Based on above discussion, we have following proposals

CU-UP indicates to CU-CP about DL data volume, MT-SDT indication per DRB in E1: DL DATA NOTIFICATION MESSAGE.

gNB CU-CP makes the decision to trigger MT-SDT depending on implementation based MT-SDT data threshold configuration and DU follows CU-CP decision for MT-SDT Paging Indication in RRC Paging Message.

In Xn Paging Message, anchor gNB CU-CP sends MT-SDT Indication and DL data volume  to neighboring gNBs within RNA.

In F1 Paging Message, CU-CP sends MT-SDT Indication and DL data volume to DUs.

## Xn based UE context retrieval enhancements for MT-SDT

In response to receiving RRC paging message including MT-SDT flag, UE will respond to MT-SDT paging message by performing RRC Resume procedure either by using the normal RACH or RA-SDT resources or CG-SDT configuration. UE will send RRC Resume Request message including either MT-SDT or MO-SDT based on UE implementation choice as resume cause. It is possible that UE may perform resume procedure in non-anchor gNB (new serving gNB) and serving gNB CU-CP will initiate UE context retrieval procedure (based on I-RNTI received from UE) by sending Xn UE Context Retrieve Message to Anchor gNB CU-CP. For both MO-SDT and MT-SDT, Anchor gNB handling of UE context transfer is same and probably there is no need to have separate flag for MO-SDT and MT-SDT in Xn UE CONTEXT RETRIEVE Procedure.

During Xn-AP UE CONTEXT RETRIEVE Procedure, reuse existing SDT Indication for both MT-SDT and MO-SDT procedures. If Anchor gNB has DL available for transmission, it forwards DL data towards serving gNB.

Similar to MO-SDT related UE context transfer between serving gNB and Anchor gNB , for MT-SDT also it is possible to have UE context retrieval can be with anchor relocation and without anchor relocation. Call flow 2 and Call flow 3 in this section describes typical MT-SDT related UE context transfer procedures with and without Anchor relocation by making use of existing MO-SDT context transfer procedures.

Call Flow 2 : MT-SDT UE context retrieval procedure with Anchor relocation

Call Flow 3 : MT-SDT UE context retrieval procedure without anchor relocation

RAN3 need to discuss above stage-2 call flows for MT-SDT UE context retrieval.

We propose,

RAN3 to discuss stage-2 call flows for i) MT-SDT with anchor relocation and ii) MT-SDT without anchor relocation.

## Interaction between MT-SDT procedure and Non-SDT data arrival

In R17, MO-SDT is configurable to UE per SRB and DRB level as part of the RRC Release Message. MO data/signalling related to MO-SDT radio bearer will be subjected to MO-SDT procedure, for radio bearers not configured as MO-SDT, data/signalling can’t be transmitted using MO-SDT procedure. In such cases, UE is expected to transition to RRC_CONNECTED state for data transfer. During the ongoing R17 MO-SDT procedure, if non-SDT data is received in DL or UL, last serving gNB terminates current ongoing MO-SDT and triggers UE to transition into RRC_INACTIVE state by using the RRC Release Message including the suspend configuration.

In Rel-17, if a gNB receives DL non-SDT data/signaling, or receives UAI with non-SDT data indication from the UE, the last serving gNB (in case of no anchor relocation) terminates the ongoing MO-SDT and moves the UE to RRC_INACTIVE state by sending the RRCRelease message

But data/signaling received on non-SDT bearers may have higher priority than ongoing MT-SDT radio bearers small data. In such scenarios, it is reasonable to trigger UE to transition into RRC_CONNECTED state as soon as possible instead of R17 procedure to release UE into RRC_INACTIVE state. There are different options available to trigger UE into RRC_CONNECTED state when non-SDT data/signalling is available and RAN3 has to discuss which option to be adopted.

Data/signaling on non-SDT bearers might be high priority, therefore it is beneficial to move the UE into RRC_CONNECTED upon the arrival of DL non-SDT data or signaling (instead of moving it into RRC_INACTIVE) or provide a mechanism to speed up the resume from RRC_INACTIVE if UE is put into RRC_INACTIVE

Thus, we propose

RAN3 to discuss mechanism to handle DL non-SDT data arrival during MT-SDT for the case of partial UE context transfer. Potential solutions include:

- Option 1: 	Upon detecting DL non-SDT data/signaling arrival, anchor gNB makes the decision for  complete UE context relocation to serving gNB (earlier it did just partial context relocation) and serving gNB triggers UE transition into RRC_Connected state by sending RRC Resume message. Anchor gNB also sends an indication to the receiving gNB for DL non-SDT data arrival.

- Option 2: 	Anchor gNB sends an indication to the serving gNB for DL non-SDT data arrival and serving gNB makes the decision for context relocation and move to RRC_Connected state. Detailed signalling FFS.

- Option 3: 	Anchor gNB sends non-SDT data/signalling arrival indication to serving gNB and serving gNB decides to send RRC Release message to UE including CFRA resources to UE (i.e., CFRA resources to UE to speed up Resumption to Connected state)

# Summary

Based on the discussion, we have following observations:

Observation 1.	RAN2 agreed RBs configured for SDT are common for MO-SDT and MT-SDT

Observation 2.	CU-CP has all information required to make MT-SDT decision and trigger MT-SDT than DU.

Observation 3.	In Rel-17, if a gNB receives DL non-SDT data/signaling, or receives UAI with non-SDT data indication from the UE, the last serving gNB (in case of no anchor relocation) terminates the ongoing MO-SDT and moves the UE to RRC_INACTIVE state by sending the RRCRelease message

Observation 4.	Data/signaling on non-SDT bearers might be high priority, therefore it is beneficial to move the UE into RRC_CONNECTED upon the arrival of DL non-SDT data or signaling (instead of moving it into RRC_INACTIVE) or provide a mechanism to speed up the resume from RRC_INACTIVE if UE is put into RRC_INACTIVE

Based on these observations and discussion above, we have the following proposals:

Proposal 1.	Reuse existing SDT indicator per DRB ID within E1 BEARER CONTEXT SETUP and MODIFICATION Signaling procedures for both MO-SDT and MT-SDT indication.

Proposal 2.	CU-UP indicates to CU-CP about DL data volume, MT-SDT indication per DRB in E1: DL DATA NOTIFICATION MESSAGE.

Proposal 3.	gNB CU-CP makes the decision to trigger MT-SDT depending on implementation based MT-SDT data threshold configuration and DU follows CU-CP decision for MT-SDT Paging Indication in RRC Paging Message.

Proposal 4.	In Xn Paging Message, anchor gNB CU-CP sends MT-SDT Indication and DL data volume  to neighboring gNBs within RNA.

Proposal 5.	In F1 Paging Message, CU-CP sends MT-SDT Indication and DL data volume to DUs.

Proposal 6.	During Xn-AP UE CONTEXT RETRIEVE Procedure, reuse existing SDT Indication for both MT-SDT and MO-SDT procedures. If Anchor gNB has DL available for transmission, it forwards DL data towards serving gNB.

Proposal 7.	RAN3 to discuss stage-2 call flows for i) MT-SDT with anchor relocation and ii) MT-SDT without anchor relocation.

Proposal 8.	RAN3 to discuss mechanism to handle DL non-SDT data arrival during MT-SDT for the case of partial UE context transfer. Potential solutions include:

Option 1: 	Upon detecting DL non-SDT data/signaling arrival, anchor gNB makes the decision for  complete UE context relocation to serving gNB (earlier it did just partial context relocation) and serving gNB triggers UE transition into RRC_Connected state by sending RRC Resume message. Anchor gNB also sends an indication to the receiving gNB for DL non-SDT data arrival.

Option 2: 	Anchor gNB sends an indication to the serving gNB for DL non-SDT data arrival and serving gNB makes the decision for context relocation and move to RRC_Connected state. Detailed signalling FFS.

Option 3: 	Anchor gNB sends non-SDT data/signalling arrival indication to serving gNB and serving gNB decides to send RRC Release message to UE including CFRA resources to UE (i.e., CFRA resources to UE to speed up Resumption to Connected state)

# Annex: RAN2#121 Agreements

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)