3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231269

Electronic meeting, Apr. 17th – Apr. 26th, 2023

Title: 	(TPs for SON BLCRs for TS 38.300) SHR

Source: 	Huawei

Agenda item:	10.2.1

Document Type:	discussion

Introduction

In last meeting, there were some progresses on the scenarios to be studied for inter-system inter-RAT SHR. The agreements and FFSs are showed as the following:

RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18.

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

Option 3: The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR

WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell .

Further information from the UE is FFS depending on if and how we support correlation mechanism and UE context retrieval mechanism.

RAN3 should specify mechanisms on how to correlate NR SHR and LTE RLF Report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE?

ACCESS AND MOBILITY INDICATION procedure is adopted as SHR forwarding mechanism for inter-RAT SHR?

In this paper, for the agreed scenarios and FFSs, we will give some potential solutions.

Discussion

Content of inter-RAT SHR

Correlation of NR SHR and LTE RLF Report

Further information from the UE is FFS depending on if and how we support correlation mechanism and UE context retrieval mechanism.

RAN3 should specify mechanisms on how to correlate NR SHR and LTE RLF Report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE?

In last RAN3#119 meeting, it was agreed:

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

- Option 3: The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR

According to the above agreement on the inter-RAT SHR forwarding scheme and the existing RLF report forwarding scheme, we can have the following forwarding procedures for both inter-RAT SHR and RLF for the intra-system inter-RAT HO from NR to LTE.

Fig. 1 forwarding procedures for both inter-RAT SHR and RLF report

As illustrated in the above figure, the inter-RAT HO is performed from source NR node to target LTE node. Upon the successful inter-RAT HO, the inter-RAT SHR is generated in NR format. Shortly, RLF occurs at the target LTE node and LTE RLF report is generated in LTE format.

The inter-RAT SHR encoded in NR format will be reported to NR node, while the RLF report encoded in LTE format can be reported to both NR and LTE node. It is possible that both the NR SHR and LTE RLF report are reported to the same NR reception node. Technically speaking, the NR reception node is not required and/or has no enough information to know whether the received NR SHR and LTE RLF report are should be correlated.

According to the forwarding scheme for R18 inter-RAT SHR, the NR reception node will forward the NR SHR to the source NR node which generates the SHR trigger condition that triggers the inter-RAT SHR. For the LTE RLF report, the reception node will first forward it to the target LTE node to which the failure LTE cell belongs. For the shortly RLF case, the target LTE node will forward the LTE RLF report and/or source NR C-RNTI to the source NR node. The source NR node will receive both the NR SHR and LTE RLF report for root cause analysis.

Observation 1: For the inter-RAT HO from NR to LTE, the source NR node will receive both inter-RAT NR SHR and LTE RLF report in case that the SHR is triggered and RLF occurs shortly after successful HO.

To correlate the NR SHR and LTE RLF report at the source NR node, some information should be used to deduce that the NR SHR and LTE RLF report were generated for the same mobility of the UE. As we all know, for the LTE RLF report, the source NR node may receive the source C-RNT provided by the target LTE node. Besides, there is the time information to derive the HO related information in the RLF report.

Observation 2: The source NR node will get the source C-RNTI and related time information of the LTE RLF report.

Currently, the C-RNTI information in the inter-RAT SHR from NR to LTE is still FFS. To assist the source NR node to correlate the NR SHR and LTE RLF report, an intuitive idea is to introduce the source NR C-RNT into the NR SHR. It is possible that the source NR C-RNTI may be reused in the source NR node later. Consequently, it is reasonable to include the time information, e.g., the time elapsed since the HO command until SHR retrieval, to further help the source NR node to uniquely identify the UE. With the unique UE information in the NR SHR, the source NR node can also try to find the related UE context. Moreover, based on the same source NR C-RNTI and the time information, the source NR node can correlate NR SHR and LTE RLF report and perform potential optimization for mobility parameters.

Observation 3: If the source NR node can get the source C-RNTI and the related time information from NR SHR, it can find the related UE context and correlate NR SHR and LTE RLF report based on the above information.

Proposal 1: Introduce source NR C-RNTI and time information between HO command and SHR retrieval into the inter-RAT SHR from NR to LTE.

Proposal 2: It is up to implementation for the source NR node to correlate NR SHR and LTE RLF Report in case there is an RLF shortly after a successful inter-RAT HO from NR to LTE.

In RAN3#117bis e-meeting, it was agreed to consider the following information into the inter-RAT SHR from NR to LTE:

- Source NR cell information

- Target LTE cell information

- Measurement results for source, target and neighbours

- Cause to indicate which inter-RAT SHR triggering condition was met

- FFS on Target C-RNTI

- UE location Information

FFS whether the presence of Target C-RNTI IE in inter-RAT SHR is related to the decision on supporting T304 trigger

It is straightforward way to consider the above information for inter-RAT SHR from LTE to NR with T304 trigger only. Besides, in last meeting, there was a WA on the content of inter-RAT SHR from LTE to NR.

WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell .

Therefore, we can directly make the WA as an agreement for inter-RAT SHR from LTE to NR.

Proposal 3: Make the WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell as an agreement.

For the cause to indicate which inter-RAT SHR triggering condition was met, it should support the inter-RAT SHR from LTE to NR case and set to T304 trigger related valued. For the C-RNTI in the inter-RAT SHR from LTE to NR, it should be the target one in case of T304 trigger triggering the SHR. Besides, taking into similar consideration for inter-RAT SHR from NR to LTE, we believe it is beneficial to introduce the time information between HO command and SHR retrieval.

Proposal 4: The content of inter-RAT SHR from LTE to NR includes:

- Cause to indicate which inter-RAT SHR triggering condition was met, e.g., T304 trigger

- Target NR C-RNTI

- time information between HO command and SHR retrieval

Finally, it seems beneficial to inform RAN2 of the wanted information to check whether RAN2 can support them.

Proposal 5: Send LS to RAN2 to include:

- time between HO command and SHR retrieval

- source C-RNTI for inter-RAT SHR from NR to LTE

- Cause to indicate which inter-RAT SHR triggering condition was met, e.g., T304 trigger for inter-RAT SHR from LTE to NR

- target C-RNTI for inter-RAT SHR from LTE to NR

Forwarding of inter-RAT SHR

In R17 SHR, the ACCESS AND MOBILITY INDICATION procedure is used to forward intra-RAT SHR over Xn interface, while the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG. Generally speaking, it is desirable to reuse the above existing procedures to forward the inter-RAT SHR.

Proposal 6a: ACCESS AND MOBILITY INDICATION procedure is adopted as SHR forwarding mechanism for inter-RAT SHR over Xn.

Proposal 6b: The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer are adopted as SHR forwarding mechanism for inter-RAT SHR over NG.

Forwarding scheme for R17 intra-NR SHR

Similar as the discussion of the forwarding schemes for R18 inter-RAT SHR, there are three potential options to forward the R17 intra-NR SHR:

•	Option 1: The receiving node forwards the inter-RAT SHR to source NR node, then source NR node forwards the inter-RAT SHR to target LTE node

•	Option 2: The receiving node forwards the inter-RAT SHR to target LTE node, then target LTE node forwards the inter-RAT SHR to source NR node

•	Option 3: The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR

For R18 inter-RAT SHR forwarding, RAN3 agreed option 3. The receiving node will forward the inter-RAT SHR to the corresponding node responsible for the mobility. As indicated in the above analysis, we prefer to introduce new IEs including source C-RNTI and time information between HO command and SHR retrieval into the R17 SHR to support inter-RAH SHR function. Accounting for that the ASN.1 for R17 has been frozen in RAN2, option 3 cannot work for R17 UE. The same issue also occurs in option 1.

In the R17 SHR, there is the target C-RNTI. The target C-RNTI is useful for the target node to identify the UE. It is noticed that option 2 can reuse the existing R17 target C-RNTI. If it is the source NR node responsible for the SHR, the target node should perform the mapping between the target C-RNTI and the related UE context. In this way, the target node finds the related UE context, then transfers the mapped source C-RNTI and/or mobility information to the source node. The kind of lookup function is supported in case of RLF for MRO. Here we are talking about the successful handover. This may bring additional complexity on the target node in case that only T310/312 triggering conditions are configured. In addition, accounting for that the ASN.1 for R17 has been frozen, there will be no impact on the R17 SHR in RAN2.

Observation 4a: Option 2 can reuse the R17 target C-RNTI, and would work for R17 SHR but requires the optional source C-RNTI from target node over Xn.

Observation 4b: Option 1 and 3 require the UE to provide additional source C-RNTI. These options will not work for legacy R17 UE.

Proposal 7: Option 2 can be considered as the forwarding scheme for R17 intra-NR SHR.

Conclusion

In this paper, we discuss miscellaneous enhancements for intra-system inter-RAT SHR, and we have the following observations and proposals:

Observation 1: For the inter-RAT HO from NR to LTE, the source NR node will receive both inter-RAT NR SHR and LTE RLF report in case that the SHR is triggered and RLF occurs shortly after successful HO.

Observation 2: The source NR node will get the source C-RNTI and related time information of the LTE RLF report.

Observation 3: If the source NR node can get the source C-RNTI and the related time information from NR SHR, it can find the related UE context and correlate NR SHR and LTE RLF report based on the above information.

Observation 4a: Option 2 can reuse the R17 target C-RNTI, and would work for R17 SHR but requires the optional source C-RNTI from target node over Xn.

Observation 4b: Option 1 and 3 require the UE to provide additional source C-RNTI. These options will not work for legacy R17 UE.

Proposal 1: Introduce source NR C-RNTI and time information between HO command and SHR retrieval into the inter-RAT SHR from NR to LTE.

Proposal 2: It is up to implementation for the source NR node to correlate NR SHR and LTE RLF Report in case there is an RLF shortly after a successful inter-RAT HO from NR to LTE.

Proposal 3: Make the WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell as an agreement.

Proposal 4: The content of inter-RAT SHR from LTE to NR includes:

- Cause to indicate which inter-RAT SHR triggering condition was met, e.g., T304 trigger

- Target NR C-RNTI

- time information between HO command and SHR retrieval

Proposal 5: Send LS to RAN2 to include:

- time between HO command and SHR retrieval

- source C-RNTI for inter-RAT SHR from NR to LTE

- Cause to indicate which inter-RAT SHR triggering condition was met, e.g., T304 trigger for inter-RAT SHR from LTE to NR

- target C-RNTI for inter-RAT SHR from LTE to NR

Proposal 6a: ACCESS AND MOBILITY INDICATION procedure is adopted as SHR forwarding mechanism for inter-RAT SHR over Xn.

Proposal 6b: The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer are adopted as SHR forwarding mechanism for inter-RAT SHR over NG.

Proposal 7: Option 2 can be considered as the forwarding scheme for R17 intra-NR SHR.

A draft LS also including SHR aspects are included in the annex.

Reference

[1] R3-226803, Summary of Offline Discussion on CB: # 17_SHR_SPCR

Annex_draft LS to RAN2

Title:	LS on intra-system inter-RAT SHR

Response to:

Release:	Rel-18

Work Item:	         NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Henrik Olofsson

E-mail Address:	henrik.olofsson@huawei.com

- Overall Description:

RAN3 has discussed the potential enhancements for inter-RAT HO from NR to LTE for RAN2 to provide:

- Consider the following potential enhancements for inter-RAT HO from NR to LTE:

  - time between HO command and SHR retrieval

  - source C-RNTI for inter-RAT SHR from NR to LTE

RAN3 also discussed the scenario of inter-RAT HO from LTE to NR and identified the potential requirements for RAN2 to provide for the inter-RAT SHR:

- SHR (Successful HO Report) for intra-system inter-RAT, HO from LTE to NR only for T304 trigger will be considered;

- Consider the following potential enhancements for inter-RAT HO from LTE to NR:

  - time between HO command and SHR retrieval

  - Cause to indicate which inter-RAT SHR triggering condition was met, e.g., T304 trigger for inter-RAT SHR from LTE to NR

  - target C-RNTI for inter-RAT SHR from LTE to NR

## 2. Actions:

To RAN2:

ACTION: RAN3 kindly asks RAN2 to take the above into consideration and start analysis on possible Uu impacts.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #120		22 – 26 May 2022			Incheon, KR

TSG-RAN3 Meeting #121		21 – 25 Aug 2022			Toulouse, FR

Annex – TP to TS38.300 for intra-system inter-RAT SHR

Start of the change

#### 15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE supports Successful Handover Report based on configuration by network, if received, and makes the Successful Handover Report available to the network as specified in TS 38.331 [12].

The UE generates Successful Handover Report in NR format. The UE stores the Successful Handover Report until the Successful Handover Report is fetched by the network or for 48 hours after the Successful Handover Report is recorded.

For intra-RAT case, when the target NR node fetches the Successful HO Report from UE, it may forward the information to the source node, i.e the node handling the cell reported as source cell in this Successful HO Report, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

For inter-RAT case, the NR node receiving the Successful Handover report from UE, it forwards the Successful HO Report to the node which configured the Successful HO Report trigger that triggered the Successful HO Report, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

Upon retrieval of a Successful Handover Report, the receiving node may analyse whether its mobility configuration needs adjustment.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.

End of the change