3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231280

Online, April 17 – 26, 2023

Agenda item:	23.2

Source: 	Qualcomm Incorporated

Title: 	Integrity of NR Positioning Technologies

Document for:		Discussion and Decision

# 1.	Introduction

At RAN#98-e, the Work Item on "Expanded and Improved NR Positioning" was agreed which includes the following objective for "RAT-dependent Integrity" [1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

In this contribution, we review the integrity for the NR Positioning Technologies and discuss the RAN3 specification impacts.

# 2.	Integrity for NR Positioning Technologies

##### Integrity

Integrity is generally defined as a measure of the level of trust that can be placed in the correctness of the information supplied by a navigation system for a specific application. Integrity of a navigation system includes the ability to provide users timely information on the level of trust that can be placed in the evaluated position.

##### Protection Level (PL)

This trustworthiness is defined by the protection level associated with a given navigation solution, which provides an upper bound on the position error with a probability derived from the integrity requirement. The protection levels are computed based on the expected behaviour of the error sources encountered in a positioning system. A general definition of the Protection Level (PL) for the fault-free case is as follows:

| PL=KmdσPE+B | (1) |
| --- | --- |
| Kmd=Q-1IR2 | (2) |
| Q=12πu∞e-t2/2dt | (3) |

where σPE is the user position error standard deviation at receiver level and B is a bias term related to all possible biases included in the measurements. Single measurement variances can be represented as follows:

| σi2=σA,i2+σB,i2+σC,i2+… | (4) |
| --- | --- |

where σx,i2 is the variance of the specific error source (which depends on the NR positioning method) for TRP i. The Q-function is the tail distribution function of the standard normal distribution. In other words, Q(t) is the probability that a normal (Gaussian) random variable will obtain a value larger than t standard deviations. IR in equation (2) is the Integrity Risk, which is defined as the probability of providing information that is out of the specified tolerance in a defined period of time without issuing a warning to the user (the management of "warnings" depends on the application). For example, in case of Integrity Risk of 10-7, Kmd = 5.33.

Essentially, the Protection Level defines the value that, with a very high probability, bounds the error of the navigation solution. Therefore, the complementary probability, the Integrity Risk, that the error in the navigation solution exceeds its associated protection level must be small enough to not exceed a threshold known as the Target Integrity Risk, which is application specific. The Protection Level is a real-time and dynamic quantity which may vary from one output epoch to the next.

Observation 1:	To leverage the PL definition, the individual error source distributions are modelled as Gaussian distribution.

##### Error Sources

To define the integrity risk of a positioning system, it is necessary to understand its error sources, threat models, frequency of occurrences and potential failure modes. Many threats could render a positioning system (location service)  unavailable. RAN1 summarized the error sources of the NR positioning technologies in Table 6.1.1-1 of TR 38.859 [2], which is repeated below.

Error sources for LMF-based and UE-based positioning integrity modes

|  | DL TDOA | UL TDOA | Multi-RTT | UL AoA | DL AoD |
| --- | --- | --- | --- | --- | --- |
| LMF-based | RSTD measurement TRP location Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | RTOA measurement TRP location Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | UE Rx-Tx time difference measurement gNB Rx-Tx time difference measurement TRP location | Angle of arrival measurement TRP location ARP location (e.g., ARPLocationInformation in TS 38.455) | TRP location DL-PRS RSRPP of the first path or RSRP |
| UE-based | TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) |  |  |  | TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) |

The above error sources can be grouped into UE and TRP measurement errors and network/system errors:

- UE measurement errors:

-	RSTD measurement

-	UE Rx-Tx time difference measurement

-	DL-PRS RSRP/RSRPP

- TRP measurement errors:

-	gNB Rx-Tx time difference measurement

-	RTOA measurement

-	Angle of arrival measurement

- Network/System errors:

-	TRP/ARP location

- 	Inter-TRP synchronization

Item (1) above may affect LPP signalling only, and therefore, has no impact on RAN3 specifications.

Item (2) above may affect NRPPa Measurement Information Transfer procedures and is discussed in section 3 below.

Item (3) above resulted in following agreement at RAN2#121 [3]:

Agreements:

TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or posSIB.

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope. Other aspects of determining the TRP error sources are left to deployment and implementation.

with potential RAN3 impact which is further discussed below.

Observation 2:	To support integrity for the NR positioning technologies, the potential RAN3 impacts include:

-	specification of the TRP measurement error statistics/bounds (gNB Rx-Tx, RTOA, AOA);

-	specification of the TRP/ARP location and synchronization error statistics/bounds.

##### Integrity Bounds and Residual Risks

To properly quantify the Protection Level, the network ensures that the probability of an unflagged incidence (not "Do Not Use" (DNU)) of an error exceeding the corresponding bound is less than a specified threshold [4]:

| 𝑃(𝐼𝑛𝑡𝑒𝑔𝑟𝑖𝑡𝑦 𝐸𝑣𝑒𝑛𝑡) = 𝑃(𝐸𝑟𝑟𝑜𝑟>𝐵𝑜𝑢𝑛𝑑 ¦ 𝑁𝑂𝑇 𝐷𝑁𝑈) <= Threshold | (5) |
| --- | --- |

which is typically decomposed into Fault and Fault-free cases:

| PIntegrity Event=PError>Bound NOT DNU, Fault+PError>Bound NOT DNU, NOT Fault | (6) |
| --- | --- |

Each of these cases can be assigned a probability upper bound:

| PError>Bound NOT DNU, Fault<=Residual Risk | (7) |
| --- | --- |
| P(Error>Bound ¦ NOT DNU, NOT Fault)<=IRallocation | (8) |

which results in the 'Integrity Principle of Operation' defined in TS 38.305 [5]:

| P(Error > Bound ¦ NOT DNU) <= Residual Risk + IRallocation | (9) |
| --- | --- |

for all the errors which have corresponding integrity assistance data available and where the corresponding DNU flag(s) are set to false. This decomposes the risk into a fixed part to be provided in the Assistance Data (Residual Risk), plus a variable component that scales with the Bound (IRallocation):

| Bound = mean + K * stdDev | (10) |
| --- | --- |
| K = normInv(IRallocation / 2) | (11) |
| irMinimum ≤ IRallocation ≤ irMaximum | (12) |

(Compare equations (1)-(3))

##### Gaussian Overbounding

The bound formula (10)-(12) describe a bounding model including a mean and standard deviation. Each error of the NR location technologies (subscript A, B, C, … in equation (4)) will be distributed according to some statistical distribution. In general, it is not possible to know the true distribution of the errors. Different errors may also have a different distribution (e.g., RTDs vs. ARP locations) and the true distribution highly dependents on implementation (e.g., TRP clock steering/adjustment algorithms, etc.). The true distribution can be approximated with a Gaussian distribution that is conservative, i.e., it "overbounds" the true distribution and never under-estimates the probability of an error of a given magnitude. The goal is to find a Gaussian distribution that can be used instead of "true distribution" in the integrity calculations and that will result in an upper bound of the integrity risk. One principle is to inflate the sigma, such that the error distribution (characterized by σ) overbounds all reasonable error distributions out to the probabilities assumed in the computation of the PLs. The inflation generates conservative Gaussian models, called "overbounds".

Observation 3:	The process of replacing an actual distribution with a simplified, conservative error model is called overbounding.

Observation 4:	To leverage the PL definition, the idealized overbound must be Gaussian (see equations (1)-(3) and (10)-(12)).

Since real error distributions tend to have non-zero mean and "fat tails" that would require a very conservative value of σ to overbound the distribution (resulting in over-inflated Protection Levels), a pair of bounds can be used, rather than a single bound, to represent the actual error distribution. The true distribution is bounded by two non-zero mean Gaussians, shifted by -𝜇 and +𝜇. This allows for a tighter bound that is still an overbound across the whole distribution.

In effect, distribution anomalies are converted into biases (e.g., [6]). The "Paired Overbounding" theorem guarantees bounding after a convolution operation (i.e., probability distribution for a sum of errors).

Observation 5:	Paired Overbounding uses a pair of bounds, rather than a single bound, to represent the actual error distribution.

In order to implement the paired-Gaussian bound, it is necessary to incorporate the bias parameters for each error source into the PL computation as shown in equations (1) and (10).

The difference compared to GNSS is solely on the different error sources applicable to the various NR positioning technologies (see Error Sources above).

Observations 1, 3-5 above resulted in the following agreement at RAN2#121 [3]:

Agreements:

RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

##### Network Integrity Monitoring

How the final protection level is derived by a positioning engine is left to implementation for the Rel-17 GNSS integrity and several approaches can be used. The equation (4) can be used to describe the overall error contribution for each measurement. The terms σx,i2 could be derived from real-time reference network measurements. For GNSS, such reference networks are typically maintained by GNSS service providers and how an LMF obtains the data is left to deployment in Rel-17. For NR positioning technologies, a similar reference receiver network would be required to monitor the system to ensure its integrity. For NR, this could for example be realized using PRUs, but is also proposed to be left to deployment/implementation.

Proposal 1:	How an LMF obtains the TRP/ARP location and synchronization error statistics/bounds (e.g., using a reference receiver network/PRUs, etc.) should be left to implementation/deployment (similar to Rel-17 GNSS integrity).

The information of the network/system error sources (e.g., TRP/ARP location, inter-TRP synchronization) needs to be available at the position calculation engine, which in case of UE-based mode is located in the UE and in case of UE-assisted/network-based mode located in the LMF.

Observation 6:	The Protection Level is determined by the entity which hosts the position calculation engine; i.e., by the UE for UE-based mode or LMF for UE-assisted/network-based mode. The same integrity assistance data need to be available at the LMF for both positioning modes but need to be provided to the UE for UE-based mode.

# 3.	LMF-based PL calculation

To determine the PL by an LMF for UE-assisted mode or network-based positioning methods, the LMF requires the overbounding standard deviation of the UE and TRP measurement error (e.g., RSTD, RTOA, Rx-Tx, AoA, DL-PRS RSRP, DL-PRS RSRPP), which provides an upper bound on the standard deviation of the nominal measurement error σx,i (equation (4)).  That is, under fault-free condition, the standard deviation of the measurement error is no greater than σx,i. In addition, a bias bound may be needed (e.g, equation (1),(10)), which provides the mean value for an overbounding model that bounds the measurement mean error.

Observation 7:	To leverage the PL definition, the LMF requires the overbounding mean and standard deviation of the UE and TRP measurement error (e.g., RSTD, RTOA, Rx-Tx, AoA, DL-PRS RSRP, DL-PRS RSRPP) for LMF-based PL calculation.

Obviously, the UE and TRP measurement errors are not observable and need to be approximated/estimated via e.g., standard error of the mean which estimates the variability across multiple samples of a population:

| σ=(xi-x)2N | (13) |
| --- | --- |

This standard deviation may serve as a measure of uncertainty and requires a number N of samples. Therefore, periodic measurements are required, where either the UE/TRP determines the sample statistics or the LMF uses the periodic reports from the UE/TRP to determine sample statistics.

The sample statistics may be derived or used by an e.g., Kalman Filter whose details depends on LMF implementation/algorithms used. Instead of deriving the measurement statistics at the UE/TRP and reporting to an LMF, it seems more flexible and implementation-independent if an LMF determines any statistics from the (already specified) UE/TRP measurement report directly. I.e., a LMF could request periodic reporting from UEs and TRPs and could derive any required statistics from the  measurement reports.

Observation 8:	For LMF-based PL calculation ("LMF based integrity" in the Work Item objective), there are two general options:
Option 1:  The UE/TRP determines the sample statistics/error bounds and reports them to the LMF, or 
Option 2:  the LMF determines the sample statistics/error bounds using (existing) periodic UE/TRP measurement reporting.

Option 2 in Observation 8 above would have no additional RAN3 specification impacts. However, the mechanism for determining the sample statistics/error bounds of the TRP measurements should be aligned with the mechanism for determining the UE measurements sample statistics/error bounds, which is under RAN2's responsibility.

Proposal 2:	Whether the TRP measurement error statistics/bounds (for RTOA, gNB Rx-Tx time difference, AoA) are determined by the TRP or by the LMF should be aligned with the UE measurement error statistics/bounds determination, which is under RAN2's responsibility.

Both Options in Observation 8 are equivalent and would allow the determination of measurement error bounds (e.g., using equation (13)). However, Option 2 would not require specifying a particular method/equation for determining the error statistics/bounds and therefore, could be up to LMF implementation. For example, the LMF could decide which measurement items are used to determine the bounds (e.g., UL AoA, multiple AoA, measurement quality, LOS/NLOS Information, etc.), or how many samples are used or needed (e.g., N in equation (13), etc.). Therefore, Option 2 in Observation 8 could be implementation dependent and would be most flexible (i.e., no "hard coding" of the method/definition would be needed). For example, changing the method for determining the measurement error statistics/bounds and adapting it to new/updated position calculation algorithms would not require standards changes.

Proposal 3:	For LMF-based PL calculation ("LMF based integrity" in the Work Item objective), the LMF requests periodic UE and TRP measurement reporting and the LMF determines any desired sample statistics/bounds from the measurements. Inform RAN2 about RAN3's preference.

Observation 9:	If Proposals 1 and 3 can be agreed, the Work Item Objective for "RAT-dependent Integrity" would have no new RAN3 specification impacts.

# 4.	Summary

In this contribution, we reviewed the integrity for the NR positioning technologies and discussed the potential RAN3 specification impacts.

The following Observations and Proposals were made.

Observation 1:	To leverage the PL definition, the individual error source distributions are modelled as Gaussian distribution.

Observation 2:	To support integrity for the NR positioning technologies, the potential RAN3 impacts include:

-	specification of the TRP measurement error statistics/bounds (gNB Rx-Tx, RTOA, AOA);

-	specification of the TRP/ARP location and synchronization error statistics/bounds.

Observation 3:	The process of replacing an actual distribution with a simplified, conservative error model is called overbounding.

Observation 4:	To leverage the PL definition, the idealized overbound must be Gaussian (see equations (1)-(3) and (10)-(12)).

Observation 5:	Paired Overbounding uses a pair of bounds, rather than a single bound, to represent the actual error distribution.

Observation 6:	The Protection Level is determined by the entity which hosts the position calculation engine; i.e., by the UE for UE-based mode or LMF for UE-assisted/network-based mode. The same integrity assistance data need to be available at the LMF for both positioning modes but need to be provided to the UE for UE-based mode.

Observation 7:	To leverage the PL definition, the LMF requires the overbounding mean and standard deviation of the UE and TRP measurement error (e.g., RSTD, RTOA, Rx-Tx, AoA, DL-PRS RSRP, DL-PRS RSRPP) for LMF-based PL calculation.

Observation 8:	For LMF-based PL calculation ("LMF based integrity" in the Work Item objective), there are two general options:
Option 1:  The UE/TRP determines the sample statistics/error bounds and reports them to the LMF, or 
Option 2:  the LMF determines the sample statistics/error bounds using (existing) periodic UE/TRP measurement reporting.

Observation 9:	If Proposals 1 and 3 can be agreed, the Work Item Objective for "RAT-dependent Integrity" would have no new RAN3 specification impacts.

Proposal 1:	How an LMF obtains the TRP/ARP location and synchronization error statistics/bounds (e.g., using a reference receiver network/PRUs, etc.) should be left to implementation/deployment (similar to Rel-17 GNSS integrity).

Proposal 2:	Whether the TRP measurement error statistics/bounds (for RTOA, gNB Rx-Tx time difference, AoA) are determined by the TRP or by the LMF should be aligned with the UE measurement error statistics/bounds determination, which is under RAN2's responsibility.

Proposal 3:	For LMF-based PL calculation ("LMF based integrity" in the Work Item objective), the LMF requests periodic UE and TRP measurement reporting and the LMF determines any desired sample statistics/bounds from the measurements. Inform RAN2 about RAN3's preference.

# References

[1]		RP-223549, "New WID on Expanded and Improved NR Positioning".

[2]		3GPP TR 38.859: "Study on expanded and improved NR positioning".

[3]		R2-2301904, "Report from session on positioning and sidelink relay", Session Chair (MediaTek) 						(RAN2#121).

[4]		R2-2110141, "Discussion on GNSS Integrity Assistance Data", Swift Navigation, Mitsubishi Electric 					Corporation, Intel Corporation, Ericsson.

[5]		3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]		J. Rife, S. Pullen, B. Pervan, and P. Enge, "Paired Overbounding and Application to GPS Augmentation", 			PLANS 2004. Position Location and Navigation Symposium.