3GPP TSG-RAN WG3 #119bis-e		R3-221321

17th – 26th April 2023

Agenda Item:	11.4

Source:	CATT

Title:	Discussion on Left-over issues

Document for:	Discussion

- Introduction

In last ran3 meeting, the following agreements have been achieved. And it is captured in chair Notes [1]:

RAN3 to await the further progress of SA4 and ITU-T before proceeding further on the topic of RVQoE value.

In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

Introduce the deactivation of RAN visible QoE information transfer via F1. No need to introduce pause/resume mechanism in Release 18.

In this contribution, we provide some analysis on open issue.

- Discussion

2.1 event-based triggers of RVQoE for mobility

For the metrics Buffer Level, we agreed to introduce a threshold-based trigger for RVQoE reporting. For the metrics Playout Delay for Media Startup, we don’t think it needed to introduce any other trigger because it is one time event-trigger metrics.

On whether to introduce event-based trigger for buffer level RVQoE reporting, we think threshold-based triggers is enough because the handover may deteriorate the buffer level value. So during handover, the buffer level value has high possibility to meet the defined threshold. So for mobility, threshold-based triggers may also be used to detect mobility events.

Proposal 1: It is proposed to not introduce event-based triggers of RVQoE for mobility.

2.2 Overload scenario

In last RAN3 meeting, RAN3 decide to introduce assistance information as below:

In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

In NG interface, QoE measurement configuration is provided from CN to RAN in INITIAL CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message. In QoE measurement configuration includes lists of UE Application Layer Measurement Configuration Information which is identified by QoE Reference. We think assistance information shall be configured per QoE Reference.

Proposal 2: It is proposed to introduce assistance information per QoE Reference in QMC Configuration Information in in INITIAL CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message.

We think priorities of QoE measurements shall be used as part of assistance information. RAN can use the priorities to decide how to handle QoE measurement in case of overload.

Proposal 3: It is proposed to introduce a priority as assistance information for QoE reference.

When receiving QoE activation information associated with the assistance information, RAN3 can decide how to handle the QoE activation in case of overload. The proper method may be:

- activate QoE measurement normally;

- delay QoE activation until the overload is solved;

- reject QoE activation, i.e. QoE activation failure;

But we notice that QoE activation failure is not supported by current specification, i.e. NG-RAN cannot inform CN about the failure of QoE activation. So, we propose RAN3 to discuss whether the scenario is reasonable and shall be supported or not.

Proposal 4: It is proposed to introduce QoE activation failure indication from NG-RAN to CN.

- Conclusion

According to the analysis in section 2, we have:

Proposal 1: It is proposed to not introduce event-based triggers of RVQoE for mobility.

Proposal 2: It is proposed to introduce assistance information per QoE Reference in QMC Configuration Information in in INITIAL CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message.

Proposal 3: It is proposed to introduce a priority as assistance information for QoE reference.

Proposal 4: It is proposed to introduce QoE activation failure indication from NG-RAN to CN.

- Reference

[1] RAN3_119_agenda_20230303_1830