3GPP TSG-RAN WG3 #119bis-e		                                                                 R3-231359

Online 17th – 26th Apr 2023

Title: 	Discussion on PCI collision avoidance for mobile IAB

Source: 	ZTE

Agenda item:	13.4

Document for:	Discussion and Approval

Introduction

During RAN3#117bis-e meeting, the PCI collision issue was discussed and the following agreements were reached:

PCI Space Partitioning is performed by OAM and up to implementation. As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message. PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively. FFS for the PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs. PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

In this paper, we will discuss the PCI collision issue and give our analysis on the potential enhancements and RAN3 impacts.

Discussion

The PCI of mobile IAB-DU cell may collide with other cells due to the movement of mobile IAB-MT. That means PCI collision may occur during partial migration or full migration. Here we discuss the procedure for PCI collision avoidance during partial migration and full migration respectively.

- Scenario 1: partial migration

As agreed in RAN3#118 meeting, the mIAB-MT’s source donor CU can send the info (i.e. gNB ID) on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO. And then, the DU’s donor can be aware of the gNB ID of the MT’s target donor. When there is Xn connection between the DU’s donor CU and MT’s target donor, the DU’s donor CU can be aware of PCIs used in the serving cells and neighbour cells of the MT’s target donor CU via existing XnAP signaling, e.g., Xn SETUP/NG-RAN NODE CONFIGURATION UPDATE. Thus the DU’s donor can detect potential PCI collision for the mobile IAB-DU cell. And then if the DU’s donor detects the PCI collision, it can send the new PCI(s) for the IAB-DU cell via F1 signalling, e.g., GNB-CU CONFIGURATION UPDATE message. After receiving new PCI(s) for the mobile IAB node from the F1-terminating donor, the mobile IAB node activates the 2nd set of cells using the new PCI(s). And then the DU’s donor can send RRCreconfiguration message for the UEs to migrate the UEs from the 1st set of cells to the 2nd set of cells. Considering that there is no need for UE migration in partial migration, there is no need to have two logical DUs in the mobile IAB node.

Observation 1: During partial migration, if there is Xn connection between the DU’s donor CU and MT’s target donor, the DU’s donor CU can detect the PCI collision and reconfigure PCI for the mobile IAB cell via existing F1AP message.

During RAN3#119 meeting, it was agreed that for scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF. That means NGAP based partial migration needs to be supported when there is no Xn connection between the F1-terminating donor and the MT’s target donor. During partial migration where there is no Xn connection between the F1-terminating donor and MT’s target donor, it is not clear how could the F1-terminating donor detect the PCI collision. The reason is that the F1-terminating donor is not able to obtain the PCIs of the serving cells and its neighbouring cells of the MT’s target donor since there is no Xn connection.

Observation 2: Partial migration needs to be supported when there is no Xn connection between the F1-terminating donor and the MT’s target donor. In this case, it is not clear how could the F1-terminating donor detect the PCI collision since the F1-terminating donor is not able to obtain the PCIs of the serving cells and its neighbouring cells of the MT’s target donor.

In this situation, it should be discussed how could the PCI collision be detected during partial migration when there is no Xn connection between the F1-terminating donor and the MT’s target donor. Some companies propose that the F1-terminating IAB-donor can detect the PCI collision based on the UE’s report in this case. In our understanding, the UE report method means UEs served by the mobile IAB node report the detected PCI and corresponding NCGI after the mobile IAB node moves into the coverage of the neighbour cell with the same PCI. In this situation, the UE report method doesn’t help since PCI collision is detected after the collision occurs, which is too late. As we know, it is captured in the objective that we need to investigate the avoidance of PCI collision rather than detect the PCI collision after the collision occurs.

Proposal 1: RAN3 to discuss how could PCI collision be detected during partial migration where there is no Xn connection between the F1-terminating donor and the MT’s target donor.

- To solve this problem, the following two options could be considered during partial migration where there is no Xn connection between the F1-terminating donor and the MT’s target donor:

- Option 1: the PCI of mobile IAB cell is configured by DU’s donor via F1

- In this option, the DU’s donor needs to coordinate with the MT’s donor so that the DU’s donor can detect the PCI collision for mobile IAB cell. That means NGAP needs to be enhanced to support the PCI collision detect and PCI reconfiguration during partial migration where there is no Xn connection between the F1-terminating donor and the MT’s target donor. For example, PCIs used in the serving cells and neighbour cells of the MT’s target donor may need to be transferred via NGAP from the MT’s target donor to DU’s donor.

- Observation 3: In partial migration where there is no Xn connection between the DU’s donor and the MT’s donor, the DU’s donor needs to coordinate with the MT’s donor so that the DU’s donor can detect the PCI collision for mobile IAB cell and NGAP needs to be enhanced to support that.

- Option 2:  the PCI of mobile IAB cell is configured by MT’s donor via RRC

- In option 2, the PCI(s) of the mobile IAB-DU cell(s) are sent by MT’s source donor CU to the MT’s target donor CU during HO preparation phase of the mobile IAB-MT. In this way, the MT’s target donor can detects potential PCI collision in advance. And the target donor CU could re-assign new PCI(s)for the mobile IAB-DU cell and send the new PCI(s) to the mobile IAB-MT via the RRCReconfiguration message if potential PCI collision is detected. Upon receiving the new PCI(s) via the RRC message from source donor CU, the mobile IAB-MT delivers the new PCI(s) of mobile IAB cell to the co-located mobile IAB-DU. And then the mobile IAB node activates the 2nd set of cells using the new PCI(s).

In a sum, more standardization effort and spec impact (i.e. NG impact) is needed if PCI collision detection and reconfiguration of mobile IAB cell is performed by DU’s donor in partial migration where there is no Xn connection between the DU’s donor and the MT’s donor. Similar issue exists in TAC configuration of mobile IAB cell as analyzed in [1]. We suggest that unified solution is adopted for the PCI/TAC configuration of mobile IAB cell, i.e. the PCI collision detection and reconfiguration is performed by MT’s donor during partial migration where there is no Xn connection between the DU’donor and the MT’s donor. Considering that the reconfiguration of PCI by MT’s donor via RRC needs to be discussed in RAN2, we suggest to send an LS to RAN2 to trigger the corresponding work in RAN2.

Observation 4: More standardization effort and spec impact (i.e. NG impact) is needed if PCI collision detection and reconfiguration of mobile IAB cell is performed by DU’s donor in partial migration where there is no Xn connection between the DU’s donor and the MT’s donor.

Proposal 2: PCI collision can be detected by the MT’s target IAB-donor and new PCI(s) are allocated by the MT’s target IAB-donor during the MT HO preparation phase in partial migration where there is no Xn connection between the DU’s donor and the MT’s donor.

Proposal 3: Send an LS to RAN2 to inform the progress in RAN3 and trigger the corresponding work in RAN2.

- Scenario 2: full migration

During the full migration, the target logical DU is activated upon the trigger of F1 setup procedure. In this situation, it needs to be discussed how could PCI of the target logical DU cell be configured. During DU migration, mobile IAB node needs to establish F1 connection with DU’s target donor. During F1 connection establishment procedure, mobile IAB-DU’s target donor can configure PCIs for cells of target logical DU. The DU’s target donor CU may request a new PCI from OAM or select a new PCI value from the preconfigured PCI list, and then send it to the mobile IAB-DU, e.g., via F1 SETUP RESPONSE message.

Proposal 4: During DU migration, mobile IAB-DU’s target donor can configure PCIs for cells of target logical DU during F1 connection establishment procedure.

Conclusion

In this paper, we discussed the PCI collision issue and gave our analysis on the potential enhancements and RAN3 impacts. The following observations and proposals are presented:

Observation 1: During partial migration, if there is Xn connection between the DU’s donor CU and MT’s target donor, the DU’s donor CU can detect the PCI collision and reconfigure PCI for the mobile IAB cell via existing F1AP message.

Observation 2: Partial migration needs to be supported when there is no Xn connection between the F1-terminating donor and the MT’s target donor. In this case, it is not clear how could the F1-terminating donor detect the PCI collision since the F1-terminating donor is not able to obtain the PCIs of the serving cells and its neighbouring cells of the MT’s target donor.

Proposal 1: RAN3 to discuss how could PCI collision be detected during partial migration where there is no Xn connection between the F1-terminating donor and the MT’s target donor.

- Observation 3: In partial migration where there is no Xn connection between the DU’s donor and the MT’s donor, the DU’s donor needs to coordinate with the MT’s donor so that the DU’s donor can detect the PCI collision for mobile IAB cell and NGAP needs to be enhanced to support that.

Observation 4: More standardization effort and spec impact (i.e. NG impact) is needed if PCI collision detection and reconfiguration of mobile IAB cell is performed by DU’s donor in partial migration where there is no Xn connection between the DU’s donor and the MT’s donor.

Proposal 2: PCI collision can be detected by the MT’s target IAB-donor and new PCI(s) are allocated by the MT’s target IAB-donor during the MT HO preparation phase in partial migration where there is no Xn connection between the DU’s donor and the MT’s donor.

Proposal 3: Send an LS to RAN2 to inform the progress in RAN3 and trigger the corresponding work in RAN2.

Proposal 4: During DU migration, mobile IAB-DU’s target donor can configure PCIs for cells of target logical DU during F1 connection establishment procedure.

Reference

- R3-231358 Discussion on enhancements to IAB node migration in mobile IAB scenario ZTE