3GPP TSG-RAN WG3 Meeting #119	R3-231398

Athens, GR, 27 Feb – 03 Mar, 2023

Title: 	(TPs to TS 38.300, 401, 413, 423,470, 473 BL CRs) Multicast Reception for RRC_INACTIVE state UEs

Source: 	Huawei, CBN

Agenda item:	15.3

Document Type:	other

# 1. Introduction

In RAN3#119 meeting, there was one one agreement achieved about multicast reception for RRC_INACTIVE state UEs, and the other parts of this objective were not discussed due to limit of time.

Support a per UE per MBS session indication from CN to RAN.

In this contribution, we will further discuss the issue about multicast reception for RRC_INACTIVE state UEs.

# 2. Discussion

## 2.1 Assistance Information

There are some conclusions about the assistance information according to the agreed CRs S2-2303396 and S2-2303395 in SA2#155 meeting:

S2-2303395:

-	Optionally, MBS assistance information indicating that a UE is preferred to be kept connected when the related MBS Session the UE joined is active, which contains the following information:

| MBS assistance information | Indicates that the UE is preferred to be kept connected when the related MBS session the UE joined is active, which contains the related MBS Session ID(s) |
| --- | --- |

-	the AF may provision MBS Session Assistance Information, which indicates that a UE is preferred to be kept connected when the related MBS Session the UE joined is active.

Table 7.2.9a-1: MBS Session Assistance information

| Parameters | Description |
| --- | --- |
| MBS Session Assistance information | A list of UEs represented by GPSIs that are preferred to be kept connected when the MBS Session represented by the indicated MBS session ID the UE joined is active |

S2-2303396:

-	MBS assistance information for the MBS session, the MBS assistance information for the MBS session is an optional parameter and associated with one MBS session, which consists of an indication that the UE is preferred to be kept in connected when the related MBS session that the UE joined is active. When the NG-RAN node receives this information, the NG-RAN may determine to keep the UE in RRC_CONNECTED state even if the MBS session data is supported to be received in RRC_INACTIVE state.

Editor's Note:	The MBS assistance information protocol details require RAN WG feedback, e.g. whether the indication is enough.

NOTE 1:	 How the NG-RAN nodes perform those decisions is up to NG-RAN implementation.

- Per the MBS session that the UE joined, the related "MBS assistance information for the MBS session" is provided to NG-RAN by the SMF if the MBS assistance information is available in the SMF and the MBS session that the UE joined is included in the MBS assistance information. The SMF gets from the UDM the "MBS assistance information", which is provisioned by the AF via the NEF to the UDM as part of the MBS subscription data and includes all the MBS session ID(s), where the UE is preferred to be kept connected when the related MBS session that the UE joined is active (as specified in clause 6.4). The SMF provides the "MBS assistance information for the MBS session" to the NG-RAN as part of the associated PDU session information within the N2 SM information in the procedures where the associated PDU session information need be sent to NG-RAN node, e.g. PDU Session modification for UE joining, handover procedure.

However, it is still FFS that which interpretation on the indication from CN to RAN should be adopted in last RAN3 meeting.

Interpretation 1: UE is preferred to be kept in RRC connected when receiving the related MBS session data. Interpretation 2: The IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception.

From our view, the interpretation 1 is aligned with SA2 progress, and as it is up to the NG-RAN node to determine how to use the received information, the mentioned “preferential treatment” in the interpretation 2 seems over specified.

Proposal 1-1: RAN3 adopt the Interpretation 1 and introduce the MBS Assistance Information IE as ENUMERATED (true, …) to inform the recommendation from core network.

Proposal 1-2: introduce the the MBS Assistance Information IE in NGAP: MBS Session Setup Request List IE and MBS Session Setup or Modify Request List IE.

Upon receiving the MBS assistance information, based on the agreed SA2 CR S2-2303846, the source NG-RAN node provides it to the target NG-RAN during mobility:

S2-2303846:

- During the N2 based handover preparation phase, the SMF provides the MBS assistance information in the MBS session information to target NG-RAN.

- The MBS session information transferred from source NG-RAN towards target NG-RAN also include MBS assistance information for the MBS session if such information is available at the source RAN node.

Editor's note: 	It is up to RAN WG to confirm that the MBS session information transferred from source NG-RAN towards target NG-RAN includes MBS assistance information.

It is obvious that during N2 based handover procedure MBS assistance information will be delivered from CN to the target NG-RAN node, and this can be supported by the proposal 1-2 above. And during Xn based handover procedure or Retrieve UE context procedure,  if both the source NG-RAN and target NG-RAN support delivery of multicast MBS session data to UEs in RRC_INACTIVE state, the MBS assistance information should be transferred from the source NG-RAN to the target NG-RAN.

Note that currently the MBS Session Information List IE is included in both XnAP:HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message, the straight forward solution is to include the MBS assistance information in the MBS Session Information List IE.

Proposal 2-1: in XnAP, introduce the MBS Assistance Information IE in HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message.

Proposal 2-2: include the MBS Assistance Information IE in the XnAP: MBS Session Information List IE.

On the other hand, in the S2-2303846, the following statements can be found:

S2-2303846:

-	For the Xn based handover procedure, if the target RAN node supports MBS, after the successful handover the SMF triggers the modification of the associated PDU session to provide the MBS assistance information for the MBS session within the MBS session information to target NG-RAN.

-	During the connection resume procedure, if the target RAN node supports MBS, after acknowledging the path switch request, the SMF triggers the modification of the associated PDU session to provide the MBS assistance information for the MBS session within the MBS session information to target NG-RAN.

But actually, as the SMF is aware of the MBS Support Indicator of the NG-RAN node, in case the source NG-RAN node supports the feature, the source NG-RAN node will provide the MBS assistance information to the target NG-RAN node, there is no need for the SMF to unnecessarily trigger the PDU session modification procedure after Xn moblity.

Proposal 2-3: in case the source NG-RAN node supports the feature of multicast reception in RRC_INACTIVE, there is no need for the SMF to provide MBS assistance information to the target NG-RAN node via PDU Session Modifciation procedure, which is redundantly and costly.

## 2.2 PTM Configuration

In the last RAN2 meeting, some agreements for the PTM configuration are listed as follows:

UE shall join in the multicast session before receiving multicast in RRC INACTIVE. If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured. When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE. Multicast MCCH configuration is provided via new SIB. Optionally, Multicast MCCH configuration for the serving cell can also be provided in dedicated signalling. Understanding is we are not optimizing mobility case because of this. Serving cell will not provide the PTM configuration of neighbour cells from other gNBs. FFS whether the network can provide PTM configuration for intra-gNB cells.

New SIB for multicast MCCH configuration

According to the above agreement, RAN2 agreed to introduce a new SIB for multicast MCCH configuration, which can be named as SIBX in this paper. Simillar with the handling of SIB20 for broadcast MCCH configuration, the new SIBX should also be generated by the gNB-DU. Considering the impact of F1AP, the new SIBX could also be provided in the gNB-DU System Information IE, which is contained in F1 SETUP REQUEST message and GNB-DU CONFIGURATION UPDATE message.

Proposal 3-1: similar to the handling of SIB20, the gNB-DU is responsible to encode the new SIBX, and introduce the new SIBX in the F1AP: gNB-DU System Information IE.

For current broadcast PTM configuration in MCCH, the configurations of PDCP and neighbour cell list are provided in MBS CU to DU RRC Information IE, and other lower layer configurations are generated by gNB-DU. Therefore, similar to the MBS CU to DU RRC Information provided in the BROADCAST CONTEXT SETUP REQUEST message, the MULTICAST CONTEXT SETUP/MODIFICATION REQUEST message should also be extended to provide similar information from CU to DU, e.g. a new IE named Multicast CU to DU RRC Information and the details of this IE is subject to  RAN2 further discussion.

Proposal  3-2: to support multicast MCCH, introduce a new F1AP: Multicast CU to DU RRC Information IE in the MULTICAST CONTEXT SETUP/MODIFICATION REQUEST messages transferred from gNB-CU to gNB-DU, and the details of this IE is subject to RAN2 further discussion.

Provide PTM configuration by RRCRelease with suspendconfig

On the other hand, RAN2 agreed that “RRCRelease message with suspendconfig can be used to deliver the PTM configuration”, to support this, the gNB-CU needs to know the latest PTM configurations of all the joint multicast sessions of the UE. Thus, there are two potential solutions:

Solution 1: UE associated F1AP signalling solution

In this solution, before releasing the UE, the gNB-CU asks the gNB-DU to provide the latest PTM configurations all the joint multicast sessions of the UE via the F1AP UE Context Modification procedure, and then the gNB-CU provides these configurations to the UE in RRCRelease message.

Solution 2: MC Session associated F1AP signalling solution

In this solution, the gNB-DU provides the PTM configuration of each MBS session to the gNB-CU in the Multicast Session Setup/Modification procedures, e.g. via introducing a  new IE named as Multicast DU to CU RRC Information. And then the gNB-CU includes the PTM configurations of the MBS sessions joined by the UE in the RRCRelease message.

Considering that the PTM configurations of the same MBS Session for different UEs are the same, asking the gNB-DU to provide lastest PTM configuration before release each UE to RRC_Inactive is not efficient and unnecessary. Therefore it is preferred to adopt solution 2 to provide PTM configuration of each MBS session from DU to CU in the Multicast Session Setup/Modification procedures.

Proposal 3-3: to support providing PTM configuration in RRCRelease, include the PTM configuration in MULTICAST CONTEXT SETUP/ MODIFICATION RESPONSE message.

Based on RAN2 agreements that “Serving cell will not provide the PTM configuration of neighbour cells from other gNBs.”, there is no need to coordinate PTM configurations among gNBs. And in RAN2, it is still FFS whether the network can provide PTM configuration for intra-gNB cells, from RAN3 point of view, based on the proposal 3-2 above, the gNB-DU should be able toprovide PTM configuration of its own cells, we need further discuss if and how to support  intra CU inter DU scenario.

Proposal 3-4: further discuss if and how to provide PTM configurations of neighbour cells in intra CU inter DU scenario.

## 2.3 State transition and mobility

Session activation

For multicast session activation, RAN2#119bis-e has the following agreements.

Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS). As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

Considering that currently the CN triggered group paging for idle UEs will also be received by the inactive UEs, and the Group paging may be used for session activation or session release, and in case of sesson release, the UEs should access to the network, therefore, from RAN3 point of view, if the CN can indicate to the NG-RAN node about whether the CN triggered group paging is for session activation, the NG-RAN node can further indicate to the UE over radio about whether to keep in RRC_INACTIVE or not, via new indication in Uu paging or MCCH which is subject to RAN2 discussion.

Proposal 4-1: the CN needs to indicate session activation in the NGAP: MULTICAST GROUP PAGING message, to enable the gNB to notify UEs about session activation and remaining in RRC Inactive state for multicast reception.

Session deactivation

In the RAN2#119bis meeting, it is agreed that

If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

For group paging scheme, when the NG-RAN node receives NGAP: MULTICAST SESSION DEACTIVATION REQUEST  message, the gNB-CU sends MULTICAST GROUP PAGING message to the gNB-DU, containing a deactivation indicator.

On ther other hand, the MCCH scheme can modify or delete the associated PTM configuration, which can be seen an implicit deactivation indication. Therefore, gNB-CU needs to send MULTICAST CONTEXT MODIFICATION REQUEST message to gNB-DU, to enable gNB-DU to modify/delete the multicast PTM configuration.

From RAN3 view, the detailed impacts of F1AP signalling needs to wait for RAN2 conclusions on session deactivation.

Proposal 4-2: wait for RAN2 further progress on multicast session deactivation.

Mobility

For service continuity within RNA, RAN2 has made the following agreements in RAN2#119bis meeting as follows:

Multicast service continuity after cell reselection in RRC_INACTIVE state (i.e. without resuming RRC connection) will be supported (if the configuration of the new cell is available for the UE). FFS whether there are cases where the UE needs to resume the connection. FFS RAN3 impacts due to inter-gNB mobility. Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

It is assumed that in the case where the target NG-RAN node has established the MBS session, and the multicast service is ongoing and inactive reception is enabled. If the RRC_INACTIVE state UE receiving the multicast service in the source NG-RAN node reselects to such a target NG-RAN node, the UE can acquire the available PTM configuration via MCCH and keep receiving the multicast service in RRC_INACTIVE state.

If the UE is in RRC_CONNECTED state and handover to such a cell which is congested, the handover may be failed (admission control failure due to congestion at target). Then, during the Xn based handover procedure, if the source NG-RAN node receives the HANDOVER PREPARATION FAILURE message with failure cause that network is congested, the UE could be sent to RRC_INACTIVE state, and in case the UE reselect to the target cell, it will be able to continue receiving the multicast service in the target cell in RRC_INACTIVE state. To enable this, there is a need for the target NG-RAN node to indicate whether the joined MBS sessions of the UE are ongoing or not in its cells for RRC_inactive reception, e.g. in HANDOVER PREPARATION FAILURE message.

Proposal 4-3: in case of HO Failure, the target NG-RAN node should indicate to the source NG-RAN node about whether the joined MBS sessions of the UE are ongoing or not in its cells for RRC_inactive reception.

# 3. Proposals

In this contribution, we discuss the support of multicast reception by UEs in RRC_INACTIVE state, and get the following proposals:

About Assistance Information

Proposal 1-1: RAN3 adopt the Interpretation 1 and introduce the MBS Assistance Information IE as ENUMERATED (true, …) to inform the recommendation from core network.

Proposal 1-2: introduce the the MBS Assistance Information IE in NGAP: MBS Session Setup Request List IE and MBS Session Setup or Modify Request List IE.

Proposal 2-1: in XnAP, introduce the MBS Assistance Information IE in HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message.

Proposal 2-2: include the MBS Assistance Information IE in the XnAP: MBS Session Information List IE.

Proposal 2-3: in case the source NG-RAN node supports the feature of multicast reception in RRC_INACTIVE, there is no need for the SMF to provide MBS assistance information to the target NG-RAN node via PDU Session Modifciation procedure, which is redundantly and costly.

About PTM Configuration

Proposal 3-1: similar to the handling of SIB20, the gNB-DU is responsible to encode the new SIBX, and introduce the new SIBX in the F1AP: gNB-DU System Information IE.

Proposal  3-2: to support multicast MCCH, introduce a new F1AP: Multicast CU to DU RRC Information IE in the MULTICAST CONTEXT SETUP/MODIFICATION REQUEST messages transferred from gNB-CU to gNB-DU, and the details of this IE is subject to RAN2 further discussion.

Proposal 3-3: to support providing PTM configuration in RRCRelease, include the PTM configuration in MULTICAST CONTEXT SETUP/ MODIFICATION RESPONSE message.

Proposal 3-4: further discuss if and how to provide PTM configurations of neighbour cells in intra CU inter DU scenario.

About State transition and mobility

Proposal 4-1: the CN needs to indicate session activation in the NGAP: MULTICAST GROUP PAGING message, to enable the gNB to notify UEs about session activation and remaining in RRC Inactive state for multicast reception.

Proposal 4-2: wait for RAN2 further progress on multicast session deactivation.

Proposal 4-3: in case of HO Failure, the target NG-RAN node should indicate to the source NG-RAN node about whether the joined MBS sessions of the UE are ongoing or not in its cells for RRC_inactive reception.

With the proposals above, we provide the corresponding TPs in the following sections.

# Annex A. TP to TS 38.300

-----------------Start of the Changes-------------------

### 16.10.5	Multicast Handling

>>> skip unchanged part

#### 16.10.5.4	Reception of MBS Multicast data

For multicast service, gNB may deliver Multicast MBS data packets using the following methods:

-	PTP Transmission: gNB individually delivers separate copies of MBS data packets to each UEs independently, i.e., gNB uses UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

-	PTM Transmission: gNB delivers a single copy of MBS data packets to a set of UEs, e.g., gNB uses group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI.

If a UE is configured with both PTM and PTP transmissions, a gNB dynamically decides whether to deliver multicast data by PTM leg and/or PTP leg for a given UE based on the protocol stack defined in clause 16.10.3, based on information such as MBS Session QoS requirements, number of joined UEs, UE individual feedback on reception quality, and other criteria. The same QoS requirements apply regardless of the decision. During an active multicast MBS session enabled for reception in RRC_Inactive state, the gNB shall keep the PTM transmission.

>>> skip unchanged part

#### 16.10.5.X Multicast Reception in RRC_INACTIVE mode

The MBS multicast may also be received by the UEs in RRC_INACTIVE state, the gNB makes the decision on whether to move a UE to RRC_INACTIVE mode to receive a multicast service, based on e.g. the QoS parameters of the service, the UE radio capability, cell load, and the MBS assistance information, etc.

During Xn Handover or Retrieve UE Context procedures, the MBS assistance information is provided from the source/old gNB to the target/new gNB. In case the source/old gNB does not support multicast reception in RRC_INACTIVE mode, the core network provides the MBS assistance information to the target gNB afterwards.

During NG Handover, the MBS assistance information is provided from the core network to the target gNB during Handover Resource Allocation procedure.

-----------------End of the Changes-------------------

# Annex B. TP to TS 38.401

-----------------Start of the Changes-------------------

## 7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

### 7.7.1	Support of dynamic PTP and PTM switching

NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

During an active multicast MBS session enabled for reception in RRC_Inactive state, the gNB-DU shall keep the PTM transmission.

-----------------End of the Changes-------------------

# Annex C. TP to TS 38.413 BL CR

-----------------Start of the Changes-------------------

#### 9.3.1.211	MBS Session Setup Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup Request List |  | 1 |  |  | - |  |
| >MBS Session Setup Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS Assistance Information | O |  | 9.3.1.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

#### 9.3.1.212	MBS Session Setup or Modify Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup or Modify Request List |  | 1 |  |  | - |  |
| >MBS Session Setup or Modify Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup or Modify Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup or Modify Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS QoS Flow To Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows to be released. | - |  |
| >>MBS Assistance Information | O |  | 9.3.1.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

-----------------Next of the Changes-------------------

#### 9.3.1.xxx	MBS Assistance Information

This IE provides the MBS Assistance Information as defined in TS 23.247 [44].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Assistance Information | M |  | ENUMERATED (true, …) |  |

-----------------Next of the Changes-------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

// skip unchanged part

id-ManagementBasedMDTPLMNModificationList,

id-MaskedIMEISV,

id-MBS-AreaSessionID,

id-MBS-AssistanceInfo,

id-MBS-ServiceArea,

id-MBS-SessionID,

id-MBS-DistributionReleaseRequestTransfer,

id-MBS-DistributionSetupRequestTransfer,

id-MBS-DistributionSetupResponseTransfer,

-----------------Next of the Changes-------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

// skip unchanged part

id-MaximumIntegrityProtectedDataRate-DL,

id-MBS-AreaSessionID,

id-MBS-AssistanceInfo,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

// skip unchanged part

MBS-AreaSessionID ::= INTEGER (0..65535, ...)

MBS-AssistanceInfo ::= ENUMERATED {

true,

...

}

MBS-DataForwardingResponseMRBList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-DataForwardingResponseMRBItem

MBS-DataForwardingResponseMRBItem ::= SEQUENCE {

mRB-ID 								MRB-ID,

dL-Forwarding-UPTNLInformation		UPTransportLayerInformation,

mRB-ProgressInformation				MRB-ProgressInformation			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { MBS-DataForwardingResponseMRBItem-ExtIEs} }	OPTIONAL,

...

}

// skip unchanged part

MBSSessionSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetupRequestItem

MBSSessionSetupRequestItem ::= SEQUENCE {

mBS-SessionID									MBS-SessionID,

mBS-AreaSessionID								MBS-AreaSessionID																	OPTIONAL,

associatedMBSQosFlowSetupRequestList			AssociatedMBSQosFlowSetupRequestList												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { MBSSessionSetupRequestItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInfo		CRITICALITY ignore			EXTENSION MBS-AssistanceInfo		PRESENCE	optional	},

...

}

MBSSessionSetuporModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetuporModifyRequestItem

MBSSessionSetuporModifyRequestItem ::= SEQUENCE {

mBS-SessionID										MBS-SessionID,

mBS-AreaSessionID									MBS-AreaSessionID									OPTIONAL,

associatedMBSQosFlowSetuporModifyRequestList		AssociatedMBSQosFlowSetuporModifyRequestList		OPTIONAL,

mBS-QosFlowToReleaseList 							QosFlowListWithCause								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{MBSSessionSetuporModifyRequestItem-ExtIEs}}			OPTIONAL,

...

}

MBSSessionSetuporModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInfo		CRITICALITY ignore			EXTENSION MBS-AssistanceInfo		PRESENCE	optional	},

...

}

-----------------Next of the Changes-------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

// skip unchanged part

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-MBS-AssistanceInfo									ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Changes-------------------

# Annex D. TP to TS 38.423 BL CR

-----------------Start of the Changes-------------------

#### 9.2.1.36	MBS Session Information List

This IE contains NG-RAN MBS session resource context related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Information Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >MBS Session ID | M |  | 9.2.3.146 |  | - |  |
| >MBS Area Session ID | O |  | 9.2.3.148 | MBS Area Session ID of the UE at the NG-RAN node from which the UE context is transferred | - |  |
| >Active MBS Session Information | O |  |  |  | - |  |
| >>MBS QoS Flows to Add List |  | 1..<maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.2.3.10 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | - |  |
| >>MBS Service Area | O |  | 9.2.3.150 |  | - |  |
| >>MBS Mapping and Data Forwarding Request Info from source NG-RAN node | O |  | 9.2.1.39 |  | - |  |
| >MBS Assistance Information | O |  | 9.2.3.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions. Value is 256. |
| maxnoofMBSQoSFlows | Maximum no. of QoS flows allowed within one MBS session. Value is 64. |

-----------------Next  of the Change-------------------

#### 9.2.3.xxx	MBS Assistance Information

This IE provides the MBS Assistance Information as defined in TS 23.247 [46].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Assistance Information | M |  | ENUMERATED (true, …) |  |

-----------------Next  of the Change-------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

// skip unchanged part

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-MBS-AssistanceInfo,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

// skip unchanged part

MBS-FrequencySelectionArea-Identity ::= OCTET STRING (SIZE(3))

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-AssistanceInfo ::= ENUMERATED {

true,

...

}

MBS-MappingandDataForwardingRequestInfofromSource ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-MappingandDataForwardingRequestInfofromSource-Item

// skip unchanged part

MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

mBS-Area-Session-ID		MBS-Area-Session-ID														OPTIONAL,

active-MBS-SessioInformation			Active-MBS-SessionInformation							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-SessionInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInfo			CRITICALITY ignore			EXTENSION MBS-AssistanceInfo		PRESENCE	optional	},

...

}

-----------------Next  of the Change------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

// skip unchanged part

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-MBS-AssistanceInfo																				ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Changes-------------------

# Annex E. TP to TS 38.470

-----------------Start of the Changes-------------------

### 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of the NR-MIB message. In case broadcast of SIB1 and other SIBs is needed, the gNB-DU is responsible for the encoding of the SIB1 message, SIB10, SIB12, SIB13, SIB14, SIB15, SIB17, SIB18, SIB20 and SIBX, and the gNB-CU is responsible for the encoding of other SIBs. The gNB-DU may re-encode SIB9. The gNB-DU is responsible for the generation of the SystemInformation message.

NOTE:	The SIB19 is generated by the gNB-DU.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g. the positioning related SIBs. The gNB-CU notifies gNB-DU about the SIBs, and the gNB-DU signals them directly.

To support Msg3 based on-demand SI as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested other SIs.

-----------------End of the Changes-------------------

# Annex F. TP to TS 38.473 BL CR

-----------------Start of the Changes-------------------

#### 9.2.14.2	MULTICAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a multicast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Multicast MRBs To Be Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRBs to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast CU to DU RRC Information | M |  | FFS |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | Reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | FFS |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

-----------------Next of the Changes-------------------

#### 9.3.1.18	gNB-DU System Information

This IE contains the system information generated by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MIB message | M |  | OCTET STRING | MIB message, as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| SIB1 message | M |  | OCTET STRING | SIB1 message, as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| SIB12 message | O |  | OCTET STRING | SIB12, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB13 message | O |  | OCTET STRING | SIB13, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB14 message | O |  | OCTET STRING | SIB14, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB10 message | O |  | OCTET STRING | SIB10, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB17 message | O |  | OCTET STRING | SIB17, as defined in subclause 6.3.1 in TS 38.331 [8] | YES | ignore |
| SIB20 message | O |  | OCTET STRING | SIB20, as defined in subclause 6.3.1 in TS 38.331 [8] | YES | ignore |
| SIB15 message | O |  | OCTET STRING | SIB15, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIBX message | O |  | OCTET STRING | SIBX, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |

-----------------Next of the Change-------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

// skip unchanged part

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastCUtoDURRCInformation,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

// skip unchanged part

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastCUtoDURRCInformation,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

// skip unchanged part

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP REQUEST

--

-- **************************************************************

MulticastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupRequestIEs}},

...

}

MulticastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory  }|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea					CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI								PRESENCE mandatory  }|

{ ID id-MulticastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	MulticastMRBs-ToBeSetup-List		PRESENCE mandatory  }|

{ ID id-MulticastCUtoDURRCInformation	CRITICALITY ignore	TYPE	MulticastCUtoDURRCInformation	PRESENCE optional   },

...

}

MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetup-ItemIEs} }

MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	MulticastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

----------------- Next of the Change -------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

// skip unchanged part

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceF1		MulticastF1UContextReferenceF1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession						MBS-Area-Session-ID				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}} OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastCUtoDURRCInformation		::= SEQUENCE {

FFS

iE-Extensions				ProtocolExtensionContainer { { MulticastCUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

MulticastCUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-ToBeSetup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-DU      UPTransportLayerInformation,

mbsProgressInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-ToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

----------------- Next of the Change -------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

// skip unchanged part

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-MulticastCUtoDURRCInformation					ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Changes-------------------