3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231424

Online, 17th – 26th April, 2023

Agenda item:	 10.2.1

Source:		Lenovo

Title:		SON enhancements for SHR

Document for:		Discussion and Decision

1	Introduction

In RAN3#117bis-e meeting [1], agreements for inter-RAT SHR were achieved as following:

T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE.

RAN3 thinks that at least the following parameters can be useful for optimizing inter-RAT successful handover from NR to LTE. LS RAN2 to confirm and request support. Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision.

- Source NR cell information

- Target LTE cell information

- Measurement results for source, target and neighbours

- Cause to indicate which inter-RAT SHR triggering condition was met

- FFS on Target C-RNTI

- UE location Information

FFS whether the presence of Target C-RNTI IE in inter-RAT SHR is related to the decision on supporting T304 trigger

RAN3 thinks that all CHO related information in intra-NR SHR (e.g., time from CHO configuration to execution) is not applicable for inter-RAT SHR.

RAN3#118 meeting [2] achieved agreements as following:

WA: Support inter-RAT SHR from LTE to NR at least for T304 if no impact on LTE.

WA: In R18, UE context retrieval in source Node is supported for inter-RAT SHR, the details are FFS.

This WA does not limit to the forwarding mechanism.

RAN3#119 meeting [3] achieved further agreements as following::

RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18.

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell .

RAN3 should specify mechanisms on how to correlate NR SHR and LTE RLF Report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE?

Further information from the UE is FFS depending on if and how we support correlation mechanism and UE context retrieval mechanism.

ACCESS AND MOBILITY INDICATION procedure is adopted as SHR forwarding mechanism for inter-RAT SHR?

In this paper, we would further discuss SON enhancements for SHR.

2	Discussion

  - 2.1 intra-NR SHR

In RAN3#119 meeting, it has agreed that for inter-RAT SHR, the forwarding mechanism is that the receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR. But for intra-NR SHR, it is FFS. It is better to have a common solution for both inter-RAT and intra-RAT SHR, i.e. the receiving node forwards the intra-NR SHR to corresponding node which generates the SHR trigger condition that triggers the intra-NR SHR, since SHR forwarding between source node and target node is not essential when SHR is generated due to one configured trigger condition is fulfilled.

Proposal 1: The receiving node forwards the intra-NR SHR to corresponding node which generates the SHR trigger condition that triggers the intra-NR SHR.

In R17, target C-RNTI is introduced in intra-NR SHR to correlate NR SHR and NR RLF Report in case that there is a RLF shortly after a successful intra-NR HO. When T304 triggers intra-SHR, if we agree that the receiving node forwards the intra-NR SHR to corresponding node which generates the SHR trigger condition that triggers the intra-NR SHR, i.e. the receiving node forwards the intra-NR SHR to the target gNB, based on the target C-RNTI which is included in both intra-NR SHR and NR RLF Report, the target gNB can performs correlation without any spec impact.

Figure 1: correlate in target node when T304 triggers intra-NR SHR

Observation: When T304 triggers intra-SHR, the target gNB can correlate the intra-NR SHR and NR RLF Report based on target C-RNTI.

However, when T310/T312 triggers intra-SHR, if we agree that the receiving node forwards the intra-NR SHR to corresponding node which generates the SHR trigger condition that triggers the intra-NR SHR, i.e. the receiving node forwards the intra-NR SHR to the source gNB, how network performs correlation needs further discussion.

As Figure 2 shows, one option is that source gNB performs correlation, instead of target C-RNIT, source C-RNTI is needed in both intra-NR SHR and NR RLF report.

Figure 2: correlate in source node when T310/T312 triggers intra-NR SHR

Another solution is that target gNB performs correlation, as Figure 3 shows, source gNB needs to forward the received intra-NR SHR to target gNB, based on the target C-RNTI, target gNB can identify the intra-NR SHR and NR RLF Report are generated in a same HO procedure for a same UE. But the question is whether and when the source gNB forwards the recevied intra-NR SHR to target gNB.

When T310/T312 triggers intra-NR SHR, correlation in source node seems straightforward and simple from network point of view.

Figure 3: correlate in target node when T310/T312 triggers intra-NR SHR

Proposal 2: When T310/T312 triggers intra-NR SHR, source gNB correlates intra-NR SHR and NR RLF Report in case that there is a RLF shortly after a successful intra-NR HO, source C-RNTI is needed in both intra-NR SHR and NR RLF report.

  - 2.2 inter-RAT SHR from NR to LTE

For inter-RAT SHR from NR to LTE, only T310 and T312 related triggers are to be considered. Based on RAN3#119 meeting’s agreements, the receiving node forwards the inter-RAT SHR to source NR node when T310/T312 triggers the inter-RAT SHR. For the case that there is a RLF occurring shortly after a successful inter-RAT HO from NR to LTE, network correlation is also needed similar as in intra-NR HO procedure.

If source NR node performs correlation for inter-RAT SHR and LTE RLF Report, as Figure 4 shows, source C-RNTI is needed in both inter-RAT SHR and LTE RLF report.

Figure 4: correlate in source node when T310/T312 triggers inter-RAT SHR from NR to LTE

If target LTE node performs correlation, target C-RNTI is needed in inter-RAT SHR. As Figure 5 shows, source NR node needs to forward explicit information included in the inter-RAT SHR to the target LTE node. Based on the target C-RNTI, target LTE node can identify the inter-RAT SHR and RLF Report are generated in a same HO procedure for a same UE. But the question is whether and when the source NR node explicitly forwards the information included in inter-RAT SHR to target LTE node.

When T310/T312 triggers inter-RAT SHR from NR to LTE, correlation in source node seems straightforward and simple from network point of view.

Figure 5: correlate in target node when T310/T312 triggers inter-RAT SHR from NR to LTE

Proposal 3: When T310/T312 triggers inter-RAT SHR from NR to LTE, source gNB correlates inter-RAT SHR and LTE RLF Report in case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE, source C-RNTI is needed in both inter-RAT SHR and LTE RLF report.

  - 2.3 inter-RAT SHR from LTE to NR

For inter-RAT SHR from LTE to NR, only T304 related trigger is agreed to be considered. Based on RAN3#119 meeting’s agreements, the receiving node forwards the inter-RAT SHR to target NR node when T304 triggers the inter-RAT SHR. For the case that there is a RLF occurring shortly after a successful inter-RAT HO from LTE to NR, network correlation is also needed. The simple way is target C-RNTI is included in the inter-RAT SHR, based on the target C-RNTI which is included in both inter-RAT SHR and RLF Report, the target NR node can correlate the two reports.

Figure 6: correlate in target node when T304 triggers inter-RAT SHR from LTE to NR

Proposal 4: When T304 triggers inter-RAT SHR from LTE to NR, target gNB correlates inter-RAT SHR and NR RLF Report in case that there is a RLF shortly after a successful inter-RAT HO from LTE to NR, target C-RNTI is needed in inter-RAT SHR.

3	Conclusion

This paper mainly discussed SON enhancements for SHR, the observation and proposals were achieved as following:

Observation: When T304 triggers intra-SHR, the target gNB can correlate the intra-NR SHR and NR RLF Report based on target C-RNTI.

Proposal 1: The receiving node forwards the intra-NR SHR to corresponding node which generates the SHR trigger condition that triggers the intra-NR SHR.

Proposal 2: When T310/T312 triggers intra-NR SHR, source gNB correlates intra-NR SHR and NR RLF Report in case that there is a RLF shortly after a successful intra-NR HO, source C-RNTI is needed in both intra-NR SHR and NR RLF report.

Proposal 3: When T310/T312 triggers inter-RAT SHR from NR to LTE, source gNB correlates inter-RAT SHR and LTE RLF Report in case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE, source C-RNTI is needed in both inter-RAT SHR and LTE RLF report.

Proposal 4: When T304 triggers inter-RAT SHR from LTE to NR, target gNB correlates inter-RAT SHR and NR RLF Report in case that there is a RLF shortly after a successful inter-RAT HO from LTE to NR, target C-RNTI is needed in inter-RAT SHR.

Reference

[1] Chairman notes for RAN3#117bis-e meeting

[2] Chairman notes for RAN3#118 meeting

[3] Chairman notes for RAN3#119 meeting