3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231479

Online, April. 17th – Apr. 26th, 2023

Agenda Item:	13.2

Source: 	Canon Research Centre France

Title:	Discussion on MT and DU Migrations of Mobile IAB-node

Document for:	Discussion

# Introduction

In this paper, we discuss the DU and MT migrations of a mobile IAB-node based on the following agreements from the previous RAN3 meetings [1], [2], [3] and [4]:

At RAN3#117: - RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT executes inter-donor migration. - The mobile IAB-node may perform multiple consecutive partial migrations without inter-donor migration of its mobile IAB-DU. At RAN3#117bis: - mIAB-DU migration and mIAB-MT handover can be executed independently from each other. Details on the scenarios need to be further discussed - For partial migration of mIAB-node, the inter-donor HO of mIAB-MT is decided and triggered by the donor CU serving the mIAB-MT. - The donor CU serving the mIAB-DU decides whether to execute mIAB-DU migration or inter-donor F1 transport migration for the mIAB-DU. - For inter-donor partial migration, the donor CU serving the mIAB-DU is informed about the mIAB-MT HO. FFS on signalling details concerning the indication. - For partial migration of mIAB-node, the donor CU serving the mIAB-DU and the target donor CU for the mIAB-MT HO can directly exchange Xn IAB Transport Migration messages, in case direct Xn connectivity exists (or is established) between the two donor CUs. At RAN3#118: - WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. This WA is subject to validation that the impact involved is affordable. - The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO. - The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. - The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. At RAN3#119: - In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU.

# Discussion

## Concurrent MT and DU migrations

In a previous contribution [5], we mentioned that in case of concurrent MT handover and DU migration for a mobile IAB-node, it is the DU migration that may suffer from delay or failure due to the modification of the backhaul path to communicate with the mobile IAB-node.

To be more precise on a potential issue, it is considered the case where the mobile IAB-node has successfully performed the F1 setup procedure with a target F1 donor-CU for DU migration purpose, and that this target F1 donor-CU is different from the non-F1 donor-CU for the mobile IAB-node. If meanwhile, a MT migration of the mobile IAB-node has occurred toward a target non-F1 donor-CU (still different from the target F1 donor-CU), the target F1 donor-CU may not be aware of this new non-F1 donor-CU to perform the transport migration management (TMM) procedure.

From [4], it can be noted that the following agreement was taken:

In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU,

However, this agreement may not be accurate enough to prevent any deadlock situation in case of concurrent MT and DU migrations. Indeed, it does not say when the target F1 donor-CU (i.e. the target logical mIAB-DU’s CU) is informed about the non-F1 donor-CU (i.e the mIAB-MT’s CU ID). If the target F1 donor-CU was informed before the MT migration, then the target F1 donor-CU will trigger the TMM procedures toward a donor-CU that may no longer be the non-F1 donor-CU.

Therefore, the above agreement could be completed by the following:

Proposal 1: if the mIAB-MT’s CU is changed during a DU migration, the target logical mIAB-DU’s CU needs to be informed about the target mIAB-MT’s CU ID and the mIAB-MT ID, before it can initiate the TMM procedures towards the target mIAB-MT’s CU.

From the above example, it appears that some adaptations of MT and DU migration procedures may be required to avoid deadlock situations and to limit the impact of concurrent MT and DU migrations to some delay to complete the procedures.

To go forward, there are 3 options:

- option 1: to consider independent procedures as a working assumption, to define the procedures accordingly, and then in a second step to refine (if necessary) the procedures to support concurrent DU and MT migrations,

- option 2: to consider concurrent DU and MT migrations as a working assumption, then to define the procedures accordingly,

- option 3: to not support concurrent DU and MT migrations, to define the procedures independently, and in a second step to add the necessary mechanisms to avoid this concurrence.

We have observed that a mobile IAB-MT migration is driven by the radio conditions, and it should not be delayed to maintain backhaul connectivity and to avoid service interruption at the served UEs. It means that a mobile IAB-MT handover may be triggered and executed while a migration of the co-located mobile IAB-DU is on-going.

Besides, option 2 seems more efficient than option 1 when considering the support of concurrent DU and MT migrations.

Therefore, if concurrent DU and MT migrations have to be supported, then the procedures for MT and DU migrations are defined taking into account the possible concurrence of DU and MT migrations (option 2).

Proposal 2: the procedures for MT and DU migrations are defined taking into account the possible concurrence of DU and MT migrations.

## Providing identification information to a target F1 donor-CU

One question to address is which node informs the target F1 donor-CU for the DU migration of a mobile IAB-node about the identification information related to the non-F1 donor-CU (i.e. the mIAB-MT’s CU ID and the mIAB-MT ID).

The 3 following options may be considered for the providing node:

- Option 1: the source F1 donor-CU. In this case, the source F1 donor-CU may relay to the target F1 donor-CU the identification information previously received from the source non-F1 donor-CU at the MT migration of the mobile IAB-node.

- Option 2: the non-F1 donor-CU. In this case the source F1 donor-CU should have provided to the non-F1 donor-CU the identification information related to the target F1 donor-CU.

- Option 3: the migrating IAB-node. In this case, the non-F1 donor-CU should have provided the identification information to the IAB-node.

Proposal 3: The 3 following options may be considered to provide the identification information related to the non-F1 donor-CU (i.e. the mIAB-MT’s CU ID and the mIAB-MT ID) to the target F1 donor-CU for the DU migration of a mobile IAB-node:

- Option 1: the source F1 donor-CU,

- Option 2: the source non-F1 donor-CU,

- Option 3: the migrating IAB-node.

RAN3 to discuss the above options.

Option 2 may be a suitable option as it is in line with the following agreement:

The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the:

•	gNB ID of the target donor CU for the mIAB-MT HO.

•	ID(s) of the mIAB-MT.

# Conclusion

In this contribution, we have discussed a potential issue with concurrent DU and MT migrations and some options to specify the MT and DU migrations for mobile IAB. Besides, we have listed the possible options to inform the target F1 donor-CU about the mIAB-MT’s CU ID and the mIAB-MT ID.

Proposal 1: if the mIAB-MT’s CU is changed during a DU migration, the target logical mIAB-DU’s CU needs to be informed about the target mIAB-MT’s CU ID and the mIAB-MT ID, before it can initiate the TMM procedures towards the target mIAB-MT’s CU.

Proposal 2: the procedures for MT and DU migrations are defined taking into account the possible concurrence of DU and MT migrations.

Proposal 3: The 3 following options may be considered to provide the identification information related to the non-F1 donor-CU (i.e. the mIAB-MT’s CU ID and the mIAB-MT ID) to the target F1 donor-CU for the DU migration of a mobile IAB-node:

- Option 1: the source F1 donor-CU,

- Option 2: the source non-F1 donor-CU,

- Option 3: the migrating IAB-node.

RAN3 to discuss the above options.

# References

[1] Chairman notes of RAN3#117e meeting

[2] Chairman notes of RAN3#117-bis-e meeting

[3] Chairman notes of RAN3#118 meeting

[4] Chairman notes of RAN3#119 meeting

[5] R3-230136